3GPP TSG-RAN WG4 Meeting 109	R4-2402872

Athens, Greece, Feb 26th  – 1st March, 2024

Title: 	WF on [109][330] Netw_Energy_NR_demod

Agenda Item:	8.26.5

Source: 	Huawei, HiSilicon

Document for:	Approval

1 PDSCH requirements

Issue 1-1: Whether to introduce SSB less Scell requirements for inter-band CA

- Agreement

  - RAN4 to evaluate the performance of SSB-less SCells CA performance, if the observed performance constitutes a practical operating point of the feature, or if SSB-less SCell compliant UE shall be tested with normal CA requirements.

Issue 1-2: Simulation assumptions

- Agreement

  - Use following simulation assumptions:

Table 1: Proposed PDSCH SSB-less SCell FR1 inter-band CA test setup, based on legacy CA demodulation testing

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | {TDLA30-10} |
| Antenna configuration |  |  | 2x2, ULA Low |
| Fraction of maximum throughput (%) |  |  | 70 (Only SCell measured) |
| SCS/CBW |  |  | 30kHz/40MHz (106 PRB) |
| MCS |  |  | {16QAM, 0.48 (MCS13)} |
| Channel estimation |  |  | DM-RS based: non-ideal/practical |
| TO Estimation & Compensation |  |  | As per implementation |
| TO |  |  | {-0.9, -0.4, 0, 0.4, 0.9} CP |
| FFT window shift target |  |  | {As per implementation} |
| SSB/TRS |  |  | PCell: SSB and TRS (optional) SCell: TRS only |
| Precoder |  |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Fill remaining DL symbols in the slot. 12 in normal DL slot 4 in special slot |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports |  | {1000} |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmissions |  |  | 4 |
| TDD UL-DL pattern |  |  | [30kHz SCS: FR1.30-1] |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |

- The RTD can be described as following figure:

2 CSI requirements

Issue 2-1: Whether to introduce CSI requirements for power/spatial domain adaption

- Agreement

  - Not to introduce CSI requirements power/spatial domain adaption