3GPP TSG-RAN WG4 Meeting # 110	R4-2402912

# Athens, Greece,  Febuary 26th - March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_RF_FR2_req_Ph3-Perf) Draft CR for 38.141-2 on FR2-1 PUSCH 256QAM demodulation requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_Ph3-Perf |  |  |  |  |  | Date: |  |  | 2024-01-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The FR2-1 PUSCH 256QAM demodulation requirements were discussed in previous RAN4 meeting. The simulation assumptions and test cases are agreed. So it is reasonable to add corresponding requirements. The draft CR R4-2321190 has been endorsed in RAN4#109. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following changes are added: - Adding new rows for FR2-1 PUSCH 256QAM demodulation requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FR2-1 PUSCH 256QAM demodualtion requirements are not available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

########################## Start of change #1 ############################

##### 8.2.1.5.2	Test requirement for BS type 2-O

The throughput measured according to clause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5.2-1 to 8.2.1.5.2-10.

Table 8.2.1.5.2-1: Test requirements for PUSCH with 70% of maximum throughput, 50 MHz Channel Bandwidth, 60 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | -1.4 |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | -1.6 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-1 | pos0 | Yes | 12.6 |
|  |  |  |  |  |  |  | No | 12.1 |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | 11.3 |
|  |  |  |  |  |  |  | No | 11.3 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-1 | pos0 | Yes | 14.3 |
|  |  |  |  |  |  |  | No | 13.7 |
|  |  |  |  |  | G-FR2-A5-6 | pos1 | Yes | 14.0 |
|  |  |  |  |  |  |  | No | 13.5 |
|  |  | Normal | TDLD30-35 Low | 70 % | TBD | pos0 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
|  |  |  |  |  | TBD | pos1 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-6 | pos0 | No | 2.3 |
|  |  |  |  |  | G-FR2-A3-18 | pos1 | No | 2.0 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A7-1 | pos0 | Yes | 16.0 |
|  |  |  |  |  |  |  | No | 15.1 |
|  |  |  |  |  | G-FR2-A7-6 | pos1 | Yes | 14.6 |
|  |  |  |  |  |  |  | No | 13.8 |

Table 8.2.1.5.2-2: Test requirements for PUSCH with 70% of maximum throughput, 100 MHz Channel Bandwidth, 60 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | -1.5 |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | -1.8 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-2 | pos0 | Yes | 12.8 |
|  |  |  |  |  |  |  | No | 11.8 |
|  |  |  |  |  | G-FR2-A4-12 | pos1 | Yes | 11.8 |
|  |  |  |  |  |  |  | No | 11.2 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-2 | pos0 | Yes | 14.8 |
|  |  |  |  |  |  |  | No | 13.9 |
|  |  |  |  |  | G-FR2-A5-7 | pos1 | Yes | 14.3 |
|  |  |  |  |  |  |  | No | 13.7 |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-7 | pos0 | No | 2.3 |
|  |  |  |  |  | G-FR2-A3-19 | pos1 | No | 2.0 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A7-2 | pos0 | Yes | 16.8 |
|  |  |  |  |  |  |  | No | 15.7 |
|  |  |  |  |  | G-FR2-A7-7 | pos1 | Yes | 14.6 |
|  |  |  |  |  |  |  | No | 13.9 |

Table 8.2.1.5.2-3: Test requirements for PUSCH with 70% of maximum throughput, 50 MHz Channel Bandwidth, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-3 | pos0 | No | -1.2 |
|  |  |  |  |  | G-FR2-A3-15 | pos1 | No | -1.5 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-3 | pos0 | Yes | 12.2 |
|  |  |  |  |  |  |  | No | 11.5 |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | 11.5 |
|  |  |  |  |  |  |  | No | 11.1 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-3 | pos0 | Yes | 14.3 |
|  |  |  |  |  |  |  | No | 13.7 |
|  |  |  |  |  | G-FR2-A5-8 | pos1 | Yes | 13.8 |
|  |  |  |  |  |  |  | No | 13.6 |
|  |  | Normal | TDLD30-35 Low | 70 % | TBD | pos0 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
|  |  |  |  |  | TBD | pos1 | Yes | TBD |
|  |  |  |  |  |  |  | No | TBD |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-8 | pos0 | No | 2.2 |
|  |  |  |  |  | G-FR2-A3-20 | pos1 | No | 2.1 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A7-3 | pos0 | Yes | 15.0 |
|  |  |  |  |  |  |  | No | 14.4 |
|  |  |  |  |  | G-FR2-A7-8 | Pos1 | Yes | 14.7 |
|  |  |  |  |  |  |  | No | 13.9 |

Table 8.2.1.5.2-4: Test requirements for PUSCH with 70% of maximum throughput, 100 MHz Channel Bandwidth, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-4 | pos0 | No | -1.8 |
|  |  |  |  |  | G-FR2-A3-16 | pos1 | No | -1.9 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-4 | pos0 | Yes | 12.5 |
|  |  |  |  |  |  |  | No | 11.1 |
|  |  |  |  |  | G-FR2-A4-14 | pos1 | Yes | 11.7 |
|  |  |  |  |  |  |  | No | 11.1 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-4 | pos0 | Yes | 14.1 |
|  |  |  |  |  |  |  | No | 13.5 |
|  |  |  |  |  | G-FR2-A5-9 | pos1 | Yes | 14.0 |
|  |  |  |  |  |  |  | No | 13.4 |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-9 | pos0 | No | 2.2 |
|  |  |  |  |  | G-FR2-A3-21 | pos1 | No | 2.0 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A7-4 | pos0 | Yes | 14.7 |
|  |  |  |  |  |  |  | No | 14.0 |
|  |  |  |  |  | G-FR2-A7-9 | pos1 | Yes | 14.3 |
|  |  |  |  |  |  |  | No | 13.7 |

Table 8.2.1.5.2-5: Test requirements for PUSCH with 70% of maximum throughput, 200 MHz Channel Bandwidth, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-5 | pos0 | No | -1.5 |
|  |  |  |  |  | G-FR2-A3-17 | pos1 | No | -1.8 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A4-5 | pos0 | Yes | 11.9 |
|  |  |  |  |  |  |  | No | 11.5 |
|  |  |  |  |  | G-FR2-A4-15 | pos1 | Yes | 11.8 |
|  |  |  |  |  |  |  | No | 11.3 |
|  |  | Normal | TDLA30-75 Low | 70 % | G-FR2-A5-5 | pos0 | Yes | 14.7 |
|  |  |  |  |  |  |  | No | 14.0 |
|  |  |  |  |  | G-FR2-A5-10 | pos1 | Yes | 14.3 |
|  |  |  |  |  |  |  | No | 13.9 |
| 2 |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-10 | pos0 | No | 2.2 |
|  |  |  |  |  | G-FR2-A3-22 | pos1 | No | 1.9 |
|  |  | Normal | TDLA30-300 Low | 70 % | G-FR2-A7-5 | pos0 | Yes | 14.8 |
|  |  |  |  |  |  |  | No | 14.1 |
|  |  |  |  |  | G-FR2-A7-10 | pos1 | Yes | 14.4 |
|  |  |  |  |  |  |  | No | 13.8 |

Table 8.2.1.5.2-6: Test requirements for PUSCH with 30% of maximum throughput, 50 MHz channel bandwidth, 60 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 30 % | G-FR2-A4-1 | pos0 | Yes | 4.6 |
|  |  |  |  |  |  |  | No | 4.1 |
|  |  |  |  |  | G-FR2-A4-11 | pos1 | Yes | 4.3 |
|  |  |  |  |  |  |  | No | 3.7 |

Table 8.2.1.5.2-7: Test requirements for PUSCH with 30% of maximum throughput, 50 MHz channel bandwidth, 120 kHz SCS in FR2-1

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 30 % | G-FR2-A4-3 | pos0 | Yes | 4.6 |
|  |  |  |  |  |  |  | No | 4.2 |
|  |  |  |  |  | G-FR2-A4-13 | pos1 | Yes | 4.3 |
|  |  |  |  |  |  |  | No | 3.8 |

Table 8.2.1.5.2-8: Test requirements for PUSCH with 70% of maximum throughput, 100 MHz Channel Bandwidth, 120 kHz SCS in FR2-2

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-650 | 70 % | G-FR2-A3B-1 | pos1 | No | 0.4 |
|  |  | Normal | TDLA30-650 | 70 % | G-FR2-A4-21 | pos1 | Yes | 11.8 |
|  |  | Normal | TDLD30-200 | 70 % | G-FR2-A5-11 | pos1 | Yes | 13.1 |
| 2 |  | Normal | TDLA30-650 | 70 % | G-FR2-A3B-6 | pos1 | No | 4.5 |
|  |  | Normal | TDLA30-650 | 70 % | G-FR2-A7-11 | pos1 | Yes | 14.2 |

Table 8.2.1.5.2-9: Test requirements for PUSCH with 70% of maximum throughput, 400 MHz Channel Bandwidth, 120 kHz SCS in FR2-2

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA10-650 | 70 % | G-FR2-A3B-2 | pos1 | No | 0.4 |
|  |  | Normal | TDLA10-650 | 70 % | G-FR2-A4-22 | pos1 | Yes | 11.8 |
|  |  | Normal | TDLD10-200 | 70 % | G-FR2-A5-12 | pos1 | Yes | 13.2 |
| 2 |  | Normal | TDLA10-650 | 70 % | G-FR2-A3B-7 | pos1 | No | 4.6 |
|  |  | Normal | TDLA10-650 | 70 % | G-FR2-A7-12 | pos1 | Yes | 14.6 |

Table 8.2.1.5.2-10: Test requirements for PUSCH with 70% of maximum throughput, 400 MHz Channel Bandwidth, 480 kHz SCS in FR2-2

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (annex J) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA10-650 | 70 % | G-FR2-A3B-3 | pos1 | No | 0.1 |
|  |  | Normal | TDLA10-650 | 70 % | G-FR2-A4-23 | pos1 | Yes | 11.4 |
|  |  | Normal | TDLD10-200 | 70 % | G-FR2-A5-13 | pos1 | Yes | 13.4 |
| 2 |  | Normal | TDLA10-650 | 70 % | G-FR2-A3B-8 | pos1 | No | 4.0 |
|  |  | Normal | TDLA10-650 | 70 % | G-FR2-A7-13 | pos1 | Yes | 13.8 |

NOTE:	If the above Test Requirement differs from the Minimum Requirement then the Test Tolerance applied for this test is non-zero. The Test Tolerance for this test and the explanation of how the Minimum Requirement has been relaxed by the Test Tolerance is given in annex C.

### 8.2.2	Performance requirements for PUSCH with transform precoding enabled

########################## End of change #1 ############################