3GPP TSG-RAN4 Meeting # 110	R4-2402416

# Athens, Greece,  26th February  - 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.115-1: Addition of test procedures for NCR demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds test procedures and test requirements for NCR demodulation according to the agreed work-split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test requirements and procedures added for PDSCH and PDCCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test procedures or requirements for conducted NCR demodulation testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 7, 7.1, sub-sections |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.106 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7 	Conducted performance requirements

## 7.1	NCR-MT Performance requirements

### 7.1.1	General

#### 7.1.1.1	Scope and definitions

Conducted performance requirements specify the ability of the NCR-MT to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the antenna connectors (NCR-MT type 1-C) or TAB connector(s) (NCR type 1-H).

Conducted performance requirements for the NCR-MT are specified for the fixed reference channels and the propagation conditions defined in TS 38.106 [2] annex F and annex G, respectively.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in a slot on a single antenna connector or TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

### 7.1.2	Demodulation performance requirements

#### 7.1.2.1	General

#### 7.1.2.2	Performance requirements for PDSCH

##### 7.1.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by two means; a minimum required throughput for a given SNR and a minimum SNR at which 1% BLER for the first transmission is achieved. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex F. The throughput performance requirements assume HARQ re-transmissions, whereas the first transmission BLER requirements do not consider HARQ.

##### 7.1.2.2.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.1.

##### 7.1.2.2.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput  and 1st transmission BLER under multipath fading propagation conditions for a given SNR.

##### 7.1.2.2.4	Method of test

7.1.2.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

7.1.2.2.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors  for diversity reception via a combining network as shown in annex D.9 or D.11.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex F and the test parameters in table 7.1.2.2.4.2-1.

Table 7.1.2.2.4.2-1: Test parameters for testing PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active BWP index |  |  | 1 |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| PDSCH DMRS configuration | Antenna ports indexes |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PT-RS configuration |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: Point A coincides with minimum guard band as specified in TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing. |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 81.2.2.5.1-1 is achieved at the IAB-MT input.

6)	For each of the reference channels in table 7.1.2.2.5.1-1 measure the throughput and the 1st transmission BLER.

##### 7.1.2.2.5	Test requirement

The throughput measured according to clause 7.1.2.2.4.2 shall not be below the limits for the SNR levels specified in table 7.1.2.2.5-1.

Table 7.1.2.2.5-1: Minimum performance PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test metric | SNR (dB) |
| 1 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 70% TPUT | -0.1 |
| 2 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 1% BLER | 5.7 |
| 3 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 70% TPUT | -3.1 |
| 4 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 1% BLER | 0.9 |
| 5 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 70% TPUT | 7.6 |
| 6 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 1% BLER | 13.7 |
| 7 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 70% TPUT | 4.3 |
| 8 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 1% BLER | 8.5 |
| 9 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 70% TPUT | -0.2 |
| 10 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 1% BLER | 5.1 |
| 11 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 70% TPUT | -3.1 |
| 12 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 1% BLER | 0.3 |
| 13 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 70% TPUT | 7.6 |
| 14 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 1% BLER | 12.7 |
| 15 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 70% TPUT | 4.4 |
| 16 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 1% BLER | 7.8 |

#### 7.1.2.3	Performance requirements for PDCCH

##### 7.1.2.3.1	Definition and applicability

The performance requirement of PDCCH is determined by a maximum allowed missed detection rate for a given SNR. The required missed detection rate is expressed for the FRCs listed in annex F.

##### 7.1.2.3.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.2.

##### 7.1.2.3.3	Test purpose

The test shall verify the receiver's ability to achieve missed detection rate under multipath fading propagation conditions for a given SNR.

##### 7.1.2.3.4	Method of test

7.1.2.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

7.1.2.3.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors for diversity reception via a combining network as shown in annex D.9 or D.11.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 7.1.2.3.4.2-1.

Table 7.1.2.3.4.2-1: Test parameters for testing PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 7.1.2.3.5.1-1is achieved at the NCR-MT input.

6)	For each of the reference channels in table 7.1.2.3.5.1-1, applicable for the NCR-MT, measure the missed detection.

##### 7.1.2.3.5	Test requirement

For the parameters specified in Table 7.1.2.3.4.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 7.1.2.3.5-1 for SCS supported by the NCR-MT.

Table 871.2.3.5-1: Minimum performance for PDCCH

| Test number | Bandwidth (MHz) |  | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x2 | 1 | 8.7 |
| 2 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x4 | 1 | 2.8 |
| 3 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x2 | 1 | 6.1 |
| 4 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x4 | 1 | 0.8 |
| 5 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 | 0.6 |
| 6 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 | -1.7 |
| 7 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x2 | 1 | 7.6 |
| 8 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x4 | 1 | 2.7 |
| 9 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x2 | 1 | 5.5 |
| 10 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x4 | 1 | 1.3 |
| 11 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 | 0.1 |
| 12 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 | -3.3 |

## D.9	NCR-MT type 1-C PDCCH and PDSCH demodulation testing

NCR-MT under test

Channel

Simulator

Channel

Simulator

Channel

Simulator

Channel

Simulator

NCR-MT tester

Tx 1

Tx 2

feedback

AWGN GeneratorAWGN Generator

AWGN GeneratorAWGN Generator

Synchronization source (if used see NOTE 2)

RX A

RX B

Figure D.9-1: Functional set-up for performance requirements for PDSCH and PDCCH transmission on two antenna ports in multipath fading conditions (2 Rx case shown)

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR-MT node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR-MT node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

## D.10 	NCR-MT type 1-C CQI testing

NCR-MT under test

AWGN Generator

AWGN GeneratorAWGN Generator

NCR tester

Synchronization source (if used see NOTE 1)

RX A

RX B

Figure D.10-1: Functional set-up for performance requirements for CQI in static conditions for NCR-MT with Rx diversity (2 Rx case shown)

NOTE 1:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 2:	It is left up to implementation how L1/L2 is configured for testing.

## D.11	NCR-MT type 1-H PDCCH and PDSCH demodulation testing

transceiver unit array

#1

#2

#K

transceiver array boundary

Transceiver array boundary connector TAB(n)

Load

Channel

Simulator

Channel

Simulator

Channel

Simulator

Channel

Simulator

NCR-MT tester

Tx 1

Tx 2

feedback

AWGN GeneratorAWGN Generator

AWGN GeneratorAWGN Generator

Synchronization source (if used see NOTE 2)

Figure D.11-1: Functional set-up for performance requirements for PDSCH and PDCCH transmission on two antenna ports in multipath fading conditions (2 Rx case shown)

NOTE 1:	The feedback could be done as an RF feedback, either using NR channels or using other means, or as a digital feedback. The HARQ Feedback should be error free.

NOTE 2:	In tests performed with signal generators, a synchronization signal may be provided between the NCR-MT node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR-MT node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 3:	It is left up to implementation how L1/L2 is configured for testing.

## D.12 	NCR-MT type 1-H CQI testing

transceiver unit array

#1

#2

#K

transceiver array boundary

Transceiver array boundary connector TAB(n)

Load

AWGN Generator

AWGN GeneratorAWGN Generator

NCR tester

Synchronization source (if used see NOTE 1)

Figure D.12-1: Functional set-up for performance requirements for CQI in static conditions for NCR-MT with Rx diversity (2 Rx case shown)

NOTE 1:	In tests performed with signal generators, a synchronization signal may be provided between the NCR node and the signal generator, or a common (e.g., GNSS) source may be provided to both NCR node and the signal generator, to enable correct timing of the wanted signal. The method of synchronization with the TE is left to test implementation.

NOTE 2:	It is left up to implementation how L1/L2 is configured for testing.