3GPP TSG-RAN4 Meeting #110		R4-2403283

Athens, Greece, Feb 26 – Mar 01, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on correction for Rel-18 RSTD and PRS-RSRP requirements for RedCap in RRC connected state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction on the UE capabilities signal for RedCap positioning Missing requirements for RedCap measurement delay with FH |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include - Replace [TBD] with the correct UE capability in 9.9A.2.1 and 9.9A.3.1 - Complete the missing requirements for RSTD and PRS-RSRP measurements for RedCap with FH in 9.9A.2.6.1 and 9.9A.3.6.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RSTD and PRS-RSRP measurement requirements for RedCap will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9A.2 and 9.9A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2402903 |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE 1----------------------------

### 9.9A.2	RSTD measurements for RedCap

#### 9.9A.2.1	Introduction

The requirements in clause 9.9A.2 shall apply provided the RedCap UE has received NR-DL-TDOA-RequestLocationInformation message from LMF via LPP [34] requesting the RedCap UE to measure and report DL RSTD measurements defined in TS 38.215 [4].

The requirements in clause 9.9A.2.5 shall apply provided the RedCap UE does not support, or support but not configured, to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via [NR-DL-AoD-ProvideCapabilities].

The requirements in clause 9.9A.2.6 shall apply provided the RedCap UE supports and is configured to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via [NR-DL-AoD-ProvideCapabilities].

#### 9.9A.2.6	Measurements Period Requirements with FH

##### 9.9A.2.6.1	Measurements Period Requirements with FH with MG

When physical layer receives last of NR-TDOA-ProvideAssistanceData message and NR-TDOA-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure DL RSTD measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCap UE capability specified in Clause 9.9A.2.3) DL RSTD measurements, defined in TS 38.215 [4], during the measurement period TRSTD,Total defined in Clause 9.9A.2.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSTD measurement samples, where

-	Nsample= 2 if the RedCap UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= Nhop,i*Lper-hop,i

where,

-	Nhop,i is the number of hops that UE can do in an MG occasion as defined in the following, and

-	Lper-hop,i is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as indicated in UE capability [TBD].

Table 9.9A.4.8-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| [RRTFH≤2 symbols] | [(≤ 4, 12)] | [2] | [6] |
|  | [All others] | [1] | [14] |
| [2 symbols <RRTFH≤6 symbols] | [(≤ 6, any)] | [1] | [14] |
|  | [(12, 12)] | [½] | [28] |
| [RRTFH>6 symbols] | [Any combination] | [½] | [28] |

The number of hops within a single MG occasion Nhop is defined as

Nhop=minNhops,effect,Nhop,max

where

-	Nhop,max is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1)

-	Nhops,effect is the effective number of Rx hops within a MG instance,

-	Nhops,effect=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.4.8-1.

### 9.9A.3	PRS-RSRP measurements for RedCap

#### 9.9A.3.1	Introduction

The requirements in clause 9.9A.3 shall apply provided the RedCap UE has received a message from LMF via LPP [34] requesting the RedCap UE to measure and report PRS-RSRP measurements defined in TS 38.215 [4].

The requirements in clause 9.9A.3.5 shall apply provided the RedCap UE does not support, or support but not configured, to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via [NR-DL-AoD-ProvideCapabilities].

The requirements in clause 9.9A.3.6 shall apply provided the RedCap UE supports and is configured to measure PRS resources with Frequency Hopping (FH), subject to UE capability indicated via [NR-DL-AoD-ProvideCapabilities].

#### 9.9A.3.6	Measurements Period Requirements with FH

##### 9.9A.3.6.1	Measurements Period Requirements with FH with MG

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], requesting RedCap UE to measure PRS-RSRP measurement with FH, the RedCap UE shall be able to measure multiple (up to the RedCapUE capability specified in Clause 9.9A.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured positioning frequency layers, within TPRS-RSRP,total ms as defined Clause 9.9A.3.5.1 with using the following definition for Nsample and Lavailable_PRS,i:

Nsample is the number of PRS RSRP measurement samples, where

-	Nsample= 2 if the UE supports supportedDL-PRS-ProcessingSamples-RRC-CONNECTED [34], and the LMF requests the UE to perform positioning measurements with reduced number of samples.

-	Nsample= 4 otherwise.

Measurement sample under FH is defined as a PRS measurement over multiple hops within a single measurement gap occasion.

Lavailable_PRS,i is the time duration of available PRS in the positioning frequency layer i to be measured during a MG, and is calculated by:

Lavailable_PRS,i= Nhop,i*Lper-hop,i

where,

-	Nhop,i is the number of hops that UE can do in an MG occasion as defined in the following, and

-	Lper-hop,i is the time duration of available PRS resources in the positioning frequency layer i in each hop to be measured during Tavailable_PRS,i, and is calculated in the same way as PRS duration K defined in clause 5.1.6.5 of TS 38.214 [26]. For calculation of Lper-hop,i, only the PRS resources unmuted and fully or partially overlapped with the sampling duration in each hop are considered;

The sampling duration per hop is the first Tsample,hop symbols in each hop, where Tsample,hop= Thop-RRTFH, Thop is the applicable length per hop as defined in Table 9.9A.4.8-1, and RRTFH is the retuning time between Rx hops as indicated in UE capability [TBD].

Table 9.9A.4.8-1: Applicable number of hops per slot and applicable length of each hop

| RRTFH | (comb size, Number of PRS symbols) | Applicable number of hops per slot Nhopsslot | Applicable length per hop (Tper-hop) in number of symbols |
| --- | --- | --- | --- |
| [RRTFH≤2 symbols] | [(≤ 4, 12)] | [2] | [6] |
|  | [All others] | [1] | [14] |
| [2 symbols <RRTFH≤6 symbols] | [(≤ 6, any)] | [1] | [14] |
|  | [(12, 12)] | [½] | [28] |
| [RRTFH>6 symbols] | [Any combination] | [½] | [28] |

The number of hops within a single MG occasion Nhop is defined as

Nhop=minNhops,effect,Nhop,max

where

-	Nhop,max is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1)

-	Nhops,effect is the effective number of Rx hops within a MG instance,

-	Nhops,effect=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.4.8-1.

----------------------END OF CHANGES 1----------------------------