3GPP TSG-RAN WG4 Meeting # 110	R4-2403347

Athens, Greece, Feb. 26 – March 1, 2024

Title: 	WF on RRM requirements for NR_SL_enh2_part1

Agenda Item:	8.22.5

Source: 	LG Electronics

Document for:	Approval

- Maintenance for SL-U

Issue 1-1: Condition for timing error requirement

<Agreement>

- For UE transmit timing Te requirements, remove the redundant condition that SyncRef UE is available at UE with at least one S-SSB position available during the last 1280ms.

Issue 1-2: Condition for initiation/cease of SLSS transmission in SL-U

< Agreement>

- For initiation/cease of SLSS transmission, remove the scenario when UE is in coverage with a serving cell on a NR sidelink unlicensed carrier

Issue 1-3: Requirements for fast sync SyncRef UE detection

< Agreement>

- Revise the description as agreed below

  - When GNSS is not available and only if UE additionally drops a maximum of 30% of its SLSS transmission, the UE shall be able to identify newly detectable intra-frequency SyncRef UE within T’detect,SyncRef UE_V2X_CCA seconds when all S-SSB periods selected for SyncRefUE identification are available during the T’detect,SyncRef UE_V2X_CCA seconds. T’detect,SyncRef UE_V2X_CCA is defined as 1.6 seconds at S-SSB Ês/Iot ≥ 0 dB.