3GPP TSG RAN WG4 Meeting #110	R4-24xxxxx

Athens, Greece, 26 February – 01 March, 2024

Agenda Item:	8.14.4

Source: 	Ericsson

Title:	WF on RedCap positioning and bandwidth aggregation for positioning measurements

Document for:	Approval

# Introduction

This WF captures the aggrements reached on the issues listed in [110][217] NR_pos_enh2_part1 thread.

# Topic #2: RedCap positioning core requirements

UE feature list update

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know … | Applicable to … | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_CONNECTED | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 41-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-1A | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 41-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2 | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_CONNECTED | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2A | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/falsel # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |

Agreement

- Agree on proposed changes in R4-2401199.

## Sub-topic 2-1: Core requirements for Rx FH.

Issue 2-1-1: Number of hops within a single MG occasion

Agreements

- The requirements shall support the following Rx frequency hopping cases:

  - 1 hop per slot

  - 2 hops per slot

  - 1 hop every 2 slots

- The number of Rx hops measured by the UE in a MG instance is given by

  - Nhops=minNhops,effect, Nhops, max

where

    - Nhops, max is the maximum number of Rx hops signaled in the UE capability (FG 41-5-1).

    - Nhops, effect is the effective number of Rx hops within a MG instance.

Issue 2-1-3: How to use Rel. 17 core requirement as baseline

Agreements

- The time duration of available PRS is derived by:

  - Lavailable_PRS,i=Nhop*Lper_hop

where,

    - Nhop is the number of hops in a single MG occasion.

    - Lper_hop is the time duration of available PRS per hop (i.e., excluding RF retuning time).

Issue 2-1-4: Minimum PRS bandwidth for Rx FH

Agreement

    - The minimum PRS BW expected to be measured with Rx hopping is given by

minBWPRS,Nhops∙BWhop-BWoverlap∙Nhops-1

where

      - BWPRS is determined by the min. among 1) the configured PRS BW, 2) UE capability (Component 1 of FG 41-5-1), and 3) total BW of all hops requested by LMF.

      - BWhop is the BW per hop signaled in the UE capability.

      - BWoverlap is the minimum hop overlap signaled in the UE capability.

      - Nhops is the number of Rx hops measured by the UE within a MG instance.

# Topic#3: PRS/SRS bandwidth aggregation core requirements

## Sub-topic 3-1: PRS aggregation for positioning measurements.

Issue 3-1-3: Nominal channel spacing for PRS aggregation

Agreements

- A note on nominal channel spacing can be added in the accuracy requirements to clarify the applicability of the requirements.

  - Simulations assumptions in R4-2321461 are already covering the nominal channel spacing.

- FFS: whether any further clarification is needed in the core requirements.

Issue 3-1-4: Impact of PRS collision with other signals on PRS bandwidth aggregation requirement

Agreement

- Do not define UE behavior for the case of PRS collision with other signals on PRS bandwidth aggregation. For the case of PRS collision with other signals a longer measurement period can be expected.

## Sub-topic 3-3: SRS aggregation for positioning measurements.

Issue 3-3-2: Interruption due to guard period for SRS aggregation

Agreements

- If no RAN1 solution is defined to handle the impact of SRS transmission for BW aggregation on other channels/signals the RAN4 will define interruption requirements for SRS transmission for BW aggregation on CC without PUSCH/PUCCH

- Send LS to RAN1 to explain the technical issue and check if there will be a RAN1 solution to handle it.

- LS to RAN1 in R4-24xxxxx is agreed.

# Topic #4: Work plan for performance requirements.

Issue 3-1: Rapporteur input to work plan for completion of performance requirement.

Conclusion: the work plan in R4-2402685 can be approved.

# Topic #5: RedCap positioning performance requirements

Issue 5-1-1: Side conditions and channel models for RSTD and UE Rx-Tx measurements for 1Rx UE without Rx FH

Agreements

- The brackets in the side conditions for 1Rx without FH under fading propagation condition can be removed, i.e., the side conditions should be:

  - RSTD accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values of -6 dB for the reference cell and -10 dB for the target cell and is based on 4 samples.

  - UE Rx-Tx accuracy requirement for 1Rx RedCap UE under fading propagation condition is defined for SINR values of -3 dB and -10 dB and is based on 4 samples.

Issue 5-1-2: Side conditions and channel models for RSRP measurements for 1Rx UE without Rx FH

Agreements

- PRS-RSRP accuracy requirements for 1 Rx RedCap UE without Rx FH:

  - Side conditions: -3 dB, -10 dB.

  - 4 samples.

  - Generic requirements, not limited to specific propagation conditions.

Issue 5-1-3: Side conditions and channel models for RSRPP measurements for 1Rx UE without Rx FH

Agreements

- PRS-RSRPP accuracy requirements for 1 Rx RedCap UE without Rx FH:

  - Side conditions: -3 dB, -10 dB,

  - 2-tap channel model from Rel-17 with Nsamples = 4.

Issue 5-1-4: Accuracy requirements for Rx FH and without Rx FH

Agreements

- Accuracy requirements will be defined also for the case with FH.

  - The BW set will be extended for the FH case in the accuracy requirements.

  - FFS: whether the accuracy requirements are the same or different for the cases with and without FH.

Issue 5-2-1: Channel model for Rx FH in FR2

Agreements

- Accuracy requirement for RedCap positioning with FH in FR2 is defined for TDL-C instead of TDL-A (60 ns delay spread, 300 Hz).

  - Update simulation assumptions from R4-2314460 accordingly (Ericsson will request a tdoc number and updated the sim. assumptions).

  - Updated simulation assumptions are captured in R4-24xxxxx.

# Topic #6: PRS/SRS bandwidth aggregation performance requirements

Issue 6-2-3: Bandwidth configuration to define accuracy requirements for positioning measurements based on bandwidth aggregation

Agreements

- For PRS BW aggregation for RSTD and UE Rx-Tx, the requirements are to be defined at least for the following per-PFL BWs:

  - 15 kHz SCS: 104 PRBs (20 MHz)

  - 30 kHz SCS: 132 PRBs (50 MHz), 272 (100 MHz)

  - 60 kHz SCS FR1: 64 PRBs (50 MHz), 132 (100 MHz)

  - 60 kHz SCS FR2: 64 PRBs (50 MHz), 132 (100 MHz)

  - 120 kHz SCS: 64 PRBs (100 MHz), 128 (200 MHz)

- Note: The aggregated PFLs will have the same SCS.

- FFS: Other BWs.

Issue 6-2-4: Separate accuracy requirement for positioning measurements based on bandwidth aggregation depending on the number of PFLs

Agreements

- Accuracy requirements shall cover 2 and 3 PFLs.

  - FFS: For the same BW, the same accuracy requirements (table) apply for 2 PFLs and 3 PFLs

  - Accuracy requirements can be different for different BWs.

Issue 6-2-5: Measurements for which accuracy requirements are defined

Agreements

- For PRS/SRS BW aggregation, RAN4 will define accuracy requirements for:

  - PRS-based RSTD and UE Rx-Tx,

  - PRS-RSRP and PRS-RSRPP (existing requirements shall apply for both).

# References

- R4-2400753, Topic summary for [110][217] NR_pos_enh2_part1, Ericsson

- R4-2403259,	Ad-hoc minutes #1 for NR positioning, Intel.

- R4-2403260,	Ad-hoc minutes #2 for NR positioning, Ericsson.

- R4-2403384,	Ad-hoc minutes #3 for NR positioning, Ericsson.