3GPP TSG-RAN WG4 Meeting #110															R4-2403542

Athens, Greece, Feb. 26th – Mar. 1st, 2024

Agenda item:			8.5.5

Source:	Moderator (MediaTek inc.)

Title:	WF on NR_MG_enh2_part1

Document for:	Information

# Introduction

In this document,

- < Agreement > represents the decisions made in this meeting

- < Way forward > represents the next step in later meetings (open issue for discussion)

# Topic #1: CR handling (AI 8.5.1)

### Sub-topic 1-1: Draft CRs handling

| T-doc number | Company | WF |
| --- | --- | --- |
| R4-2402882 | MediaTek inc. | Revised to R4-2403358 (from R4-2402882) |
| R4-2400375 | Apple | The CR can be merged with other CRs. |
| R4-2401028 | CMCC | Revised to R4-2403359 (from R4-2401028) |
| R4-2401200 | xiaomi | The CR can be merged with other CRs. |
| R4-2401864 | Nokia, Nokia Shanghai Bell | Revised to R4-2403360 (from R4-2401864) |
| R4-2402164 | Huawei, HiSilicon | The CR can be merged with other CRs. |
| R4-2400377 | Apple | Revised to R4-2403361 (from R4-2400377) |
| R4-2400999 | OPPO | Revised to R4-2403362 (from R4-2400999) |
| R4-2401866 | Nokia, Nokia Shanghai Bell | The CR can be merged with other CRs. |

# Topic #2: Case 1 requirements (Pre-configured MG and concurrent MG) (AI 8.5.1.1)

### Sub-topic 2-1: Collision handling for dynamic collisions

Sub-topic description: This sub-topic covers issues related to the collision cases for concurrent gaps with Pre-MG. The summary of the issues on this topic are provided below:

Scenario 1: the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

Scenario 2: pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

Scenario 3: pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG.

Scenario 4: One pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision (This scenario is for Pre-MG + Pre-MG).

Figure: the collision scenarios for concurrent gaps with Pre-MG during dynamic collision.

Issue 2-1-1: [Case 1] - [Scenario 1] Further clarification on the agreement from scenario 1?

- Background:

  - Agreements from dynamic collision:

    - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG and UE should continue the measurement within the MG#2

      - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used

      - The collision scenario in this issue is depicted in the figure below:

< Way forward >

  - Option 1:

    - The time point when Pre-MG activation/deactivation take effects shall be updated as: ‘Activation of Pre-MG takes effect from the first complete MG occasion after the activation and deactivation delay if the time difference between the last overlapping MG and first Pre-MG occasion to be changed is larger than 5ms. Otherwise, the first Pre-MG occasion shall be kept as deactivated also’

  - Option 2:

    - For Scenario 1, subclause 8.19.5.3 should state that the pre-configured gap activation shall be applied 5ms after the ending point of the overlapping measurement gap.

Issue 2-1-2: [Case 1] - [Scenario 2] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

- Background:

  - Agreement from the previous meeting:

    - [Case 1] - [Scenario 2] When the pre-configured MG deactivation procedure is overlapped with one of concurrent gap occasion during the dynamic collision (i.e. Pre-MG has higher priority than the MG)

      - When a pre-MG deactivation and a Type-2 MG collide, and the pre-MG has higher priority, UE should drop the colliding Type-2 MG occasion

  - The collision scenario in this issue is depicted in the figure below:

< Way forward >

  - Option 1:

    - For Scenario 2, subclause 8.19.5.3 should state that the Pre-MG deactivation delay requirements are the same as for single Pre-MG in 8.19.2.1 or 8.19.3 or 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation and RRC reconfiguration, respectively, i.e. no overlap of deactivation procedure and gap occasion.

Issue 2-1-3: [Case 1] - [Scenario 3] When the pre-configured MG activation procedure is overlapped with one of concurrent gap occasion where the MG has higher priority than the Pre-MG

- Background:

  - Agreement from online session [R4-2317305]

    - The UE continues the measurement within the overlapped concurrent gap occasion (MG#2), i.e. existing priority rule applies without any change.

  - The collision scenario in this issue is depicted in the figure below:

< Way forward >

  - Option 1:

    - Requirements in subclauses 9.1.12.4 and 8.19.5.3 should be same as for Scenario 1.

Issue 2-1-4: [Case 1] - [Scenario 4] When one pre-configured MG deactivation procedure is overlapped with another pre-configured MG activation procedure during the dynamic collision

Moderator’s note: this issue is a mix between an existing issue of fully overlapping activation/deactivation Pre-MG with collision a Pre-MG gap in the concurrent gap with Pre-MG.

- Background:

  - Agreements from fully overlap with activation/deactivation [R4-2310175]:

      - For Case 1 (Pre-configured MG and multiple concurrent MGs), under the assumption that the baseline requirement considers collisions on Pre-MG is only considered when Pre-MG is activated, extend the delay by T1 ms for fully overlapped simultaneous activation/deactivation for Pre-MG + Pre-MG

      - T1 = 2ms.

      - FFS if this activation delay collide with existing gaps

      - An illustration example is captured below [R4-2306330]:

  - Agreements from dynamic collision:

    - A collision between a change in the status of a pre-configured MG (MG#1) and a gap instance happens when the change occurs ≤ 4 ms before the start or ≤ 4 ms after the end of a gap instance of an activated concurrent MG (MG#2) the Pre-MG status and dropping rule shall be applied 5ms after the overlapping MG [and UE should continue the measurement within the MG#2]

      - TBD whether same Pre-MG activation delay requirements as Rel-17 can still be re-used

      - The collision scenario in this issue is depicted in the figure below:

< Way forward >

  - if fully overlapped simultaneous activation/deactivation for Pre-MG + Pre-MG collides with activated Pre-MG:

    - Option 1:

      - For Scenario 4, directly apply the agreements decided for the fully simultaneous multiple Pre-MG activation/deactivation is fine, no need to touch the agreements for dynamic collision of Scenario 1/2/3, i.e.

        - The new status of two Pre-MG are applied after the extended T1;

        - The dropping rule is only applicable for the activated status of Pre-MG.

    - Option 2:

      - RAN4 not to define UE behaviour and requirements for Scenario 4.

Issue 2-1-5: [Case 1] - [Scenario 3A – New issue] When the Pre-MG deactivation procedure collides with concurrent gap occasion where Pre-MG has lower priority than concurrent MG

< Way forward >

  - Option 1:

    - Requirements in subclauses 9.1.12.4 and 8.19.5.3 should be same as for Scenario 1.

Issue 2-1-6: [Case 1] - [Scenario 4A – New issue] When the Pre-MG 1 activation procedure collides with Pre-MG 2 deactivation procedure where Pre-MG1 has higher priority than Pre-MG2

< Way forward >

  - Option 1:

    - For Scenarios 4A and 4B, subclause 9.1.12.3 should refer to subclause 8.19.5.2 in case of full overlap of Pre-MG activation and other Pre-MG deactivation, assuming that no requirements will be defined for partial overlap.

Issue 2-1-7: [Case 1] - [Scenario 4A – New issue] When the Pre-MG 1 activation procedure collides with Pre-MG 2 deactivation procedure where Pre-MG1 has lower priority than Pre-MG2

< Way forward >

- Option 1:

  - For Scenarios 4A and 4B, subclause 9.1.12.3 should refer to subclause 8.19.5.2 in case of full overlap of Pre-MG activation and other Pre-MG deactivation, assuming that no requirements will be defined for partial overlap.

### Sub-topic 2-2: Feature list for NR UE – Pre-MG

Issue 2-2-1: [Case 1] R18 NR UE features (Concurrent gap with Pre-MG in a FR)

Issue 2-2-2: [Case 1] R18 NR UE features (2 Pre-MG configuration with simultaneous activation / deactivation):

Issue 2-2-3: [Case 1] R18 NR UE features (Dynamic collision):

<Way Forward>

Session Chair: 
Define in total 3 capabilities for MG enh partl (including both case 1 and case 2), with the condition below. FFS Details.
For 32-5, the capability can be further discussed based on R17 progress. Before any agreement reached for Rel-17, 32-5 is not considered.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gaps with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Details in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-2] | Two Pre-MG configuration with simultaneous activation/deactivation | Support configurations of two Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | UE activation/deactivation time for simultaneous Pre-MG is undefined | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-3] | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | Yes | No | UE is not expected to meet RRM requirements for dynamic collisions | Per UE | No | No | N.A |  | Optional with capability signalling |

### Sub-topic 2-3: Others

Issue 2-3-1: [Case 1] Pre-MG association clarification

< Way forward >

  - Option 1:

    - When NW configures a Pre-MG1 and a Pre-MG2/Type-2 MG in ConMGs, the MO associated with Pre-MG1 will be measured within activated Pre-MG2/Type-2 MG if Pre-MG1 is deactivated and the MO is fully overlapping with activated Pre-MG2/Type-2 MG.

      - Option 1a:

        - FFS: whether it need to be captured in spec

  - Option 2:

    - No need to introduce implicit association for concurrent gaps with Pre-MG.

    - Option 2a:

      - The case that Pre-MG1 is deactivated whereas the associated MO still has to be performed with MG is corner case.

Issue 2-3-2: [Case 1] Pre-MG association collisions related issue

< Way forward >

  - Option 1:

    - The measurement dropping rules when the concurrent measurement gaps are collided can be optimized, e.g.

      - Option 1). Only there is overlapping among the [SSBs+Xms] to be measured by these collided concurrent gaps, UE needs to drop the measurement with the lower priority gap. Otherwise, UE can perform these measurements sequentially because UE can return to each of carriers one by one.

  - Option 2:

    - There is no need to continue discussion on optimizations of the collision handling for concurrent gaps in Rel-18.

# Topic #3: Case 2 requirements (NCSG and concurrent MG) (AI 8.5.1.2)

### Sub-topic 3-1: Rel-17 UE behavior for deactivated SCell measurements with NCSG

< Way forward >

As per Chairman guidance, this issue will be discussed in thread Rel-17 maintenance.

### Sub-topic 3-2: Rel-18 UE behavior for deactivated SCell measurements with NCSG

- Agreement from previous meetings:

< Agreement >: - New in Rel-18 - When Type-2 MG and NCSG are both configured, some serving cell MOs may associated to the NCSG and some are not. - Question 1: What is the expected UE behaviour (assume SMTC partially overlapped with NCSG) - Option 1: skip gap association, all deactivated Scells are measured within NCSG. (This implies some new rule to override the existing gap association rule) - Option 2: Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule) - Deactivated Scell MO associated with NCSG is measured within NCSG - Deactivated Scell MO not associated with NCSG is measured outside NCSG - Question 2: Whether additional UE capability indication is needed

Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18)

< Way forward >

  - Option 1:

    - Still follow the gap association, i.e., (This implies we follow Rel-17 gap association rule)

      - Deactivated Scell MO associated with NCSG is measured within NCSG

      - Deactivated Scell MO not associated with NCSG is measured outside NCSG

  - Option 1a:

    - Based on the principle of reusing the gap association rule to determine in which MG the deactivated SCell MO would be performed, when the deactivated SCell switches to be activated, still reuse the R17 conditions to decide whether this SCell can be measured with the NCSG. That is, keep alignment with the understanding of R17 UE behaviours

  - Option 2:

    - When the SCell is deactivated, the deactivated SCell’s MO will be measured within NCSG if the SMTC is partially or fully overlapped with NCSG regardless of gap association.

  - Option 3:

    - The Rel-18 UE behavior (assume SMTC partially overlapped with NCSG) can follow the gap association, i.e., deactivated SCell MO associated with NCSG is measured within NCSG and the MO not associated with NCSG is measured outside NCSG.

  - Option 4:

    - A new indication shall be introduced enable support of NCSG for deactivated SCell only.

  - Option 5:

    - No need to distinguish the UE indication for activated status and deactivated status separately for a single serving cell.

  - Option 6:

    - RAN4 can defer the issue below to Rel19.

  - Option 7:

    - The collision case between measured deactivated SCell MO’s outside NCSG, if allowed in Rel-17, and concurrent MG occasions needs to be considered for Case 2 requirements in Rel-18.

### Sub-topic 3-3: UE capabilities for Concurrent gaps with NCSG and NCSG + NCSG

Issue 3-3-1: [Case 2] R18 NR UE features (Concurrent gap with NCSG):

Issue 3-3-2: [Case 2] R18 NR UE features (2 NCSG configuration):

<Way Forward>

Session Chair: 
Define in total 3 capabilities for MG enh partl (including both case 1 and case 2), with the condition below. FFS Details.
For 32-5, the capability can be further discussed based on R17 progress. Before any agreement reached for Rel-17, 32-5 is not considered.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-4 | Concurrent gaps with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Details in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | UE behaviour is undefined if the network configures concurrent MGs where at least one of the gaps is a NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-5] | Two NCSG configuration in a FR | Support configurations of two NCSG in the same FR | 32-4 | Yes | No | UE behaviour is undefined if the network configures two NCSGs in the same FR | Per UE | No | No | N.A |  | Optional with capability signalling |

# Topic #4: Performance Part 1 (Pre-MG/NCSG and concurrent MG) (AI 8.5.3)

### Sub-topic 4-1: Performance principles for Case 1 and Case 2

Issue 4-1-1: Which general configuration shall be defined for the test cases?

< Agreement from online session >

  - support the following principles for case 1 and case 2:

    - Only define test case in NR SA in both FR1 and FR2:

    - Do not introduce the test for L1 impact:

    - Define a minimum set of test cases for SSB-based measurement:

    - Only define test case under non-DRX:

    - Define test case without SBI reporting:

    - On SSB-only test cases, RAN4 does not consider simultaneous per-UE gap and per-FR gap configurations:

    - Do not define test cases with simultaneously FR1 and FR2 gaps configured:

    - Test cases are limited to single serving carrier:

    - Only use mandatory gap patterns to define test cases:

    - Verify gap dropping behaviour without introducing additional test cases:

    - For case 1 and case 2, Define test for partial overlap, and fully non-overlap is used if no core requirements for partial overlap in some cases.

### Sub-topic 4-2: Test cases for Case 1

Issue 4-2-1: [Case 1] Which scenarios shall RAN4 RRM define test cases for concurrent MG with pre-MG

Agreement:

  - The following scenarios be considered in the test cases:

    - Pre-MG + Type-2 MG

    - Pre-MG + Pre-MG

      - Under the following conditions:

        - Simultaneous multiple activation, FFS non-simultaneous multiple activation

        - Without dynamic collision

    - Dynamic collision with Pre-MG + Type-2 MG

    - FFS other scenarios, options for further discussion:

      - Pre-MG + Type-1 MG

Issue 4-2-2: [Case 1] Testing procedure

< Way forward >

It is not clear whether this is needed or not.

Issue 4-2-3: [Case 1] Test cases list for Case 1 (Work split Case 1)

< Tentative agreement >

Define the following test cases:

| No | Test case category | Test purpose | Volunteering company |
| --- | --- | --- | --- |
| Con-Pre-MG TC1 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap and autonomous activation/deactivation of Pre-MG + Type-2 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of Cell2 within the required period - Pre-MG activation/deactivation delay - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period - Verify the gap collision rule | MediaTek |
| Con-Pre-MG TC2 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and network-controlled activation/deactivation of Pre-MG + Type-2 | - When pre-MG being deactivated at the beginning of testing, UE can report the results of Cell2 within the required period - Pre-MG activation/deactivation delay - After pre-MG being activated by network-control, UE can report the results of Cell2 and Cell3 within the required period - Verify the gap collision rule | Huawei |
| Con-Pre-MG TC3 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap with Pre-MG and autonomous activation/deactivation of two Pre-MG for FR2 | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE autonomously, UE can report the results of Cell2 and Cell3 within the required period. | Xiaomi |
| Con-Pre-MG TC4 | Event triggered reporting test on intra-frequency in FR1 with concurrent gap with Pre-MG and network-controlled activation/deactivation of two Pre-MG for FR1 | - Verify that the UE correctly activates and deactivates the pre-MG and makes correct measurement and reporting of an event with activated and deactivated pre-MG - Multiple Pre-MG activation/deactivation delay - Verify that the UE makes correct reporting of an event: - After pre-MG being activated by UE network-controlled, UE can report the results of Cell2 and Cell3 within the required period. | CMCC |

< Way Forward >

FFS the following test cases:

| No | Test case category | Test purpose | Volunteering company |
| --- | --- | --- | --- |
| Con-Pre-MG TC5 | Inter-frequency measurement with autonomous activation/deactivation of Pre-MG in FR1 with dynamic collision | - Verify the gap association; · Verify the dynamic gap collision when Pre-MG activation • the UE shall NOT report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Type2 MG occasions. • the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasion overlapped in dynamic collision |  |
| Con-Pre-MG TC6 | Inter-frequency measurement with network-controlled activation/deactivation of Pre-MG in FR2 with dynamic collision | · Verify the gap association; · Verify the dynamic gap collision when Pre-MG activation • the UE shall NOT report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Type2 MG occasions. • the UE shall be able to receive PDSCH and report corresponding valid ACK/NACK for those PDSCHs scheduled in the slots overlapped with the Pre-MG occasion overlapped in dynamic collision |  |

### Sub-topic 4-3: Test cases for Case 2

Issue 4-3-1: [Case 2] Which scenarios shall RAN4 RRM define test cases for NCSG and concurrent MG (Case 2)

<Tentative agreement>

The following scenarios be considered in the test cases:

    - NCSG + Type-2 MG

    - NCSG + NCSG

      - FFS other scenarios, options for further discussion:

        - NCSG + Type-1 MG

Issue 4-3-2: [Case 2] Test cases list for Case 2 (Work split Case 2)

< Tentative agreement >

Define the following test cases:

| No | Test case category | Test purpose | Volunteering companies |
| --- | --- | --- | --- |
| Con-NCSG TC1 | Event triggered reporting test on inter-frequency in FR1 with concurrent gap and NCSG | - Inter-frequency cell search/measurement delay is met for Cell2 in NCSG and Inter-frequency cell search/measurement delay for Cell3 in NCSG, and - UE receives data in Cell1 meeting scheduling restriction requirements, and - Verify the gap collision rule | Ericsson |
| Con-NCSG TC2 | Event triggered reporting test on intra-frequency in FR2 with concurrent gap and NCSG | · Same as Con-NCSG TC 1, yet for intra-f in FR2. | CATT |

< Way Forward >

FFS the following test cases:

| No | Test case category | Test purpose | Volunteering companies |
| --- | --- | --- | --- |
| Con-NCSG TC3 | Event triggered reporting test on inter-frequency in FR2 with concurrent gap and NCSG of two NCSGs | · Inter-frequency cell search delay is met for Cell2 in NCSG1 and Inter-frequency cell search/measurement delay for Cell3 in NCSG2, and · UE receives data in Cell1 meeting scheduling restriction requirement - Verify the gap collision rule |  |
| Con-NCSG TC4 | Event triggered reporting test on deactivated SCell in FR1 with concurrent gap and NCSG | · Intra-frequency cell search/measurement delay for deactivated SCC is met for Cell2 in NCSG, and Inter-frequency cell search/measurement delay for Cell3 in MG · UE receives data in Cell1 meeting scheduling restriction requirements, and UE will not cause any interruption on Cell1 outside VIL windows. |  |

------- End of Document -------