3GPP TSG-RAN WG4 Meeting #110	R4-2403631

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-2 on FR2 sTxMP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2024-1-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The beam peak is not clear for sTxMP since multiple beam exists. Cosidering the test method for sTxMP is not decided yet, it is better to refine the misleading wording. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the following wording: “The requirement is verified in beam peak direction.” To: “The requirement is verified in beam peak direction for each of the indicated joint/UL TCI states scheduled for simultaneous transmission. ” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current wording will cause misleading in requirement verification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2K.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R4-2401515 |  |  |  |  |  |  |  |  |

***************************** START OF CHANGES ************************************

### 6.2K.4	Configured transmitted power for simultaneous transmission to multiple directions

A UE configured for simultaneous transmission to multiple directions can configure two maximum output powers. The configured UE maximum output power PCMAX,f,c,k for each of joint/UL TCI states k (k=0,1) indicated for simultaneous transmission to multiple directions of carrier f and serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [11].

The configured UE maximum output power PCMAX,f,c,k shall be set such that the corresponding measured peak EIRP PUMAX,f,c,k is within the following bounds

PPowerclass + ΔPIBE – MAX(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP + ΔMBP,n, P-MPRf,c,k) – MAX{T(MAX(MPRf,c,k, A- MPRf,c,k,) + ΔMPRSTxMP), T(P-MPRf,c,k)} – ΔTSTxMP  ≤ PUMAX,f,c,k ≤ EIRPmax

and PUMAX,f,c, the corresponding measured peak EIRP for carrier f of a serving cell c, aggregated over all indicated joint/UL TCI states in a given direction, satisfies over all directions

PUMAX,f,c ≤ EIRPmax

while the corresponding measured total radiated power PTMAX,f,c is bounded by

PTMAX,f,c ≤ TRPmax

with PPowerclass the UE minimum peak EIRP as specified in sub-clause 6.2K.1, EIRPmax the applicable maximum EIRP as specified in sub-clause 6.2.1, MPRf,c,k as specified in sub-clause 6.2K.2 , A-MPRf,c,k as specified in sub-clause 6.2K.3, ΔMBP,n the peak EIRP relaxation as specified in clause 6.2.1 and TRPmax the maximum TRP for the UE power class as specified in sub-clause 6.2.1. ΔPIBE is 1.0 dB if UE declares support for mpr-PowerBoost-FR2-r16, UL transmission is QPSK, MPRf,c = 0 and when NS_200 applies and the network configures the UE to operate with mpr-PowerBoost-FR2-r16 otherwise ΔPIBE is 0.0 dB. ΔMPRSTxMP is [3.0] dB if two TCI states are indicated for simultaneous transmission to multiple directions, 0.0 dB otherwise. ΔTSTxMP is specified in sub-clause 6.2K.1.The requirement is verified in beam peak direction for each of the indicated joint/UL TCI states scheduled for simultaneous transmission.

P-MPRf,c,k is the power management maximum output power reduction P-MPRf,c for each of indicated joint/UL TCI states. P-MPRf,c is defined in clause 6.2.4.

The tolerance T(ΔP) for applicable values of ΔP (values in dB) is specified in Tables 6.2.4-1 and 6.2.4-2.

********************************** END OF CHANGE ************************************