3GPP TSG-RAN WG4 Meeting # 110							R4-2403635

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	10

Source: 	CMCC

Title: 	Rel-18 RAN4 UE feature list for NR (version 3)

Document for:	Approval

- Introduction

This contribution includes the RAN4 UE feature list for Rel-18 NR. The previous Rel-18 RAN4 UE feature list is R4-2321797 and R4-2321993.

- NR_ENDC_RF_FR1_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_ENDC_RF_FR1_enh2 | 27-1 | TxDiversity for 4Tx | Indicates UE supports Tx diversity for 4Tx for the band configured. |  | Yes | N/A | UE doesn’t support 4Tx Tx diversity for the band configured | Per FS | No | FR1 only | N/A |  | Optional with capability signalling |
| 27. NR_ENDC_RF_FR1_enh2 | 27-2 | LowerMSD for inter-band NR CA and EN-DC | Indicates whether the UE supports lower maximum sensitivity degradation when the band is the victim band with sensitivity degradation as specified in 38.101-1 and 38.101-3. The victim band and associated aggressor band(s) are within at least one of inter-band CA or EN-DC band combinations supported by the UE. This feature includes following components: 1) The aggressor band which causes sensitivity degradation to the victim band. 2) The additional aggressor band only when the sensitivity degradation to the victim band is caused by IMD of another two bands. 3) MSD type, including harmonic, harmonic mixing, cross band isolation, IMD2, IMD3, IMD4, IMD5 and ‘all’. Value ‘all’ indicates the MSD capability class is applicable for all MSD types defined in this release, which are applicable to the associated victim band/aggressor band(s). 4) The applicable power class for the lower MSD capability. 5) The lower MSD capability class as specified in 7.3A.7 in 38.101-1 and in 7.3B.2.3.7 in 38.101-3. | No | Yes | N/A | The UE shall comply with the minimum requirements for MSD. | Per band | No | FR1 only | Support mixture of FDD/TDD |  | Optional with capability signalling |

- NR_channel_raster_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_channel_raster_enh | 28-1 | Enhanced channel raster | The UE supports the requirements for UE channel bandwidths located on the enhanced channel raster of a band as specified in TS 38.101-1, TS 38.101-5 |  | Yes |  | [N/A (not defined)] Option 1: UE may not support requirements for UE specific channel bandwidths located on enhanced channel raster New option 2: UE may not support requirements for UE specific channel bandwidths located on enhanced channel raster; configuring a narrower UE-specific channel bandwidth inside a wider gNB channel bandwidth may not be possible. | Per Band | No | FR1 only | The feature is supported for applicable bands in FDD-TDD and FR1/FR2 combinations | Applies only for bands with a 100 kHz channel raster for both TN and NTN. Should be early implementable from Rel-16. | Mandatory with capability signaling for all Rel-18 UEs for certain bands as defined in 38.101-1 and 38.101-5 FFS: Mandatory [with or without] capability signaling for (e)RedCap UEs from [Rel-17 or Rel-18] for all applicable bands supported by the UE. Optional otherwise. |

- NR_RF_FR2_req_Ph3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_RF_FR2_req_Ph3 | 29-1 | Beam correspondence in initial access and RRC_INACTIVE | R18 and onward UE shall support beam correspondence in initial access and RRC_INACTIVE and satisfy the corresponding spherical coverage requirement for initial access and RRC_INACTIVE as specified in 38.101-2 |  | No |  | UE performance in initial access and RRC_INACTIVE in FR2 cannot be guaranteed | N/A | N/A | FR2 only |  |  | Mandatory without capability signaling from Rel-18 |

- NR_FR2_multiRX_DL

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_FR2_multiRX_DL | 30-1 | Supports scheduling restriction relaxation and measurement restriction relaxation | - Supports simultaneous reception of CSI-RS for layer 1 measurement and PDSCH with different QCL Type-D on overlapping OFDM symbols. - Supports Simultaneous layer 1 measurement of CSI-RS overlapping with another CSI-RS with different QCL Type-D on overlapping OFDM symbol(s). | 16-2c, 23-5-1, [at least one of 16-2a, 16-2b-1, 16-2b-2 and 16-2b-3] | Yes | N/A |  | Per FSPC | TDD only | FR2-1 only |  | [Note: It can be supported for all power classes excepted PC6. ] |  |
| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer 1 measurement | - Supports beam sweeping factor reduction for SSB-based layer 1 measurement when the UE is in multi-RX operation. | 16-2c, 23-5-1 | [No] | N/A |  | Per band | N/A | FR2-1 only |  | Candidate values for Component 2: {2,4,6} for FR2-1 [Note: It can be supported for all power classes excepted PC6.] |  |

- NR_RRM_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_RRM_enh3 | 31-1 | Enhanced L3 measurement reporting for unknown SCell activation if the valid L3 measurement results are available | Support of reporting valid L3 measurement results triggered by the SCell activation command |  | Yes | N/A | UE does not support reporting valid L3 measurement results triggered by SCell activation command | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported, including single SCell activation, single PUCCH SCell activation, and multiple SCell activation with/without PUCCH SCell. | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-2 | Beam sweeping factor reduction for FR2 unknown SCell activation | Support of reducing beam sweeping factor for cell detection if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure Support of reducing beam sweeping factor for SSB based L1-RSRP measurement if UE has full set (N=8) of beam sweeping during AGC settling part during FR2-1 unknown SCell activation procedure |  | Yes | N/A | UE does not support beam sweeping factor reduction for cell detection during FR2-1 unknown SCell activation. UE does not support beam sweeping factor reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation. | Per Band | TDD onl | FR2-1 only | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. Candidate values for beam sweeping reduction for cell detection during FR2-1 unknown SCell activation are 1,2,4, or 6. [Agreed in WF R4-2310081] Candidate values for beam sweeping reduction for SSB based L1-RSRP measurement during FR2-1 unknown SCell activation are 0,1,2,3,4,5,6, or 7. [Agreed in WF R4-2310081] | Optional with capability signaling |
| 31. NR_RRM_enh3 | 31-3 | Shorter measurement interval for unknown SCell activation | (1) Support of using SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in measurement object for enhanced unknown SCell activation requirement. (2) Support of performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation |  | Yes | N/A | UE does not use SSB periodicity instead of SMTC periodicity for the measurement interval during unknown SCell activation when the SMTC is only configured in MO for enhanced unknown Scell activation requirement. UE does not support performing L1-RSRP measurement in non-DRX mode even DRX is configured during unknown SCell activation | Per UE | No | No | N/A | UE is required to meet the shortened SCell activation delay requirement in TS38.133 [section 8.x.y] if the feature is supported. | Optional with capability signaling |

- NR_MG_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | 32-1 | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-2] | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Unknown activation time for simultaneous Pre-MG is expected | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | 32-3 | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap or Pre-MG. Revised wording: (Support configuration with a pre-MG colliding with another MG or pre-MG, while the first pre-MG has higher priority.) | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | 32-4 | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-5] | 2 NCSG configuration in a FR | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No | N.A |  | Optional with capability signalling |
|  | [32-6] | [Inter-RAT EUTRAN measurements without gap and outside active DL BWP] | 1. Support of requirements of inter-RAT EUTRAN measurements outside active DL BWP without gap with or without interruption. | [32-8] | Yes | NA | UE behaviour of supporting inter-RAT EUTRAN measurements without gap is known to network | [Per UE] | No | No | N.A |  | Optional with capability signalling |
|  | 32-7 | Inter-RAT EUTRAN measurement without gap [and within active DL BWP] | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP | [32-8] | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
|  | 32-8 | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | [32-6 or 32-7] | Yes | No | Undefined UE measurement behavior and when to allow scheduling restriction | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. - #0 and #1 are mandatory, if UE supports EMW feature. - FFS other conditions, e.g., UE supports Case b-1 or b-2 Other patterns are optional | Optional with capability signalling |
|  | 32-9 | Simultaneous reception of NR data and EUTRAN CRS within BWP with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology | 32-7 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
|  | [32-10] | Simultaneous reception of NR data and EUTRAN CRS outside BWP with different numerology | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS with CRS not contained within UE’s active DL BWP and PDCCH or PDSCH reception from the serving cell with a different numerology. | 32-6 | Yes | No | scheduling restriction is applicable | Per UE | No | FR1 only | NA |  | Optional with capability signalling |

Table 2: Rel-18 LTE UE features for NR_MG_enh2 WI.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 32. NR_MG_enh2 | [x-y] | InterRAT-BandInfoNR-r18 | Support of inter-RAT NR measurements without gap with or without interruption. This feature has a defined RRC signalling: ‘InterRAT-BandInfoNR-r18’ |  | Yes | NA | The UE does not support inter-RAT NR measurements without gap with or without interruption for performing inter-RAT NR measurement without gap | Per UE | No | No | NA | # [Component 1 candidate value: “{no-gap-with-interruption, no-gap-no-interruption}”] | Optional with capability signalling |

- NonCol_intraB_ENDC_NR_CA

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NonCol_intraB_ENDC_NR_CA | 33-1 | Support of intra-band non-collocated NR CA operation | Indicates the UE supports TDD-TDD intra-band non-collocated NR-CA operation with MTTD/MRTD requirements according to Table 7.5.4.1/Table 7.6.4-2 in 38.133 [5] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in 38.101-1 [2]. And the UE also supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1 [2]. |  | Yes | N/A | Intra-band non-collocated NR CA operation is not supported. The UE supports TDD-TDD intra-band NR-CA operation with MRTD according to Table 7.6.4-1 in 38.133 and UE RF requirements for intra-band NR-CA except for 7.10A in 38.101-1. | Per BC | N/A | FR1 only | N/A | Supported for band n77/n78 only | Optional with capability signaling |
| 33. NonCol_intraB_ENDC_NR_CA | 33-2 | Support network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16 | For Rel-18, introduce a new UE capability which indicates support network control of requirement applicability for UE supporting interBandMRDC-WithOverlapDL-Bands-r16. This field is only applicable to the UE indicating interBandMRDC-WithOverlapDL-Bands-r16.. | 2-19 (Rel-16 RAN4 feature) | Yes | N/A | The UE will support “interBandMRDC-WithOverlapDL-Bands-r16” only which means in Rel-18 the network can’t enforce modification on the UE types | per UE | N/A | FR1 Only | N/A |  | Optional with capability signalling |

- NR_HST_FR2_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34.NR_HST_FR2_enh | 34-1 | Support of NR FR2 HST with simultaneous DL reception with two different QCL TypeD RSs | 1) Support of enhanced RF requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception with two different QCL TypeD RSs 2) Support of enhanced RRM requirement to support FR2-1 PC6 UEs with simultaneous DL signals reception associated with two different QCL TypeD RSs | 22-1 | Yes | N/A | UE does not support FR2 high speed train scenario with simultaneous DL reception with two different QCL TypeD RSs | Per Band | No | FR2 only | N/A | A single indication element is used to indicate for the components for 34-1. The UE capability reported in this feature is applied when highSpeedDeploymentTypeFR2-r17 is configured by network as bidirectional | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-2 | Enhanced FR2 HST RRM requirements for intra-band CA and inter-frequency measurements in connected mode | 1) Support of the RRM requirement for intra-band CA operation in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 2) Support of the RRM requirement for enhanced inter-frequency measurements in connected mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | Yes | N/A | The performance of intra-frequency measurement on SCC and/or inter-frequency measurements in connected mode for NR FR2 HST scenario cannot be guaranteed | Per band | No | FR2 only | N/A |  | Optional with capability signaling |
| 34.NR_HST_FR2_enh | 34-3 | Enhanced FR2 HST RRM requirements for inter-frequency measurement in Idle and Inactive mode | Support of the RRM requirement for inter-frequency measurements in idle and Inactive mode to support FR2 high speed up to 350 km/h, as specified in TS 38.133 | 22-1 | No | N/A | The performance of inter-frequency measurement in idle and Inactive mode for FR2 HST scenario cannot be guaranteed | N/A | No | FR2 only | N/A |  | Optional without capability signaling |
| 34.NR_HST_FR2_enh | 34-4 | Support of enhanced MAC CE for TCI state switch indication for FR2 HST | 1. Support of enhanced one-shot large UL transmit timing adjustment requirement to support FR2-1 PC6 UEs, as specified in TS 38.133 based on [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] 2. Support of enhanced TCI state switching delay requirewments based on new MAC CE for TCI state switch indication named as [the cross-RRH TCI state indication for UE-specific PDCCH MAC CE] in HST FR2 scenario, as specified in TS 38.133. | 22-1 | Yes | N/A | UE does not support enhanced MAC CE for TCI state switch indication for FR2 HST | Per Band | No | FR2 only | N/A | The naming of MAC CE for [the cross-RRH TCI state indication for UE-specific PDCCH] depends on RAN2 | Optional with capability signalling |

- NR_ATG

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. NR_ATG | 35-1 | Enhanced RRM requirements for measurements in IDLE and INACTIVE modes | Indicate the support of enhanced inter-frequency cell re-selection requirements for ATG (as specific in TS 38.133 Table 4.2D.2.4-2) |  | no | N/A | If UE does not support the capability, legacy measurement requirements (as specified in TS 38.133, Table 4.2D.2.4-1) are applied. | Per UE | No | FR1 only | N/A |  | Optional without capability signaling |
| 35. NR_ATG | 35-2 | Antenna type | Indicate whether UE supports the RF and RRM requirements with antenna array as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. If the field is absent, the RF and RRM requirements with omni-directional antenna applies as specified in TS 38.101-1 section 6.1J, 7.1J and TS 38.133. |  | yes | N/A | If UE does not support this feature group, performance cannot be guaranteed. | Per Band | No | FR1 only | N/A |  | Mandatory with capability signaling for UE supports NR communication via ATG |
| 35. NR_ATG | 35-3 | Rated max output power | Indicate the support of rated maximum output power at maximum modulation order and full PRB configurations |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s maximum output power. | Per band | No | FR1 only | N/A | Value range from 23dBm to 40dBm with 1dB as granularity | Mandatory with capability signaling for UE supports NR communication via ATG |
| 35. NR_ATG | 35-4 | ATG specific P-max | Indicate the support of ATG specific P-max configured by network. |  | no | N/A | If UE does not support ATG specific P-max value, ATG UE can’t identify configured maximum output power PCMAX,f,c | Per UE | No | FR1 only | N/A | Value range from -21dBm to 42dBm | Mandatory without capability signaling for UE supports NR communication via ATG |

- NR_demod_enh3

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | [1) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for total 2 layers across target and co-scheduled UEs with 2 RX antennas 2) R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO transmissions for up to 2,3, or 4 total layers across target and co-scheduled UEs with 4 RX antennas.] | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | TBD | No | FR1 only | N/A |  | Optional with capability signaling |
| 36. NR_demod_enh3 | 36-1 | MU-MIMO Interference Mitigation advanced receiver | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, for MU-MIMO up to maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 2 RX and 4RX antennas, when co-scheduled UE(s)’ modulation order is signaled. | 3-4 | Yes | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO | Per UE Note: UE supports R-ML on MU-MIMO on single carrier operation. UE optionally supports R-ML on MU-MIMO on one or more carriers in CA operation |  |  |  |  |  |
| 36. NR_demod_enh3 | 36-2a | MU-MIMO Interference Mitigation advanced receiver with modulation order detection | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and 4RX] when co-scheduled UE(s)’ modulation order is not signaled | 36-1 | No | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO with modulation order detection | N/A |  |  |  |  |  |
| 36. NR_demod_enh3 | 36-2b | MU-MIMO Interference Mitigation advanced receiver with modulation order detection | R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression for MU-MIMO [for 2 layers across target and co-scheduled UEs with 2RX and maxNumberMIMO-LayersPDSCH layers across target and co-scheduled UEs with 4RX] when co-scheduled UE(s)’ modulation order is not signaled | 36-1 | No | N/A | UE not capable of advanced receiver to suppress inter-user inference in MU-MIMO with modulation order detection | N/A |  |  |  |  |  |

- NR_pos_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37. NR_pos_enh2 | 37-1 | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_CONNECTED | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 41-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-1A | Support of reduced number of samples for PRS based positioning measurements with frequency hopping for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in PRS based positioning measurements with frequency hopping | RAN1 feature 28-1, 27-3-1, 41-5-1 | No | NA | RedCap UE does not support reduced number of samples for PRS based positioning measurements with frequency hopping | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2 | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_CONNECTED | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |
| 37. NR_pos_enh2 | 37-2A | Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation for RRC_IDLE and RRC_INACTIVE | 1. Support of reduced number of samples in positioning measurements with PRS bandwidth aggregation | Component 1 RAN1 feature 41-4-1 | No | NA | UE does not support reduced number of samples in positioning measurements with PRS bandwidth aggregation | Per Band | No | No | NA | # Component 1 candidate value: true/false # Need for the LMF to know if the feature is supported: True | Optional with capability signalling |

- NR_MC_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | 38-1 | Switching period for dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL up to two TAGs | UE to indicate support of dynamic UL Tx switching across up to 4 bands for inter-band UL CA, or SUL. switchingPeriodFor2T-r18 indicates the length of 2Tx-2Tx switching period. switchingPeriodFor1T-r18 indicates the length of 1Tx-2Tx switching and/or 1Tx-1Tx switching period, as specified in TS 38.101-1. n35us represents 35 us, n140us represents 140us, and n210us represents 210us, as specified in TS 38.101-1. |  | Yes |  | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination | Per BC, details are up to RAN2 | No need | Applicable only to FR1 |  |  | Optional with capability signaling |
| 38. NR_MC_enh | 38-2 | Application of DL interruptions due to dynamic UL Tx switching | 1. Capability to indicate that for the band where DL interruption is needed, the RRM interruption requirements defined in RAN4 shall be applied for duplex mode combinations except the combinations - SUL+TDD - TDD+TDD CA with the same UL-DL pattern | 38-1 | Yes | N/A | UL Tx switching where DL interruption is needed cannot be supported. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Field encoded as a bit map, where bit N is set to "1" if DL interruption on band N will occur during uplink Tx switching as specified in TS 38.133 [5]. The leading / leftmost bit (bit 0) corresponds to the first band of this band combination, the next bit corresponds to the second band of this band combination and so on. | Optional with capability signaling |
| 38. NR_MC_enh | 38-3 | Switching Period for unaffected Band for Dual UL | SwitchingPeriodUnaffectedBandDualUL indicate for a given band pair {band X and band Y}, whether/how the switching period is to be applied on band X, Y, Z, when a UL Tx switching is triggered from band pair {band X and band Z} to band pair {band Y and band Z}, as defined in 38.101-1. If absent for band Z, the UE is not required to transmit on any UL bands during the switching period reported for the band pair of band X and band Y, as defined in 38.101-1 - maintainedUL-Trans-r18 indicates that the UE is capable of uplink transmission on band Z and is not required to transmit on band X and Y during the switching period reported for the band pair of band X and band Y, as specified in 38.101-1. - periodOnULBands-r18 indicates the switching period to be applied on any UL bands as specified in 38.101-1. n35us represents 35 us, n140us represents 140us, and n210us represents 210us. - Band Z corresponds to the zth entry in the uplinkTxSwitchingPeriodUnaffectedBandDualUL-List-r18, which includes the UL band of this band combination excluding band X and band Y listed in the same order of the band combination. | 38-1 | Yes | N/A | UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination. | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 3 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-4 | [Additional switching Period for Dual UL] | [1. Indicate additionally the supported Tx switching period for switching between a band pair and another band pair or another band, when Rel-18 UL Tx switching is configured by uplinkTxSwitchingMoreBands-r18. If the capability is not reported, the switching period reported in switchingPeriodFor2T-r18 or switchingPeriodFor1T-r18 applies, as specified in TS 38.214 and TS 38.101-1.] | 38-1 | Yes | N/A | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination.] | Per BC | No | FR1 only | Support mixture of FDD/TDD | Component 1 candidate value: {35us, 140 us, 210us} | Optional with capability signaling |
| 38. NR_MC_enh | 38-5 | [Improved switching period for four-band switching case] | [1. Support the capability that the switching period can be improved to min {max(Tswitch_A-C, Tswitch_B-D), max(Tswitch_A-D, Tswitch_B-C)} assuming UE’s preferred (switched-from, switched-to) band pairs for parallel UL transmission switching for a band combination consisting of four different bands.] | 38-1 | Yes | N/A | [Network can only assume the maximum switch period] | Per BC | No | FR1 only | Support mixture of FDD/TDD | [Note: Detailed information can refer to the LS to RAN2 in R4-2317609] | Optional with capability signalling |
| 38. NR_MC_enh | [38-6] | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among up to 4 bands | 1. Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused | 38-1 | Yes | N/A | The existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2217741. | Optional with capability signalling |

Apple proposal:

| 38. NR_MC_enh | [38-6] | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among 3 bands | 1. Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused | 38-1 | Yes | N/A | The existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2217741. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 38. NR_MC_enh | [38-7] | UL-MIMO coherence capability for dynamic Tx switching between 2Tx-2Tx switching among 4 bands | 1. Apply UL-MIMO coherence for the 2Tx-capable UL band(s). Rel-17 signalling on UL-MIMO coherence capability for 2Tx-2Tx switching is reused | 38-1 | Yes | N/A | The existing Rel-15 per band UE capability pusch-TransCoherence is applicable to each of the 2Tx-capable UL band(s) for Tx switching | Per BC | No | FR1 only | Support mixture of FDD/TDD | Note: Detailed information can refer to the LS to RAN2 in R4-2217741. | Optional with capability signalling |

- NR_Mob_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-1 | SSB based L1-RSRP measurements for multiple cells with RTD > CP | Capability of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or the same active BWP is larger than CP length of the cell on the frequency layer. | 45-1from RAN1 Rel-18 feature list or 39-2 or 39-2a | Yes | No | NW does not know which requirements UE will follow | [Per UE/Per BC] | No | Yes | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-2 | SSB based inter-frequency L1-RSRP measurements without measurement gaps | Capability of SSB based inter-frequency L1-RSRP measurements without measurement gaps (without interruption on serving cell(s)) for LTM | 45-1a from RAN1 Rel-18 feature list and 9-4 | Yes | No | UE does not support inter-frequency L1-RSRP measurements without measurement gaps | [Per UE/Per BC] | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-2a | SSB based inter-frequency L1-RSRP measurements with measurement gaps | Capability of SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM | 45-1a from RAN1 Rel-18 feature list | Yes | No | UE does not support inter-frequency L1-RSRP measurements with measurement gaps | Per UE | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-3-1 | Number of frequency layers for L1-RSRP measurement | - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list and/or 39-2 2. Component 2: 39-2a | Yes | No | NW does not know the max number of frequency layers UE can measure | [Per UE/Per BC] | No | Yes | No | 1. Candidate values Component 1: {1,2,3,4,5,6,7,8} 2. Candidate values Component 2: {1,2,3,4,5,6,7,8} | Mandatory with capability signaling if UE supports 45-1, 39-2 and/or 39-2a |
| 39. NR_Mob_enh2 | 39-3-2 | Number of neighbour cells to be measured per frequency layer | - The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps The max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list or 39-2 2. Component 2: 39-2a | Yes | No | There is no limitation on the number of neighbour cells per frequency layer for L1 measurement. | [Per UE/Per BC] | No | [Yes] | N/A | 1. Candidate values Component 1: {1,2,3,4,5,6,7,8} 2. Candidate values Component 2: {1,2,3,4,5,6,7,8} Note: it is RAN4 understanding that RAN1 feature 45-1 and 45-1a is for number of cell that can be configured for L1 measurement. What RAN4 is discussing here is for number of cells on which UE can actually perform L1 measurement. | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | 45-1 from RAN1 Rel-18 feature list or 39-2 | Yes | No | There is no limitation on the number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | [Per band/BC] | No | No | N/A | [candidate values: {1,2,3,4,5,6,7,8,9,10,11,12,13,14,15,16}] | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-3-4] | Number of SSB resources for L1-RSRP measurement within a slot | The max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement | 45-1 from RAN1 Rel-18 feature list or 39-2 | Yes | No | There is no limitation on the number of cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | [Per band/Per BC] | No | Yes | N/A | Candidate value: {1,2,3,4,5,6,7,8} Note: It is also counted in FG 2-24 | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 1. Component 1: 45-1 from RAN1 Rel-18 feature list or 39-2 2. Component 2: 39-2a | Yes | No | There is no limitation on the number of SSB resources per frequency layer for L1 measurement. | [Per UE/Per BC] | No | Yes | No | Candidate value of Component 1: {1,2,3,4,5,6,7,8} Candidate value of Component 2: {1,2,3,4,5,6,7,8} | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | 45-1 from RAN1 Rel-18 feature list or 39-2 | Yes | No | There is no limitation on the number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | [Per band/BC] | No | Yes | No | Candidate values: {2,4,8,12,16,32,64} | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-4 | Interruption on DL slot(s) due to PDCCH- ordered RACH transmission | Capability on whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission | 45-5 | Yes | No | UE does not cause interruptions on DL slots on serving cells due to PDCCH-ordered RACH transmission | Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination | No | No | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-4a | Interruption due to RF retuning for PDCCH- ordered RACH | Indicates the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE’s configured UL BWP(s) of active serving cells | 45-5 | Yes | No | Network does not know whether UE supports the case that RACH bandwidth is outside of any configured BWP and the corresponding length of the interruption | Per band pair (between the target band for RACH transmission and band under UE’s current band combo) per band combination | No | No | N/A | Candidate values for interruption length Y = 0.25, 0.5, 1 and 2 | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-5 | RF/BB preparation time for PDCCH-order RACH | Indicates the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE’s configured UL BWP(s) of active serving cells | 45-5 | Yes | No | Network does not know whether UE supports the case that RACH bandwidth is outside of any configured BWP and the corresponding RF/BB preparation time | [Per band pair] (between the target band for RACH transmission and band under UE’s current band combo) per band combination | No | No | N/A | Candidate values: { [1ms,3ms,5ms,10ms] } | Optional with capability signaling |
| 39. NR_Mob_enh2 | [39-6] | Early processing of an LTM candidate cell RRC configuration | Indicates the number of cells, [FFS: including both SpCell and SCell], on which UE supports early processing of an LTM candidate cell RRC configuration before cell switch. | 45-3a or 45-4a in RAN1 feature list | Yes | No | TLTM_RRC-processing delay (refer to TS 38.133) will not be skipped, i.e., 10ms | [Per UE] | No | Yes | N/A | Candidate values: {1,2,…,TBD } | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-7 | Faster UE processing time during cell switch | Capability of reduced TLTM_processing delay (refer to TS 38.133)]. - Support of reduced TLTM_processing for cell switch from FR1 to FR1. - Support of reduced TLTM_processing for cell switch from FR2 to FR2. - Support of reduced TLTM_processing for cell switch from FR1/FR2 to FR2/FR1. |  | Yes | No | TLTM_processing delay will not be reduced, i.e., 20ms for intra-FR cell switch and 40ms for inter-FR cell switch | Per UE | No | No | N/A | Candidate values of Component 1 and component 2: {10ms, 15ms} Candidate values of Component 3: {20ms, 30ms} | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-8 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup |  | Yes | N/A | UE does not support measurement validation during connection setup/resume | Per-UE | No | FFS | N/A |  | Optional with capability signaling |
| 39. NR_Mob_enh2 | 39-9 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation and report based on non-EMR measurement during connection setup/resume for fast CA/DC setup |  | Yes | N/A | UE does not support measurement validation during connection setup/resume | Per-UE | No | FFS | N/A |  | Optional with capability signaling |

- NR_NTN_enh

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40.NR_NTN_enh | 40-1 | VSAT UE type in NTN | Support of fixed or mobile VSAT (Very Small Aperture Terminal) UE type a) Type 1: a fixed VSAT, which is allowed to access to a cell for MSS (mobile satellite service) or FSS (fixed satellite service) from regulation perspective. b) Type 2: a mobile VSAT, which is allowed to access to an MSS cell from regulation perspective. A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 must indicate support of this capability with only one type. If this capability is absent, a mobile VSAT is supported by default. |  | Yes | N/A | The network doesn’t know the VSAT UE type and cannot decide whether it’s allowed to handover this UE to an FSS cell. | Per UE | N/A | FR2 | N/A | Support receiving access control indication in system information | Optional with capability signalling |
| 40.NR_NTN_enh | 40-2 | Beam steering | Support of beam steering capability - Type 1: Fully electronically-steered beam UEs - Type 2: Fully mechanically-steered beam UEs A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 must indicate support of this capability with only one type. |  | Yes | N/A | Beam steering is not supported. | Per UE | N/A | FR2 | N/A | The capability is not applicable for UE other than VSAT. | Optional with capability signaling |

- NR_cov_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Netw_Energy_NR (input after adhoc)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 42.Netw_Energy_NR | 42-1 | SCell without SS/PBCH block for inter-band CA | Support of SCell without SS/PBCH block for inter-band CA |  | Yes | NA | UE cannot support SCell without SS/PBCH block for inter-band CA | per FS | NA | FR1 only | NA | For each band within the BC, UE indicates if it supports the SSB-less operation when this band is the reference band and other band(s) in the BC as the SSB-less band(s). If UE indicate “support” for this band, it means all other bands within the BC can be configured as SSB-less bands. | Optional with capability signaling |

- NR_DualTxRx_MUSIM

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 44. 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC | 44-1 | TxDiversity for 2Tx | Indicates UE supports 2Tx Tx diversity for the band configured |  | Yes | N/A | UE doesn’t support 2Tx Tx diversity for the band configured | Per FS | No | FR1 only | N/A | Early implementable from Rel-17 | Optional with capability signalling |

- NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-1 | Power class for sidelink CA | Indicates power class the UE supports when operating according to this band combination used for sidelink. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClassSidelink-r16 in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1 | Per BC | No | FR1 only | N/A |  | Optional with capability signalling |
| 45. NR_SL_enh2 | 45-2 | SL reception in intra-carrier guard band | Capability of reception in the non-zero intra-cell guardband between contiguous RB sets in SL wideband carrier operation wider than 20MHz when LBT is successful only in a subset of RB sets |  | Yes | Yes | UE cannot receive in the intra-cell guard band specified in 38.101-1 | Per band | No | FR1 only | N/A | The candidate values are true and false | Optional with capability signalling |
| 45. NR_SL_enh2 | 45-3 | Power class for sidelink unlicensed | This parameter indicates the supported power class for this band used for sidelink unlicensed. If the field is absent, the UE supports the default power class in TS 38.101-1, Table 6.2E.1F-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1, e.g., power class 5 | Per Band | No | FR1 only | N/A |  | Optional with capability signalling |

# xx. NR_netcon_repeater

ZTE propose to capture NCR MT FGs in RAN4 UE feature list (R4-2402517).

Recommended WF:

Since no new FGs are introduced to NCR MT, if companies agree to capture the applicable FGs for NCR MT, recommend to capture following information in RAN4 UE feature list instead of the tables:

The following FGs in TR38.822 are applicable to NCR MT:

- Rel-15 FGs: FG 1-1~FG 1-11, FG 2-1~FG 2-17, FG 3-1~FG 3-4 (details refer to R4-2402517)

- Rel-16 and Rel-17 FGs: All of R16 and R17 features are optional for NCR-MT except for CA, DC related feature , unlicensed band and, HPUE Duty cycle,  MPR related feature which is not applicable for NCR-MT.

Agreement:

Capture following information in LS content:

The following FGs in TR38.822 are applicable to NCR MT:

- Rel-15 FGs: FG 1-1~FG 1-11, FG 2-1~FG 2-17, FG 3-1~FG 3-4 (details refer to R4-2402517)

- Rel-16 and Rel-17 FGs: All of R16 and R17 features are optional for NCR-MT except for CA, DC related feature , unlicensed band and, HPUE Duty cycle,  MPR related feature which is not applicable for NCR-MT.