3GPP TSG-RAN WG4 Meeting # 110												R4-2403648

Athens, Greece, Feb 26 – Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR |  | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-1 Annex: NTN VSAT related FRC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR introduces VSAT requirements for NTN Ka bands according to the agreed work split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the Annnex with FRC for NTN VSAT receiver requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN ka-bands won’t be correctly supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

Annex A (normative):
Measurement channels

A.1	General

A.1.1	Throughput definition

The throughput values defined in the measurement channels specified in Annex A, are calculated and are valid per codeword. For multi-codeword transmissions, the throughput referenced in the minimum requirements is the sum of throughputs of all codewords.

A.2	UL reference measurement channels

A.3	DL reference measurement channels

A.3.1	General

The transport block size (TBS) determination procedure is described in clause 5.1.3.2 of TS 38.214 [12].

Unless otherwise stated, no user data is scheduled on slot #0 within 20 ms in order to avoid SSB and PDSCH transmissions in one slot and simplify test configuration.

A.3.2	Reference measurement channels for PDSCH performance requirements

For PDSCH reference channels if more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit).

A.3.2.1	FDD

A.3.2.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.1.1-1: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 FDD |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |
| Allocated slots per 2 frames | Slots | 19 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 4 |  |  |  |  |
| Modulation |  | QPSK |  |  |  |  |
| Target Coding Rate |  | 0.30 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  | 18 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 3904 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |
| For Slots i = 1,…, 19 | CBs | 1 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 12480 |  |  |  |  |
| For Slots i =1,…, 9, 12, …, 19 | Bits | 13104 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 3.709 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |

Table A.3.2.1.1-2: PDSCH Reference Channel for FDD (16QAM)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-2.1 FDD |  |  |  |  |  |
| Channel bandwidth | MHz | 10 |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |  |
| Number of allocated resource blocks | PRBs | 52 |  |  |  |  |  |
| Number of consecutive PDSCH symbols |  | 12 |  |  |  |  |  |
| Allocated slots per 2 frames | Slots | 19 |  |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |  |
| Number of DMRS REs |  | 12 |  |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 13064 |  |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |  |
| For Slots i = 1,…, 19 | Bits | 24 |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | CBs | N/A |  |  |  |  |  |
| For Slots i = 1,…, 19 | CBs | 2 |  |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |  |
| For Slot i = 0 | Bits | N/A |  |  |  |  |  |
| For Slots i = 10, 11 | Bits | 26208 |  |  |  |  |  |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 27456 |  |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 12.411 |  |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames |  |  |  |  |  |  |  |

A.3.2.1.2	Reference measurement channels for SCS 60 kHz FR2-NTN

[

Table A.3.2.1.2-1: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 7) |  | 23 /24 | 23 / 24 | 23 / 24 |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 4224 | 8456 | 16896 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | CBs | 1 | 2 | 3 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} (NOTE 5) | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} (NOTE 6) | Bits | 14256 | 28512 | 57024 |
| Max. Throughput averaged over 1 frame (NOTE 8) | Mbps | TBD | TBD | TBD |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {3,4,5,6,7} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 8) = {0,1,2} for i from {0,…,79} together with the TDD UL-DL configuration specified in A2.3. NOTE 7: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 8: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |

Table A.3.2.1.2-2: PDSCH Reference Channel for FDD (16/64QAM)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 6) |  | 23 / 24 | 23 / 24 | 23 / 24 |
| MCS index |  |  |  |  |
| Modulation |  |  |  |  |
| Target Coding Rate |  |  |  |  |
| Maximum number of HARQ transmissions |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits |  |  |  |
| Transport block CRC | Bits |  |  |  |
| LDPC base graph |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,79} | CBs |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | CBs |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits |  |  |  |
| Max. Throughput averaged over 1 frame (NOTE 7) | Mbps |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. NOTE 6: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 7: Throughput is averaged over 2nd frame of RMC |  |  |  |  |

]

A.3.2.1.3	Reference measurement channels for SCS 120 kHz FR2-NTN

[Table A.3.2.1.3-1: PDSCH Reference Channel for FDD (QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 7) |  | 47 / 48 | 47 / 48 | 47 / 48 | 47 / 48 |
| MCS index |  | 4 | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 2088 | 4224 | 8456 | 16896 |
| Transport block CRC | Bits | 16 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | CBs | 1 | 1 | 2 | 3 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} (NOTE 5) | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} (NOTE 6) | Bits | 6912 | 14256 | 28512 | 57024 |
| Max. Throughput averaged over 1 frame (NOTE 8) | Mbps | TBD | TBD | TBD | TBD |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms NOTE 4: Slot i is slot index per 2 frames NOTE 5: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {7,…,15} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 6: When this DL RMC used together with the UL RMC for the transmitter requirements requiring at least one sub frame (1ms) for the measurement period, Slot i, if mod(i, 16) = {0,…,6} for i from {0,…,159} together with the TDD UL-DL configuration specified in A2.3. NOTE 7: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 8: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |  |

Table A.3.2.1.3-2: PDSCH Reference Channel for FDD (16/64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame (NOTE 6) |  |  |  |  |  |
| MCS index |  |  |  |  |  |
| Modulation |  |  |  |  |  |
| Target Coding Rate |  |  |  |  |  |
| Maximum number of HARQ transmissions |  |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits |  |  |  |  |
| Transport block CRC | Bits |  |  |  |  |
| LDPC base graph |  |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits |  |  |  |  |
| Max. Throughput averaged over 1 frame (NOTE 7) | Mbps |  |  |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. NOTE 6: First number corresponds to the number slots allocated in the first frame of the RMC; second number corresponds to the number slots allocated in the second frame of the RMC. NOTE 7: Throughput is averaged over 2nd frame of RMC. |  |  |  |  |  |

]

<End of the change>