3GPP TSG-RAN WG4 Meeting #110 																											R4-2403662

Athens, Greece, February 26 – March 1, 2024

Title: 	WF on coverage enhancement for part 2

Agenda Item:	8.19

Source: 	Nokia

Document for:	Approval

# Topic 1 – Feature List

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| powerBoostRel18 | 41-2 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions without modified spectrum flatness requirement | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK without modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1.The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | 1-6, 1-7 | Yes | N/A | UE cannot power boost without modified spectrum flatness requirement | Per FS | NO | FR1 only | N/A | FFS – RAN4 is still discussing the applicable scenarios | Optional with capability signalling |
| powerBoostTSRel18 | 42-3 | Power boosting for DFT-s-OFDM pi/2 BPSK and QPSK transmissions with modified spectrum flatness requirement shaping | 1. Support of UE power boosting for DFT-s-OFDM pi/2 BPSK and QPSK with modified spectrum flatness requirement for PC3 and PC2 MPR reduction, when applicable as defined in 6.2 of TS 38.101-1. The power boosting is only enabled when signalled via RCC powerBoostPi2BPSKRel18 for BPSK and powerBoostQPSKRel18 for QPSK | 1-6, 1-7 | Yes | N/A | UE cannot power boost with modified spectrum flatness requirement | Per FS | NO | FR1 only | N/A | FFS – RAN4 is still discussing the applicable scenarios | Optional with capability signalling |

WF:

- RAN4 is still discussing the applicable scenarios.

  - The common understanding in RAN4 is that power boosting is only supported for single CC UL at least in Rel-18.