3GPP TSG-RAN WG4 Meeting #110	R4-2403741

# Athens, Greece, 26th Feb – 1st Mar, 2024

Source:	Huawei, HiSilicon

Title:	TP for TR 37.718-11-21: DC_3(n)AA_n77A

Agenda item:			7.6.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 37.718-11-21 to include DC_3(n)AA_n77A.

# 2. Text Proposal

# ---Start of changes---

## 6.x	DC_3(n)_n77

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| NR and E-UTRA DC Configuration | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3(n)_n77 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.x.1-2: Inter-band NE-DC configurations within FR1 (three bands)

| NE-DC configuration | Uplink NE-DC configuration |
| --- | --- |
| DC_3(n)AA_n77A | DC_3(n)AA1 DC_n77A_3A |
| NOTE1: Only single switched UL is supported. |  |

### 6.x.2	Channel bandwidths per operating band for DC

|  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR and E-UTRA DC Configuration | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3(n)AA_n77A | 3 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  | n3 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  |  |  |  |  |  |  |  |  |  |
|  | n77 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

### 6.x.3	Co-existence studies

Same as DC_(n)3AA-n78A

### 6.x.4	∆TIB and ∆RIB values

For DC_3(n)_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band NE-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3(n)_n77 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.x.4-2: ΔRIB

| Inter-band NE-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3(n)_n77 | 0.2 | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.x.5	MSD

The MSD values are taken from DC_(n)3AA-n78A in TS 38.101-3.

Table 6.x.2.2-1: MSD test points for Scell due to dual uplink operation for NE-DC in NR FR1 (three bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NE-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3(n)AA_n77A | 3 | 1740 | 5 | 25 | 1835 | 31.9 | IMD24 |
|  | n3 | N/A | 5 | N/A | 1840 | [28.9] | IMD24 |
|  | n77 | 3575 | 10 | 50 | 3575 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

---End of changes---