# 3GPP TSG-RAN WG4 Meeting # 110                                                             R4-2403746

# Athens, Greece, Feb. 26th – Mar. 1st , 2024

# Source:	ZTE Corporation

# Title:	TP for TR38.718-02-01_CA_n1A-n78C

# Agenda Item:	7.10.2

# Document for:	Approval

- Introduction

The CA_n1A-n78C_BCS4 and 5 were requested and included in the WID, this contribution provides a text proposal to introduce CA_n1A-n78C in TR38.718-02-01.

The fallback have already been defined in the spec.

- Reference

- [1] TR38.718-02-01,Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2),v0.9.0

- Text Proposal

----- Start of TP -----

  - 5.x		CA_n1-n78

    - 5.x.1	Common for 1 band UL and 2 bands UL CA

      - 5.x.1.1 Operating bands for CA

Table 5.x.1.1-1: CA band combination CA_n1-n78

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78 | n1 | 1920 MHz | – | 1980 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3400 MHz | – | 3800 MHz | TDD |

      - 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n1-n78

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n78C | CA_n1A-n78C | n1 | See n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n78 | CA_n78C_BCS4 and 5 |  |

      - 5.x.1.3	UE Co-existence studies

The co-existence studies for UL CA_n78C are studied as in the table 5.x.1.3.1-1:

Table 5.x.1.3.1-1: Co-existence studies for Uplink Intra-Band Contiguous CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10 | 10 | 200 | 3300 | 3800 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 3300 | 3320 | 3500 | 3800 | 3780 | 3600 |
| 2nd order IMD products | IfU1L-fU2LI | IfU1L-fU3LI | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges | 20 | 200 | 6620 | 7580 |  |  |
| 3rd order IMD products | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges | 3100 | 4000 | 9920 | 11380 |  |  |
| 4th order IMD products | I2*fU1L-2*fU2LI | I2*fU1H-2*fU3H I | 3*fU1L-fU3L | 3*fU1H-fU3H | 3*fU1L+fU2L | 3*fU1H+fU2H |
| Interference ranges | 40 | 400 | 6400 | 7800 | 13220 | 15180 |
| 5th order IMD products | I3*fU1L-2*fU3LI | I3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges | 2900 | 4200 | 9700 | 11600 | 16520 | 18980 |
| 6th order IMD products | I3*fU1L-3*fU2LI | I3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fU1H-fU3H |
| Interference ranges | 60 | 600 | 6200 | 8000 | 13000 | 15400 |
| 7th order IMD products | I4*fU1L-3*fU3LI | I4*fU1H-3*fU3HI | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges | 2700 | 4400 | 9500 | 11800 | 16300 | 19200 |

It can be seen that there are no co-existence issues caused by CA_n78C impact band n1 DL.

      - 5.x.1.4	∆TIB and ∆RIB values

The TIB,c and RIB of CA_n1-n78 are already defined in the spec TS38.101-1.

      - 5.x.1.5	REFSENs requirements

According to the co-existence study, no additional MSD requirements should be defined.

      - 5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exception issue.

    - 5.x.2		Specific for 2 bands UL CA

      - 5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n1A-n78C | 23 | +2/-3 |

      - 5.x.2.2		UE co-existence studies

In this section, we only focus on the triple beat IMDs caused by UL CA_n1A-n78C. For double beat IMDs caused by CA_n1A-n78A, it was already covered in the co-existence studies of the fallback configurations of DL CA_n1A-n78A with UL single carrier or UL CA_n1A-n78A.

The analysis of the triple beat IMD is found in Table 5.x.2.3-1:

Table 5.x.2.3-1: Band n1 and Band n78 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 3300 | 3320 | 3500 | 3800 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 3780 | 3600 |  | 10 |
| 1st order TB | IfU3L -fU1L- fSCCL¦ | IfU2L -fU1L + fSCCL¦ | IfU2L -fU1L- fSCCH¦ | IfU3L -fU1L + fSCCH¦ |  | Max ch. separation |
| Ranges | 1720 | 1940 | 1960 | 2180 |  | 200 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L +fU1L+fSCCL¦ | IfU1H +fU2H+fSCCH¦ |  |  |
| Ranges | 4640 | 5660 | 8540 | 9560 |  |  |

It can be seen that 1st order TB IMDs would fall into band n1 DL Rx. However, according to the following agreements in the WF R4-2220556:

- For two-band inter-band CA or DC combinations, the precondition is that:

- Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1. The decision tree in Figure 1 provides guidelines to decide if TB detection analysis is needed.

- The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in Table 1.

In the other words, only same band group or adjacent band group TB IMDs issues shall be considered.

For band n1 and n78, band n1 belongs to FR1-2(MB) group while band n78 belongs to FR1-4(VHB), which means there is no need to define the additional TB IMDs requirements.

      - 5.x.2.3		REFSENS requirements

Based on the co-existence , although there are IMD issues, there is no need to define the additional REFSEN exception requirements.

----- End of TP -----