3GPP TSG-RAN WG4 Meeting #110 	R4-2403758

# Athens, Greece, Feb. 26th  - Mar. 1st

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | <CR#> | rev | <Rev#> | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to 38.101-3 – Add CA_n48A-n258A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, US Cellular |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_2BDL_xBUL |  |  |  |  |  | Date: |  |  | 2024-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding band combination per operator request |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding CA_n48A-n258A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Band combination not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A, 5.5A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521 series |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**************************** Start of changes ************************************

Table 5.2A.1-1: Band combinations for inter-band CA between FR1 and FR2 (two bands)

| NR CA Band |  | NR Band |
| --- | --- | --- |
| CA_n1-n2571 |  | n1, n257 |
| CA_n1-n2581 |  | n1, n258 |
| CA_n2-n2601 |  | n2, n260 |
| CA_n2-n2571 |  | n2, n257 |
| CA_n2-n2581 |  | n2, n258 |
| CA_n2-n2611 |  | n2, n261 |
| CA_n3-n2571 |  | n3, n257 |
| CA_n3-n2581 |  | n3, n258 |
| CA_n5-n2581 |  | n5, n258 |
| CA_n5-n2601 |  | n5, n260 |
| CA_n5-n2611 |  | n5, n261 |
| CA_n7-n2581 |  | n7, n258 |
| CA_n7-n2571 |  | n7, n257 |
| CA_n8-n2571 |  | n8, n257 |
| CA_n7-n2601 |  | n7, n260 |
| CA_n8-n2581 |  | n8, n258 |
| CA_n12-n2601 |  | n12, n260 |
| CA_n12-n2571 |  | n12, n257 |
| CA_n12-n2581 |  | n12, n258 |
| CA_n14-n2601 |  | n14, n260 |
| CA_n30-n2601 |  | n30, n260 |
| CA_n12-n2611 |  | n12, n261 |
| CA_n25-n2571 |  | n25, n257 |
| CA_n25-n2581 |  | n25, n258 |
| CA_n25-n2601 |  | n25, n260 |
| CA_n25-n2611 |  | n25, n261 |
| CA_n26-n2581 |  | n26, n258 |
| CA_n28-n2571 |  | n28, n257 |
| CA_n28-n2581 |  | n28, n258 |
| CA_n34-n2581 |  | n34, n258 |
| CA_n38-n2571 |  | n38, n257 |
| CA_n38-n2581 |  | n38, n258 |
| CA_n39-n2571 |  | n39, n257 |
| CA_n39-n2581 |  | n39, n258 |
| CA_n40-n2571 |  | n40, n257 |
| CA_n40-n2581 |  | n40, n258 |
| CA_n41-n2571 |  | n41, n257 |
| CA_n41-n2581 |  | n41, n258 |
| CA_n41-n2601 |  | n41, n260 |
| CA_n41-n2611 |  | n41, n261 |
| CA_n48-n2581 |  | n48, n258 |
| CA_n48-n2601 |  | n48, n260 |
| CA_n48-n2611 |  | n48, n261 |
| CA_n48-n2631 |  | n48, n263 |
| CA_n66-n2571 |  | n66, n257 |
| CA_n66-n2581 |  | n66, n258 |
| CA_n66-n260 |  | n66, n260 |
| CA_n66-n261 |  | n66, n261 |
| CA_n71-n2571 |  | n71, n257 |
| CA_n71-n2601 |  | n71, n260 |
| CA_n71-n2581 |  | n71, n258 |
| CA_n71-n2611 |  | n71, n261 |
| CA_n77-n2571 |  | n77, n257 |
| CA_n77-n2581 |  | n77, n258 |
| CA_n77-n2571 |  | n77, n257 |
| CA_n77-n2591 |  | n77, n259 |
| CA_n77-n2601 |  | n77, n260 |
| CA_n77-n2611 |  | n77, n261 |
| CA_n78-n2571 |  | n78, n257 |
| CA_n78-n2581 |  | n78, n258 |
| CA_n78-n2591 |  | n78, n259 |
| CA_n79-n2571 |  | n79, n257 |
| CA_n79-n2581 |  | n79, n258 |
| CA_n79-n2591 |  | n79, n2591 |
| CA_n105-n2571 | n105, n257 |  |
| CA_n105-n2581 | n105, n258 |  |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |

Table 5.5A.1-1k: Inter-band CA configurations and bandwidth combinations sets between FR1 and FR2 (two bands)

| NR CA configuration |  | Uplink CA configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 3) |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n48A-n258A |  | - | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n258 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n260A |  | CA_n48A-n260A | n48 |  | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48A-n260G | CA_n48A-n260A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48A-n260H | CA_n48A-n260A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48A-n260I | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48A-n260J | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48A-n260K | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48A-n260L | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48A-n260M | CA_n48A-n260A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48A-n260R2 | CA_n48A-n260A/R2 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R2 |  |  |  |
| CA_n48A-n260R3 | CA_n48A-n260A/R2/R3 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R3 |  |  |  |
| CA_n48A-n260R4 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R4 |  |  |  |
| CA_n48A-n260R5 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R5 |  |  |  |
| CA_n48A-n260R6 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R6 |  |  |  |
| CA_n48A-n260R7 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R7 |  |  |  |
| CA_n48A-n260R8 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R8 |  |  |  |
| CA_n48A-n260R9 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R9 |  |  |  |
| CA_n48A-n260R10 | CA_n48A-n260A/R2/R3/R4 |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n260 | CA_n260R10 |  |  |  |
| CA_n48(2A)-n260A | CA_n48A-n260A |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48(2A)-n260G | CA_n48A-n260A/G |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48(2A)-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48(2A)-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48(2A)-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48(2A)-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48(2A)-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48(2A)-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48B-n260A | CA_n48A-n260A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48B-n260G | CA_n48A-n260A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48B-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48B-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48B-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48B-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48B-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48B-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48(A-B)-n260A | CA_n48A-n260A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |  |  |
| CA_n48(A-B)-n260G | CA_n48A-n260A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260G |  |  |  |
| CA_n48(A-B)-n260H | CA_n48A-n260A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260H |  |  |  |
| CA_n48(A-B)-n260I | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260I |  |  |  |
| CA_n48(A-B)-n260J | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260J |  |  |  |
| CA_n48(A-B)-n260K | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260K |  |  |  |
| CA_n48(A-B)-n260L | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260L |  |  |  |
| CA_n48(A-B)-n260M | CA_n48A-n260A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n260 | CA_n260M |  |  |  |
| CA_n48A-n261A | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48A-n261G | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48A-n261H | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48A-n261I | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48A-n261J | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48A-n261K | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48A-n261L | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48A-n261M | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48A-n261(2A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48A-n261(2G) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48A-n261(2I) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2I) |  |  |  |
| CA_n48A-n261(2H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48A-n261(3A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48A-n261(4A) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(4A) |  |  |  |
| CA_n48A-n261(A-G) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48A-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48A-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48A-n261(A-H) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48A-n261(A-I) | CA_n48A-n261A |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48A-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48A-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48A-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48A-n261(2A-G) | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48A-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48A-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48A-n261(A-2G) | CA_n48A-n261A/G |  | n48 | 5, 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(2A)-n261A | CA_n48A-n261A |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48(2A)-n261G | CA_n48A-n261A/G |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48(2A)-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48(2A)-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48(2A)-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48(2A)-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48(2A)-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48(2A)-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48(2A)-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48(2A)-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48(2A)-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48(2A)-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48(2A)-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48(2A)-n261(2A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48(2A)-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48(2A)-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48(2A)-n261(2A) | CA_n48A-n261A |  | n48 | CA_n48(2A) BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48(2A)-n261(3A) | CA_n48A-n261A |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48(2A)-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48(2A)-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(2A)-n261(A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48(2A)-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48(2A)-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A)_BCS1 |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48(2A)-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(2A) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48(2A)_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48B-n261A | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48B-n261G | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48B-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48B-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48B-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48B-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48B-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48B-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48B-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48B-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48B-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48B-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48B-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48B-n261(2A-G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48B-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48B-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48B-n261(2A) | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48B-n261(3A) | CA_n48A-n261A |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48B-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48B-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48B-n261(A-G) | CA_n48A-n261A/G |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48B-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48B-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48B-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48B |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS1 |  | 1 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
|  |  |  | n48 | CA_n48B_BCS2 |  | 2 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48(A-B)-n261A | CA_n48A-n261A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |  |  |
| CA_n48(A-B)-n261G | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261G |  |  |  |
| CA_n48(A-B)-n261H | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261H |  |  |  |
| CA_n48(A-B)-n261I | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261I |  |  |  |
| CA_n48(A-B)-n261J | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261J |  |  |  |
| CA_n48(A-B)-n261K | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261K |  |  |  |
| CA_n48(A-B)-n261L | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261L |  |  |  |
| CA_n48(A-B)-n261M | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261M |  |  |  |
| CA_n48(A-B)-n261(A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G) |  |  |  |
| CA_n48(A-B)-n261(A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-H) |  |  |  |
| CA_n48(A-B)-n261(G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-H) |  |  |  |
| CA_n48(A-B)-n261(2A) | CA_n48A-n261A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A) |  |  |  |
| CA_n48(A-B)-n261(3A) | CA_n48A-n261A |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(3A) |  |  |  |
| CA_n48(A-B)-n261(2G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2G) |  |  |  |
| CA_n48(A-B)-n261(2H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2H) |  |  |  |
| CA_n48(A-B)-n261(A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-I) |  |  |  |
| CA_n48(A-B)-n261(G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(G-I) |  |  |  |
| CA_n48(A-B)-n261(2A-G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |  |  |
| CA_n48(A-B)-n261(2A-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |  |  |
| CA_n48(A-B)-n261(A-2G) | CA_n48A-n261A/G |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |  |  |
| CA_n48(A-B)-n261(A-G-H) | CA_n48A-n261A/G/H |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |  |  |
| CA_n48(A-B)-n261(H-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(H-I) |  |  |  |
| CA_n48(A-B)-n261(2A-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |  |  |
| CA_n48(A-B)-n261(A-G-I) | CA_n48A-n261A/G/H/I |  | n48 | CA_n48(A-B) |  | 0 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |  |  |
| CA_n48(3A)-n260A |  | CA_n48A-n260A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(3A)-n260G |  | CA_n48A-n260A/G | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48(3A)-n260H |  | CA_n48A-n260A/G/H | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48(3A)-n260I |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48(3A)-n260J |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48(3A)-n260K |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48(3A)-n260L |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48(3A)-n260M |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48(4A)-n260A |  | CA_n48A-n260A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48(4A)-n260G |  | CA_n48A-n260A/G | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48(4A)-n260H |  | CA_n48A-n260A/G/H | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48(4A)-n260I |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48(4A)-n260J |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48(4A)-n260K |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48(3A)-n260L |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48(4A)-n260M |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48C-n260A |  | CA_n48A-n260A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | 50, 100, 200, 400 |  |  |
| CA_n48C-n260G |  | CA_n48A-n260A/G | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260G |  |  |
| CA_n48C-n260H |  | CA_n48A-n260A/G/H | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260H |  |  |
| CA_n48C-n260I |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260I |  |  |
| CA_n48C-n260J |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260J |  |  |
| CA_n48C-n260K |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260K |  |  |
| CA_n48C-n260L |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260L |  |  |
| CA_n48C-n260M |  | CA_n48A-n260A/G/H/I | n48 |  | CA_n48C |  | 0 |
|  |  |  | n260 |  | CA_n260M |  |  |
| CA_n48A-n263A |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48A-n263G |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48A-n263H |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48A-n263I |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48A-n263J |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48A-n263K |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48A-n263L |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48A-n263M |  | CA_n48A-n263A | n48 |  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(2A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(2A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(2A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(2A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(2A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(2A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(2A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(2A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(2A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48B-n263A |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48B-n263G |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48B-n263H |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48B-n263I |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48B-n263J |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48B-n263K |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48B-n263L |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48B-n263M |  | CA_n48A-n263A | n48 |  | CA_n48B |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(A-B)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(A-B)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(A-B)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(A-B)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(A-B)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(A-B)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(A-B)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(A-B)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(A-B) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48C-n263A |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48C-n263G |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48C-n263H |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48C-n263I |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48C-n263J |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48C-n263K |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48C-n263L |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48C-n263M |  | CA_n48A-n263A | n48 |  | CA_n48C |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(3A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(3A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(3A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(3A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(3A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(3A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(3A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(3A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(3A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |
| CA_n48(4A)-n263A |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | 400, 800, 1600, 2000 |  |  |
| CA_n48(4A)-n263G |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263G |  |  |
| CA_n48(4A)-n263H |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263H |  |  |
| CA_n48(4A)-n263I |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263I |  |  |
| CA_n48(4A)-n263J |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263J |  |  |
| CA_n48(4A)-n263K |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263K |  |  |
| CA_n48(4A)-n263L |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263L |  |  |
| CA_n48(4A)-n263M |  | CA_n48A-n263A | n48 |  | CA_n48(4A) |  | 0 |
|  |  |  | n263 |  | CA_n263M |  |  |

***************************** End of changes ************************************