3GPP TSG-RAN WG4 Meeting # 110	R4-2403782

# Athens, GR, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | xxxx | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for TS 38.101-3 on subclause for inter-band CA configurations with more than three bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_R18_yBDL_xBUL-Core |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN4#109 meeting, a TP on spec structure for inter-band CA configuration tables was approved in R4-2321921 and the related guidelines were captured in TR 38.846 as below. - All combinations having the same number of constituent bands are categorized into one sub-clause. - For two bands inter-band CA configuration table in TS 38.101-1 5.5A.3.1, a “sub-table-group” tag is suggested to be applied for the purpose of easier retrieval. - For the other inter-band CA configuration tables which have huge configurations of more than 50 pages, the big table could be split to a limited number of up to three smaller sub-tables. The sub-tables should not have less than 40 pages and the maximum number should be 3 sub-tables after the split. With the approved guidelines, the structure of the spec for inter-band CA configurations between FR1 and FR2 should be fixed accordingly. Note that the revisions for inter-band CA configurations between FR1 and FR2 with two bands and with three bands are submitted in R4-2400916 and R4-2400920 respectively in this meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To amend the subclause structure for inter-band CA configurations between FR1 and FR2 with four bands and with five bands. - To split the huge inter-band CA configuration table with four bands into two smaller sub-tables. - To add “sub-table” tag for four bands configurations for easy retrieval. - Add the missing “NOTE 2” in the table for five bands configurations. - To move the table notes for four bands out below the last table and add a statement prior to the notes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The retrieval of a certain CA configuration in the spec will remain difficult. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5A.1, 5.5A.1.3 (new), 5.5A.1.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## 5.5A	Configuration for CA

#### 5.5A.1	Inter-band CA configurations between FR1 and FR2

Table 5.5A.1-3: Void

Table 5.5A.1-4: Void

The configurations for operating bands for CA including Band n41 also apply for the corresponding operating bands for CA with Band n90 replacing Band n41 but with otherwise identical parameters. For brevity the said configuration for operating bands for CA with Band n90 are not listed in the tables below but are covered by this specification.

The configuration tables for CA describe Bandwidth Combination Sets. Bandwidth Combination Set 4 and 5 contains all possible defined channel bandwidths for each FR1 band in the combination. The fact that BCS4 and BCS5 contains all channel bandwidths for each FR1 band does not alter if a bandwidth is mandatory or optional for a given band. Bandwidths that are identified as optional in Table 5.3.5-1 of TS 38.101-1 [2] for a given release are still optional for UEs that support BCS4 or BCS5, where the bandwidths the UE supports for each band, the maximum bandwidth and/or minimum bandwidth for the band in the band combination are indicated in the UE capabilities. Note that the minimum bandwidth is indicated only in BCS5 and BCS5 shall not be indicated together with BCS4 for a CA configuration. For inter-band CA combinations including intra-band CA and with BCS4 or BCS5 in the following configuration tables, the Bandwidth Combination Sets for the FR1 intra-band CA are BCS4 or BCS5, respectively, and the Bandwidth Combination Sets for the FR2 intra-band CA are BCS0.

<< Unchanged contents omitted >>

5.5A.1.3	Inter-band CA configurations between FR1 and FR2 (four bands)

Table 5.5A.1.3-1a

Table 5.5A.1.3-1a: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (four bands)

| NR CA configuration | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n28A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A CA_n3A-n28A CA_n3A-n257A CA_n28A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n28A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n257A/G CA_n28A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n28A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n257A/G/H CA_n28A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n28A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I |  | n1 | 5, 10,15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n41A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A CA_n3A-n41A CA_n3A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n41A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n41A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n41A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77A-n257A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A CA_n3A-n77A CA_n3A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n77A-n257G | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n77A CA_n3A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n77A-n257H | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n77A CA_n3A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n77A-n257I | CA_n1A-n3A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n3A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n77A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n77A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n77A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n77(2A)-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n77(2A)-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n77(2A)-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n77(2A)-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n77(2A)-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n77(2A)-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n77(2A)-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n77(2A)-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n79A-n257A | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A CA_n3A-n79A CA_n3A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n79A-n257G | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n79A CA_n3A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n79A-n257H | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n79A CA_n3A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25 ,30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n79A-n257I | CA_n1A-n3A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n77A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n77A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90,100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n77A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90,100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n77A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n77A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n77A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n8A-n77(2A)-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n77(2A)-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n77(2A)-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n77(2A)-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n77(2A)-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n77(2A)-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n77(2A)-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n77(2A)-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n8A-n78A-n257A | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n8A-n78A-n257D | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n1A-n8A-n78A-n257E | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257E |  |
| CA_n1A-n8A-n78A-n257F | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257F |  |
| CA_n1A-n8A-n78A-n257G | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n8A-n78A-n257H | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n8A-n78A-n257I | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n8A-n78A-n257J | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n1A-n8A-n78A-n257K | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n1A-n8A-n78A-n257L | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n1A-n8A-n78A-n257M | - |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n1A-n28A-n41A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n41A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n41A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n41A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n77A-n257A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n77A-n257G | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n77A-n257H | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n77A-n257I | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n77(2A)-n257A |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n77(2A)-n257G |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n77(2A)-n257H |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n77(2A)-n257I |  | CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n28A-n79A-n257A | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n28A-n79A-n257G | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n28A-n79A-n257H | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n28A-n79A-n257I | CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n41A-n77A-n257A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n77A-n257G | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n77A-n257H | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n77A-n257I | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n41A-n77(2A)-n257A |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n77(2A)-n257G |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n77(2A)-n257H |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n77(2A)-n257I |  | CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n41A-n79A-n257A |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n41A-n79A-n257G |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n41A-n79A-n257H |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n41A-n79A-n257I |  | CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n77A-n79A-n257A | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n77A-n79A-n257G | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G CA_n257G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n77A-n79A-n257H | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H CA_n257G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n77A-n79A-n257I | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I CA_n257G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n77(2A)-n79A-n257A |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n77(2A)-n79A-n257G |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n77(2A)-n79A-n257H |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n77(2A)-n79A-n257I |  | CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n1A-n78A-n79A-n257A | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n78A-n79A-n257G | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G CA_n257G |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n1A-n78A-n79A-n257H | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H CA_n257G/H |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n1A-n78A-n79A-n257I | CA_n1A-n78A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I CA_n257G/H/I |  | n1 | 5, 10, 15, 20 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n2A-n5A-n48A-n260A | CA_n2A-n260A CA_n5A-n260A CA_n48A-n260A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n48A-n260G | CA_n2A-n260A/G CA_n5A-n260A/G CA_n48A-n260A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n48A-n260H | CA_n2A-n260A/G/H CA_n5A-n260A/G/H CA_n48A-n260A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n48A-n260I | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n48A-n260J | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n48A-n260K | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n48A-n260L | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n48A-n260M | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n48A-n261A | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n48A-n261G | CA_n2A-n261A/G CA_n5A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n48A-n261H | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n48A-n261I | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n48A-n261J | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n48A-n261K | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n48A-n261L | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n48A-n261M | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n48A-n261(G-H) | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n48A-n261(2H) | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n48A-n261(A-G-H) | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n48A-n261(H-I) | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n48A-n261(A-G-I) | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n48A-n261(A-G) | CA_n2A-n261A/G CA_n5A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n48A-n261(A-H) | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n48A-n261(A-I) | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n48A-n261(2A-G) | CA_n2A-n261A/G CA_n5A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n5A-n48A-n261(2A-H) | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n48A-n261A/G/H |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n48A-n261(2A-I) | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n48A-n261(A-2G) | CA_n2A-n261A/G CA_n5A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n48A-n261(G-I) | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n48A-n261(2A) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n5A-n48A-n261(3A) | CA_n2A-n261A CA_n5A-n261A CA_n48A-n261A |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n5A-n48A-n261(2G) | CA_n2A-n261A/G CA_n5A-n261A/G CA_n48A-n261A/G |  | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n66A-n260A |  | CA_n2A-n260A CA_n5A-n260A CA_n66A-n260A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n66A-n260G |  | CA_n2A-n260A/G CA_n5A-n260A/G CA_n66A-n260A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n66A-n260H |  | CA_n2A-n260A/G/H CA_n5A-n260A/G/H CA_n66A-n260A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n66A-n260I |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n66A-n260J |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n66A-n260K |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n66A-n260L |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n66A-n260M |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n66A-n261A |  | CA_n2A-n261A CA_n5A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n66A-n261G |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n66A-n261H |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n66A-n261I |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n66A-n261J |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n66A-n261K |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n66A-n261L |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n66A-n261M |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n66A-n261(A-G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n66A-n261(2A) |  | CA_n2A-n261A CA_n5A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n5A-n66A-n261(3A) |  | CA_n2A-n261A CA_n5A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n5A-n66A-n261(2G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n66A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n66A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n66A-n261(2A-G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n5A-n66A-n261(2A-H) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n66A-n261(A-2G) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n66A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n66A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n66A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n66A-n261(2H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n66A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n66A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n66A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20 | 0 |
|  |  |  | n5 | 5, 10, 15, 20 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n77A-n260A |  | CA_n2A-n260A CA_n5A-n260A CA_n77A-n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n77A-n260G |  | CA_n2A-n260A/G CA_n5A-n260A/G CA_n77A-n260A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n5A-n77A-n260H |  | CA_n2A-n260A/G/H CA_n5A-n260A/G/H CA_n77A-n260A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n5A-n77A-n260I |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n5A-n77A-n260J |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n5A-n77A-n260K |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n5A-n77A-n260L |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n5A-n77A-n260M |  | CA_n2A-n260A/G/H/I CA_n5A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n5A-n77A-n261A |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n5A-n77A-n261G |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n5A-n77A-n261H |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n5A-n77A-n261I |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n5A-n77A-n261J |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n5A-n77A-n261K |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n5A-n77A-n261L |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n5A-n77A-n261M |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n5A-n77A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n5A-n77A-n261(2H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n5A-n77A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n5A-n77A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n5A-n77A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n5A-n77A-n261(A-G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n5A-n77A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n5A-n77A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n5A-n77A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n5A-n77A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n5A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n5A-n77A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n5A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n5A-n77A-n261(2A) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n5A-n77A-n261(3A) |  | CA_n2A-n261A CA_n5A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n5A-n77A-n261(2G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n5A-n77A-n261(A-2G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n5A-n77A-n261(2A-G) |  | CA_n2A-n261A/G CA_n5A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n5 | 5, 10, 15, 20, 25 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n48A-n66A-n260A |  | CA_n2A-n260A CA_n48A-n260A CA_n66A-n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n48A-n66A-n260G |  | CA_n2A-n260A/G CA_n48A-n260A/G CA_n66A-n260A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n48A-n66A-n260H |  | CA_n2A-n260A/G/H CA_n48A-n260A/G/H CA_n66A-n260A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n48A-n66A-n260I |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n48A-n66A-n260J |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n48A-n66A-n260K |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n48A-n66A-n260L |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n48A-n66A-n260M |  | CA_n2A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n48A-n66A-n261A |  | CA_n2A-n261A CA_n66A-n261A CA_n48A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n48A-n66A-n261G |  | CA_n2A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n261G |  |
| CA_n2A-n48A-n66A-n261H |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n261H |  |
| CA_n2A-n48A-n66A-n261I |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n48A-n66A-n261J |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n48A-n66A-n261K |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n48A-n66A-n261L |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n48A-n66A-n261M |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n48A-n66A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n48A-n66A-n261(2H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n48A-n66A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n48A-n66A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n48A-n66A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n48A-n66A-n261(A-G) |  | CA_n2A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n48A-n66A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n48A-n66A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n48A-n66A-n261(2A-G) |  | CA_n2A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n48A-n66A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n48A-n66A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n48A-n66A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n48A-n66A-n261(2A) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n48A-n66A-n261(3A) |  | CA_n2A-n261A CA_n48A-n261A CA_n66A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n48A-n66A-n261(2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n48A-n66A-n261(A-2G) |  | CA_n2A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n2A-n66A-n77A-n260A |  | CA_n2A-n260A CA_n66A-n260A CA_n77A-n260A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n77A-n260G |  | CA_n2A-n260A/G CA_n66A-n260A/G CA_n77A-n260A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n2A-n66A-n77A-n260H |  | CA_n2A-n260A/G/H CA_n66A-n260A/G/H CA_n77A-n260A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n2A-n66A-n77A-n260I |  | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n2A-n66A-n77A-n260J |  | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n2A-n66A-n77A-n260K |  | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n2A-n66A-n77A-n260L |  | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n2A-n66A-n77A-n260M |  | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n2A-n66A-n77A-n261A |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n2A-n66A-n77A-n261G |  | CA_n2A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n2A-n66A-n77A-n261H |  | CA_n2A-n260A/G/H CA_n66A-n260A/G/H CA_n77A-n260A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n2A-n66A-n77A-n261I |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n2A-n66A-n77A-n261J |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n2A-n66A-n77A-n261K |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n2A-n66A-n77A-n261L |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n2A-n66A-n77A-n261M |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n2A-n66A-n77A-n261(G-I) |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n2A-n66A-n77A-n261(2H) |  | CA_n2A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n2A-n66A-n77A-n261(A-G-H) |  | CA_n2A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n2A-n66A-n77A-n261(H-I) |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n2A-n66A-n77A-n261(A-G-I) |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n2A-n66A-n77A-n261(A-G) |  | CA_n2A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n2A-n66A-n77A-n261(A-H) |  | CA_n2A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n2A-n66A-n77A-n261(A-I) |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n2A-n66A-n77A-n261(2A-G) |  | CA_n2A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n2A-n66A-n77A-n261(2A-H) |  | CA_n2A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n2A-n66A-n77A-n261(2A-I) |  | CA_n2A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n2A-n66A-n77A-n261(G-H) |  | CA_n2A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n2A-n66A-n77A-n261(2A) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n2A-n66A-n77A-n261(3A) |  | CA_n2A-n261A CA_n66A-n261A CA_n77A-n261A | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n2A-n66A-n77A-n261(2G) |  | CA_n2A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n2A-n66A-n77A-n261(A-2G) |  | CA_n2A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G | n2 | 5, 10, 15, 20, 25, 30, 35 ,40 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |

Table 5.5A.1.3-1b

Table 5.5A.1.3-1b: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (four bands)

| NR CA configuration | Uplink configuration |  | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- |
| CA_n3A-n7A-n78A-n258A | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n7A-n78A-n258B | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258B |  |
| CA_n3A-n7A-n78A-n258C | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258C |  |
| CA_n3A-n7A-n78A-n258D | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258D |  |
| CA_n3A-n7A-n78A-n258E | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258E |  |
| CA_n3A-n7A-n78A-n258F | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258F |  |
| CA_n3A-n7A-n78A-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n78A-n258A/G CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258G |  |
| CA_n3A-n7A-n78A-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258H |  |
| CA_n3A-n7A-n78A-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258I |  |
| CA_n3A-n7A-n78A-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258J |  |
| CA_n3A-n7A-n78A-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258K |  |
| CA_n3A-n7A-n78A-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258L |  |
| CA_n3A-n7A-n78A-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | 5, 10, 15, 20, 25, 30, 40, 50 |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258M |  |
| CA_n3A-n7B-n78A-n258A | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | 50, 100, 200, 400 |  |
| CA_n3A-n7B-n78A-n258B | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258B |  |
| CA_n3A-n7B-n78A-n258C | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258C |  |
| CA_n3A-n7B-n78A-n258D | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258D |  |
| CA_n3A-n7B-n78A-n258E | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258E |  |
| CA_n3A-n7B-n78A-n258F | CA_n3A-n258A CA_n7A-n258A CA_n78A-n258A CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258F |  |
| CA_n3A-n7B-n78A-n258G | CA_n3A-n258A/G CA_n7A-n258A/G CA_n78A-n258A/G CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258G |  |
| CA_n3A-n7B-n78A-n258H | CA_n3A-n258A/G/H CA_n7A-n258A/G/H CA_n78A-n258A/G/H CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258H |  |
| CA_n3A-n7B-n78A-n258I | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258I |  |
| CA_n3A-n7B-n78A-n258J | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258J |  |
| CA_n3A-n7B-n78A-n258K | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258K |  |
| CA_n3A-n7B-n78A-n258L | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258L |  |
| CA_n3A-n7B-n78A-n258M | CA_n3A-n258A/G/H/I CA_n7A-n258A/G/H/I CA_n78A-n258A/G/H/I CA_n3A-n7A CA_n3A-n78A CA_n7A-n78A |  | n3 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  |  | n7 | CA_n7B |  |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n258 | CA_n258M |  |
| CA_n3A-n8A-n77A-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n8A-n77A-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n8A-n77A-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n8A-n77A-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n8A-n77A-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n3A-n8A-n77A-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n3A-n8A-n77A-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n3A-n8A-n77A-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n3A-n8A-n77(2A)-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n8A-n77(2A)-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n8A-n77(2A)-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n8A-n77(2A)-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n8A-n77(2A)-n257J | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257J |  |
| CA_n3A-n8A-n77(2A)-n257K | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257K |  |
| CA_n3A-n8A-n77(2A)-n257L | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257L |  |
| CA_n3A-n8A-n77(2A)-n257M | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n8 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257M |  |
| CA_n3A-n28A-n41A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n41A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n41A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n41A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A CA_n28A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77A-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G CA_n28A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n28A-n77A CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(2A)-n257A | CA_n3A-n257A CA_n28A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(2A)-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77(2A)-n257G | CA_n3A-n257A/G CA_n28A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(2A)-n257H | CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(2A)-n257I | CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(3A)-n257A | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(3A)-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n77(3A)-n257G | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(3A)-n257H | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(3A)-n257I | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n77 | CA_n77(3A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n78A-n257A | CA_n28A-n257A CA_n78A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n78A-n257D | - |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257D |  |
| CA_n3A-n28A-n78A-n257G | CA_n3A-n257A/G CA_n28A-n257A/G CA_n78A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n78A-n257H | CA_n3A-n257A/G/H CA_n28A-n257A/G/H CA_n78A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n78A-n257I | CA_n3A-n257A/G/H/I CA_n28A-n257A/G/H/I CA_n78A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n79A-n257A | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n79A-n257G | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n79A-n257H | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n79A CA_n28A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n79A-n257I | CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n28 | 5, 10, 15, 20 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77A-n257A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77A-n257G | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77A-n257H | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77A-n257I | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77(2A)-n257A |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77(2A)-n257G |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77(2A)-n257H |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77(2A)-n257I |  | CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n79A-n257A |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n79A-n257G |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n79A-n257H |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n79A-n257I |  | CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 10, 15, 20, 25, 30 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n77A-n79A-n257A | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n77A-n79A-n257G | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n77A-n79A-n257H | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n77A-n79A-n257I | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n3A-n77(2A)-n79A-n257A | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n77(2A)-n79A-n257G | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n3A-n77(2A)-n79A-n257H | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n3A-n77(2A)-n79A-n257I | CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n5A-n48A-n66A-n260A | CA_n5A-n260A CA_n48A-n260A CA_n66A-n260A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n260G | CA_n5A-n260A/G CA_n48A-n260A/G CA_n66A-n260A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n5A-n48A-n66A-n260H | CA_n5A-n260A/G/H CA_n48A-n260A/G/H CA_n66A-n260A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n5A-n48A-n66A-n260I | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n5A-n48A-n66A-n260J | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n5A-n48A-n66A-n260K | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n5A-n48A-n66A-n260L | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n5A-n48A-n66A-n260M | CA_n5A-n260A/G/H/I CA_n48A-n260A/G/H/I CA_n66A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n5A-n48A-n66A-n261A | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n5A-n48A-n66A-n261G | CA_n5A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n5A-n48A-n66A-n261H | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n5A-n48A-n66A-n261I | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n48A-n66A-n261J | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n48A-n66A-n261K | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n48A-n66A-n261L | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n48A-n66A-n261M | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n48A-n66A-n261(A-G) | CA_n5A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n5A-n48A-n66A-n261(A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n5A-n48A-n66A-n261(A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n5A-n48A-n66A-n261(2A-G) | CA_n5A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n5A-n48A-n66A-n261(2A-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n5A-n48A-n66A-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n5A-n48A-n66A-n261(G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n5A-n48A-n66A-n261(2A) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n5A-n48A-n66A-n261(3A) | CA_n5A-n261A CA_n48A-n261A CA_n66A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n5A-n48A-n66A-n261(2G) | CA_n5A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n5A-n48A-n66A-n261(A-2G) | CA_n5A-n261A/G CA_n48A-n261A/G CA_n66A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n5A-n48A-n66A-n261(G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n5A-n48A-n66A-n261(2H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n5A-n48A-n66A-n261(A-G-H) | CA_n5A-n261A/G/H CA_n48A-n261A/G/H CA_n66A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n5A-n48A-n66A-n261(H-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n5A-n48A-n66A-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n48A-n261A/G/H/I CA_n66A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n48 | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n5A-n66A-n77A-n260A | CA_n5A-n260A CA_n66A-n260A CA_n77A-n260A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | 50, 100, 200, 400 |  |
| CA_n5A-n66A-n77A-n260G | CA_n2A-n260A/G CA_n66A-n260A/G CA_n77A-n260A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260G |  |
| CA_n5A-n66A-n77A-n260H | CA_n2A-n260A/G/H CA_n66A-n260A/G/H CA_n77A-n260A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260H |  |
| CA_n5A-n66A-n77A-n260I | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260I |  |
| CA_n5A-n66A-n77A-n260J | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260J |  |
| CA_n5A-n66A-n77A-n260K | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260K |  |
| CA_n5A-n66A-n77A-n260L | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260L |  |
| CA_n5A-n66A-n77A-n260M | CA_n2A-n260A/G/H/I CA_n66A-n260A/G/H/I CA_n77A-n260A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n260 | CA_n260M |  |
| CA_n5A-n66A-n77A-n261A | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | 50, 100, 200, 400 |  |
| CA_n5A-n66A-n77A-n261G | CA_n5A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261G |  |
| CA_n5A-n66A-n77A-n261H | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261H |  |
| CA_n5A-n66A-n77A-n261I | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261I |  |
| CA_n5A-n66A-n77A-n261J | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261J |  |
| CA_n5A-n66A-n77A-n261K | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261K |  |
| CA_n5A-n66A-n77A-n261L | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261L |  |
| CA_n5A-n66A-n77A-n261M | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261M |  |
| CA_n5A-n66A-n77A-n261(G-H) | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-H) |  |
| CA_n5A-n66A-n77A-n261(2H) | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2H) |  |
| CA_n5A-n66A-n77A-n261(A-G-H) | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-H) |  |
| CA_n5A-n66A-n77A-n261(H-I) | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(H-I) |  |
| CA_n5A-n66A-n77A-n261(A-G-I) | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G-I) |  |
| CA_n5A-n66A-n77A-n261(A-G) | CA_n5A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-G) |  |
| CA_n5A-n66A-n77A-n261(A-H) | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-H) |  |
| CA_n5A-n66A-n77A-n261(A-I) | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-I) |  |
| CA_n5A-n66A-n77A-n261(A-2G) | CA_n5A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(A-2G) |  |
| CA_n5A-n66A-n77A-n261(2A-G) | CA_n5A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-G) |  |
| CA_n5A-n66A-n77A-n261(2A-H) | CA_n5A-n261A/G/H CA_n66A-n261A/G/H CA_n77A-n261A/G/H |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-H) |  |
| CA_n5A-n66A-n77A-n261(2A-I) | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A-I) |  |
| CA_n5A-n66A-n77A-n261(G-I) | CA_n5A-n261A/G/H/I CA_n66A-n261A/G/H/I CA_n77A-n261A/G/H/I |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(G-I) |  |
| CA_n5A-n66A-n77A-n261(2A) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2A) |  |
| CA_n5A-n66A-n77A-n261(3A) | CA_n5A-n261A CA_n66A-n261A CA_n77A-n261A |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(3A) |  |
| CA_n5A-n66A-n77A-n261(2G) | CA_n5A-n261A/G CA_n66A-n261A/G CA_n77A-n261A/G |  | n5 | 5, 10, 15, 20, 25 | 0 |
|  |  |  | n66 | 5, 10, 15, 20, 25, 30, 35, 40, 45 |  |
|  |  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  |  | n261 | CA_n261(2G) |  |
| CA_n28A-n41A-n77A-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77A-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77A-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77A-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n77(2A)-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77(2A)-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77(2A)-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77(2A)-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n79A-n257A |  | CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n79A-n257A | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n79A-n257G |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n79A-n257H |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n79A-n257I |  | CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n28 | 5, 10 | 0 |
|  |  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n77A-n79A-n257A | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n77A-n79A-n257G | CA_n257G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n77A-n79A-n257H | CA_n257G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n77A-n79A-n257I | CA_n257G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n77(2A)-n79A-n257A | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n77(2A)-n79A-n257G | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n77(2A)-n79A-n257H | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n77(2A)-n79A-n257I | CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15, 20 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n28A-n78A-n79A-n257A | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A CA_n78A-n79A CA_n78A-n257A CA_n79A-n257A |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 100, 200, 400 |  |
| CA_n28A-n78A-n79A-n257G | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G CA_n78A-n79A CA_n78A-n257A/G CA_n79A-n257A/G |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n28A-n78A-n79A-n257H | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n79A-n257A/G/H |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n28A-n78A-n79A-n257I | CA_n28A-n78A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n79A-n257A/G/H/I |  | n28 | 5, 10, 15 | 0 |
|  |  |  | n78 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n41A-n77A-n79A-n257A |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n77A-n79A-n257G |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n41A-n77A-n79A-n257H |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n41A-n77A-n79A-n257I |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n41A-n77(2A)-n79A-n257A |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
| CA_n41A-n77(2A)-n79A-n257G |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
| CA_n41A-n77(2A)-n79A-n257H |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
| CA_n41A-n77(2A)-n79A-n257I |  | CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n77 | CA_n77(2A) |  |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
| CA_n77A-n79A-n257A-n259A |  | CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A CA_n79A-n257A CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257A-n259G |  | CA_n259G CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G CA_n79A-n257A CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257A-n259H |  | CA_n259G/H CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H CA_n79A-n257A CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257A-n259I |  | CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257A-n259J |  | CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257A-n259K |  | CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257A-n259L |  | CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257A-n259M |  | CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257G-n259A |  | CA_n257G CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A CA_n79A-n257A/G CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257G-n259G |  | CA_n257G CA_n259G CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G CA_n79A-n257A/G CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257G-n259H |  | CA_n257G CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H CA_n79A-n257A/G CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257G-n259I |  | CA_n257G CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257G-n259J |  | CA_n257G CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257G-n259K |  | CA_n257G CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257G-n259L |  | CA_n257G CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257G-n259M |  | CA_n257G CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257H-n259A |  | CA_n257G/H CA_n77A-n79A CA_n77A-n257A CA_n77A-n257G/H CA_n77A-n259A CA_n79A-n257A CA_n79A-n257G/H CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257H-n259G |  | CA_n257G/H CA_n259G CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G CA_n79A-n257A/G/H CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257H-n259H |  | CA_n257G/H CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257H-n259I |  | CA_n257G/H CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257H-n259J |  | CA_n257G/H CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257H-n259K |  | CA_n257G/H CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257H-n259L |  | CA_n257G/H CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257H-n259M |  | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G/H CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n77A-n79A-n257I-n259A |  | CA_n257G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A CA_n79A-n257A/G/H/I CA_n79A-n259A | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n77A-n79A-n257I-n259G |  | CA_n257G/H/I CA_n259G CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G CA_n79A-n257A/G/H/I CA_n79A-n259A/G | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n77A-n79A-n257I-n259H |  | CA_n257G/H/I CA_n259G/H CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n77A-n79A-n257I-n259I |  | CA_n257G/H/I CA_n259G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n77A-n79A-n257I-n259J |  | CA_n257G/H/I CA_n259G/H/I/J CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n77A-n79A-n257I-n259K |  | CA_n257G/H/I CA_n259G/H/I/J/K CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n77A-n79A-n257I-n259L |  | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n77A-n79A-n257I-n259M |  | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n77A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257A-n259A |  | CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A CA_n79A-n257A CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257A-n259G |  | CA_n259G CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G CA_n79A-n257A CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257A-n259H |  | CA_n259G/H CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H CA_n79A-n257A CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257A-n259I |  | CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I CA_n79A-n257A CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257A-n259J |  | CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J CA_n79A-n257A CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257A-n259K |  | CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257A-n259L |  | CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257A-n259M |  | CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | 50, 100, 200, 400 |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257G-n259A |  | CA_n257G CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A CA_n79A-n257A/G CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257G-n259G |  | CA_n257G CA_n259G CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G CA_n79A-n257A/G CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257G-n259H |  | CA_n257G CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H CA_n79A-n257A/G CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257G-n259I |  | CA_n257G CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I CA_n79A-n257A/G CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257G-n259J |  | CA_n257G CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257G-n259K |  | CA_n257G CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257G-n259L |  | CA_n257G CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257G-n259M |  | CA_n257G CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257G |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257H-n259A |  | CA_n257G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A CA_n79A-n257A/G/H CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257H-n259G |  | CA_n257G/H CA_n259G CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G CA_n79A-n257A/G/H CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257H-n259H |  | CA_n257G/H CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H CA_n79A-n257A/G/H CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257H-n259I |  | CA_n257G/H CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257H-n259J |  | CA_n257G/H CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257H-n259K |  | CA_n257G/H CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257H-n259L |  | CA_n257G/H CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257H-n259M |  | CA_n257G/H CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G/H CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257H |  |
|  |  |  | n259 | CA_n259M |  |
| CA_n78A-n79A-n257I-n259A |  | CA_n257G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A CA_n79A-n257A/G/H/I CA_n79A-n259A | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | 50, 100, 200, 400 |  |
| CA_n78A-n79A-n257I-n259G |  | CA_n257G/H/I CA_n259G CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G CA_n79A-n257A/G/H/I CA_n79A-n259A/G | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259G |  |
| CA_n78A-n79A-n257I-n259H |  | CA_n257G/H/I CA_n259G/H CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259H |  |
| CA_n78A-n79A-n257I-n259I |  | CA_n257G/H/I CA_n259G/H/I CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259I |  |
| CA_n78A-n79A-n257I-n259J |  | CA_n257G/H/I CA_n259G/H/I/J CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259J |  |
| CA_n78A-n79A-n257I-n259K |  | CA_n257G/H/I CA_n259G/H/I/J/K CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259K |  |
| CA_n78A-n79A-n257I-n259L |  | CA_n257G/H/I CA_n259G/H/I/J/K/L CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259L |  |
| CA_n78A-n79A-n257I-n259M |  | CA_n257G/H/I CA_n259G/H/I/J/K/L/M CA_n78A-n79A CA_n78A-n257A/G/H/I CA_n78A-n259A/G/H/I/J/K/L/M CA_n79A-n257A/G/H/I CA_n79A-n259A/G/H/I/J/K/L/M | n78 | 10, 15, 20, 40, 50, 60, 80, 90, 100 | 0 |
|  |  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  |  | n257 | CA_n257I |  |
|  |  |  | n259 | CA_n259M |  |

The following notes are applied to the above tables.

NOTE 1: 	The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively.

NOTE 2:	The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to.

NOTE 3: 	The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively.

5.5A.1.4	Inter-band CA configurations between FR1 and FR2 (five bands)

Table 5.5A.1.4-1: Inter-band CA configurations and bandwidth combination sets between FR1 and FR2 (five bands)

| NR CA configuration | Uplink configuration | NR Band | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n1A-n3A-n8A-n77A-n257A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n77A-n257G | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n77A-n257H | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n77A-n257I | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n77A-n257J | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n77A-n257K | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n77A-n257L | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n77A-n257M | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n8A-n77(2A)-n257A | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n1A-n3A-n8A-n77(2A)-n257G | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257G |  |
| CA_n1A-n3A-n8A-n77(2A)-n257H | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257H |  |
| CA_n1A-n3A-n8A-n77(2A)-n257I | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257I |  |
| CA_n1A-n3A-n8A-n77(2A)-n257J | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257J |  |
| CA_n1A-n3A-n8A-n77(2A)-n257K | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257K |  |
| CA_n1A-n3A-n8A-n77(2A)-n257L | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257L |  |
| CA_n1A-n3A-n8A-n77(2A)-n257M | - | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n8 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n257 | CA_n257M |  |
| CA_n1A-n3A-n28A-n41A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A CA_n28A-n41A CA_n28A-n257A CA_n41A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n41A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n257A/G CA_n41A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n41A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n257A/G/H CA_n41A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n41A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n41A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n41A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n257A/G/H/I CA_n41A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n28A-n77A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A CA_n28A-n77A CA_n28A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n77A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n77A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n77A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n28A-n79A-n257A | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n28A-n79A-n257G | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n79A CA_n28A-n257A/G CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n28A-n79A-n257H | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n79A CA_n28A-n257A CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n28A-n79A-n257I | CA_n1A-n3A CA_n1A-n28A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n28A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n41A-n77A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n41A-n77A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n41A-n77A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n41A-n77A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n41A-n79A-n257A | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n41A-n79A-n257G | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n41A-n79A-n257H | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n41A-n79A-n257I | CA_n1A-n3A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n3A-n77A-n79A-n257A | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n3A-n77A-n79A-n257G | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n3A-n77A-n79A-n257H | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n3A-n77A-n79A-n257I | CA_n1A-n3A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |  |
|  |  | n77 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n41A-n77A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n41A-n77A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n41A-n77A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n41A-n77A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n77A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n41A-n79A-n257A | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n41A-n79A-n257G | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n41A-n79A-n257H | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n41A-n79A-n257I | CA_n1A-n28A CA_n1A-n41A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n28A-n77A-n79A-n257A | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n28A-n77A-n79A-n257G | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n28A-n77A-n79A-n257H | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n28A-n77A-n79A-n257I | CA_n1A-n28A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n28 | 5, 10, 15, 20 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n1A-n41A-n77A-n79A-n257A | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n1A-n41A-n77A-n79A-n257G | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n1A-n41A-n77A-n79A-n257H | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n1A-n41A-n77A-n79A-n257I | CA_n1A-n41A CA_n1A-n77A CA_n1A-n79A CA_n1A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n1 | 5, 10, 15, 20 | 0 |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n41A-n77A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A CA_n41A-n77A CA_n41A-n257A CA_n77A-n257A | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | 50, 100, 200, 400 |  |  |
| CA_n3A-n28A-n41A-n77A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n257A/G CA_n77A-n257A/G | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257G |  |  |
| CA_n3A-n28A-n41A-n77A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n257A/G/H CA_n77A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257H |  |  |
| CA_n3A-n28A-n41A-n77A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n77A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n77A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n257A/G/H/I CA_n77A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30, 40 | 0 |  |
|  |  | n28 | 5, 10, 15, 20, 30 |  |  |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |  |
|  |  | n257 | CA_n257I |  |  |
| CA_n3A-n28A-n41A-n79A-n257A | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A CA_n41A-n79A CA_n41A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n41A-n79A-n257G | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n79A CA_n41A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n41A-n79A-n257H | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n79A CA_n41A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n41A-n79A-n257I | CA_n3A-n28A CA_n3A-n41A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n41A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80,100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77A-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77A-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77A-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77A-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n28A-n77(2A)-n79A-n257A | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n28A-n77(2A)-n79A-n257G | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n28A-n77(2A)-n79A-n257H | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n28A-n77(2A)-n79A-n257I | CA_n3A-n28A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n77 | CA_n77(2A)_BCS0 |  |
|  |  | n79 | 40, 50, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n3A-n41A-n77A-n79A-n257A | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n3A-n41A-n77A-n79A-n257G | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n3A-n41A-n77A-n79A-n257H | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n3A-n41A-n77A-n79A-n257I | CA_n3A-n41A CA_n3A-n77A CA_n3A-n79A CA_n3A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| CA_n28A-n41A-n77A-n79A-n257A | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A CA_n77A-n79A CA_n77A-n257A CA_n79A-n257A | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | 50, 100, 200, 400 |  |
| CA_n28A-n41A-n77A-n79A-n257G | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G CA_n77A-n79A CA_n77A-n257A/G CA_n79A-n257A/G | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257G |  |
| CA_n28A-n41A-n77A-n79A-n257H | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H CA_n77A-n79A CA_n77A-n257A/G/H CA_n79A-n257A/G/H | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257H |  |
| CA_n28A-n41A-n77A-n79A-n257I | CA_n28A-n41A CA_n28A-n77A CA_n28A-n79A CA_n28A-n257A/G/H/I CA_n41A-n77A CA_n41A-n79A CA_n41A-n257A/G/H/I CA_n77A-n79A CA_n77A-n257A/G/H/I CA_n79A-n257A/G/H/I | n28 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n77 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n79 | 40, 50, 60, 80, 100 |  |
|  |  | n257 | CA_n257I |  |
| NOTE 1: The SCS of each channel bandwidth for NR FR1 and NR FR2 band refers to Table 5.3.5-1 of TS 38.101-1 and TS 38.101-2 respectively. NOTE 2: The CA configurations are given in Table 5.5A.1-1 of either TS 38.101-1 or TS 38.101-2 where unless otherwise stated BCS0 is referred to. NOTE 3: The delimiter “/” is only used in the uplink configurations for the sake of simplicity. For example, CA_nxA-nyA/B/C denotes CA_nxA-nyA, CA_nxA-nyB and CA_nxA-nyC, where nx and ny are two NR bands, ny is a FR2 band and A, B and C are the corresponding bandwidth classes respectively. |  |  |  |  |

<< End of changes >>