zz3GPP TSG-RAN WG4 Meeting #110												R4-2403877

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:			6.4

Source:	Moderator (Apple)

Title:	Ad hoc minutes on NR_2Rx_XR

Document for:	Information

# Introduction

The discussion is captured under each topic for NR_2Rx_XR.

# Topic #1:	2Rx non-REDCAP XR devices

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2400157 | Apple | Observation 1: Considering the extensive history of assumptions on NF and IM made by RAN4 when deriving REFSENS and some preliminary RF front end considerations of the new XR wearable form factor, there does not seem to be a strong technical justification to tighten the 2Rx REFSENS value for 2Rx wearable devices. Observation 2: Based on further guidance by RAN, it is feasible for RAN4 to begin the effort to define the test methodology aspects for the radiated testing of XR wearable devices. Because RAN4 develops OTA performance requirements based on measured data of commercially available devices, this effort should be postponed until a later release. To alleviate operator concerns with the coverage of 2Rx XR wearable devices, RAN4 should discuss whether a single OTA requirement for XR wearables can be defined, regardless of the number of Rx antennas used in the device. Proposal 1: A CR to 38.101-1 is needed to include the definition of the “2Rx non-REDCAP XR” UE and to update NOTE 1 to Tables 7.3.2-1a and 7.3.2-1b should be updated to include applicability to the “2Rx non-REDCAP XR.” Proposal 2: Based on the OTA considerations provided, RAN4 can initiate the work to define test methodology aspects for 2Rx non-REDCAP XR UEs in Rel-19. The effort to specify OTA performance requirements should be postponed until a later release. To accommodate operator concerns with the coverage of 2Rx, it is recommended to include the following objective to the future OTA performance requirements WID: “RAN4 should discuss whether a single OTA requirement for XR wearables can be defined, regardless of the number of Rx antennas used in the device.” |
| R4-2400552 | Meta Ireland | Proposal 1: In TS38.101-1, RAN4 defines 2Rx non-RedCap XR devices, following RP-232657, as: 2Rx Non-RedCap XR UE: A UE that is not (e)RedCap and supports only two Rx antennas in frequency bands where 4Rx is mandated. The XR device is intended to be worn on the human head. When in use, it is intended to be supported only by or behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains. Proposal 2: RAN4 reuses the existing REFSENS requirement for 2Rx non-RedCap XR UEs. Proposal 3: RAN4 update Note 1 in Table 7.3.2-1a and Table 7.3.2-1b of in TS38.101-1 to allow 2Rx relaxation and apply the existing REFSENS for XR device. |
| R4-2400553 | Meta Ireland | Proposal 1: The several key advantages of antenna design and properties of the XR glasses compared to smartphone antennas provide natural mitigation to performance impact of 2Rx vs 4Rx. The performance of XR glasses can be expected to be on-par or outperform compared with a pair of antennas of 4Rx smartphones. The antenna advantages of XR glasses include lower antenna correlation, less uplink SAR backoff, and no impact from antenna loss due to hand-held usage. Proposal 2: In order to overcome the challenges of OTA tests, RAN4 develops and utilises a theoretical model to evaluate the performance of XR devices for OTA considerations. |
| R4-2400555 | Meta Ireland | CR TS38.101-1 on XR device definition and REFSENS requirements for 2Rx non-Redcap XR device [2Rx_XR_UE] |
| R4-2400620 | Nokia, | CR 38.101-1 addition of 2Rx XR exception for REFSENS |
| R4-2400707 | Qualcomm Incorporated | Observation 1: The scope of the feasibility discussion for the conducted REFSENS tightening is limited to bands n7,n38, n41, n48, n77, n78, n79 and n104 for XR device which implements two RX ports for these bands Observation 2: Receiver noise figure can be improved by using more expensive solutions or more power or area. Observation 3: XR devices have some characteristics that have potential for better radiated performance than handheld devices How to close the RAN task and draft CRs for RAN#103, will need discussion. There maybe a need for WF for OTA part that may include some draft for new WI objectives for XR device radiated performance. For conducted part, to assist proper transparency of the agreements, we provided a proposal: Proposal: RAN4 shall not conclude on conducted refsens tightening without concluding proper specification language |
| R4-2400822 | CMCC | Observation 1: Compared with 4Rx for handheld UE, 2Rx for non-REDCAP XR devices naturally have advantages of antenna performance, e.g. better antenna correlation and antenna efficiency. Observation 2: for dense urban scenario, UL coverage is worse than DL but the coverage difference is little. DL coverage is also very essential. Proposal 1: for 2Rx non-REDCAP XR device, it’s better to define better REFSENSE requirements than that of 2Rx handheld UE. Proposal 2: it’s better to reuse the same OTA requirements as handheld UE for 2Rx non-REDCAP XR device. |
| R4-2401526 | vivo | How to reflect the new definition and corresponding requirements into 38.101-1 Proposal 1: Consider the following definition and the note in 2Rx REFSENS and exception in 38.101-1. Wearable 2Rx UE: a UE intended to be worn on the human head, and intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains. NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE and wearable 2Rx UE. Four Rx antenna ports for RedCap UE is not supported for this operating band. NOTE 11: For wearable 2Rx UE, -TBD dB is added to the REFSENS. The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104 where the UE is required to be equipped with a minimum of four Rx antenna ports except for wearable 2Rx UE. What should be the conduct requirements value should be Proposal 2: Consider a value between 0 and 1 dB for the tightening, since the implementation margin is limited and UL performance is bottleneck. How to consider OTA requirements Observation 1: The Free-space test method defined in TRP TRS WI is general for different UE types, which can cover XR free space testing. However, the phantom-based XR test method is not specified yet. Observation 2: RAN4 is working on the TRP TRS requirements for smartphone with phantom-based usage scenario. Proposal 3: RAN4 should develop phantom-based XR test method. Proposal 4: To study the OTA performance gap between XR and smartphone, the corresponding work should be well organized and proceed, which can be considered as part of Rel-19 scope in TRP TRS WI. |
| R4-2401527 | vivo | draft CR to 38.101-1 on Addition of requirements for non-Redcap XR |
| R4-2401796 | OPPO | Observation 1: To reduce the DL performance impacts, the improvement of conducted REFSENS and OTA requirements can be considered. Observation 2: There is no much difference between XR device and other UE types in RFFE which makes no much improvement can be done for conducted REFSENS. In contrast, the OTA requirements probably can be improved by proper antenna design and the missing of hand blocking effects in XR devices. Proposal 1: Keep the conducted REFSENS with 2Rx unchanged in bands where 4Rx is mandatory. Study the improvement of OTA requirements for XR devices in future releases. 3.1 Definitions For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. [Unchanged sections omitted] Vehicular UE: A UE embedded in a vehicle, permanently connected to an embedded antenna system that radiates externally for NR operating bands. NOTE: Vehicular UE does not refer to other UE form factors placed inside the vehicle. Wideband operation: For a UE that supports shared spectrum channel access, wideband operation refers to operation within a channel larger than 20 MHz in which intra-cell guard bands may be configured to distinguish individual RB-sets 2Rx non-REDCAP XR UE: A UE supporting XR feature and is intended to be worn on the human head and when in use is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.. For the REFSENS requirement definition, the changes are as below. 7.2 Diversity characteristics The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n48, n77, n78, n79, n104. For the bands n7, n38, n41, n48, n77, n78, n79 and n104, UEs other than “2Rx non-REDCAP XR UE” are required to be equipped with a minimum of four Rx antenna ports, and for “2Rx non-REDCAP XR UE”, it can be equipped with two antenna ports. This requirement applies when the band is used as a standalone band or as part of a band combination.. Proposal 2: The changes as above are proposed. |
| R4-2402421 | Huawei, HiSilicon, Telecom Italia, Telia Company, T-Mobile USA, Telefonica, BT plc, CMCC, Orange, Spark (NZ) | Proposal 1: It is proposed to define the REFSENS of 2Rx non-RedCap XR in a relative manner similar to that for 4Rx, i.e. introduce ΔRXR,2R and add it on top of 2Rx REFSENS for handheld UE. Proposal 2: The introduced ΔRXR,2R should be distinguished with different values for the bands which are mandatorily supporting 4Rx. Observation 1: OTA TRS requirement is specified in RAN4 band by band with relatively high thresholds that number of commercially available devices should be sufficient with different models from different vendors. Observation 2: For the bands which are mandatorily supporting 4Rx, RAN4 only develop the OTA requirements based on measurement of 4Rx devices. Observation 3: Mass commercial 2R/4R XR devices are not available yet to comply with the measurement campaign required by the RAN4 procedure for specifying the OTA requirements Proposal 3: It is proposed to specify one set of OTA TRS requirement for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx Proposal 4: It is proposed to adopt the existing 4Rx TRS requirement, specified for handheld UE, for 2Rx non-RedCap XR device |
| R4-2402422 | Huawei, HiSilicon, Telecom Italia, Telia Company, T-Mobile USA, Telefonica, BT plc, CMCC, Orange, Spark (NZ) | CR on Introduction of 2Rx relaxation for XR devices |
| R4-2402452 | T-Mobile USA, Deutsche Telekom, Vodafone, AT&T, Orange, Spark, Telstra, TELUS | Observation 1: All the 7 devices studied have at least one antenna port that can meet the 4Rx REFSENS requirement with a single antenna port for n41. Observation 2: All the antenna ports in the 7 devices studied when adjusted from 1Rx to 2Rx can meet the 4Rx REFSENS requirement for n41. Observation 3: All but one of the 28 antenna ports studied, when adjusted from 1Rx to 2Rx have at least 1 dB of margin relative to the 4Rx REFSENS requirement for n41. Observation 4: For the two devices studied, all antenna ports can meet the 4Rx REFSENS requirement with a single antenna port for n77. Proposal: XR UEs that utilize the 2Rx antenna allowance shall meet the 4Rx conducted REFSENS requirements for bands where 4Rx is mandatory. |
| R4-2402609 | MediaTek Inc. | Proposal 1: Capture the definition of 2Rx non-REDCAP XR Wearable UE into TS 38.101-1 as the following: “A 2Rx non-REDCAP XR Wearable UE refers to a non-handheld device intended to be worn on the human head, supported only by/behind the ears and by a nose-bridge and, due to constrained form factor/volume, equipped with only two Rx antenna ports in all supported FR1 frequency bands.” Proposal 2: Clause 7 requirements for four Rx antenna ports do not apply to a 2Rx non-REDCAP XR Wearable UE. Proposal 3: Agree not to improve conducted Rx sensitivity requirements for 2Rx non-REDCAP XR Wearable UE compared with existing nominal 2Rx assumption. |
| R4-2402635 | MediaTek Inc. | CR to TS 38.101-1 for 2Rx non-REDCAP XR devices |

The moderator can suggest a limited number of papers which could be presented.

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions..

### Sub-topic 1-1: Definition of 2RX XR devices

- Proposals

  - Option 1: 2Rx non-Redcap XR UE: A UE that is not (e)RedCap and supports only two Rx antennas in frequency bands where 4Rx is mandated. The XR device is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

  - Option 2: Two antenna port XR UEs: Intended to be worn on the human head.	When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

  - Option 3: Wearable 2Rx UE: a UE intended to be worn on the human head, and intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

  - Option 4: 2Rx non-REDCAP XR UE: A UE supporting XR feature and is intended to be worn on the human head and when in use is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

  - Option 5: 2Rx non-RedCap XR UE: A non-RedCap XR-wearable UE with only 2 Rx branches in the bands where 4Rx is specified as mandatory. A non-RedCap XR-wearable UE is intended to be worn on the human head; when in use, the UE is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

  - Option 6: 2Rx non-REDCAP XR Wearable UE:  A non-handheld device intended to be worn on the human head, supported only by/behind the ears and by a nose-bridge and, due to constrained form factor/volume, equipped with only two Rx antenna ports in all supported FR1 frequency bands.

- Recommended WF

  - Two Rx antenna port XR UE: A non-(e)RedCap XR UE that is equipped with only two Rx  antenna ports in frequency band(s) where 4 Rx antenna ports are required. The UE is intended to be worn on human head. When in use, is intended to be supported only by/behind the ears and by a nose-bridge resulting in a constrained form factor with limited volume available for Rx chains.

### Sub-topic 1-2: Definition of XR

- Proposals

  - Option 1: XR (eXtended Reality): XR is a term for different types of realities and refers to all real-and-virtual combined environments and human-machine interactions generated by computer technology and wearables. It included following representative forms and the areas interpolated among them AR, MR and VR.

  - Option 2: XR:	Extended Reality

  - Option 3: XR		eXtended Reality

- Recommended WF

  - XR		eXtended Reality

### Sub-topic 1-3: Conducted receiver sensitivity tightening

- Proposals

  - Option 1: No tightening

  - Option 2: a value between 0 and 1 dB for the tightening

  - Option 3:

Table 1: Two antenna port reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41 | -1.5 |
| n48, n77, n78, n79, n104 | -1.0 |

  - Option 4: XR UEs that utilize the 2Rx antenna allowance shall meet the 4Rx conducted REFSENS requirements for bands where 4Rx is mandatory.

Table 27.3.2-2: Two antenna port reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41 | -2.7 |
| n48, n77, n78, n79, n104 | -2.2 |

  - Option 5:

- Table 7.3.2-3: Two antenna port XR UE reference sensitivity allowance ΔRXR,2R

| Operating band | ΔRXR,2R (dB) |
| --- | --- |
| n7, n38, n41, n48, n77, n78, n79, n104 | TBD |

Discussion:

MTK: for any tightening, clarify what is front-end loss assumed?

AT&T: to seek an agreement the sensitivity tightening, need to understand the RAN4 WF on OTA performance.

Huawei: we understand how sensitivity is defined, and there is some implementation margin that can be considered for tightening. 1-1.5dB is a middle ground for compromise.

Meta: sensitivity has noise figure and implementation margin. There is no big difference between handheld UEs and XR UEs. Sensitivity should be decoupled from OTA performance. XR devices can have better OTA performance because of low antenna correlation, lack of body loss, etc. Need to focus sensitivity, and then discuss OTA.

Ericsson: in the network, it is the OTA performance that counts.

T-Mobile USA: we co-signed the Huawei CR and also submitted our own contribution. We got our own data very late. The numbers we got exceeded specification requirements. There is room for tightening.

TIM: OTA is important.

Huawei: agree OTA performance is important. Considering the current progress of OTA requirement, it takes time.

Apple: we understand OTA is defined with the RAN4 process. RAN asks RAN4 to consider the feasibility of tightening sensitivity.

QC: agree with Meta that we need to look at how the sensitivity can be tightened. Need to be careful about how to interpret the measured data.

Nokia: Defining OTA performance is hard.

Samsung: to define OTA requirement, we need to use the commercial devices to conduct measurements. Better to follow RAN decision to separate the conducted sensitivity and OTA discussion.

CMCC: to allow 2RX XR in the network, need to have some baseline assumption on the OTA performance.

Apple: the RAN WF is to focus on sensitivity and consider OTA.

DT: XR devices should have better OTA performances.

QC: we would like to look at values between Option 3 and Option 1.

Vodafone: we can look at values between Option 3 and Option 4.

Possible options for further discussion this week:

- Option A: Consider Option 3 for receiver sensitivity. Meanwhile, think about what can be agreed for OTA aspects.

- Option B: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance:

  - Proposal 3: It is proposed to specify one set of OTA TRS requirement per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4RX XR only

- Option C: Consider Option 2 for receiver sensitivity. Meanwhile, think about what can be agreed for OTA aspects.

- Any other options

- Recommended WF

  - TBD

T-Mobile USA: We upload the revision of our document. Looking at the performance of number of devices, at least one of them meet the 4Rx requirements with 1Rx. We compared the 1Rx requirement and 2Rx requirement. 2Rx requirements can be met by using 1Rx. 0.4dB as margin. It is entirely reasonable for 4Rx UE to meet 2Rx requirements. TIM proposal in RAN2 sounds like the reasonable proposal.

AT&T: For option B, it is the compromise proposal. By combining with OTA, it is RAN4 understanding how to address it by using OTA testing.

Mediatek: to T-Mobile USA, on measurement results, first issue is that there is only limited number of devices. Large margin does not mean that we should have tightened requirements. Secondly, there is one crucial part what the reasonable front loss is.

OPPO: The REFSENS is defined in RAN4 for link budget. If IM is tightened, it is put burden for UE.

CMCC: Companies have different logics behind the proposal. From our side, the existing requirement can be tightened. Mediatk and OPPO just compare with existing requirements but not consider the performance of new device. We can accept B as compromise.

Samsung: We have two aspects: conductive to further improve the performance and OTA to check the feasibility. For OTA part, it is far way to define the requirement at this stage. For OTA, it is based on measurement campaign. If following proposal 3, How can 2Rx XR UE pass the requirements based on the campaign with 4Rx UE measurement. Proposal 3 contradicts with RAN decision.

AT&T: it does not conflict with RAN decision at all. Proposal 3 is the way forward that RAN4 should do.

Telecom Italia: We have the same feeling as AT&T. In proposal B, we ask including proposal 4. That is more complete.

Meta: Generally this measurement campaign should be based on 2Rx and 4Rx devices. It is not reasonable to just base on 2Rx. We should consider both. For REFENS, it should be based on link budget. Measurement data can be achieved by handheld device. But here we are talking about the XR device, which are different.

Huawei: based on the comment, we have similar understanding as RAN task. For the conductive requirements, the specific value can be considered. For OTA, the proposal here is just the way forward, which should be based on the commercial devices. We just leave the requirements in the future. Here we just have guidance for future. We did not ask for using 4Rx data for develop the 2Rx XR requirements.

Telecom Italia: to Meta, we need the test for new devices.

T-Mobile USA: about the form factor, it is part that operators have concern on. We will loss the coverage and loss 5,6,7dB of performance.

Samsung: We need compromise to ensure performance but not guidance to kill the future. If possible, for proposal 3, we try to define UE side OTA covering both 2Rx and 4Rx including XR.

Moderator: regarding understanding of RAN task, RAN4 should consider conductive test and also OTA aspects. We are not going to define the OTA requirements. We should have measurement campaign. Secondly, we properly not overestimate the data. The data is good but data have some limitation due to small pool. Thirdly, looking option 2 proposal 4, it is not fair to specify the requirements based on one 2Rx device out of four devices.

AT&T: the measurement campaign needs minimal set of requirements. I do not see the issue with one device.

Telecom Italia: do the OTA requirements based on 4Rx UE and then consider delta.

DT: can we request 2Rx XR device have the same performance as 4Rx handheld devices? We need security.

Vivo: t

Need further discussion:

- Option D: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance:

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.

- Option E: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance:

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx handheld UE. Further evaluation based on the new measurement campaign and new form factor for 2Rx.

- Option F: Consider both Option 2 for receiver sensitivity and the following proposal for OTA performance:

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.

- Option G: 2Rx XR UE meets the 4Rx handheld UE conductive REFSENS and OTA requirements, and the default would be not to bar 2Rx XR.

### Sub-topic 1-4: OTA aspects

- Proposals

  - Option 1: Based on the OTA considerations provided, RAN4 can initiate the work to define test methodology aspects for 2Rx non-REDCAP XR UEs in Rel-19.  The effort to specify OTA performance requirements should be postponed until a later release.  To accommodate operator concerns with the coverage of 2Rx, it is recommended to include the following objective to the future OTA performance requirements WID:  “RAN4 should discuss whether a single OTA requirement for XR wearables can be defined, regardless of the number of Rx antennas used in the device.”

  - Option 2: In order to overcome the challenges of OTA tests, RAN4 develops and utilises a theoretical model to evaluate the performance of XR devices for OTA considerations.

  - Option 3: WF for OTA part that may include some draft for new WI objectives for XR device radiated performance.

  - Option 4: it’s better to reuse the same OTA requirements as handheld UE for 2Rx non-REDCAP XR device.

  - Option 5: RAN4 should develop phantom-based XR test method. To study the OTA performance gap between XR and smartphone, the corresponding work should be well organized and proceed, which can be considered as part of Rel-19 scope in TRP TRS WI.

  - Option 6: It is proposed to specify one set of OTA TRS requirement for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx. It is proposed to adopt the existing 4Rx TRS requirement, specified for handheld UE, for 2Rx non-RedCap XR device

- Recommended WF

  - TBD

Combined options:

- Option A: Consider Option 3 for receiver sensitivity. Meanwhile, think about what can be agreed for OTA aspects.

- Option B: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance:

  - Proposal 3: It is proposed to specify one set of OTA TRS requirement per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4RX XR only

- Option C: Consider Option 2 for receiver sensitivity. Meanwhile, think about what can be agreed for OTA aspects.

- Option D: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance: (AT&T, BT

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.

- Option E: Consider both Option 3 for receiver sensitivity and the following proposal for OTA performance: (TIM, CMCC

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx handheld UE. Further evaluation based on the new measurement campaign and new form factor for 2Rx.

- Option F: Consider [0.5dB] for conducted receiver sensitivity tightening and the following proposal for OTA performance: (QC, MTK, Meta, Apple

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.

- Option G: 2Rx XR UE meets the 4Rx handheld UE conductive REFSENS and 4Rx handheld OTA requirements, and the default would be not to bar 2Rx XR. (TIM, Vodafone, T-mobile USA, Orange, CMCC, AT&T, BT,

  - Note the testing aspects, such as the use of phantom, are different for 4Rx handheld UEs and 2Rx XR UE. For 4Rx handheld UEs, there are talking mode and browsing mode, and different hand phantoms.

- Option G’: 2Rx XR UE meets the 4Rx handheld UE conductive REFSENS and the following proposal for OTA performance. The default would be to bar 2Rx XR

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.

Recommended WF:

- On conducted receiver sensitivity, consider two options

  - 0.5dB tightening compared to the existing 2Rx UE conducted REFSENS, which is considered feasible by some UE vendors

  - 2Rx XR UE meets the 4Rx handheld UE conducted REFSENS

- On OTA performance:

  - It is proposed to specify OTA TRS requirements per band for both 4Rx XR and 2Rx XR for the NR bands which are mandatorily to support 4Rx based on measurement campaign of 4Rx XR, considering the performance degradation value for 2Rx based on 4Rx measurement campaign.