3GPP TSG-RAN WG4 Meeting #110-bis	R4-2404127

Changsha, China, April 15-19, 2024

Source:	Nokia

Title: 	Supporting Simulation results for PDSCH demodulation for 8Rx

Agenda item: 	6.1.3.1.1

Document for: 	Discussion

Introduction

In this contribution, we provide the simulation results for 8 Rx PDSCH CA UE demodulation requirements.

PDSCH simulation results

Test parameters

We provide in this section the parameters used for PDSCH TDD simulation which are characterized in Table 1.

Table 1: Simulation assumption for PDSCH (TDD)

| Simulation parameters |  |  |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD pattern |  | 7D1S2U |
|  |  | S=6D:4G:4U (Not schedule PDSCH in special slot for 8 layers cases) |
| SCS(kHz)/Bandwidth (MHz) |  | 30/40 |
| PDSCH configuration | Mapping type | Type A and B |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% |
|  |  | 256QAM:3% |
| Test metric |  | • Only 70% of maximum TP |
|  |  | Rank 2, 4 and 8 |
| Antenna configuration |  | • Rank 2: 2T8R |
|  |  | • Rank 4: 4T8R |
|  |  | • Rank 8: 8T8R |
| Antenna correlation |  | • Rank 2: ULA Medium B |
|  |  | • Rank 4 and 8: ULA Low |
| Propagation conditions |  | • Rank 2: TDLC300-100 |
|  |  | • Rank 4 and 8: TDLA30-10 |
| MCS |  | Rank 2: MCS19 (Table 1), MCS 20 (Table 2) Rank 4: MCS17, MCS26 Rank 8: MCS13, MCS 17 |
| Overhead |  | Overhead = 0 |
| TRS configuration |  | • Configure TRS symbols in symbol#{5, 9} |
|  |  | • NZP CSI-RS configuration of row 6 for 8 layers. |
| N1 and N2 configuration for 8Tx case |  | (N1,N2)=(4,1) |

Furthermore, we provide in this section the parameters used for PDSCH FDD simulation which are characterized in Table 2

Table 2: Simulation assumption for PDSCH (FDD)

| Simulation parameters |  |  |
| --- | --- | --- |
| Duplex mode |  | FDD |
| SCS(kHz)/Bandwidth (MHz) |  | 15/10 |
| PDSCH configuration | Mapping type | Type A and B |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM:6% |
|  |  | 256QAM:3% |
| Test metric |  | • Only 70% of maximum TP |
|  |  | Rank 2, 4 and 8 |
| Antenna configuration |  | • Rank 2: 2T8R |
|  |  | • Rank 4: 4T8R |
|  |  | • Rank 8: 8T8R |
| Antenna correlation |  | • Rank 2: ULA Medium B |
|  |  | • Rank 4 and 8: ULA Low |
| Propagation conditions |  | • Rank 2: TDLC300-100 |
|  |  | • Rank 4 and 8: TDLA30-10 |
| MCS |  | Rank 2: MCS19 (Table 1) Rank 4: MCS17 (Table 1) Rank 8 MCS17 (Table 1) |
| Overhead |  | Overhead = 0 |
| TRS configuration |  | • Configure TRS symbols in symbol#{5, 9} |
|  |  | • NZP CSI-RS configuration of row 6 for 8 layers. |
| N1 and N2 configuration for 8Tx case |  | (N1,N2)=(4,1) |

Finally, we provide in this section the parameters used for PDSCH CA simulation which are characterized in Table 3

Table 3: Simulation assumptions for CA

| Duplex Mode | SCS (kHz) | CBW (MHz) | Rank | MCS | Propagation Conditions |
| --- | --- | --- | --- | --- | --- |
| TDD | 30kHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 8 | MCS 17 (Table 1), MCS 19 (Table 1) | TDLA 30-10 Low, TDLC 300-100 Medium B |
| FDD | 15kHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50 | 8 | MCS 17 (Table 1), MCS 19 (Table 1) | TDLA 30-10 Low, TDLC 300-100 Medium B |

Simulation results: SNR 70% throughput

Table 4Error! Reference source not found. provides the SNR at which 70% throughput is achieved using the simulation parameters presented in the previous section.

Table 4: SNR at 70% for different number of layers and MCS (TDD Results).

| Rank | Antenna Configuration | MCS | Channel Correlation | Propagation Condition | SNR @ 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 2 | 2T8R | 19 (Table 1) | ULA Medium B | TDLC 300-100 | 11.5 |
| 4 | 4T8R | 17 (Table 1) | ULA Low | TDLA 30-10 | 10.3 |
| 8 | 8T8R | 17 (Both CW) (Table 1) |  |  | 20.9 |

Table 4Error! Reference source not found. provides the SNR at which 70% throughput is achieved for FDD using the simulation parameters presented in the previous section.

Table 5: SNR at 70% for different number of layers and MCS (FDD Results).

| Rank | Antenna Configuration | MCS | Channel Correlation | Propagation Condition | SNR @ 70% of max TP |
| --- | --- | --- | --- | --- | --- |
| 2 | 2T8R | 19 (Table 1) | ULA Medium B | TDLC 300-100 | 10.2 |
| 4 | 4T8R | 17 (Table 1) | ULA Low | TDLA 30-10 | 10.1 |
| 8 | 8T8R | 17 (Both CW) (Table 1) |  |  | 18.9 |

Table 6Table 4Error! Reference source not found. provides the SNR at which 70% throughput is achieved for FDD component carriers using the simulation parameters presented in the previous section, and Table 7 provides the SNR at which 70% throughput is achieved for the TDD component carriers.

Table 6: SNR at 70% for different number of layers and MCS (CA FDD Component Carrier Results).

| Duplex Mode | SCS (kHz) | CBW (MHz) | Channel | Rank | MCS | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 15 | 5 | TDCL300-100 | 2 | 19 (Table 1) | 9.7 |
|  |  | 10 |  |  |  | 9.6 |
|  |  | 15 |  |  |  | 9.7 |
|  |  | 20 |  |  |  | 9.6 |
|  |  | 25 |  |  |  | 9.8 |
|  |  | 30 |  |  |  | 9.7 |
|  |  | 35 |  |  |  | 9.6 |
|  |  | 40 |  |  |  | 9.6 |
|  |  | 45 |  |  |  | 9.7 |
|  |  | 50 |  |  |  | 9.6 |
|  |  | 5 | TDLA 30-10 | 8 | 17 (Table 1) | 19.6 |
|  |  | 10 |  |  |  | 18.9 |
|  |  | 15 |  |  |  | 19.3 |
|  |  | 20 |  |  |  | 19.7 |
|  |  | 25 |  |  |  | 19.8 |
|  |  | 30 |  |  |  | 19.8 |
|  |  | 35 |  |  |  | 20.1 |
|  |  | 40 |  |  |  | 20.4 |
|  |  | 45 |  |  |  | 20.2 |
|  |  | 50 |  |  |  | 20.1 |

Table 7: SNR at 70% for different number of layers and MCS (CA TDD Component Carrier Results).

| Duplex Mode | SCS (kHz) | CBW (MHz) | Channel | Rank | MCS | SNR (dB) of 70% of max TP |
| --- | --- | --- | --- | --- | --- | --- |
| TDD | 30 | 5 | TDCL300-100 | 2 | 19 (Table 1) | 9.8 |
|  |  | 10 |  |  |  | 9.7 |
|  |  | 15 |  |  |  | 9.9 |
|  |  | 20 |  |  |  | 10 |
|  |  | 25 |  |  |  | 9.9 |
|  |  | 30 |  |  |  | 9.7 |
|  |  | 40 |  |  |  | 9.9 |
|  |  | 50 |  |  |  | 10 |
|  |  | 60 |  |  |  | 10.1 |
|  |  | 80 |  |  |  | 10.2 |
|  |  | 90 |  |  |  | 10.1 |
|  |  | 100 |  |  |  | 10.2 |
|  |  | 5 | TDLA 30-10 | 8 | 17 (Table 1) | 19.3 |
|  |  | 10 |  |  |  | 19.6 |
|  |  | 15 |  |  |  | 19.4 |
|  |  | 20 |  |  |  | 18.9 |
|  |  | 25 |  |  |  | 19.2 |
|  |  | 30 |  |  |  | 19.2 |
|  |  | 40 |  |  |  | 19 |
|  |  | 50 |  |  |  | 19.2 |
|  |  | 60 |  |  |  | 19.3 |
|  |  | 80 |  |  |  | 19.7 |
|  |  | 90 |  |  |  | 19.6 |
|  |  | 100 |  |  |  | 19.8 |

Conclusion

In this contribution we provided the results of our simulation campaign on 8 Rx for different number of layers and MCS to support the definition of UE demodulation performance requirements for 8Rx.

References

- R4-2220613, “WF on 8RX UE demodulation and CSI requirements”, Huawei, HiSilicon, RAN WG4 #105, Toulouse, Nov. 2022

- R4-2302942, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106, Athens, Greece, 27 February - 03 March, 2023

- R4-2305888, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #106-bis-e, Online, April 17 – April 26, 2023

- R4-2309806, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #108, Incheon, Korea, 22 – 26 May 2023

- R4-2313877, “WF for 8Rx UE performance requirements”, Huawei, HiSilicon, RAN WG4 #108, Toulouse

- R4-2316914, “WF for 8Rx and 4Tx UE performance requirements”, RAN4#108-bis, Xiamen, October 2023

- R4-2321199, “WF for 8Rx UE performance requirements”, RAN4#109, Chicago, November 2023