3GPP TSG-RAN WG4 Meeting #110bis 					                   R4-2404313

Changsha, China, April 15 – 19, 2024

Agenda item:	6.5.4

Source:	Apple

Title:	Discussion on performance requirement for measurement without gaps

Document for:	Discussion

# Introduction

Test case list for measurement without gap was initially discussed in the previous RAN4 meeting. The last agreements can be found in [1].

In this contribution, we continue discussing the test list.

# Discussion

#### 2.1 test case list for NFG

The following test list was preliminarily agreed in RAN4#110. Note that modification of test case list and configuration is possible based on RAN4 consensus, which was captured in the RAN4#110 meeting minutes. Here we provide some analysis on the feasibility and necessity of each test case, as well as the corresponding configurations.

| No. # | Item of core requirements | Type of test cases | Frequency range of serving cell | SMTC configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NFG1 | Event triggered reporting and interruptions | Intra-frequency measurements without gap config but with DRX | FR1 | 20ms | No gap config | [DRX pattern TBD] | A.6.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG2 |  | Intra-frequency measurements without gap or DRX configuration | FR2 | 160ms | No gap config | No DRX | A.7.6.1.X |  |
| NFG3 |  | Inter-frequency measurements with gap config but without DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Title is incorrect. It should be no gap. |
| NFG4 |  |  | All in FR2 | 20ms | [Pattern TBD] | [DRX pattern TBD] | A.7.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG5 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |
| NFG6 |  |  | All in FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X |  |
| NFG7 |  | Intra-frequency measurements without gap or DRX configuration | FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X |  |
| NFG8 |  |  | FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |

Proposal 1: test case lit for NFG.

| No. # | Item of core requirements | Type of test cases | Frequency range of serving cell | SMTC configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [FFS: NFG1] | Event triggered reporting and interruptions | Intra-frequency measurements without gap config but with DRX | FR1 | 20ms | No gap config | [DRX pattern TBD] | A.6.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG2 |  | Intra-frequency measurements without gap or DRX configuration | FR2 | 160ms | No gap config | No DRX | A.7.6.1.X |  |
| NFG3 |  | Inter-frequency measurements without gap config but without DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Title is incorrect. It should be no gap. |
| [FFS: NFG4] |  |  | All in FR2 | 20ms | [Pattern TBD] | [DRX pattern TBD] | A.7.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG5 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |
| NFG6 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X |  |
| NFG7 |  | Intra-frequency measurements without gap or DRX configuration | FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X |  |
| NFG8 |  |  | FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |

#### 2.2 test case list for inter-RAT measurement without gap

Similarly, we also provide analysis on test case for inter-RAT measurement without gap.

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | EMW configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR1 | Event triggered reporting and interruptions | Inter-RAT EUTRAN measurements case b-1 with MG and DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | [DRX pattern TBD] | A.6.6.3.X | Infeasible since no core requirements for b-1 with interruption. < Agreement in R4#106bis-e>: Reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1 and Define requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption. |
| IR2 |  | Inter-RAT EUTRAN measurements case b-1 with MG but no DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | No DRX config | A.6.6.3.X | Same as above |
| IR3 | Event triggered reporting [without interruption] | Inter-RAT EUTRAN measurements case b-2 without MG or DRX configuration | NR FR1 | [Pattern TBD] | No gap configuration | No DRX configuration | A.6.6.3.X |  |
| IR4 | Event triggered reporting | Inter-RAT NR measurements case a-1with MG | NR FR2 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | FR1-FR2 testability issue. Need to update A.3.13A. More justification is needed on what and how to verify collision between EMW and MG. |
| IR5 | Event triggered reporting and interruptions | Inter-RAT NR measurements case a-1 with MG | NR FR1 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | More justification is needed on what and how to verify collision between EMW and MG. |

Agreements in RAN4#107 for information.

| Using scenarios | Capability indications | New RRM requirements needed | Notes |
| --- | --- | --- | --- |
| Case a-1: Inter-RAT NR wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS36.133 8.1.2.4.21&22 can be applied |
|  | “no gap but interruption allowed” | Yes | To be defined in TS36.133 |
|  | “no gap no interruption” | Yes | To be defined in TS36.133 |
| Case b-1: Inter-RAT LTE wo gap because of the vacant RF chain available | “gap” | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “ncsg” | No. | the existing requirements in TS38.133 9.4.2&9.4.3 can be reused. |
|  | “nogap-noncsg” | Yes | To be defined in TS38.133 |
| Case b-2: Inter-RAT LTE wo gap because the measurement reference signal can be contained within UE’s active BWP | “gap” | No | The existing requirements in TS38.133 9.4.2&9.4.3 can be applied |
|  | “no gap but interruption allowed” | No | Do not consider |
|  | “no gap” | Yes | To be defined in TS38.133 |

Observation 1: case b-1 with interruption is NOT supported in this WI.

Proposal 2: test case list for inter-RAT measurement without gap:

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | EMW configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR1 | Event triggered reporting and interruptions | Inter-RAT EUTRAN measurements case b-1 with MG and DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | [DRX pattern TBD] | A.6.6.3.X | Infeasible since no core requirements for b-1 with interruption. < Agreement in R4#106bis-e>: Reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1 and Define requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption. |
| IR2 |  | Inter-RAT EUTRAN measurements case b-1 with MG but no DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | No DRX config | A.6.6.3.X | Same as above |
| IR3 | Event triggered reporting [without interruption] | Inter-RAT EUTRAN measurements case b-2 without MG or DRX configuration | NR FR1 | [Pattern TBD] | No gap configuration | No DRX configuration | A.6.6.3.X |  |
| IR4 | Event triggered reporting | Inter-RAT NR measurements case a-1 [FFS: with MG] | NR FR2 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | FR1-FR2 testability issue. Need to update A.3.13A. More justification is needed on what and how to verify collision between EMW and MG. |
| IR5 | Event triggered reporting and interruptions | Inter-RAT NR measurements case a-1 [FFS: with MG] | NR FR1 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | More justification is needed on what and how to verify collision between EMW and MG. |

# Conclusion

In this contribution, we provide discussion on test case list of measurement without gaps. After discussion, the following conclusions are provided:

Proposal 1: test case lit for NFG.

| No. # | Item of core requirements | Type of test cases | Frequency range of serving cell | SMTC configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [FFS: NFG1] | Event triggered reporting and interruptions | Intra-frequency measurements without gap config but with DRX | FR1 | 20ms | No gap config | [DRX pattern TBD] | A.6.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG2 |  | Intra-frequency measurements without gap or DRX configuration | FR2 | 160ms | No gap config | No DRX | A.7.6.1.X |  |
| NFG3 |  | Inter-frequency measurements without gap config but without DRX | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Title is incorrect. It should be no gap. |
| [FFS: NFG4] |  |  | All in FR2 | 20ms | [Pattern TBD] | [DRX pattern TBD] | A.7.6.1.X | DRX related requirements are still TBD. Necessity of this test depends on conclusion of the issue. |
| NFG5 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |
| NFG6 | Event triggered reporting [without interruption] | Inter-frequency measurements without gap or DRX configuration | All in FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X |  |
| NFG7 |  | Intra-frequency measurements without gap or DRX configuration | FR1 | 20ms | No gap config | No DRX config | A.6.6.1.X |  |
| NFG8 |  |  | FR2 | 20ms | No gap config | No DRX config | A.7.6.1.X | Similar with ‘interruption’ case, no need to test both intra and inter in both FR1 and FR2. E.g. two tests with intra in FR1 and inter in FR2 are enough. |

Observation 1: case b-1 with interruption is NOT supported in this WI.

Proposal 2: test case list for inter-RAT measurement without gap:

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | EMW configuration | MG configuration | DRX configuration | Subclause | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR1 | Event triggered reporting and interruptions | Inter-RAT EUTRAN measurements case b-1 with MG and DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | [DRX pattern TBD] | A.6.6.3.X | Infeasible since no core requirements for b-1 with interruption. < Agreement in R4#106bis-e>: Reuse NeedForNCSG-InfoEUTRA-r17 to support Case b-1 and Define requirements for case when UE reports “nogap-noncsg” in NeedForNCSG-InfoEUTRA-r17 for indicating no-gap without interruption. |
| IR2 |  | Inter-RAT EUTRAN measurements case b-1 with MG but no DRX configuration | NR FR1 | [Pattern TBD] | [Pattern TBD] | No DRX config | A.6.6.3.X | Same as above |
| IR3 | Event triggered reporting [without interruption] | Inter-RAT EUTRAN measurements case b-2 without MG or DRX configuration | NR FR1 | [Pattern TBD] | No gap configuration | No DRX configuration | A.6.6.3.X |  |
| IR4 | Event triggered reporting | Inter-RAT NR measurements case a-1 [FFS: with MG] | NR FR2 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | FR1-FR2 testability issue. Need to update A.3.13A. More justification is needed on what and how to verify collision between EMW and MG. |
| IR5 | Event triggered reporting and interruptions | Inter-RAT NR measurements case a-1 [FFS: with MG] | NR FR1 | No EMW config | [Pattern TBD] | No DRX config | A.8.4.2.X | More justification is needed on what and how to verify collision between EMW and MG. |

# References

- R4-2403543, WF on NR_MG_enh2_part2, Intel