# 3GPP TSG-RAN WG4 Meeting #110bis 					       					            R4-2404318

# Changsha, China, April 15 – 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for PDCCH ordered based RACH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN1 PDCCH ordered based RACH delay requirements, TSSB and ∆RF/BB preparation are referred to TS38.133. If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - TBWPswitchDelay=0 if the active UL BWP does not change, or if a cell indicator field in the PDCCH order indicates a non-serving cell [5, TS 38.212], and TBWPswitchDelay is defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] - TSSB=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and TSSB is defined in [10, TS 38.133] otherwise - ∆RF/BB preparation=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and ∆RF/BB preparation is defined in [10, TS 38.133] otherwise. However, they haven’t been specified in TS38.133 yet. |
| Summary of change: |  | Introudce TSSB and ∆RF/BB preparation in TS38.133 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PDCCH ordered RACH delay requirements would still be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 8.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<START OF CHANGES>>

## 8.X	PDCCH ordered based RACH delay for LTM candidate cell

### 8.X.1	Introduction

The requirements in this clause apply for a UE capable of [FG 45-5: RACH-based early TA acquisition] configured to trigger RACH procedure toward target LTM candidate cell before receiving cell switch command MAC-CE.

### 8.x.2	PDCCH ordered based RACH delay

UE shall complete the RACH transmission within the delay specified in TS38.213 [3] clause 8.1, in which TSSB and ∆RF/BB preparation  are defined as the following:

-	TSSB:

-	TSSB = 0 provided the following conditions are met:

-	target LTM candidate cell is in FR1, and

-	UE has received MAC-CE to activate TCI of target candicate cell and the time between receiving the MAC-CE activating the target TCI state and the PDCCH order for RACH is at least [THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, where THARQ, TOk, Tfirst-SSB and TSSB-proc are as stated in section 8.15.3], and

-	L1-RSRP measurement period as defined in clause 9.14 and 9.15 respectivefully for intra-frequency and inter-frequency is less than or equal to 160ms.

-	Otherwise,

-	When SSB is within active BWP, TSSB is the time to the first SSB transmission after PDCCH-order RACH command is decoded by the UE.

-	When SSB is outside active BWP, TSSB is the time to the first SSB transmission overlapped with MGL after PDCCH-order RACH command is decoded by the UE.

-	∆RF/BB preparation: if PRACH bandwidth is within active UL BWP of any active serving cell, ∆RF/BB preparation = 0. If PRACH bandwidth is outside active UL BWP but within one of configured UL BWPs of any active serving cell, ∆RF/BB preparation is the DCI based BWP switching delay specified in clause 8.6. If PRACH bandwidth is outside all the configured UL BWP of all serving cell(s), ∆RF/BB preparation equals to [FG 39-5: RF/BB preparation time for PDCCH-order RACH].

<<END OF CHANGES>>