3GPP TSG-RAN WG4 Meeting #110bis	R4-2404478

Changsha, China, 15th – 19th April, 2024

Agenda Item:	9.11.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we firstly discuss following general issue from last meeting, and then we provide some initial views on RRM core requirements.

- Issue 1-2: Post deployment testing.

- Initial views on RRM core requirements

2	Discussion

2.1 Issue 1-2: Post deployment testing

Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded

Option 1 does not make sense literally as it is still focusing on “before deployment” effort, which contradicts with “post deployment” description in the issue. We agree the principle behind option 1 that an AI/ML model/functionality should be fully verified/tested before actual deployment. The performance could be guaranteed by vendors. It is unreasonable for vendor to deploy an update that would clearly decrease the performance or hardly provide any benefits.

Option 2 is a valid operative method to guarantee model/functionality performance in practice. If some unexpected performance degradation really occurs, once being identified via monitoring, the network or UE can trigger LCM to deactivate/fallback/switch the corresponding AI/ML model or functionality.

Proposal 1: Suggest to focus on option 2 for handling post deployment testing.

2.2 RRM core requirements

Based on TU budget after RAN #103, this meeting shall formally start the normative work on AI/ML-based BM and positioning. In the first meeting, we could start by identifying a general scope of impact on RRM core requirements brought by the two cases, respectively. Our initial views are provided in table 1 and 2 (Appendix). For discussion on details, RAN4 needs to wait for more RAN1 and RAN2 progress.

Proposal 2: Suggest to identify a general scope of impact on RRM core requirements brought by AI/ML-based BM and positioning. Our initial views are provided in Table 1 and 2 (Appendix).

Conclusion

In this contribution, the issue of post deployment testing is discussed with following proposal:

Proposal 1: Suggest to focus on option 2 for handling post deployment testing.

Proposal 2: Suggest to identify a general scope of impact on RRM core requirements brought by AI/ML-based BM and positioning. Our initial views are provided in Table 1 and 2 (Appendix).

Appendix

Table 1 General scope of impact on RRM core requirements brought by AI/ML-based beam management

| RRM Req. Category | RRM procedure | RAN4 impact |
| --- | --- | --- |
| IDLE state mobility – Cell selection | Cell selection | No impact |
| IDLE/INACTIVE state mobility – Cell re-selection | Measurement and evaluation of serving cell | No impact |
|  | Measurements of intra-frequency NR cells |  |
|  | Measurements of inter-frequency NR cells/inter-RAT |  |
| RRC_CONNECTED state mobility - Handover | NR Handover | No impact |
|  | Handover to other RATs |  |
|  | NR DAPS Handover |  |
|  | NR Conditional Handover |  |
|  | NR Handover with PSCell |  |
| RRC_CONNECTED state mobility - RRC Connection Mobility Control | SA: RRC Re-establishment | No impact |
|  | Random access |  |
|  | SA: RRC Connection Release with Redirection |  |
| Timing | UE Transmit timing | No impact |
|  | UE Timer accuracy |  |
|  | Timing advance |  |
|  | Cell phase synchronization accuracy |  |
|  | Maximum transmission timing difference |  |
|  | Maximum receive timing difference |  |
| Signaling characteristics | SSB /CSI-RS based Radio Link Monitoring | Discussion and Decide |
|  | Interruption | No impact |
|  | Scell Activation/Deactivation Delay |  |
|  | UE UL carrier RRC reconfiguration delay |  |
|  | Link recovery procedures (SSB/CSI-RS based beam failure indication, L1 indication, Candidate beam detection) | Discussion and Decide |
|  | Active BWP switch delay | No impact |
|  | Active TCI state switching delay | Discussion and Decide |
|  | Uplink spatial relation switch delay |  |
|  | UE-specific CBW change | No impact |
|  | Pathloss reference signal switching delay | Discussion and Decide |
|  | Active downlink TCI switching delay for unified TCI |  |
|  | Active uplink TCI switching delay for unified TCI |  |
|  | Functionality/Model monitoring procedure | FFS new requirements |
|  | Functionality/Model selection/activation/deactivation/fallback delay |  |
|  | Active model switch delay |  |
| Measurement procedure | NR intra-frequency measurements (Number of cells and number of SSB, measurement reporting requirements, etc) | Discussion and Decide |
|  | NR inter-frequency measurements | No impact |
|  | Inter-RAT measurements |  |
|  | L1-RSRP measurements for reporting | Discussion and Decide |
|  | Cross link interference measurements | No impact |
|  | L1-SINR measurements for reporting |  |
| Measurement performance requirements – NR measurements | Intra-frequency RSRP accuracy requirements for FR1/FR2 | Discussion and Decide |
|  | Inter-frequency RSRP accuracy requirements for FR1/FR2 | No impact |
|  | RSRP measurement report mapping | Discussion and Decide |
|  | Intra-frequency RSRQ accuracy requirements for FR1/FR2 | No impact |
|  | Inter-frequency RSRQ accuracy requirements for FR1/FR2 |  |
|  | RSRQ report mapping |  |
|  | Intra-frequency SINR accuracy requirements for FR1/FR2 |  |
|  | Inter-frequency SINR accuracy requirements for FR1/FR2 |  |
|  | SINR report mapping |  |
|  | L1-RSRP accuracy requirements for FR1/FR2 | Discussion and Decide |
|  | L1-SINR accuracy requirements for FR1/FR2 | No impact |

Table 2 General scope of impact on RRM core requirements brought by AI/ML-based positioning accuracy enhancement

| RRM Req. Category | RRM procedure | RAN4 impact |
| --- | --- | --- |
| Signaling characteristics | Functionality/Model monitoring procedure | FFS new requirements |
|  | Functionality/Model selection/activation/deactivation/fallback delay |  |
|  | Active model switch delay |  |
| Measurement procedure – NR measurements for positioning | RSTD measurements | Discussion and Decide |
|  | PRS-RSRP measurements |  |
|  | UE Rx-Tx time difference measurements |  |
|  | E-CID measurements |  |
|  | PRS-RSRPP measurements |  |
| Measurement performance requirements – NR measurements | RSTD measurements | Discussion and Decide |
|  | PRS-RSRP measurements |  |
|  | UE Rx-Tx time difference measurements |  |
|  | PRS-RSRPP measurements |  |
| Measurements performance requirements for NR gNB | UL-RTOA | Discussion and Decide |
|  | gNB Rx-Tx time difference |  |
|  | UL SRS RSRP measurement |  |
|  | Timing advance |  |
|  | UL SRS RSRPP measurement |  |