3GPP TSG-RAN WG4 Meeting #110-bis                                       R4-2304506

Changsha, China, 15th – 19th April, 2024

Source: 	Huawei, HiSilicon

Title: 	On sidelink intra-band non-contiguous CA in PC3 and PC2

Agenda Item:	9.9.2

Document for:	Approval

Introduction

The New WID revision [1] was approved in RAN#103 with the scope for sidelink contiguous and non-contiguous CA operation in ITS band.

- Specify the RF requirements for NR sidelink CA in n47: - Intra-band non-contiguous CA with power class 2 and power class 3, with Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz - Intra-band contiguous CA with power class 2 - Introduce necessary changes for release independence specification, if identified Note: Work for power class 3 starts first in RAN4. Whether there is any RRM core requirement to be specified will be confirmed in Q3 2024.

In this contribution, we provide our consideration and initial analysis on RAN4 work for NR sidelink intra-band non-contiguous CA with power class 3 and power class 2.

Discussion

In Rel-15 LTE V2X, the requirements of intra-band contiguous multi-carrier operation bandwidth class B and class C were defined in addition to the single carrier V2X case. It is noted that it is a full set of Tx and Rx requirements in order to support the multi-carrier operation.

Similarly, in Rel-18 the NR sidelink CA operation with PC3 was specified in RAN4, another set of requirements was defined in addition to that of Rel-16/17 V2X/Sidelink.

In the WID for sidelink CA, intra-band non-contiguous CA with PC3 and PC2 is to be considered. The work in Rel-19 would include at least

- Operating bands n47 for intra-band non-contiguous sidelink CA

- Band combination(s) for intra-band non-contiguous SL CA

- Configurations and BCS for intra-band non-contiguous SL CA

- Tx requirements mainly on

  - Maximum output power

  - MPR for PC2 and PC3

  - Configured output power

  - Update output power dynamic, transmit signal quality, out of band emission, spurious emissions

- Rx requirements mainly on

  - REFSENS

  - Maximum input level,

  - ACS locking

Proposal 1: The work in Rel-19 NR sidelink intra-band non-contiguous CA with power class 3 and power class 2 would include at least

- Operating bands n47 for intra-band non-contiguous sidelink CA

- Band combination(s) for intra-band non-contiguous SL CA

- Configurations and BCS for intra-band non-contiguous SL CA

- Tx requirements mainly on

  - Maximum output power

  - MPR for PC2 and PC3

  - Configured output power

  - Update output power dynamic, transmit signal quality, out of band emission, spurious emissions

- Rx requirements mainly on

  - REFSENS

  - Maximum input level,

  - ACS locking

In Rel-18 intra-band contiguous SL CA with PC3, the simulation assumptions were agreed to reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed roughly based on the agreement in Rel-18.

In Rel-19 intra-band non-contiguous SL CA with PC3 and PC2, considering the narrow bandwidth in ITS band n47, dualPA may not be required. Indeed, the specified bandwidth combination(s) is(are) up to operators’ demand. 10MHz + 10MHz is with much possibility. The maximum bandwidth combination would not be wider than 30MHz + 30MHz. Therefore, single PA structure is sufficient.

Proposal 2: Single PA structure for intra-band non-contiguous SL CA is sufficient in Rel-19.

The simulation assumptions can be based on intra-band contiguous SL CA in Rel-18, with minor revision as follows.

Table 1 General simulation assumptions for intra-band non-contiguous SL CA with PC2/PC3

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm and 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc or 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Proposal 3: For the MPR evaluation of intra-band non-contiguous CA with PC2/PC3, the simulation assumptions as follows should be agreed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm and 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc or 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Conclusion

This contribution provides our consideration and initial analysis on RAN4 work for Rel-18 SL evolution with the following and proposals:

Proposal 1: The work in Rel-19 NR sidelink intra-band non-contiguous CA with power class 3 and power class 2 would include at least:

- Operating bands n47 for intra-band non-contiguous sidelink CA

- Band combination(s) for intra-band non-contiguous SL CA

- Configurations and BCS for intra-band non-contiguous SL CA

- Tx requirements mainly on

  - Maximum output power

  - MPR for PC2 and PC3

  - Configured output power

  - Update output power dynamic, transmit signal quality, out of band emission, spurious emissions

- Rx requirements mainly on

  - REFSENS

  - Maximum input level,

  - ACS locking

Proposal 2: Single PA structure for intra-band non-contiguous SL CA is sufficient in Rel-19.

Proposal 3: For the MPR evaluation of intra-band non-contiguous CA with PC2/PC3, the simulation assumptions as follows should be agreed

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 23dBm and 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc or 25dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

- References

[1] RP-240840 New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band