3GPP TSG-RAN WG4 Meeting # 110-bis		                                                                R4-2404586

Changsha, Hunan Province, China, April 15th – 19th, 2024

Agenda item:	9.13.2

Source: 	Spreadtrum Communications

Title: 	Discussion on co-existence evaluation for Ambient IoT and NR/LTE

Document for:	Discussion

Introduction

In RAN#102 meeting, the SID [1] on Study on solutions for Ambient IoT in NR was approved as one of R19 RAN1-led items. In this contribution, we want to provide some initial views on the co-existence study for Ambient IoT and NR/LTE.

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception

Discussion

Coexistence scenarios

According to the SID, Ambient IoT includes two deployment topologies, namely D1T1 and D2T2, and three spectrum deployment options, including in-band to NR, in guard-band to NR/LTE or in standalone bands. With three types of devices including device 1, device2a, device 2b. CW (Carrier wave) from node(s) inside or outside the connectivity topology. CW including two spectrums, respectively are UL and DL spectrum. Ambient IoT system includes two sets aggressor vs victim, one is NR/LTE aggressor with Ambient IoT victim and the other one is Ambient IoT aggressor with NR/LTE victim. RAN4 needs to define requirements for ACLR/ACS/Spurious and/or guard RBs for the A-IoT system to ensure co-existence with NR/LTE. If we consider all the above scenarios, there will be so many co-existence simulated cases, it will result in a huge workload. Thus, in the initial stage, we should down-select some cases based on RAN1 agreements and proposals together with RAN4 past experience.

According to RAN1 chair notes [2], there are some proposals and agreements about topology and CW characteristics as follows.

Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal: For Deployment scenario 1 with topology 1, the following scenarios are used for evaluation of coverage and coexistence, D1T1-A: indoor BS + indoor AIoT device, CW inside topology, - [R2D at least in DL spectrum] - Only for device 1 and device 2a - [FFS: RF energy harvesting] - D1T1-A1: - CW Emitter in CW2D and the Reader ‘R2’ in D2R are different - CW Emitter in CW2D and the Reader ‘R1’ in R2D are the same - ‘R1’ in R2D and ‘R2’ in D2R are different - D1T1-A2: - CW Emitter and Reader ‘R’ in CW2D, D2R and R2D are the same D1T1-B: indoor BS + indoor AIoT device, CW outside topology, - For D1T1-B: - CW Emitter in CW2D and ‘R’ in D2R are different - CW Emitter in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - [R2D in at least DL spectrum] - Only for device 1 and device 2a - [FFS: RF energy harvesting] D1T1-C: indoor BS + indoor AIoT device with D2R transmission generated internally - Only for device 2b - [R2D in at least DL spectrum] - D2R in UL spectrum - [FFS: RF energy harvesting] FFS for other scenarios FFS other assumptions for each scenario Conclusion Companies are encouraged to consider Table 3.4.2 in R1-2401735 for their contributions to RAN1#116bis regarding link budget template.

For the three spectrum deployments from the perspective of coexistence interference. On one hand, in-band to NR and guard-band to NR/LTE needs to share spectrum with NR and/or LTE, which will result in serious co-existence interference issues. While, standalone bands deployment has no extra coexistence issues. However, compared with standalone bands, the spectrum resource utilization is much higher for in-band to NR and guard-band deployment.

Figure2.1-1: Three spectrum deployments

Proposal 1: For co-existence evaluation, In-band to NR deployment should be the first priority, and guard-band to NR/LTE can be second priority.

Thus, we can take in-band to NR deployment to analyse co-existence. At first, we could analyse co-existence scenarios based on D1T1-A (including two types: A1 and A2) as follows. The NR UE is outdoor.

Figure2.1-2: co-existence with NR UE for outdoor (CW in UL spectrum)                         Figure2.2-3: co-existence with NR UE for outdoor (CW in DL spectrum)

Figure2.1-4: co-existence with NR UE for outdoor (CW in DL spectrum)                         Figure2.1-5: co-existence with NR UE for outdoor (CW in UL spectrum)

Proposal 2: The following aspects and/or scenarios should be given higher priority for in-band coexistence evaluation:

- Outdoor NR UE

- Device1/2a with RF-ED

- For D1T1: R2D in DL spectrum, CW in UL spectrum

- For D2T2: R2D in UL spectrum, CW in DL spectrum (Second priority comparing with D1T1)

Proposal 3: Adopt the Table 1 as the proposed scenarios for in-band coexistence evaluation.

Table 1: A-IoT co-existence scenarios cases with in-band NR deployment

| Cases | Spectrum deployment mode | topo | CW topo | device | Spectrum for Ambient IoT | Aggressor | victim | Initial views | Priority level |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-1 | In-band | D1T1-A1 | Inside | 1/2a | R2D: DL spectrum CW: DL spectrum D2R: DL spectrum | A-IoT D2R/CW | NR DL | Needs to consider ASCS impact | 2 |
| 1-2 |  |  |  |  |  | A-IoT R2D | NR DL | Needs to consider ASCS impact | 2 |
| 1-4 |  |  |  |  |  | NR DL | A-IoT D2R | Needs to consider IBE/ALCR | 2 |
| 1-5 |  |  |  |  |  | NR DL | A-IoT R2D | Device 1 with RF-ED may be cannot distinguish NR signal. | 2 |
| 1-6 | In-band | D1T1-A1 | Inside | 1/2a | R2D: DL spectrum CW: UL spectrum D2R: UL spectrum | A-IoT D2R/CW | NR UL | Needs to consider IBE impact | 1 |
| 1-7 |  |  |  |  |  | A-IoT R2D | NR DL | Needs to consider ASCS impact | 1 |
| 1-8 |  |  |  |  |  | NR UL | A-IoT D2R/CW | Needs to consider ALCR impact | 1 |
| 1-9 |  |  |  |  |  | NR DL | A-IoT R2D | Device 1 with RF-ED may be cannot distinguish NR signal. | 1 |
| 2-1 | In-band | D1T1-A2 | Inside | 1/2a | R2D: DL spectrum CW: DL spectrum D2R: DL spectrum | A-IoT D2R/CW | NR DL | Needs to consider ASCS impact | 2 |
| 2-2 |  |  |  |  |  | A-IoT R2D | NR DL | Needs to consider ASCS impact | 2 |
| 2-4 |  |  |  |  |  | NR DL | A-IoT D2R/CW | Needs to consider IBE/ALCR | 2 |
| 2-5 |  |  |  |  |  | NR DL | A-IoT R2D | Device 1 with RF-ED may be cannot distinguish NR signal. | 2 |
| 2-6 | In-band | D1T1-A2 | Inside | 1/2a | R2D: DL spectrum CW: UL spectrum D2R: UL spectrum | A-IoT D2R/CW | NR UL | Needs to consider IBE impact | 1 |
| 2-7 |  |  |  |  |  | A-IoT D2R | NR DL | Needs to consider ASCS impact | 1 |
| 2-8 |  |  |  |  |  | NR UL | A-IoT D2R/CW | Needs to consider ALCR impact | 1 |
| 2-9 |  |  |  |  |  | NR DL | A-IoT R2D | Device 1 with RF-ED may be cannot distinguish NR signal. | 1 |
| 3-1 | In-band | D1T1-B | Outside | 1/2a | R2D: DL spectrum D2R: UL spectrum CW: UL spectrum | A-IoT D2R | NR UL | Needs to consider IBE impact | 1 |
| 3-2 |  |  |  |  |  | A-IoT R2D | NR DL | Needs to consider ASCS impact | 1 |
| 3-3 |  |  |  |  |  | NR UL | A-IoT D2R | Needs to consider ALCR impact | 1 |
| 3-4 |  |  |  |  |  | NR DL | A-IoT R2D | Device 1 with RF-ED may be cannot distinguish NR signal. | 1 |

Simulation assumptions

    - Reference frequency

The SID has proposed to FR1 licensed FDD spectrum, and according to the RAN1 some inputs, 900MHz is proposed for reference frequency band.

Proposal 4: 900MHz is adopted as the reference frequency for simulation assumption.

    - Simulation assumptions and parameters

Proposal 5: Adopt the following simulation assumptions and parameters in Section 2.2.2 as a starting point.

◆ Network layout model

For D1T1, the indoor network layout model in TR 38.803 and InF-DH defined in TR 38.901are adopted as a starting point.

For D1T1-A, NR UE and macro gNB are all in outdoors, and all A-IoT reader (gNB)/device are all indoors. Thus, the topology includes two BS deployments, namely outdoor (UMa: Table 2.2.2-2 )+ indoor(indoor Factory: Table 2.2.2-1), the Figure2.2.2-1 below shows the overall deployment block diagram. The red box represents the factory building, which is 100 m away from the base station (the distance from the restricted area of the base station), indicating the situation closest to the base station and the biggest interference.

Figure 2.2.2-1: overall deployment block diagram.

Table 2.2.2-1: Single operator layout for indoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Scenario |  | InF-DH |  |
| Network layout |  | 60m x 120m, 18BSs |  |
| Inter-site distance |  | 20m |  |
| BS antenna height |  | 8 m | ceiling |
| A-IoT device location | Outdoor/indoor | Indoor |  |
|  | LOS/NLOS | LOS and NLOS |  |
|  | A-IoT device antenna height | 1.5 m |  |
| A-IoT device distribution (horizontal) |  | uniform dropping for indoor with minimum 2D distance of 1 m |  |
| Minimum BS – A-IoT device distance (2D) |  | 0 m |  |
| Channel model |  | Indoor Factory |  |
| BS deployment |  | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m |  |

Table 2.2.2-2: Macro layout for outdoor

| Parameters |  | Values | Remark |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance |  | 200m (baseline) 300m (optional) |  |
| BS antenna height |  | 25 m |  |
| NR UE location | Outdoor/indoor | Outdoor |  |
|  | LOS/NLOS | LOS/NLOS |  |
|  | NR UE antenna height | 1.5m |  |
| NR UE distribution (horizontal) |  | Uniform dropping with minimum 2D distance of 1 m |  |
| Minimum BS – NR UE distance (2D) |  | 35 m |  |
| Channel model |  | UMa |  |
| BS deployment |  |  |  |

◆ Propagation model

Path loss

RAN1 has reached an agreement about path loss model. For D1T1, InF-DH is adopted as a starting point.

Agreement The following pathloss model is used in the coverage evaluation. - For D1T1, - InF-DH defined in TR38.901 is used. - Decide which of the following is used for each link, - NLOS - LOS - FFS: InF-SH - For D2T2, down-select from the following path loss models - InF-DL defined in TR38.901 where the BS path loss model is reused for intermediate-UE with antenna height of 1.5m - InH-Office model defined in TR38.901, (a.k.a, InH_B in Report ITU-R M.2412-0) where the BS path loss model is reused for intermediate-UE with antenna height of 1.5m - Decide which of the following is used for each link, - NLOS - LOS

More details can be shown below.

Table 2.2.2-3: Path loss model for D1T1-A/B

|  | D1T1-A1 | D1T1-A2 | D1T1-B |
| --- | --- | --- | --- |
| R2D | InF-DH | InF-DH | InF-DH |
| CW2D | InF-DH | InF-DH |  |
| NR UE to D | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) Indoor part(Indoor loss PLin, D is the building to device) |
| D2R | InF-DH | InF-DH | InF-DH |
| D to NR UE | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to device) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to device) |
| NR UE to Reader | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) |
| Reader to NR UE | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) | including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) | Including three parts: - O-to-I penetration loss model: Low-loss model - Outdoor part(Indoor loss PLin, D is UE to the building) - Indoor part(Indoor loss PLin, D is the building to Reader) |
| D to gNB | Uma+ indoor part | Uma+ indoor part | Uma+ indoor part |
| gNB to D | Uma+ indoor part | Uma+ indoor part | Uma+ indoor part |

Antenna pattern

BS antenna configuration

2 or 4 antenna elements, with (M,N,P,Mg,Ng)=(1,1,2,1,1) or (2,1,2,1,1)

Intermediate UE/A-IoT device antenna configuration

1 element (vertically polarized), Isotropic antenna gain pattern

◆Noise Figure

Noise figure for BS and intermediate UE/NR UE is assumed as 9dB. For device1, LP-WUS simulation assumption worst value of 24dB is proposed as a starting point (section 6.2.2 in TR 38.869). For device 2a/2b, 15dB is adopted as a starting point.

◆ Other simulation parameters

| Parameters | Topo1 | Topo2 |
| --- | --- | --- |
| Carrier Frequency | 900MHz | 900MHz |
| A-IoT device type | Device1/2a/2 | Device1/2a/2b |
| Channel bandwidth | 10MHz,20MHz | 10MHz,20MHz |
| BB filter order | 3/5 Butterworth filter | 3/5 Butterworth filter |
| Scheduled channel bandwidth per A-IoT (DL) | 180KHz | 180KHz |
| Scheduled channel bandwidth per A-IoT (UL) | 180KHz | 180KHz |
| The number of active A-IoT (DL) | 1 | 1 |
| The number of active A-IoT (UL) | 1 | 1 |
| Traffic model | Full buffer | Full buffer |
| BS antenna gain (dBi) | 5 | 8 |
| NR UE and intermediate UE antenna gain (dBi) | X/5 | 5/5 |
| Device antenna gain(dBi) | 0 | 0 |
| BS max TX power in dBm | 33dBm | 46dBm |
| NR UE max TX power in dBm | 23dBm | 23dBm/26dBm |
| CW max TX power in dBm | CW inside(33dBm) CW outside( FFS) | CW inside(23dBm/26dBm) CW outside( FFS) |
| Intermediate UE max TX power in dBm | / | 23dBm |
| A-IoT Device max TX power in dBm | -10dBm | -10dBm |
| RF impairment | SFO: 105ppm (for device 1/2a) CFO: 200 ppm (for device2b) | SFO: 105ppm (for device 1/2a) CFO: 200ppm (for device 2b) |
| Handover margin | 0dB | 0dB |

◆Performance metrics

For NR/LTE UE and intermediate UE, BLER should be less than 1%.

For A-IoT UE, BLER is 1% is proposed as a starting point.

Co-existence simulation methodology

Section 5.3 in TR 38.803 is proposed as a starting point.

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

In addition,

Proposal 6: The above simulations steps are adopted as a starting point.

Initial simulations results

◆UL A-IoT vs UL NR UE

In this part, we provide the initial simulation results about UL co-existence, A-IoT is in-band NR deployment. We evaluated the A-IoT UL (NR UE UL aggressor vs A-IoT UL victim) and NR UE UL (A-IoT UL aggressor vs NR UE UL victim performance in topology1.

Figure 2.4-1: layout of BS/NR UE with A-IoT device                       Figure2.4-2: deployment for in-band NR

Table 2.4-1: simulation parameters of UL co-existence

| Parameter | Value |
| --- | --- |
| Carrier frequency (GHz) | 900MHz |
| SCS for NR | 15kHz |
| Tx power of emitter (dBm) | 33 |
| Tx power of legacy UE (dBm) | 23 |
| Distance between emitter and A-IoT device(m) | 10 |
| backscatter loss(dB) | 5 |
| Transmission bandwidth of A-IoT device | 180kHz |
| Height of emitter(m) | 5 |
| Height of gNB(m) | 8 |
| Height of device(m) | 2 |
| Distance between emitter and A-IoT device(m) | 10 |
| Distance between Device and gNB(m) | 15 |
| Distance between emitter and gNB(m) | 15 |
| Path loss between A-IoT device and emitter | InF-DH NLOS in TS38.910 |
| Path loss between A-IoT device and gNB(m) | InF-DH NLOS in TS38.910 |
| Path loss between emitter and gNB(m) | InF-DH NLOS in TS38.910 |
| Path loss between NR UE and gNB(m) | InF-SH NLOS in TS38.910 |
| Coding in A-IoT UL | DNRZ |
| Bandwidth of NR UE | 10M |
| Sample rate of NR UE | 15.36MHz |
| PRB number of NR UL | 50 |
| Modulation of NR UE | 256QAM |
| BER | 10-3 |

In the process of simulation, we gradually moved the NR UE away from the BS, increasing the distance by 50 meters each time, from 100 meters until the distance between the NR UE and the BS reached 500 meters. We calculate the values of averaged EVM for every RB to get the guard RB number between NR and A-IoT compared with 3.5% of EVM. Simulation results are shown in the figures as follows. In Figure 2.4-3, the result shows that guard RB is 1 to meet the 3.5% of EVM when the distance between the NR UE and BS is 100m.The number of guard RB is increasing with the distance gradually add. The number of guard RB is 4 when the distance is 400m.

- NR UE UL performance (A-IoT UL as aggressor)

Figure 2.4-4: EVM of 256QAM when TXRX distance=100m              Figure 2.4-4: EVM of 256QAM when TXRX distance=400m

Observation 1: With the increase of distance between BS and NR UE, the number of guard RB required increases gradually. When the distance is up to 400m, the Guard RB is 4 for 256QAM of NR UE.

In addition, we gradually moved the NR UE away from the BS, increasing the distance by 50 meters each time, from 100 meters until the distance between the NR UE and the BS reached 500 meters and adjusting the bit rate of the A-IoT device. We calculate the values of averaged EVM for every RB to get the Guard RB number between NR and A-IoT compared with 3.5% of EVM. The result is shown as follows.

Figure 2.4-5: Bit rate of A-IoT device vs EVM of distance

Observation 2: There is no need to guard RB when the distance between BS and NR UE ≤ 400m and the bit rate ≤14kbps.

- A-IoT device UL performance (NR UE UL as aggressor)

Similarly, we gradually moved the NR UE away from BS, increasing the distance by 50 meters each time, from 100 meters until the distance between the NR UE and BS reached 350 meters. At the same time, we also get the result is shown in figure 2.4-7 from 10meters to 90meters.We calculate the values of averaged EVM for every RB to get the guard RB number between NR and A-IoT compared with 30% of EVM. The result is shown in Figure 2.4-6.We can see that when the distance is fixed, EVM decreases as the number of guard RB increases. And EVM is gradually reduced with the distance is increasing when the number of guard BR is fixed.

Number of Guard RB

Number of Guard RB

Figure 2.4-6: Different distance between BS and NR UE vs Guard RB          Figure 2.4-7: Different distance between BS and NR UE vs Guard RB

Observation 3: EVM is below 30% when the number of guard BR is 3 and the distance between BS and NR UE is 10meters.

In addition, we also simulated the change of BER of A-IoT device with guard RB at different distances between UE and gNB. The code rate of A-IoT is 42kbps, the result is shown in figure 2.4-9.

Number of Guard RB

Figure 2.4-8: Guard RB with In-band NR                                  Figure 2.4-9: BER of A-IoT device with different distance and guard RB

Observation 4: When the number of guard RB is5 and the distance between BS and NR UE is 10meters, the BER of A-IoT device is lower than 10-3.

Multiple A-IoT operators co-existence

To improve A-IoT system capacity and spectrum efficiency, FDMA should be studied for R2D transmission, especially for multiple A-IoT operators’ coexistence scenario. FDMA divides different frequency resource units in the frequency domain which are used for data transmission by different devices. Ideally, the frequency resource units should not overlap with each other, and a guard band may be needed between adjacent frequency resource units.

However, due to relatively large RF filter (Matching Network) bandwidth in front of RF envelop detection, e.g., 10MHz, reader may have to reserve at least 10MHz system bandwidth for A-IoT R2D transmission to avoid any interference falling into the RF envelop detector, i.e., Bsys,R2D = 10MHz. Specifically, within the wide system bandwidth, only one R2D signal can be transmitted at one instance; otherwise, if multiple R2D signals are simultaneously transmitted by FDMA within the wide system bandwidth, all others R2D signals will be absorbed by the RF envelop detector, which will greatly affect the performance. Considering that the spectrum allocated for A-IoT systems may be limited, if the total spectrum is less than twice of system bandwidth Bsys,R2D, FDMA may not be feasible without any improvements.

To increase system deployment flexibility and save the spectrum allocated for A-IoT system, FDMA divides different frequency resource units for different operators in the frequency domain which are used for different devices deployed in different operators. That is to say, the frequency resource can divide into two R2D signal channel, as shown in Figure 2.5-1. Two R2D signal channels are allocated at the edge of the A-IoT system bandwidth and for different operators, at the same time defining the system bandwidth Gap in the overlapped frequency resource, guaranteeing that there is no signals transmitted in the system bandwidth Gap, FDMA can still be achieved without introducing any interference between these two R2D channels.

Comparing with only one R2D signal channel within one system bandwidth, the total bandwidth to achieve FDMA is (2×Bsys,R2D – Gap), which obviously saves the spectrum resources and improve spectrum efficiency. Therefore, FDMA should be studied for R2D transmission for multiple A-IoT operators’ coexistence scenario.

Figure 2.5-1: one possible FDM candidate for multiple A-IoT operators scenario

Proposal 7: Multiple A-IoT operators co-existence scenario should be investigated in Rel-19 A-IoT.

Conclusions

In this paper, we provide our views on co-existence between A-IoT and NR/UE, we have made the following observations and proposals.

Proposal 1: For co-existence evaluation, In-band to NR deployment should be the first priority, and guard-band to NR/LTE can be second priority.

Proposal 2: The following aspects and/or scenarios should be given higher priority for in-band coexistence evaluation:

- Outdoor NR UE

- Device1/2a with RF-ED

- For D1T1: R2D in DL spectrum, CW in UL spectrum

- For D2T2: R2D in UL spectrum, CW in DL spectrum (Second priority comparing with D1T1)

Proposal 3: Adopt the Table 1 as the proposed scenarios for in-band coexistence evaluation.

Proposal 4: 900MHz is adopted as the reference frequency for simulation assumption.

Proposal 5: Adopt the following simulation assumptions and parameters in Section 2.2.2 as a starting point.

Proposal 6: The above simulations steps are adopted as a starting point.

Observation 1: With the increase of distance between BS and NR UE, the number of guard RB required increases gradually. When the distance is up to 400m, the Guard RB is 4 for 256QAM of NR UE.

Observation 2: There is no need to guard RB when the distance between BS and NR UE ≤ 400m and the bit rate ≤14kbps.

Observation3: EVM is below 30% when the number of guard BR is 3 and the distance between BS and NR UE is 10meters.

Observation 4: When the number of guard RB is5 and the distance between BS and NR UE is 10meters, the BER of A-IoT device is lower than 10-3.

Proposal 7: Multiple A-IoT operators co-existence scenario should be investigated in Rel-19 A-IoT.

Reference

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things)in NR, Huawei, INC.TSG-RAN 102

- RAN1#116bis, Session notes for 9.4 Study on solutions for Ambient IoT in NR