3GPP TSG-RAN WG4 Meeting # 110-bis									       R4-2404762

Changsha, Hunan Province, China, 15th April – 19th April 2024

Title: 	MSD considerations on CA_n71(2A)

Agenda Item:	5.19.2

Source: 	Murata Manufacturing Co. Ltd

Document for:	Approval

1	Introduction

In the RAN4#110 meeting, a way forward [1] was approved with the following way forward.

In this contribution, we present our views on test point selection and MSD values for CA_n71(2A).

- Discussion

2.1 MSD Test Point Selection

In the RAN4#110 meeting, one company proposed PC3 test point simplification and PC2 test point candidates for CA_n71(2A) [2]. Table 1, extracted from [2], shows test point candidates for CA_n71(2A) PC3 and PC2 REFSENS.

Table 1: CA_n71(2A) REFSENS one UL test point candidates, extracted from [2]

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNC (dB) | Candidate | BCS | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10.0 | 5 (RBstart = 2) TC 1.106 | 22.2 | #1: Legacy worst-case MSD (Obs 1.1) | All | near full overlap IMD3 |
|  |  | 20MHz + 5MHz | 10.0 | 20 (RBstart = 0) TC 1.104 | FFS | #2: Alternative to legacy (Obs.1.3) |  | IMD3 full overlap |
|  |  | 25MHz + 5MHz | 5.0 | 20 (RBstart = 8) TC 1.102 | FFS | #3: Rel-18 UEs that support symmetrical UL/DL CBW (Obs. 1.4) | 4/5 only |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band (Obs. 1.2) |  |  |  |  |  |  |  |  |

The configurations of each test point candidate can be illustrated in Figure 1 below:

Figure 1: Configurations of each test point candidate

Same findings are observed from the above configurations; for candidate #1, SCC DL is affected by near full overlap PCC IM3. Regarding candidate #2 and #3, PCC IM3 fully falls into SCC DL.

Observation 1: The proposed test points in [2] are reasonable.

2.2 MSD Consideration

Based on proposals 1 and 2, we performed MSD estimation for CA_n71(2A) PC3, PC2 1Tx, and PC2 2Tx.

2.2.1 Power Amplifier Calibration

For PC3, PA is calibrated to deliver -30 dBc ACLR using full allocation 20 MHz with DFTs-QPSK at MPR 1dB.

For PC2, PA is calibrated to deliver -31dBc ACLR using full allocation 20MHz with DFTs-QPSK at MPR 1dB.

2.2.2 Tx Noise Levels

PC3 and PC2(1x26dBm) Tx Noise levels affecting the Rx band of SCC are summarized in Table 2.

Table 2: PC3 and PC2(1x26dBm) Tx noise levels.

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | PC3 | PC2 (1x26dBm) |
| --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  | [dBm/MBW] | [dBm/MBW] |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10 | 5 (RBstart = 2) | -17.2 | -14.3 |
|  |  | 20MHz + 5MHz | 10 | 20 (RBstart = 0) | -15.6 | -13.7 |
|  |  | 25MHz + 5MHz | 5 | 20 (RBstart = 8) | -15.5 | -13.8 |

2.2.3 MSD Estimation

We evaluated MSD values for CA_n71(2A) PC3, PC2(1x26dBm), and PC2(2x23dBm) using the following assumptions:

- Tx-Rx duplexer isolation: 55dB

- Tx-antenna duplex rejection in Rx range: 45 dB

- Primary antenna to diversity antenna isolation: 10 dB

- MRC combining and -1 dB Signal to-Noise Ratio (SNR)

Figure 2: CA_n71(2A) PC3, PC2(1x26dBm) and PC2(2x23dBm) MSD analysis.

Proposal 1: The following MSD values are proposed to take into consideration for CA_n71(2A) PC3, PC2(1x26dBm) and PC2(2x23dBm).

Table 3: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands for 7.3A.2.2-1

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 20MHz + 5MHz(NOTE 11) | Wgap=10.0 | 20 (RBstart = 0) | [27.0] | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) | [27.1] |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |  |

Table 4: intra-band non-contiguous CA with one uplink configuration for reference sensitivity for PC2 in FDD bands-Table 7.3A.2.2-1a

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | BCS | IMD order | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10 | 5 (RBstart = 2) | [25.1] | [26.3] | All | near full overlap IMD3 | FDD |
|  |  | 20MHz + 5MHz | 10 | 20 (RBstart = 0) | [28.9] | [31.1] | 4/5 only | IMD3 full overlap | FDD |
|  |  | 25MHz + 5MHz | 5 | 20 (RBstart = 8) | [28.8] | [31.2] |  |  |  |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

- Conclusion

In this paper, we considered the test point to CA_n71(2A) and provided the MSD estimation results.

Observation 1: The proposed test points in [2] are reasonable.

Proposal 1: The following MSD values are proposed to take into consideration for CA_n71(2A) PC3, PC2(1x26dBm) and PC2(2x23dBm).

Table 3: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands for 7.3A.2.2-1

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap /[MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap=10.0 | 5 (RBstart = 2) | 22.2 | FDD |
|  |  | 20MHz + 5MHz(NOTE 11) | Wgap=10.0 | 20 (RBstart = 0) | [27.0] | FDD |
|  |  | 25MHz + 5MHz(NOTE 11) | Wgap=5.0 | 20 (RBstart = 8) | [27.1] |  |
| NOTE 7: The carrier centre frequency of SCC in the DL operating band is configured closer to the UL operating band. NOTE 11: Bandwidth Combination Set 4/5 |  |  |  |  |  |  |

Table 4: intra-band non-contiguous CA with one uplink configuration for reference sensitivity for PC2 in FDD bands-Table 7.3A.2.2-1a

| CA configuration | SCS | Aggregated channel bandwidth (PCC+SCC) | Wgap [MHz] | UL PCC allocation | ΔRIBNCX (dB) | ΔRIBNCY (dB) | BCS | IMD order | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | (kHz) |  |  |  |  |  |  |  |  |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | 10 | 5 (RBstart = 2) | [25.1] | [26.3] | All | near full overlap IMD3 | FDD |
|  |  | 20MHz + 5MHz | 10 | 20 (RBstart = 0) | [28.9] | [31.1] | 4/5 only | IMD3 full overlap | FDD |
|  |  | 25MHz + 5MHz | 5 | 20 (RBstart = 8) | [28.8] | [31.2] |  |  |  |
| NOTE X: Applicable to UE supporting PC2 with single Tx. NOTE Y: Applicable to UE supporting PC2 with dual Tx. |  |  |  |  |  |  |  |  |  |

- References

- R4-2403628, WF on PC2 HPUE MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc., Huawei, Qualcomm Inc.

- R4-2400365, PC2 CA_n71(2A) MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc.