| 3GPP TR 37.718-11-11 V1.3.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-18 Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR Maintenance	8

5	DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL): General Part	9

5.1 	Maximum Sensitivity Degradation (MSD) analysis	9

5.1.1	 Inter-band EN-DC/NE-DC within FR1	9

5.1.2	 Inter-band EN-DC/NE-DC including FR2	9

5.2 	Interference analysis for other system	9

5.3 	Spurious emission band UE co-existence for EN-DC/NE-DC	10

5.4 	Handling of Band specific requirements for inter-band EN-DC including FR2	10

5.5 	Table Format for ∆TIB and ∆RIB values	10

6	DC of 1 LTE band (1DL/1UL) and 1 NR band: Specific Band Combination Part	12

6.1	Inter-band DC within FR1	12

6.1.1	DC_8_n78	12

6.1.2	DC_3_n28	14

6.1.3	DC_1_n26	15

6.1.4	DC_3_n26	17

6.1.5	DC_7_n26	19

6.1.6	DC_28_n20	21

6.1.7	DC_28_n38	23

6.1.8	DC_8_n38	25

6.1.9	DC_5_n25	27

6.1.10	DC_5_n41	28

6.1.11	DC_71_n25	31

6.1.12	DC_7_n12	34

6.1.13	DC_71_n7	35

6.1.14	DC_1_n105	38

6.1.15	DC_3_n105	41

6.1.16	DC_7_n105	42

6.1.17	DC_71_n12	45

6.1.18	DC_5_n28	47

6.1.19	DC_14_n41	49

6.1.20	DC_40_n3	51

6.1.21	DC_40_n7	53

6.1.22	DC_20_n40	55

6.1.23	DC_8_n1	57

6.1.24	DC_41_n79	59

6.2	Inter-band DC including FR2	62

6.2.1	DC_X_nY	62

6.3	Intra-band contiguous DC	63

6.3.1	 DC_3(n)	63

6.4	Intra-band non-contiguous DC	64

6.4.1	DC_1_n1	64

Annex A (informative): Change history	66

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is a technical report for Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) under Rel-18 time frame.

The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) for the Rel-18 band combinations in Table 1-1, 1-2, 1-3 and 1-4, including EN-DC and NE-DC combinations. The actual requirements are added to the corresponding technical specifications.

Table 1-1: Release 18 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink EN-DC configuration |
| --- | --- |
| DC_3(n)AA | DC_3(n)AA1 |
| DC_8B_n78A | DC_8A_n78A DC_8B_n78A |
| DC_3C_n28A | DC_3C_n28A |
| DC_1A_n26A | DC_1A_n26A |
| DC_3A_n26A | DC_3A_n26A |
| DC_3C_n26A | DC_3A_n26A DC_3C_n26A |
| DC_7A_n26A | DC_7A_n26A |
| DC_7C_n26A | DC_7A_n26A DC_7C_n26A |
| DC_28A_n20A | DC_28A_n20A |
| DC_1A_n1A | DC_1A_n1A1 |
| DC_28A_n38A | DC_28A_n38A |
| DC_8A_n38A | DC_8A_n38A |
| DC_5A_n25A | DC_5A_n25A |
| DC_5A_n41A | DC_5A_n41A |
| DC_71A_n25A | DC_71A_n25A |
| DC_7A_n12A | DC_7A_n12A |
| DC_71A_n7A | DC_71A_n7A |
| DC_1A_n105A | DC_1A_n105A |
| DC_3A_n105A | DC_3A_n105A |
| DC_7A_n105A | DC_7A_n105A |
| DC_71A_n12A | DC_71A_n12A1 |
| DC_5A_n28A | DC_5A_n28A |
| DC_14A_n41A | DC_14A_n41A |
| DC_40A_n3A | DC_40A_n3A |
| DC_40A_n7A | DC_40A_n7A |
| DC_20A_n40A | DC_20A_n40A |
| DC_8B_n1A | DC_8B_n1A |
| DC_41A_n79C | DC_41A_n79C |
| NOTE 1: Only single switched UL is supported |  |

Table 1-2: Release 18 EN-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink EN-DC configuration |
| --- | --- |

Table 1-3: Release 18 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) within FR1

| DC combination | Uplink NE-DC configuration |
| --- | --- |

Table 1-4: Release 18 NE-DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL) including FR2

| DC combination | Uplink NE-DC configuration |
| --- | --- |

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[4]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for Dual Connectivity (DC) of 1 LTE band and 1 NR band under Rel-18 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	DC of 1 LTE band (1DL/1UL) and 1 NR band (1DL/1UL): General Part

5.1 	Maximum Sensitivity Degradation (MSD) analysis

5.1.1	 Inter-band EN-DC/NE-DC within FR1

For each inter-band EN-DC/NE-DC band combination, sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference, harmonic mixing, cross band isolation, and intermodulation interference from another band part of the same DC configuration. For 2UL inter-band DC, the harmonic and IMD products can be calculated by the formula shown in Table 5.1.1-1.

Table 5.1.1-1: Formulation for Harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |

Note that, for each DC band combination, MSD table shall be provided for interference that requires sensitivity degradation for an Rx band according to the same manner as specified in section 7.3B in TS 38.101-3.

5.1.2	 Inter-band EN-DC/NE-DC including FR2

For each inter-band EN-DC/NE-DC band combination, sensitivity degradation is allowed for a band if it is impacted by UL harmonic interference from another band part of the same DC configuration. Note that for each DC band combination in section 6, MSD table shall be provided for interference that requires sensitivity degradation for a Rx band according to the same manner as specified in section 7.3B in TS 38.101-3.

5.2 	Interference analysis for other system

When 2UL inter-band EN-DC/NE-DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems shown in Table 5.2-1.

Table 5.2-1: Harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Harmonic/IMD |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 |  |  |  |
| Galileo | 1559 | - | 1591 |  |  |  |
| GLONASS | 1591 | - | 1610 |  |  |  |
| GPS | 1563 | - | 1587 |  |  |  |
| ISM band (2.4GHz) | 2400 | - | 2483.5 |  | US/Europe |  |
|  | 2400 | - | 2494 |  | Asia |  |
| ISM band (5GHz) | 5150 | - | 5925 |  | US |  |
|  | 5150 | - | 5350 |  | Europe |  |
|  | 5470 | - | 5725 |  |  |  |
|  | 5150 | - | 5825 |  | Asia |  |

To mitigate the impact of interference to other system is implementation dependent, thus it is optional in section 6 for each DC band combination.

5.3 	Spurious emission band UE co-existence for EN-DC/NE-DC

Table 5.3-1 specifies the requirements for the specified EN-DC configurations for coexistence with protected bands.

Table 5.3-1: Spurious emissions for uplink inter-band EN-DC (two bands)

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_XA_nYA | E-UTRA Band xx, yy, zz NR band aa, bb, cc | FDL_low | - | FDL_high | -50 | 1 |  |
| NOTE1: The protected E-UTRA/NR bands are the common set of the protected bands for the constituted bands. |  |  |  |  |  |  |  |

Note that for the NE-DC configurations that have a corresponding specified EN-DC configuration, the requirements in the Table 6.5B.3.3.2-1 of TS 38.101-3 apply on each component carrier with all component carriers are active.

5.4 	Handling of Band specific requirements for inter-band EN-DC including FR2

Unless otherwise stated, for inter-band EN-DC configurations including FR2, there are no additional requirements of spurious emission band UE co-existence, MSD analysis, and ∆TIB and ∆RIB values.

- For spurious emission band UE co-existence, no requirements for FR2 NR bands to protect E-UTRA and FR1 NR bands are applied to the constituent FR2 NR bands. Spurious emission band UE co-existence requirements specified in TS 36.101 are applied to the constituent E-UTRA bands for the EN-DC configuration.

- For spurious emission band UE co-existence table, although the requirements are specified, it seems that they are not testable under the condition RAN4 originally expected. In addition, the impact of the harmonics and IMD issues between bands below 6GHz and bands above 24.25GHz inside UEs had been ignored. Hence at least the co-existence between bands below 6GHz and bands above 24.25GHz should not have any issues if the assumption that no impact of harmonic and IMD on co-existence between FR1 and FR2 is right.

- MSD for E-UTRA and FR2 NR bands of inter-band DC combinations is set to N/A.

- ΔTIB,c and ΔRIB,c for E-UTRA and FR2 NR bands of inter-band EN-DC combinations is set to zero.

5.5 	Table Format for ∆TIB and ∆RIB values

For inter-band EN-DC configurations, the table format for ∆TIB and ∆RIB values is shown in Table 5.5-1 and Table 5.5-2.

Table 5.5-1:  ΔTIB,c due to EN-DC (two bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| DC_X_nY |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 5.5-2:  ΔRIB,c due to EN-DC (two bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| DC_X_nY |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_1_n77 the band order from left to right is 1 and n77. |  |  |

For inter-band NE-DC configurations, the table format for ∆TIB and ∆RIB values is shown in Table 5.5-3 and Table 5.5-4.

Table 5.5-3: New template for ΔTIB,c due to NE-DC (two bands)

| Inter-band NE-DC configuration | ΔTIB,c for NR band / E-UTRA band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| DC_nx_y |  |  |
| NOTE *: “-” denotes ΔTIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

Table 5.5-4: New template for ΔRIB,c due to NE-DC (two bands)

| Inter-band NE-DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)* |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration** |  |
| DC_nx_y |  |  |
| NOTE *: “-” denotes ΔRIB,c = 0. NOTE **: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

6	DC of 1 LTE band (1DL/1UL) and 1 NR band: Specific Band Combination Part

6.1	Inter-band DC within FR1

6.1.1	DC_8_n78

6.1.1.1	Configuration for DC

Table 6.1.1.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_8B_n78A7 | DC_8A_n78A DC_8B_n78A | No |
| NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |  |

6.1.1.2	Maximum output power for DC

Table 6.1.1.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8A_n78A DC_8B_n78A | 23 | +2/-3 |

6.1.1.3	Spurious emission band UE co-existence for DC

Note that the spurious emission band UE co-existence for DC_8A_n78A is already specified in the Rel.15 specification, no additional impact is expected.

6.1.1.4	MSD analysis for DC

Table 6.1.1.4-1 lists Band 8 +Band n78 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.1.4-1: Band 8 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 9900 | 11400 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 13200 | 15200 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 16500 | 19000 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2385 | 2920 | 4180 | 4715 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2040 | 1470 | 5685 | 6720 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5060 | 5630 | 7480 | 8515 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1160 | 555 | 8985 | 10520 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5940 | 6545 | 10780 | 12315 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 5840 | 4770 | 8360 | 9430 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 14320 | 12285 | 360 | 280 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14080 | 16115 | 6820 | 7460 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9640 | 8070 | 3855 | 4960 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11660 | 13230 | 9240 | 10345 |

Table 6.1.1.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 8 and band n78,

Table 6.1.1.4-2: Band 8 and Band n78 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 | 13200 | 15200 | 16500 | 19000 |

Based on Table 6.1.1.4-1, 6.1.1.4-2,

- 4rd order harmonics generated Band 8 uplink by may fall into Rx frequencies of band n78

- 4rd order IMD generated by dual uplink of Band 8 + Band n78 may fall into own Rx of band 8

The corresponding MSD requirements for the harmonics are already specified in the specifications when introducing DC_8A_n78A, no additional MSD requirement is needed for DL DC_8B_n78A with UL DC_8A_n78A. For the MSD requirements for IMD interference of DC_8B_n78A, the requirements for DC_8A_n78A can be referred as below.

And note that exceptions for UL1/DL4 harmonic mixing are not specified in the specifications.

Table 6.1.1.4-3: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8B_n78A | 8 | 897.5 | 5 | 25 | 942.5 | 8.3 | FDD | IMD4 |
|  | n78 | 3635 | 10 | 50 | 3635 | N/A | TDD | N/A |

Based on the WF R4-2220556, the pre-condition of the triple beat analysis for the two bands inter-band UL EN-DC with intra-band UL is that the two uplink bands are part of the same band group or belong to adjacent band groups as defined in Table 6.1.1.6-1

Table 6.1.1.6-1 : Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

Since the two bands in the UL DC_8B_n78A are not in the same band group or belong to adjacent band groups in the above table, it can be concluded that there is no triple beat issue, and no additional MSD is needed.

6.1.1.5	∆TIB and ∆RIB values

For DC_8B_n78A, the TIB,c and RIB values are already covered by the DC_8_n78, as in the tables below.

Table 6.1.1.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_8_n78 | 0.6 | 0.8 |

Table 6.1.1.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_8_n78 | 0.2 | 0.5 |

6.1.1.6		Self-interference analysis

The analysis for DC_8A_n78A in TR 37.863-01-01 section 6.41.6 can be referred.

6.1.2	DC_3_n28

6.1.2.1	Configuration for DC

Table 6.1.2.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3C_n28A | DC_3C_n28A | No |

As DL_3C_n28A_UL_3A_n28A has been completed since Rel-16, in Rel-18 we aim to introduce configuration DL_3C_n28A_UL_3C_n28A.

6.1.2.2	Maximum output power for DC

Table 6.1.2.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3C_n28A | 23 | +2/-3 |

6.1.2.3	Spurious emission band UE co-existence for DC

For DC_3_n28, the corresponding spurious emission band UE co-existence for DC have been specified in TS 38.101-3-h60. There is no need to further study it for DL_3C_n28A_UL_3C_n28A.

6.1.2.4	MSD analysis for DC

Referring to clause 7.3.3.2 from TR 38.862 [Y], it’s known how to detect a triple beat issue in the FDD band DL or a third band DL.

Mainly consider the 1st order triple beat α (TX22TX1) of 3rd order non-linearity -> this is shown in the table as triple beat order 1.

-  Detection -> Duplex Offset<1∙ULCAAggBW+TXMBW2 +RXBW2.

For 20MHz+20MHz band 3C, the LTE nominal channel spacing is 19.8MHz and the UL CA aggregated channel bandwidth is 39.8MHz.

Since UL configuration of band n28 was restricted as 25RB for each DL bandwidth REFSENS requirement, the UL RB allocation for band n28 can be 25RB, but closet to DL Rx band. It’s agreed to assume 25MHz channel bandwidth for band n28 in WF R4-2214426. Thus, the following MSD test configuration and value are proposed as below.

Table 6.1.2.4-1: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ENDC band combination | NR/LTE band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
|  | n28 | 715.5 | 25 | 25(RBSTART=108) | 770.5 | 11 | FDD | 1st order triple beat α (TX22TX1) i.e. IMD3 |
| DC_3C_n28AX | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 |  |  |  |
| NOTE X: This test configuration to ensure the B3 self-interference would not interrupt the testing. |  |  |  |  |  |  |  |  |

6.1.2.5	∆TIB and ∆RIB values

For DC_3_n28, the corresponding ∆TIB and ∆RIB values have been specified in TS 38.101-3-h60. There is no need to further study it for DL_3C_n28A_UL_3C_n28A.

6.1.3	DC_1_n26

6.1.3.1	Configuration for DC

Table 6.1.3.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_1A_n26A | DC_1A_n26A | No |

6.1.3.2	Maximum output power for DC

Table 6.1.3.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_1A_n26A | 23 | +2/-3 |

6.1.3.3	Spurious emission band UE co-existence for DC

Table 6.1.3.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_1_n26 | E-UTRA Band 1, 3, 5, 11, 18, 19, 21, 26, 31, 40, 42, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 738 |  | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | 803 | -40 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.3.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.3.4-1

Table 6.1.3.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1166 | 1071 | 2734 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 352 | 222 | 2991 | 3146 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3548 | 3678 | 4654 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2332 | 2142 | 5468 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 462 | 627 | 4911 | 5126 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4527 | 6574 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7106 | 6831 | 1476 | 1276 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + *fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8494 | 8769 | 5176 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – *fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4312 | 4062 | 1293 | 1518 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7388 | 7638 | 6282 | 6507 |

Based on Table 6.1.3.4-1, it can be seen that

- 2nd harmonics might fall into Rx frequencies of band n77

- 3rd harmonics might fall into Rx frequencies of band 41, 46 and 47.

- 3rd order IMD products may fall into Rx frequencies of band 22, 42, 43, 48, 49, n77, n78 and n79.

- 4th order IMD products may fall into Rx frequencies of band 4, 10, 23, 40, 46, 65, 66, 71 and n79, and also own Rx frequencies of band 1.

- 5th order IMD products may fall into Rx frequencies of band 11, 32, 45, 46, 50, 74, 75, 76 and n77.

Based on above, the 4th order IMD may fall into Rx frequencies of band n1.

Based on the similarities with the existing EN-DC combination DC_1A_n5A, there is no need to add MSD value.

6.1.3.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_1_n5 as specified in TS 38.101-3.

Table 6.1.3.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_1_n26 | 0.3 | 0.3 |

Table 6.1.3.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_1_n26 | 0 | 0 |

6.1.4	DC_3_n26

6.1.4.1	Configuration for DC

Table 6.1.4.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3A_n26A | DC_3A_n26A | Yes |
| DC_3C_n26A | DC_3A_n26A DC_3C_n26A | Yes |

6.1.4.2	Maximum output power for DC

Table 6.1.4.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n26A DC_3C_n26A | 23 | +2/-3 |

6.1.4.3	Spurious emission band UE co-existence for DC

Table 6.1.4.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_3_n26 | E-UTRA Band 1, 5, 11, 18, 19, 21, 26, 31, 39, 40, 43, 50, 51, 65, 73, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 42 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.4.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.4.4-1

Table 6.1.4.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 6840 | 7140 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 971 | 861 | 2524 | 2634 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low+1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low –3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low –3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high +3*fy_high¦ | ¦2*fy_low+ 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

Table 6.1.4.4-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n26 | 814 | 849 | 859 | 894 | 1718 | 1788 | 2577 | 2682 | 3436 | 3576 | 4295 | 4470 |

Based on Table 6.1.4.4-1, it can be seen that

- 2nd order IMD products may fall into Rx frequencies of band 26.

- 4th order IMD products may fall into Rx frequencies of band 3.

- 5th order IMD products may fall into Rx frequencies of bands 26.

Based on Table 6.1.4.4-2, it can be seen there is the 2nd harmonic mixing. However, no MSD is required for this 2nd harmonic mixing due to the DL victim band is a low band.

Based on the co-existence studies above, MSD is required. MSD values are defined based on discussions in R4-2219700 and reusing from DC_3_n5.

Table 6.1.4.4-2: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_3A_n26A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | n26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
|  | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | n26 | 838 | 5 | 25 | 883 | 24 |  | IMD23 |
| DC_3C_n26A | 3 | 1720 | 20 | 1 (RBSTART=0) | 1815 | N/A | FDD | N/A |
|  |  | 1739.8 | 20 | 1 (RBSTART=99) | 1834.8 | N/A |  |  |
|  | n26 | 841.5 | 15 | 25(RBSTART=54) | 886.5 | 18.9 | FDD | IMD3 |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.1.4.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_3_n5 as specified in TS 38.101-3.

Table 6.1.4.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_3_n26 | 0.3 | 0.3 |

Table 6.1.4.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_3_n26 | 0 | 0 |

6.1.5	DC_7_n26

6.1.5.1	Configuration for DC

Table 6.1.5.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A_n26A | DC_7A_n26A | Yes |
| DC_7C_n26A | DC_7A_n26A DC_7C_n26A | Yes |

6.1.5.2	Maximum output power for DC

Table 6.1.5.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n26A DC_7C_n26A | 23 | +2/-3 |

6.1.5.3	Spurious emission band UE co-existence for DC

Table 6.1.5.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7_n26 | E-UTRA Band 1, 2, 3, 4, 5, 12, 13, 14, 17, 26, 29, 30, 31, 34, 40, 42, 43, 50, 51, 65, 66, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

6.1.5.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.5.4-1

Table 6.1.5.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1756 | 1651 | 3314 | 3419 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 942 | 802 | 4151 | 4326 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low+1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low –3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low –3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high +3*fy_high¦ | ¦2*fy_low+ 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

Based on Table 6.1.5.4-1, it can be seen that

- 3rd order IMD products may fall into Rx frequencies of band n26.

- 5th order IMD products may fall into Rx frequencies of bands n26 and 7.

Based on the co-existence studies above, MSD is required. MSD values can be reused from DC_7_n5.

Based on WF R4-2217096:

- For UL DC_7C_n26A, a 1st order TB product may affect band 26 DL channel.

- However, the 2 UL bands are not part of the same band group or do not belong to adjacent band groups. Therefore, there is no need to capture a triple-beat MSD test point

Table 6.1.5.4-2: Reference sensitivity exceptions for PCell due to dual uplink operation for EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_7A_n26A DC_7C_n26A | 7 | 2547 | 10 | 50 | 2667 | N/A | FDD | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 12 |  | IMD33 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD | IMD5 |
|  | n26 | 816.5 | 5 | 25 | 861.5 | N/A |  | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

6.1.5.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_7_n5 as specified in TS 38.101-3.

Table 6.1.5.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_7_n26 | 0.3 | 0.3 |

Table 6.1.5.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_7_n26 | 0 | 0 |

6.1.5.6	REFSENS requirements

Based on the co-existence studies in [R4-2216088], there are 3rd harmonic mixing near miss from band n26 DL into band n7 UL MSD value based on the Skyworks discussion paper R4-2215516: MSD for DC_7_n26.

Table 6.1.5.6-1: Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL EN-DC FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 7 | n263 | 20 | 15 | 25 (RBstart=75) | 5 | [2.0] | NOTE X | UL1/DL3 near-miss |
| NOTE X: The requirements should be verified for the lowest NR ARFCN of the affected DL (lower) band and for the highest NR ARFCN of the UL (higher) band |  |  |  |  |  |  |  |  |

6.1.6	DC_28_n20

6.1.6.1	Configuration for DC

Table 6.1.6.1-1: Inter-band DC configurations (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed | DL interruption allowed (Note 14) |
| --- | --- | --- | --- |
| DC_28A_n20A8,11,13 | DC_28A_n20A | No |  |
| NOTE 8: The frequency range in band n28 / 28 is restricted for this band combination to 703 - 733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when DC_20_n28/ DC_28_n20/ CA_20-28/ CA_n20-n28 is a subset of a higher order band combination. NOTE 11: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for apply when the maximum power spectral density imbalance between downlink carriers is within 6 dB. For UEs indicating interBandMRDC-WithOverlapDL-Bands-r16, the power imbalance requirement defined in clause 7.6B.2.6 apply. For these UEs, the power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration NOTE 13: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements apply for synchronized DL carriers with a maximum receive time difference ≤ 3 usec. The requirements also apply for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

6.1.1.2	Maximum output power for DC

Table 6.1.6.2-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_28A_n20A |  |  |  |  | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |

6.1.1.3	Spurious emission band UE co-existence for DC

The requirements for spurious emission band UE coexistence that exist for DC_20_n28 in 38.101-3 can be re-used for DC_28_n20 as per table 6.1.6.3-1.

Table 6.1.6.3-1: 2UL Band 28 + Band n20 Spurious Emission Coexistence Requirements

| EN-DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n20 | E-UTRA Band 3, 7, 8, 31, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 32, 38, 42, 43, 65, 75, 76 | FDL_low | - | FDL_high | -50 | 1 | 2 |

6.1.6.4	MSD analysis for DC

Table 6.1.6.4-1 lists the Band 28A + Band n20A 2UL DC 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.6.4-1: Band 28 and Band n20 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fn_low | fn_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fn_low | 2* fn_high |
| 2nd harmonics frequency limits (MHz) | 1406 – 1496 |  | 1664 – 1724 |  |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fn_low | 3* fn_high |
| 3rd harmonics frequency limits (MHz) | 2109 – 2244 |  | 2496 – 2586 |  |
| 2nd order IMD products | ¦fn_low – fx_high¦ | ¦fn_high – fx_low¦ | ¦fn_low + fx_low¦ | ¦fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 84 – 159 |  | 1535 – 1610 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fn_high¦ | ¦2*fx_high – fn_low¦ | ¦2*fn_low – fx_high¦ | ¦2*fn_high – fx_low¦ |
| IMD frequency limits (MHz) | 544 – 664 |  | 916 – 1021 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fn_low¦ | ¦2*fx_high + fn_high¦ | ¦2*fn_low + fx_low¦ | ¦2*fn_high + fx_high¦ |
| IMD frequency limits (MHz) | 2238 – 2358 |  | 2367 – 2472 |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fn) | (fx_high + max BW fn) | (fn_low – max BW fx) | (fn_high + max BW fx) |
| IMD frequency limits (MHz) | 683 – 768 |  | 812 – 882 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fn_high¦ | ¦3*fx_high – 1*fn_low¦ | ¦3*fn_low – 1*fx_high¦ | ¦3*fn_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1247 – 1412 |  | 1748 – 1883 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fn_high¦ | ¦2*fx_high –2* fn_low¦ | ¦2*fx_low +2* fn_low¦ | ¦2*fx_high +2* fn_high¦ |
| IMD frequency limits (MHz) | 168 – 318 |  | 3070 – 3220 |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fn_low¦ | ¦3*fx_high + 1*fn_high¦ | ¦3*fn_low + 1*fx_low¦ | ¦3*fn_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2941 – 3106 |  | 3199 – 3334 |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fn_high¦ | ¦fx_high – 4*fn_low¦ | ¦fn_low – 4*fx_high¦ | ¦fn_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2580 – 2745 |  | 1950 – 2160 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fn_high¦ | ¦2*fx_high - 3*fn_low¦ | ¦2*fn_low - 3*fx_high¦ | ¦2*fn_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1000 – 1180 |  | 385 – 580 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fn_low¦ | ¦fx_high + 4*fn_high¦ | ¦fn_low + 4*fx_low¦ | ¦fn_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4031 – 4196 |  | 3644 – 3854 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fn_low¦ | ¦2*fx_high + 3*fn_high¦ | ¦2*fn_low + 3*fx_low¦ | ¦2*fn_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3902 – 4082 |  | 3773 – 3968 |  |

Based on Table 6.1.6.4-1,

-	2nd order harmonics may fall into Rx frequencies of bands 1, 4, 10, 23, 65 and 66.

-	3rd order harmonics may fall into Rx frequencies of band 11, 21, 32, 38, 41, 45, 50, 51, 69, 74, 75, 76, 90, 91, 92, 93 and 94.

-	2nd order IMD may fall into Rx frequencies of band 24.

-	3rd order IMD may fall into Rx frequencies of bands 8, 30, 40 and 71.

-	4th order IMD may fall into Rx frequencies of bands 3, 9, 35, 39, 52, 77 and 78.

-	5th order IMD may fall into Rx frequencies of bands 1, 2, 4, 7, 10, 25, 31, 34, 36, 38, 41, 43, 48, 49, 65, 66, 69, 70, 72, 73, 77, 78, 87, 88 and 90.

When a 2UL inter-band DC UE is operating with other systems such as Wi-Fi, Bluetooth and GNSS, the harmonics and intermodulation products can have an impact on these systems. Table 6.1.6.4-2 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.6.4-2: 2UL Band 28 + Band n20 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD2 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD2 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD2 |
| GPS | 1563 | - | 1587 | Yes |  | IMD2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | Yes | US/Europe | IMD3 |
|  | 2400 | - | 2494 | Yes | Asia | IMD3 |
| ISM band (5GHz) | 5150 | - | 5925 | No | US |  |
|  | 5150 | - | 5350 | No | Europe |  |
|  | 5470 | - | 5725 | No |  |  |
|  | 5150 | - | 5825 | No | Asia |  |

6.1.6.5	∆TIB and ∆RIB values

Table 6.1.6.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_28_n20 | 0.5 | 0.5 |

Table 6.1.6.5-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_28_n20 | - | - |

6.1.7	DC_28_n38

6.1.7.1	Configuration for DC

Table 6.1.7.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_28A_n38A | DC_28A_n38A | No |

6.1.7.2	Maximum output power for DC

Table 6.1.7.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_28A_n38A | 23 | +2/-3 |

6.1.7.3	Spurious emission band UE co-existence for DC

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_28_n38 | E-UTRA Band 2, 3, 5, 8, 20, 27, 31, 34, 40, 52, 72 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 9, 11 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.7.4	MSD analysis for DC

Table 6.1.7.4-1 lists Band 28 +Band n38 2UL DC up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.7.4-1: Band 28 and Band n38 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 2570 | 2620 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1917 | 1822 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1214 | 1074 | 4392 | 4537 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3976 | 4116 | 5843 | 5988 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3834 | 3644 | 6546 | 6736 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 511 | 326 | 6962 | 7157 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4679 | 4864 | 8413 | 8608 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9777 | 9532 | 422 | 192 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10983 | 11228 | 5382 | 5612 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6454 | 6214 | 2896 | 3131 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9116 | 9356 | 7249 | 7484 |

Table 6.1.7.4-2 lists up to 5th order harmonics products for dual connectivity between band 28 and band n38,

Table 6.1.7.4-2: Band 28 and Band n38 UL harmonic products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 2812 | 2992 | 3515 | 3740 |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 | 12850 | 13100 |

Table 6.1.7.4-3 lists up to 5th order harmonics mixing products for dual connectivity between band 28 and band n38,

Table 6.1.7.4-3: Band 28 and Band n38 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 28 | 703 | 748 | 758 | 803 | 1516 | 1606 | 2274 | 2409 | 3032 | 3212 | 3790 | 4015 |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 | 12850 | 13100 |

6.1.7.5	∆TIB and ∆RIB values

For DC_28_n38, the TIB,c and RIB values from CA_n28-n38 are reused, see tables below.

Table 6.1.7.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_28_n38 | 0.3 | 0.3 |

Table 6.1.7.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_28_n38 | 0 | 0 |

6.1.7.6		Self-interference analysis

Based on the results in Table 6.1.7.4-1, 6.1.7.4-2 and 6.1.7.4-3 there are no MSD requirements needed.

6.1.8	DC_8_n38

6.1.8.1	Configuration for DC

Table 6.1.8.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_8A_n38A | DC_8A_n38A | No |

6.1.8.2	Maximum output power for DC

Table 6.1.8.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8A_n38A | 23 | +2/-3 |

6.1.8.3	Spurious emission band UE co-existence for DC

Table 6.1.8.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_8A_n38A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 40, 50, 51, 65, 67, 68, 72, 74, 75, 76 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 22, 42, 43, 52 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 5, 7, 22 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 5, 22 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 7: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. |  |  |  |  |  |  |  |

6.1.8.4	MSD analysis for DC

Table 6.1.8.4-1 lists Band 8 +Band n38 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.8.4-1: Band 8 and Band n38 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 880 | 915 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 12850 | 13100 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1655 | 1740 | 3450 | 3535 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 860 | 740 | 4225 | 4360 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4330 | 4450 | 6020 | 6155 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20 | 175 | 6795 | 6980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3480 | 3310 | 6900 | 7070 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5210 | 5365 | 8590 | 8775 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9600 | 9365 | 1090 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6100 | 5880 | 2395 | 2600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11160 | 11395 | 6090 | 6280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9470 | 9690 | 7780 | 7985 |

Table 6.1.8.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 8 and band n38,

Table 6.1.8.4-2: Band 8 and Band n38 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 8 | 880 | 915 | 925 | 960 | 1850 | 1920 | 2775 | 2880 | 3700 | 3840 | 4625 | 4800 |
| n38 | 2570 | 2620 | 2570 | 2620 | 5140 | 5240 | 7710 | 7860 | 10280 | 10480 | 12850 | 13100 |

Based on Table 6.1.8.4-1, 6.1.8.4-2,

- There is no UL Harmonic interference

- 5th order IMD products may fall into Rx frequencies of band 8

- 5th order IMD products may fall into Rx frequencies of band n38, but n38 is a TDD band so no RX during TX.

- There is no harmonic mixing between the two bands

For the MSD requirements for IMD interference of DC_8A_n38A, the requirements for DC_38A_n8A can be reused.

Table 6.1.8.4-3: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_8A_n38A | 8 | 887.5 | 5 | 25 | 932.5 | 8.1 | FDD | IMD5 |
|  | n38 | 2617.5 | 5 | 25 | 2617.5 | N/A | TDD | N/A |

6.1.8.5	∆TIB and ∆RIB values

For DC_8A_n38A, the TIB,c and RIB values reused from CA_n8A-n38A, as in the tables below.

Table 6.1.8.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_8_n38 | 0.6 | 0.3 |

Table 6.1.8.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_8_n38 | - | - |

6.1.9	DC_5_n25

6.1.9.1	Configuration for DC

Table 6.1.9.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_5A_n25A | DC_5A_n25A | No |

6.1.9.2	Maximum output power for DC

Table 6.1.9.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n25A | 23 | +2/-3 |

6.1.9.3	Spurious emission band UE co-existence for DC

Table 6.1.9.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n25 | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 26, 28, 29, 30, 42, 48, 53, 54, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.9.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.9.4-1

Table 6.1.9.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1648 | 1698 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 5550 | 5745 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 7400 | 7660 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1091 | 1001 | 2674 | 2764 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 267 | 152 | 2851 | 3006 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3498 | 3613 | 4524 | 4679 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2182 | 2002 | 5348 | 5528 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 557 | 697 | 4701 | 4921 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4322 | 4462 | 6374 | 6594 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6836 | 6551 | 1546 | 1381 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8224 | 8509 | 5146 | 5311 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4097 | 3852 | 1153 | 1358 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7198 | 7443 | 6172 | 6377 |

Based on Table 6.1.9.4-1, it can be seen there is no IMD and harmonic issues for this combination.

6.1.9.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_n5-n25 as specified in TS 38.101-1.

Table 6.1.9.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_5_n25 | 0.3 | 0.3 |

Table 6.1.9.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_5_n25 | 0 | 0 |

6.1.10	DC_5_n41

6.1.10.1	Configuration for DC

Table 6.1.1.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_5A_n41A | DC_5A_n41A | No |

6.1.10.2	Maximum output power for DC

Table 6.1.1.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n41A | 23 | +2/-3 |

6.1.10.3	Spurious emission band UE co-existence for DC

Table 6.1.10.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5A_n41A | E-UTRA Band 1, 2, 3, 4, 5, 8, 10, 12, 13, 14, 17, 24, 25, 26, 28, 29, 30, 34, 40, 42, 45, 48, 50, 51, 54, 65, 66, 70, 71, 73, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 52 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.10.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.10.4-1

Table 6.1.10.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 824 | 849 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1648 | 1698 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2472 | 2547 | 7488 | 8070 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3296 | 3396 | 9984 | 10760 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4120 | 4245 | 12480 | 13450 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1866 | 1647 | 3320 | 3539 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1042 | 798 | 4143 | 4556 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4144 | 4388 | 5816 | 6229 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3732 | 3294 | 6640 | 7078 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 218 | 51 | 6639 | 7246 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4968 | 5237 | 8312 | 8919 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9936 | 9135 | 900 | 606 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10808 | 11609 | 5792 | 6086 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6422 | 5790 | 2445 | 2908 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9136 | 9768 | 7464 | 7927 |

Table 6.1.10.4-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 5 | 824 | 849 | 869 | 894 | 1738 | 1788 | 2607 | 2682 | 3476 | 3576 | 4345 | 4470 |
| n41 | 2469 | 2690 | 2469 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Based on Table 6.1.10.4-1, it can be seen that

-	3rd and 5th order IMD products may fall into Rx frequencies of band 5.

-	5th order IMD products may fall into Rx frequencies of bands n41.

-	There is a 3rd order harmonic of band 5 uplink in band n41

Based on Table 6.1.10.4-2, it can be seen there is the 3rd harmonic mixing.

Based on the co-existence studies above, MSD is required. MSD values are reused from DC_26_n41.

Table 6.1.10.4-3: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_5A_n41A | 5 | 839 | 5 | 25 | 884 | 15.6 | FDD | IMD33 |
|  | n41 | 2562 | 10 | 50 | 2562 | N/A | TDD | N/A |
| NOTE 3: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |  |

Table 6.1.10.4-4 lists the MSD due to 3rd harmonic required for EN-DC configuration DC_5A_n41A (reused from DC_26_n41).

Table 6.1.10.4-4: MSD due to 3rd harmonic for DC_5A_n41A

| MSD due to harmonic for the DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
|  |  | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB | dB |
| 5 | n418,9 | NA | 10.3 | 8.4 | 7.4 | 6.5 | 5.5 | 5 | 4.6 | 4.3 | 3.9 | 3.1 | 2.7 |
| NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 9: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.10.4-5 lists the B5 UL configuration for n41 MSD test due to 3rd order harmonic interference.

Table 6.1.10.4-5: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference

| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | n41 | - | 16 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

Table 6.1.10.4-6 lists the MSD values for DC_5A_n41A due to 3rd order harmonic mixing (reused from DC_26_n41).

Table 6.1.10.4-6: MSD due to 3rd order harmonic mixing for DC_5A_n41A

| UL band | DL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |
| --- | --- | --- | --- | --- | --- |
|  |  | dB | dB | dB | dB |
| n41 | 54 | N/A | N/A | 24.3 | 24.3 |
| NOTE 4: The requirements should be verified for DL EARFCN of the victim (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (lower) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |

Table 6.1.10.4-7 summarizes the n41 UL configuration for B5 3rd order harmonic mixing MSD test.

Table 6.1.10.4-7 n41 uplink configuration for B5 3rd order harmonic mixing MSD test

| NR Band / Channel bandwidth of the high band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE-NR DC band combination | UL band | SCS | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz |
| DC_5_n41 | n41 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |

6.1.10.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_5_41 as specified in TS 36.101.

Table 6.1.10.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_5_n41 | 0.6 | 0.3 |

Table 6.1.10.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_5_n41 | 0.2 | 0 |

6.1.11	DC_71_n25

6.1.11.1	Configuration for DC

Table 6.1.11.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_71A_n25A | DC_71A_n25A | No |

6.1.11.2	Maximum output power for DC

Table 6.1.11.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n25A | 23 | +2/-3 |

6.1.11.3	Spurious emission band UE co-existence for DC

Table 6.1.11.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_71_n25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 53, 54, 66, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 48, 70 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29, 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.11.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.11.4-1

Table 6.1.11.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 1850 | 1915 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1326 | 1396 | 3700 | 3830 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 5550 | 5745 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 7400 | 7660 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 9250 | 9575 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1252 | 1152 | 2513 | 2613 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 589 | 454 | 3002 | 3167 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3176 | 3311 | 4363 | 4528 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2504 | 2304 | 5026 | 5226 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 74 | 244 | 4852 | 5082 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3839 | 4009 | 6213 | 6443 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6997 | 6702 | 942 | 737 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8063 | 8358 | 4502 | 4707 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4419 | 4154 | 1606 | 1841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6876 | 7141 | 5689 | 5924 |

Table 6.1.11.4-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 71 | 663 | 698 | 617 | 652 | 1234 | 1304 | 1851 | 1956 | 2468 | 2608 | 3085 | 3260 |
| n25 | 1850 | 1915 | 1930 | 1995 | 3860 | 3990 | 5790 | 5985 | 7720 | 7980 | 9650 | 9975 |

Based on Table 6.1.11.4-1, it can be seen there:

-	There is no IMD

-	There is 3rd harmonic from 71 UL into band n25 DL.

-	There is 3rd harmonic mixing from 71 DL into band n25 UL

Based on the co-existence studies above, MSD is required. MSD values are reused from DC_71_n2:

Table 6.1.11.4-3: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 71 | n2511 | 4.6 | 1.0 | 0.7 | 0.6 | 0.5 | 0.3 | 0.2 |  |  |  |  |  |  |
|  | n2512 | 1.7 | 1.0 | 0.7 | 0.6 | 0.5 | 0.3 | 0.2 |  |  |  |  |  |  |
| NOTE 11: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2/Band n25 is located with its upper edge at 1990 MHz/1995 MHz NOTE 12: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2/Band n25 is located with its upper edge at 1990 MHz /1995 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.11.4-4: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 71 | n25 | 15 | 254 85 | 254 85 | 204 85 | 204 85 | 204 85 | 204 85 | 204 85 |  |  |  |  |  |  |
| NOTE 4: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2/Band n25 is located with its upper edge at 1990 MHz/1995 MHz NOTE 5: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2/Band n25 is located with its upper edge at 1990 MHz/1995 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.11.4-5: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n25 | 714 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.11.4-6: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n25 | 71 | 15 | 25 | 50 | 50 | 50 |  |  |  |  |  |  |  |

6.1.11.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_71_n2 as specified in TS 38.101-3.

Table 6.1.11.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_71_n25 | 0.3 | 0.3 |

Table 6.1.11.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_71_n25 | 0 | 0 |

6.1.12	DC_7_n12

6.1.12.1	Configuration for DC

Table 6.1.12.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A_n12A | DC_7A_n12A | No |

6.1.12.2	Maximum output power for DC

Table 6.1.12.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n12A | 23 | +2/-3 |

6.1.12.3	Spurious emission band UE co-existence for DC

Table 6.1.12.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_7_n12 | E-UTRA Band 2, 5, 13, 14, 17, 26, 27, 30, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 50, 51, 66 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.12.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.12.4-1

Table 6.1.12.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 699 | 716 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1398 | 1432 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2097 | 2148 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2796 | 2864 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3495 | 3580 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1871 | 1784 | 3199 | 3286 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1172 | 1068 | 4284 | 4441 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3898 | 4002 | 5699 | 5856 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3742 | 3568 | 6398 | 6572 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 473 | 352 | 6784 | 7011 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4597 | 4718 | 8199 | 8426 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9581 | 9284 | 364 | 226 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10699 | 10996 | 5296 | 5434 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6312 | 6068 | 2852 | 3043 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8898 | 9142 | 7097 | 7288 |

Based on Table 6.1.12.4-1, it can be seen there is no IMD and harmonic issues for this combination.

6.1.12.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from CA_7_12 as specified in TS 36.101.

Table 6.1.12.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_7_n12 | 0.3 | 0.3 |

Table 6.1.12.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_7_n12 | 0 | 0 |

6.1.13	DC_71_n7

6.1.13.1	Configuration for DC

Table 6.1.13.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_71A_n7A | DC_71A_n7A | No |

6.1.13.2	Maximum output power for DC

Table 6.1.13.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n7A | 23 | +2/-3 |

6.1.13.3	Spurious emission band UE co-existence for DC

Table 6.1.13.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_71_n7 | E-UTRA Band 4, 5, 12, 13, 14, 17, 26, 30, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 70 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.13.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.13.4-1

Table 6.1.13.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 663 | 698 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1326 | 1396 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 1989 | 2094 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 2652 | 2792 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 3315 | 3490 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1802 | 3163 | 3268 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1244 | 1104 | 4302 | 4477 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3826 | 3966 | 5663 | 5838 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3814 | 3604 | 6326 | 6536 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 581 | 406 | 6802 | 7047 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4489 | 4664 | 8163 | 8408 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9617 | 9302 | 292 | 82 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10663 | 10978 | 5152 | 5362 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6384 | 6104 | 2906 | 3151 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8826 | 9106 | 6989 | 7234 |

Based on Table 6.1.13.4-1, 4th order harmonic of Band 71 Tx may fall into the own Rx Band of Band n7.

MSD values due to 4th order harmonic for DC_71_n7 are reused from DC_7_n71

Table 6.1.13.4-2: MSD due to UL harmonic for EN-DC in NR FR1

| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 71 | n76,7 | 14.6 | 11.7 | 10.1 | 9 | 8 | 7.1 | 6.3 | 5.5 |  | 4 |  |  |  |  |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.13.4-3: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the high band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz | 10 MHz | 15 MHz | 20 Hz | 25 MHz | 30 MHz | 35 MH | 40 MHz | 45 MH | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| 71 | n7 | 8 | 16 | 25 | 25 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |

6.1.13.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_7_n71 as specified in TS 38.101-1.

Table 6.1.13.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_71_n7 | 0.6 | 0.3 |

Table 6.1.13.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_71_n7 | 0.2 | 0 |

6.1.14	DC_1_n105

6.1.14.1	Configuration for DC

Table 6.1.14.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_1A_n105A | DC_1A_n105A | No |

6.1.14.2	Maximum output power for DC

Table 6.1.14.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_1A_n105A | 23 | +2/-3 |

6.1.14.3	Spurious emission band UE co-existence for DC

Table 6.1.14.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Protected band | MBW (MHz) | Protected band |
| DC_1_n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29, 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.14.4	MSD analysis for DC

Table 6.1.14.4-1 lists Band 1 +Band n105 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.14.4-1: Band 1 and Band n105 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 663 | 703 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 1326 | 1406 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 1989 | 2109 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 2652 | 2812 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 3315 | 3515 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1317 | 1217 | 2583 | 2683 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3137 | 3297 | 654 | 514 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4503 | 4663 | 3246 | 3386 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5057 | 5277 | 9 | 189 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2434 | 2634 | 5166 | 5366 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6423 | 6643 | 3909 | 4089 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 892 | 672 | 7257 | 6977 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1731 | 1971 | 4614 | 4354 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4572 | 4792 | 8343 | 8623 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5829 | 6069 | 7086 | 7346 |

Table 6.1.14.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 1 and band n105,

Table 6.1.14.4-2: Band 1 and Band n105 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 | 8440 | 8680 | 10550 | 10850 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on Table 6.1.14.4-1, 6.1.14.4-2,

- There is no direct UL Harmonic interference, but there is a near miss collision of UL3 of n105 into DL1 of band 1.

- 3rd order IMD products may fall into Rx frequencies of band n105

- 3rd order harmonic mixing may fall into the Rx frequencies of n105 (1_n105 UL1/DL3)

For the MSD requirements for IMD interference of DC_1A_n105A, the requirements for DC_1A_n71A can be reused.

Table 6.1.14.4-3: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_1A_n105A | 1 | 1968 | 5 | 25 | 2158 | N/A | FDD | N/A |
|  | n105 | 673 | 5 | 25 | 622 | 15.1 | FDD | IMD3 |

6.1.14.5	∆TIB and ∆RIB values

For DC_1A_n105A, the TIB,c and RIB values reused from CA_n1A-n105A, as in the tables below.

Table 6.1.14.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_1_n105 | 0.3 | 0.6 |

Table 6.1.14.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_1_n105 | - | 0.3 |

6.1.14.6	REFSENS requirements

Based on the co-existence studies above, MSD is required. MSD values for UL harmonic are reused from similar near-miss cases:

Table 6.1.14.6-1: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n105 | 13 | 1.4 | 1.1 | 0.8 | 0.3 |  |  |  |  |  |  |  |  |  |
| NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.14.6-2: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n105 | 1 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |

Based on the co-existence there are 3rd order harmonic mixing from band 1 UL into band n105 DL. MSD value is based on the DC_1A_n71A.

Table 6.1.14.6-3: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 1 | n1054 | 26.8 | 23.6 | 21.2 | 15.6 |  |  |  |  |  |  |  |
| NOTE 4: The requirements should be verified for DL EARFCN or NR ARFCN of the victim (lower) band (superscript LB) such that with the DL carrier frequency in the lower band and fULHB the UL carrier frequency in the higher band, both in MHz. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.14.6-4: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 1 | n105 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |

6.1.15	DC_3_n105

6.1.15.1	Configuration for DC

Table 6.1.15.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3A_n105A | DC_3A_n105A | No |

6.1.15.2	Maximum output power for DC

Table 6.1.15.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3A_n105A | 23 | +2/-3 |

6.1.15.3	Spurious emission band UE co-existence for DC

Table 6.1.15.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Protected band | MBW (MHz) | Protected band |
| DC_3_n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 71 | FDL_low | - | FDL_high | -50 | 1 | 5 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.15.4	MSD analysis for DC

Table 6.1.15.4-1 lists Band 3 +Band n105 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.15.4-1: Band 3 and Band n105 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1785 | 663 | 703 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 1326 | 1406 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 1989 | 2109 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 2652 | 2812 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 3315 | 3515 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1122 | 1007 | 2373 | 2488 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2717 | 2907 | 459 | 304 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4083 | 4273 | 3036 | 3191 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4427 | 4692 | 204 | 399 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2014 | 2244 | 4746 | 4976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5793 | 6058 | 3699 | 3894 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1102 | 867 | 6477 | 6137 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1311 | 1581 | 4029 | 3724 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4597 | 7503 | 7843 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 5409 | 5679 | 6456 | 6761 |

Table 6.1.15.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 3 and band n105,

Table 6.1.15.4-2: Band 3 and Band n105 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 3 | 1710 | 1785 | 1805 | 1880 | 3610 | 3760 | 5415 | 5640 | 7220 | 7520 | 9025 | 9400 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on Table 6.1.15.4-1, 6.1.15.4-2,

- There is no UL Harmonic interference

- There is no intermodulation products that interfere the receive bands

- There is no Harmonic mixing causing interference

6.1.15.5	∆TIB and ∆RIB values

For DC_3A_n105A, the TIB,c and RIB values reused from CA_n25-n71.

Table 6.1.15.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_3_n105 | 0.3 | 0.6 |

Table 6.1.15.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_3_n105 | - | 0.3 |

6.1.16	DC_7_n105

6.1.16.1	Configuration for DC

Table 6.1.16.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_7A_n105A | DC_7A_n105A | No |

6.1.16.2	Maximum output power for DC

Table 6.1.16.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_7A_n105A | 23 | +2/-3 |

6.1.16.3	Spurious emission band UE co-existence for DC

Table 6.1.16.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Protected band | MBW (MHz) | Protected band |
| DC_7_n105 | E-UTRA Band 5, 26 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 5 |
|  | Frequency range | 2570 | - | 2575 | 1.6 | 5 | 5, 6, 7 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 5, 6, 7 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 5, 6 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 and Table 6.5A.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.16.4	MSD analysis for DC

Table 6.1.16.4-1 lists Band 7 +Band n105 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.16.4-1: Band 7 and Band n105 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 663 | 703 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5000 | 5140 | 1326 | 1406 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 1989 | 2109 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 2652 | 2812 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 3315 | 3515 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1907 | 1797 | 3163 | 3273 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4297 | 4477 | 1244 | 1094 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5663 | 5843 | 3826 | 3976 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6797 | 7047 | 581 | 391 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3594 | 3814 | 6326 | 6546 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8163 | 8413 | 4489 | 4679 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 312 | 82 | 9617 | 9297 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2891 | 3151 | 6384 | 6094 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5152 | 5382 | 10663 | 10983 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6989 | 7249 | 8826 | 9116 |

Table 6.1.16.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 7 and band n105,

Table 6.1.16.4-2: Band 7 and Band n105 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 7 | 2500 | 2570 | 2620 | 2690 | 5240 | 5380 | 7860 | 8070 | 10480 | 10760 | 13100 | 13450 |
| n105 | 663 | 703 | 612 | 652 | 1224 | 1304 | 1836 | 1956 | 2448 | 2608 | 3060 | 3260 |

Based on Table 6.1.16.4-1, 6.1.16.4-2,

- There is 4th order UL Harmonic interference of n105 that fall into band 7 RX frequencies.

- There are no intermodulation products that interfere the receive bands

- There is Harmonic mixing causing interference for 7 UL into n105 DL as 1UL/4DL

Based on the co-existence studies above, MSD is required. MSD values for UL harmonic are reused from DC_7_n71:

Table 6.1.16.4-3: Reference sensitivity exceptions (MSD) due to UL harmonic for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n105 | 76,7 | 14.6 | 11.7 | 10.1 | 9 |  |  |  |  |  |  |  |  |  |
| NOTE 6: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 7: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.16.4-4: Uplink configuration for reference sensitivity exceptions due to UL harmonic interference for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| n105 | 7 | 15 | 8 | 16 | 25 | 25 |  |  |  |  |  |  |  |  |  |

Based on the co-existence studies above, MSD is required. MSD values for Harmonic mixing are reused from DC_8_n78 that also captures 4th order products:

Table 6.1.16.4-5: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 7 | n105 | 5.7 | 4.0 | 3.0 | 2.7 |  |  |  |  |  |  |  |

Table 6.1.16.4-6: Uplink configuration for reference sensitivity exceptions due to receiver harmonic mixing for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band / UL RB allocation of the agressor band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 7 | n105 | 15 | 25 | 50 | 75 | 100 |  |  |  |  |  |  |  |

6.1.16.5	∆TIB and ∆RIB values

For DC_7A_n105A, the TIB,c and RIB values reused from DC_7A_n71A, as in the tables below.

Table 6.1.16.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_7_n105 | 0.3 | 0.6 |

Table 6.1.16.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_7_n105 | - | 0.2 |

6.1.17	DC_71_n12

6.1.17.1	Configuration for DC

Table 6.1.17.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_71A_n12A | DC_71A_n12A18,19 | Yes |
| NOTE 18: Only single switched UL is supported. NOTE 19: The implementation with 4 antennas is targeted for FWA form factor for this band combination. |  |  |

6.1.17.2	Maximum output power for DC

Table 6.1.17.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_71A_n12A7 | 23 | +2/-3 |
| NOTE 7: Only single switched UL is supported. |  |  |

6.1.17.3	Spurious emission band UE co-existence for DC

Note that only Single Tx Switched UL mode is supported for this combination, spurious emission band UE co-existence requirement for band 71 and n12 respectively is applied.

Table 6.1.17.3-1: Requirements for band n12 spurious emissions for UE co-existence

| NR Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 48, 53, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 50, 51, 66, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

Table 6.1.17.3-2: Requirements for band 71 spurious emissions for UE co-existence

| E-UTRA Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 54, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5.3.1-2 are permitted for each assigned NR carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2 MHz + N x LCRB x RBsize kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth. |  |  |  |  |  |  |  |

6.1.17.4	MSD analysis for DC

From the agreements captured in R4-2306479, the MSD requirements due to cross band isolation are given in table below.

Table 6.1.17.4-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| 71 | n12 | 683 | 20 | 15 | 20 (RBstart=86) | 731.5 | 5 | 8.2 | ACLR2 |
| n12 | 71 | 706.5 | 15 | 15 | 20 (RBstart=59) | 649.5 | 5 | 3.8 | >ACLR2 |

6.1.17.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_12_n71 as specified in TS 38.101-3.

Table 6.1.17.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_71_n12 | 1 | 1 |

Table 6.1.17.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_71_n12 | 0.8 | 0.8 |

6.1.18	DC_5_n28

6.1.18.1	Configuration for DC

Table 6.1.18.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_5A_n28A | DC_5A_n28A | No |

6.1.18.2	Maximum output power for DC

Table 6.1.18.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_5A_n28A | 23 | +2/-3 |

6.1.18.3	Spurious emission band UE co-existence for DC

Referring to the spurious emission band UE-to-UE co-existence requirements for DC_28_n5 specified in clause 6.5B.3.3.2 of TS 38.101-3 V18.2.0, This clause specifies the requirements for EN-DC coexistence with protected bands. When both constituent bands (LTE band 5 and NR band n28) have common coexistence band protection requirements as specified in clause 6.5.3.2 of TS 38.101-1 and clause 6.6.3.2 of 36.101, the requirements are also applied to the EN-DC configuration. The additional protected frequency ranges are shown in Table 6.1.18.3-1.

Table 6.1.18.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_5_n28 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 9 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 5, 17 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 14 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 5 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 5 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
| NOTE 1: Void NOTE 2: Void NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: Void NOTE 11: Void NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: Void NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: Void |  |  |  |  |  |  |  |

6.1.18.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.18.4-1.

Table 6.1.18.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4120 | 4245 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 76 | 146 | 1527 | 1597 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 557 | 672 | 900 | 995 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2230 | 2345 | 2351 | 2446 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1260 | 1420 | 1724 | 1844 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2933 | 3093 | 3175 | 3295 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 292 | 152 | 3054 | 3194 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 2693 | 2548 | 2168 | 1963 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 1141 | 976 | 596 | 411 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 3999 | 4144 | 3636 | 3841 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 3878 | 4043 | 3757 | 3942 |

There is no MSD issue for DC_5_n28.

There is no harmonic issue for DC_5_n28.

6.1.18.5	∆TIB and ∆RIB values

For DC_5_n28, TIB,c and RIB,c values will be the same as for DC_28_n5 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.18.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5_n28 | 5 | 0.5 |
|  | n28 | 0.5 |

Table 6.1.18.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5_n28 | 5 | 0 |
|  | n28 | 0 |

6.1.18.6	self-interference analysis

A B5 Tx noise level of ~ -42.2dBm in the DL band n28 upper 5MHz channel was measured. The analysis can be found below.

The MSD was proposed below.

Table 6.1.18.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation for PC3 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) | source |  |
| 5 | n28 | 829 | 10 | 15 | 25 (RBstart=0) | 800.5 | 5 | 11.4 | >ACLR2 |

6.1.19	DC_14_n41

6.1.19.1 Configuration for DC

Table 6.1.19.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_14A_n41A | DC_14A_n41A | No |

6.1.19.2 Maximum output power for DC

Table 6.1.19.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_14A_n41A | 23 | +2/-3 |

6.1.19.3	Spurious emission band UE co-existence for DC

Table 6.1.19.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_14_n41 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |

6.1.19.4	MSD analysis for DC

Table 6.1.19.4-1 lists Band 14 +Band n41 2UL DC up to 5th order harmonics and up to 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.19.4-1: Band 14 and Band n41 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 788 | 798 | 2496 | 2690 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1576 | 1596 | 4992 | 5380 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2364 | 2394 | 7488 | 8070 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3152 | 3192 | 9984 | 10760 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3940 | 3990 | 12480 | 13450 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1698 | 1902 | 3284 | 3488 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1114 | 900 | 4194 | 4592 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4072 | 4286 | 5780 | 6178 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 326 | 102 | 6690 | 7282 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3804 | 3396 | 6568 | 6976 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4860 | 5084 | 8276 | 8868 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9972 | 9186 | 696 | 462 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6494 | 5892 | 2598 | 3016 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10772 | 11558 | 5648 | 5882 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9064 | 9666 | 7356 | 7774 |

Table 6.1.19.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 14 and band n41,

Table 6.1.19.4-2: Band 14 and Band n41 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 14 | 788 | 798 | 758 | 768 | 1516 | 1536 | 2274 | 2304 | 3032 | 3072 | 3790 | 3840 |
| n41 | 2496 | 2690 | 2496 | 2690 | 4992 | 5380 | 7488 | 8070 | 9984 | 10760 | 12480 | 13450 |

Based on Table 6.1.19.4-1, 6.1.19.4-2,

- There is no UL Harmonic interference.

- There are no intermodulation products that interfere the receive bands.

- There is no Harmonic mixing interference.

6.1.19.5	∆TIB and ∆RIB values

For DC_14A_n41A, the TIB,c and RIB value is reused from DC_12A_n41A as in the tables below

Table 6.1.19.5-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)7 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |
| DC_14_n41 | 0.3 | 0.3 |

Table 6.1.19.5-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |
| DC_14_n41 | - | - |

6.1.20	DC_40_n3

6.1.20.1	Configuration for DC

Table 6.1.20.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_40A_n3A | DC_40A_n3A | No |

6.1.20.2	Maximum output power for DC

Table 6.1.20.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_40A_n3A | 23 | +2/-3 |

6.1.20.3	Spurious emission band UE co-existence for DC

Referring to the Spurious emission band UE-to-UE co-existence requirements for DC_3_n40 specified in clause 6.5B.3.3.2 of TS 38.101-3 V18.2.0, This clause specifies the requirements for EN-DC coexistence with protected bands. When both constituent bands (LTE band 40 and NR band n3) have common coexistence band protection requirements as specified in clause 6.5.3.2 of TS 38.101-1 and clause 6.6.3.2 of 36.101, the requirements are also applied to the EN-DC configuration. The additional protected frequency ranges are shown in Table 6.1.20.3-1.

Table 6.1.20.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_40A_n3A | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 1: Void NOTE 2: Void NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz NOTE 4: Void NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1, Table 6.6.3.1A-1 in TS 36.101 [4] or in Table 6.5.3.1-1 in TS 38.101-1 [2] from the edge of the channel bandwidth. NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Applicable when the assigned E-UTRA or NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 10: Void NOTE 11: Void NOTE 12: This requirement is applicable only for the following cases: A: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB; B: for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth; C: for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 13: Void NOTE 14: This requirement is applicable for 5 and 10 MHz E-UTRA or NR channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart < 48. NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable in the case of a 10 MHz E-UTRA or NR carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 18: Void NOTE 19: Void NOTE 20: Void. NOTE 21: Void NOTE 22: Void |  |  |  |  |  |  |  |

6.1.20.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.20.4-1.

Table 6.1.20.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 690 | 515 | 4010 | 4185 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2815 | 3090 | 1020 | 1270 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6310 | 6585 | 5720 | 5970 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5115 | 5490 | 2730 | 3055 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8610 | 8985 | 7430 | 7755 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1030 | 1380 | 8020 | 8370 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 4840 | 4440 | 7890 | 7415 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 755 | 330 | 3780 | 3330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9140 | 9540 | 10910 | 11385 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9730 | 10155 | 10320 | 10770 |

There is no MSD issue for DC_40_n3.

There is no harmonic issue for DC_40_n3.

6.1.20.5	∆TIB and ∆RIB values

For DC_40_n3, TIB,c and RIB,c values will be the same as for DC_3_n40 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.20.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n3 | n3 | 0.5 |
|  | 40 | 0.5 |

Table 6.1.20.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_40_n3 | n3 | 0 |
|  | 40 | 0 |

6.1.20.6	Self-interference analysis

There is no self-interference.

6.1.21	DC_40_n7

6.1.21.1	Configuration for DC

Table 6.1.21.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_40A_n7A | DC_40A_n7A | No |

6.1.21.2	Maximum output power for DC

Table 6.1.21.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_40A_n7A | 23 | +2/-3 |

6.1.21.3	Spurious emission band UE co-existence for DC

Referring to the Spurious emission band UE-to-UE co-existence requirements for DC_7_n40 specified in clause 6.5B.3.3.2 of TS 38.101-3 V18.2.0, This clause specifies the requirements for EN-DC coexistence with protected bands. When both constituent bands (LTE band 40 and NR band n7) have common coexistence band protection requirements as specified in clause 6.5.3.2 of TS 38.101-1 and clause 6.6.3.2 of 36.101, the requirements are also applied to the EN-DC configuration.

6.1.21.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.21.4-1.

Table 6.1.21.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 7500 | 7710 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 10000 | 10280 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 100 | 270 | 4800 | 4970 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 2030 | 2300 | 2600 | 2840 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7100 | 7370 | 7300 | 7540 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 4330 | 4700 | 5100 | 5410 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9400 | 9770 | 9800 | 10110 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 540 | 200 | 9600 | 9940 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7980 | 7600 | 7100 | 6630 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3110 | 2700 | 2200 | 1760 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 12300 | 12680 | 11700 | 12170 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 12100 | 12510 | 11900 | 12340 |

IMD3 of UL DC_40_n7 may fall into the DL band of band n7.

There is no harmonic issue for DC_40_n7.

6.1.21.5	∆TIB and ∆RIB values

For DC_40_n7, TIB,c and RIB,c values will be the same as for DC_7_n40 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.21.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_40_n7 | n7 | 0.5 |
|  | 40 | 0.6 |

Table 6.1.21.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_40_n7 | n7 | 0 |
|  | 40 | 0.5 |

6.1.21.6	Self-interference analysis

For the MSD due to cross band isolation, it’s proposed to specify the following MSD referring to the DC_7_n40 and CA_n7-n40.

Table 6.1.21.6-1: Reference sensitivity exceptions (MSD) due to cross band isolation and uplink/downlink configurations for PC3 EN-DC in NR FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n7 | 40 | 2525 | 50 | 15 | 45 (RBSTART=0) | 2397.5 | 5 | 3.7 | >ACLR2 |
| 40 | n7 | 2390 | 20 | 15 | 100 (RBSTART=0) | 2622.5 | 5 | 22.3 | >ACLR2 |

For the MSD due to IMD3 interference from DC_40_n7, it’s proposed to specify the following MSD referring to the DC_7_n40.

Table 6.1.21.6-2: MSD test points for PCell due to dual uplink operation for PC3 EN-DC in NR FR1 (two bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA or NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_40_n7 | n7 | 2510 | 5 | 25 | 2630 | 23 | IMD3 |
|  | 40 | 2390 | 5 | 25 | 2390 | N/A | N/A |

6.1.22	DC_20_n40

6.1.22.1	Configuration for DC

Table 6.1.22.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_20A_n40A | DC_20A_n40A | No |

6.1.22.2	Maximum output power for DC

Table 6.1.22.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_20A_n40A | 23 | +2/-3 |

6.1.22.3	Spurious emission band UE co-existence for DC

his clause specifies the requirements for EN-DC coexistence with protected bands. When both constituent bands (LTE band 20 and NR band n40) have common coexistence band protection requirements as specified in clause 6.5.3.2 of TS 38.101-1 and clause 6.6.3.2 of 36.101, the requirements are also applied to the EN-DC configuration. There is no additional protected frequency range.

#### 6.1.22.4	MSD analysis for DC

For 2UL/2DL UE coexistence study 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 6.1.22.4-1.

Table 6.1.22.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2300 | 2400 | 832 | 862 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 4600 | 4800 | 1664 | 1724 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 6900 | 7200 | 2496 | 2586 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 9200 | 9600 | 3328 | 3448 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 11500 | 12000 | 4160 | 4310 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1568 | 1438 | 3132 | 3262 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3738 | 3968 | 736 | 576 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5432 | 5662 | 3964 | 4124 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6038 | 6368 | 96 | 286 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7732 | 8062 | 4796 | 4986 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2876 | 3136 | 6264 | 6524 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 1148 | 928 | 8768 | 8338 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2014 | 2304 | 5536 | 5176 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5628 | 5848 | 10032 | 10462 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7096 | 7386 | 8564 | 8924 |

Table 6.1.22.4-2 lists up to 5th order harmonics mixing products for dual connectivity between band 20and band n40,

Table 6.1.22.4-2: Band 20 and Band n40 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 20 | 832 | 862 | 791 | 821 | 1582 | 1642 | 2373 | 2463 | 3164 | 3284 | 3955 | 4105 |
| n40 | 2300 | 2400 | 2300 | 2400 | 4600 | 4800 | 6900 | 7200 | 9200 | 9600 | 11500 | 12000 |

There is no IMD issue for DC_20_n40.

There is 3rd order harmonic issue for victim DL band 20 when band n40 transmit the UL signal.

6.1.22.5	∆TIB and ∆RIB values

For DC_20_n40, TIB,c and RIB,c values will be the same as for CA_n20-n40 in 38.101-3. The TIB,c and RIB,c values are given in the tables below.

Table 6.1.22.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20_n40 | 20 | 0.3 |
|  | n40 | 0.3 |

Table 6.1.22.5-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_20_n40 | 20 | 0 |
|  | n40 | 0.5 |

#### 6.1.22.6	Self-interference analysis

Referring to CA_n20A-n40A, the MSD due to receiver harmonic mixing for DC_20A_n40A can be specified below.

Table 6.1.22.6-1: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n40 | 20 | 5 | 15 | 25 (RBstart=0) | 5 | 27.8 | NOTE 4 | UL1/DL3 |
| n40 | 20 | 20 | 15 | 100 (RBstart=0) | 20 | 20.3 | NOTE 4 | UL1/DL3 |

6.1.23	DC_8_n1

6.1.23.1	Configuration for DC

Table 6.1.23.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_8B_n1A | DC_8A_n1A DC_8B_n1A | No |

Note that the DL DC_8B_n1A with UL DC_8A_n1A configuration is already specified in the Rel.18 specification via draft CR approach.

6.1.23.2	Maximum output power for DC

Table 6.1.23.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_8B_n1A | 23 | +2/-3 |

6.1.23.3	Spurious emission band UE co-existence for DC

Note that the spurious emission band UE co-existence for DC_8_n1 is already specified in the current specification, no additional impact is expected.

6.1.23.4	MSD analysis for DC

Since the DL DC_8B_n1A with UL DC_8A_n1A configuration is already specified, the coexistence analysis provided in this section is focused on the UL DC_8B_n1A which contains intra-band UL CA component. As band 8 and band n1 belong to adjacent band groups, FR1-1 & FR1-2, further analysis on intra-band IMD & triple beat analysis is needed.

In Table 6.1.23.4-1, up to 9th order IMD ranges for 2CCs on intra-band CA_8B are listed, with the corresponding figure in figure 6.1.23.4-1.

Table 6.1.23.4-1: Co-existence studies for intra-band UL CA_8B part of UL DC_8B_n1A

| Configuration | Channel BW | Minimum Channel separation | Maximum Instantaneous UL BW | Minimum frequency | Maximum frequency | (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 3+3, 10+10 | 0 | 20 | 880 | 915 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 880 | 883 | 900 | 915 | 912 | 895 |
| 2nd | I fU1L - fU2L I | I fU1L - fU3L I | fU1L + fU2L | fU1H + fU2H | - | - |
| Interference ranges | 3 | 20 | 1763 | 1827 | - | - |
| 3rd | 2*fU1L - fU3L | 2*fU1H - fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 860 | 935 | 2643 | 2742 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 6 | 40 | 1740 | 1850 | 3523 | 3657 |
| 5th | I 3*fU1L - 2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L - fU3L | 4*fU1H - fU3H | 4*fU1L + fU2L | 4*fU1H + fU2H |
| Interference ranges | 840 | 955 | 2620 | 2765 | 4403 | 4572 |
| 6th | I 3*fU1L - 3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L - 2*fU3L | 4*fU1H - 2*fU3H | 5*fU1L - fU3L | 5*fU1H - fU3H |
| Interference ranges | 9 | 60 | 1720 | 1870 | 3500 | 3680 |
| 7th | I 4*fU1L - 3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L - 2*fU3L | 5*fU1H - 2*fU3H | 6*fU1L - fU3L | 6*fU1H - fU3H |
| Interference ranges | 820 | 975 | 2600 | 2785 | 4380 | 4595 |
| 9th | I 5*fU1L - 4*fU3L I | I 5*fU1H-4*fU3H I | 6*fU1L - 3*fU3L | 6*fU1H - 3*fU3H | 7*fU1L - 2*fU3L | 7*fU1H - 2*fU3H |
| Interference ranges | 800 | 995 | 2580 | 2805 | 4360 | 4615 |

Figure 6.1.23.4-1: Co-existence studies for intra-band UL CA_8B part of UL DC_8B_n1A

Based on Table 6.1.23.4-1,

- 3rd, 5th, 7th, 9th IMD generated by two CCs from UL band 8 might fall into own Rx of band 8, considering 1RB+1RB scenario, however, currently such requirement is not considered in the LTE specification.

- No IMD generated by two CCs from UL band 8 will fall into own Rx of band n1.

Then in Table 6.1.23.4-2, triple beat IMD products for UL DC_8B_n1A are listed, with the corresponding figure in figure 6.1.23.4-2.

Table 6.1.23.4-2: Band 8 and Band n1 triple beat IMD products

| CC location | fU1L | fU2L | fU3L | fU1H |  | CBW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 880 | 883 | 900 | 915 |  | 3+3, 10+10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min ch. separation |
| Frequency | 1920 | 1980 | 912 | 895 |  | 0 |
| 1st order TB | IfU3L-fU1L-fSCCL¦ | IfU2L-fU1L-fSCCH¦ | IfU2L-fU1L+fSCCL¦ | IfU3L-fU1L+fSCCH¦ |  | Max ch. separation |
| Ranges | 1900 | 1977 | 1923 | 2000 |  | 20 |
| 1st order TB | IfU2L+fU1L-fSCCH¦ | IfU1H+fU2H-fSCCL¦ | IfU2L+fU1L+fSCCL¦ | IfU1H+fU2H+fSCCH¦ |  |  |
| Ranges | 217 | 93 | 3683 | 3807 |  |  |

Figure 6.1.23.4-1: Co-existence studies triple beat

Based on Table 6.1.23.4-2, there is no TB falling into the Rx frequency range of the band 8 or band n1.

Based on the above, it can be concluded that no additional MSD is needed.

6.1.23.5	∆TIB and ∆RIB values

For DC_8B_n1A, the TIB,c and RIB values are already covered by the DC_8_n1, which is already specified in the current specification.

6.1.24	DC_41_n79

6.1.24.1	Configuration for DC

Table 6.1.24.1-1:  Inter-band EN-DC configurations within FR1 (two bands)

| EN-DC configuration | Uplink EN-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_41A_n79A6,7 DC_41A_n79C6,7 | DC_41A_n79A DC_41A_n79C | No |
| NOTE 6: The frequency range below 2506 MHz for Band 41 is not used in this combination. NOTE 7: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability. |  |  |

Note that the DL DC_41A_n79C with UL DC_41A_n79A configuration is already specified.

6.1.24.2	Maximum output power for DC

Table 6.1.24.2-1: Maximum output power for inter-band EN-DC (two bands)

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_41A_n79C | 23 | +2/-3 |

6.1.24.3	Spurious emission band UE co-existence for DC

Note that the spurious emission band UE co-existence for DC_41_n79 is already specified in the current specification, no additional impact is expected.

6.1.24.4	MSD analysis for DC

Since the DL DC_41A_n79C with UL DC_41A_n79A configuration is already specified, the coexistence analysis provided in this section is focused on the UL DC_41A_n79C which contains intra-band UL CA component. First we look into the bandwidth combination of the CA_n79C in the current specification.

Table 6.1.24.4-1:  BCS for CA_n79C in current specification (TS 38.101-1 V18.4.0)

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

Then, with consideration of the minimum and maximum possible aggregated CBW, 10+10 and 100+100 (MHz) from Table 6.1.24.4-1, and based on the guidelines in TS 38.846, up to 9th order IMD ranges for 2CCs on intra-band CA_n79C are listed in Table 6.1.24.4-2 below, with the corresponding figure in figure 6.1.24.4-1.

Table 6.1.24.4-2: Co-existence studies for intra-band UL CA_n79C part of UL DC_41A_n79C

| Configuration | Channel BW | Minimum Channel separation | Maximum Instantaneous UL BW | Minimum frequency | Maximum frequency | (MHz) |
| --- | --- | --- | --- | --- | --- | --- |
| Data | 10+10, 100+100 | 0 | 20 | 880 | 915 | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency | 4400 | 4410 | 4600 | 5000 | 4990 | 4800 |
| 2nd | I fU1L - fU2L I | I fU1L - fU3L I | fU1L + fU2L | fU1H + fU2H | - | - |
| Interference ranges | 10 | 200 | 8810 | 9990 | - | - |
| 3rd | 2*fU1L - fU3L | 2*fU1H - fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H | - | - |
| Interference ranges | 4200 | 5200 | 13210 | 14990 | - | - |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges | 20 | 400 | 8600 | 10200 | 17610 | 19990 |
| 5th | I 3*fU1L - 2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L - fU3L | 4*fU1H - fU3H | 4*fU1L + fU2L | 4*fU1H + fU2H |
| Interference ranges | 4000 | 5400 | 13000 | 15200 | 22010 | 24990 |
| 6th | I 3*fU1L - 3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L - 2*fU3L | 4*fU1H - 2*fU3H | 5*fU1L - fU3L | 5*fU1H - fU3H |
| Interference ranges | 30 | 600 | 8400 | 10400 | 17400 | 20200 |
| 7th | I 4*fU1L - 3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L - 2*fU3L | 5*fU1H - 2*fU3H | 6*fU1L - fU3L | 6*fU1H - fU3H |
| Interference ranges | 3800 | 5600 | 12800 | 15400 | 21800 | 25200 |
| 9th | I 5*fU1L - 4*fU3L I | I 5*fU1H-4*fU3H I | 6*fU1L - 3*fU3L | 6*fU1H - 3*fU3H | 7*fU1L - 2*fU3L | 7*fU1H - 2*fU3H |
| Interference ranges | 3600 | 5800 | 12600 | 15600 | 21600 | 25400 |

Figure 6.1.24.4-1: Co-existence studies for intra-band UL CA_n79C part of UL DC_41A_n79C

Based on Table 6.1.24.4-2,

- No IMD generated by two CCs from UL band n79 will fall into own Rx of band 41.

Then since the two UL bands are both TDD bands, there is no triple beat IMD issue for this combination.

Based on the above, it can be concluded that no additional MSD is needed.

6.1.24.5	∆TIB and ∆RIB values

The TIB,c and RIB values for DC_41A_n79C are already  specified in the current specification.

6.2	Inter-band DC including FR2

< Editor’s note: Note that based on the agreement in RAN4#95-e (R4-2008931), it is allowed to propose configurations for EN-DC including FR2 with one draft CR. >

6.2.1	DC_X_nY

6.2.1.1	Configuration for DC

Table 6.2.1.1-1:  Inter-band EN-DC configurations including FR2 (two bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_XA_nYA | DC_XA_nYA |

6.2.1.2	Spurious emission band UE co-existence for DC

< Editor’s note: this section is optional. >

6.2.1.3	MSD analysis for DC

< Editor’s note: this section is optional. >

6.2.1.4	∆TIB and ∆RIB values

< Editor’s note: this section is optional. >

6.3	Intra-band contiguous DC

6.3.1	 DC_3(n)

6.3.1.1	Channel bandwidths per operating band for DC

Table 6.3.1.1-1: Supported channel bandwidths per DC configuration

|  |  | NR –E-UTRA configuration / Bandwidth combination set |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink NE-DC configuration | Uplink NE-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | BCS |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths NR for carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_3(n)AA | DC_3(n)AA1 | 5, 10, 15, 20 | 5,10,15,20, 25, 30 |  | 50 | 0 |
|  |  |  | 5,10,15,20, 25, 30 | 5, 10, 15, 20 |  |  |
| NOTE 1: Only single switched UL is supported |  |  |  |  |  |  |

6.3.1.2	Configuration for DC

Table 6.3.1.2-1:  Intra-band contiguous NE-DC configurations

| NE-DC configuration | Uplink NE-DC configuration | Single UL allowed |
| --- | --- | --- |
| DC_3(n)AA | DC_3(n)AA6 | YES6 |
| NOTE 6: Only single switched UL is supported |  |  |

6.3.1.3	Maximum output power for DC

Table 6.3.1.3-1: Maximum output power for NE-DC (continuous sub-blocks)

| NE-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_3(n)AA3 | 23 | +2/-3 |
| NOTE 3: Only single switched UL is supported |  |  |

6.3.1.4	Spurious emission band UE co-existence for DC

Note that only single switched UL mode is supported for this combination hence no additional spurious emission band UE co-existence requirement is needed.

6.3.1.5	MSD analysis for DC

MSD configuration and value is already cover by DC_(n)3AA in TR 37.717-11-11.

6.3.1.6	MPR, AMPR requirements

Note that only single switched UL mode is supported for this combination hence no additional MPR or A-MPR requirement is needed.

6.4	Intra-band non-contiguous DC

6.4.1	DC_1_n1

6.4.1.1	Channel bandwidths per operating band for DC

Table 6.4.1.1-1: Supported channel bandwidths per DC configuration

| E-UTRA – NR configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_1A_n1A | DC_1A_n1A2 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 40 | 0 |
| NOTE 2: Only single switched UL is supported. |  |  |  |  |  |  |

6.4.1.2	Configuration for DC

Table 6.4.1.2-1: Intra-band non-contiguous EN-DC configurations

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_1A_n1A | DC_1A_n1A5 | Yes5 |
| NOTE 5: Only single switched UL is supported. |  |  |

6.4.1.3	Maximum output power for DC

Table 6.4.1.3-1: Maximum output power for EN-DC (non-continuous sub-blocks)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n1A4 |  |  |  |  | 23 | +2/-3 |
| NOTE 4: Only single switched UL is supported. |  |  |  |  |  |  |

6.4.1.4	Spurious emission band UE co-existence for DC

No additional co-existence requirements needed.

6.4.1.5	MSD analysis for DC

No additional analysis required.

6.4.1.6	∆RIBNC values

The reference sensitivity requirements for CA_n1(2A) in 38.101-1 have been re-used here.

Table 6.4.1.6-1: Intra-band non-contiguous EN-DC with one uplink configuration on NR for reference sensitivity (NR uplink carrier is closer to the E-UTRA downlink carrier than it is to the NR downlink carrier)

| DC configuration | Aggregated channel bandwidth |  | Wgap / (MHz) | UL E-UTRA allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
|  | NR | E-UTRA |  |  |  |  |
| DC_1A_n1A | 5 MHz | 5 MHz | 0.0 < Wgap ≤ 50.0 | 25 | 0.5 | FDD |

6.4.1.7	MPR, A-MPR requirements

No additional MPR or A-MPR requirements needed as this combination is SUO.

Annex A (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-08 | RAN4-104e | R4-2212906 |  |  |  | Initial TR skeleton | 0.0.0 |
| 2022-08 | RAN4-104e | R4-2212907 |  |  |  | included TPs/pCRs: R4-2215086 DC_3(n)AA | 0.1.0 |
| 2022-10 | RAN4-104-bis-e | R4-2215750 |  |  |  | included TPs/pCRs: R4-2217099 Triple beat MSD of UL DC_3C_n28A R4-2215769 DC_8B_n78A R4-2217061 DC_1_n26 R4-2217102 DC_3_n26 R4-2217103 DC_7_n26 R4-2217064 DC_1_n1 R4-2217065 DC_28_n20 R4-2217087 Table templates and error corrections | 0.2.0 |
| 2022-11 | RAN4-105 | R4-2218550 |  |  |  | included TPs/pCRs: R4-2220566 Updates for DC_3_n26 R4-2220763 DC_28_n38 R4-2220767 DC_8_n38 R4-2219304 Updates for DC_7_n26 R4-2219758 Delta TIB and RIB templates for NE-DC combinations | 0.3.0 |
| 2023-03 | RAN4-106 | R4-2300827 |  |  |  | included TPs/pCRs: R4-2303623 DC_5A_n25A R4-2303624 DC_5A_n41A R4-2303625 DC_7A_n12A R4-2303626 DC_71A_n7A R4-2303627 DC_71A_n25A | 0.4.0 |
| 2023-04 | RAN4-106-bis-e | R4-2305574 |  |  |  | included TPs/pCRs: R4-2305575 support of uplink DC_8B_n78A R4-2306496 DC_1_n105 R4-2306497 DC_3_n105 R4-2306498 DC_7_n105 | 0.5.0 |
| 2023-05 | RAN4-107 | R4-2309361 |  |  |  | included TPs/pCRs: R4-2309534 DC_71A_n12A | 0.6.0 |
| 2023-08 | RAN4-108 | R4-2313560 |  |  |  | included TPs/pCRs: R4-2314826 DC_5A_n28A | 0.7.0 |
| 2023-09 | RAN-101 | RP-232304 |  |  |  | TR as provided for information to TSG RAN | 1.0.0 |
| 2023-10 | RAN4-108-bis | R4-2316231 |  |  |  | included TPs/pCRs: R4-2316099 DC_14A_n41A R4-2316194 DC_40A_n3A R4-2316195 DC_40A_n7A R4-2317670 DC_20A_n40A | 1.1.0 |
| 2024-02 | RAN4-110 | R4-2401973 |  |  |  | included TPs/pCRs: R4-2402025 Uplink DC_8B_n1A | 1.2.0 |
| 2024-04 | RAN4-110-bis | R4-2404783 |  |  |  | included TPs/pCRs: R4-2405397 support of uplink DC_41A_n79C | 1.3.0 |