# 3GPP TSG-RAN WG4 Meeting # 110-bis	R4-2404840

# Changsha, China, 15th – 19th April, 2024

Agenda item:			7.3.6

Source:	Moderator (MediaTek inc.)

Title:	Topic summary for [110bis][229] IoT_NTN_enh

Document for:	Information

# Introduction

Briefly introduce background, the scope of this summary (e.g. list of treated agenda items).

This document is the topic summary for RRM requirements for R18 IoT (Internet of Things) NTN (non-terrestrial network) enhancements, including the following topics covered

- Topic#1: RRM core requirements (AI 7.3.3)

- Topic#2: RRM performance requirements (AI 7.3.4)

Recommended issues for online discussion:

- Issues on Topic#2 (perf part), following the order of issues.

- Issues on Topic#1.

# Topic #1: RRM core requirements (AI 7.3.3)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2405754 | Nokia, Nokia Shanghai Bell | Observation 1: As GNSS-MG are expected to be frequently configured and triggered, current specification would ensue in frequently having non-applicable requirements for RLM. Observation 2: In many situations, when eDRX is configured, there is no need to have additional time for the UE to enlarge the RLM measurement delay. Proposal 1: Refine the specification text such that, for the cases where the GNSS-MG is smaller than the eDRX cycle, the RLM requirements are still applicable. Proposal 2: When the GNSS-MG is shorter than the (e)DRX cycle and it collides with the on Duration part of one (e)DRX cycle, the time to evaluate requirements might be extended. Proposal 3: When the UE is configured with eDRX cycle, and the GNSS-MG is larger than the eDRX cycle, the requirements applicable right after the GNSS-MG shall be corresponding to a DRX cycle of [1.28] s. Proposal 4: Capture in specification that for NB-IoT it is up for UE implementation which frequencies to be measured/prioritized in RRC_CONNECTED. Proposal 5: NB-IoT UE shall start time-based neighbor cell measurements: a. If no t-serviceStartNeigh is provided, [Y] DRX Cycles before t-service b. If t-serviceStartNeigh is provided for the neighbor cells, measurements shall start at tinitiate_inter = min( [Y] DRX Cycles, tService-tServiceStartNeigh) before t-service Proposal 6: When a UE starts intra-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), the UE skips the serving cell measurements when the following side conditions are met: a. When skipping serving cell measurements reduces the value of Ksatellite (i.e. no neighbor cell is configured for measurements in the same satellite as the serving cell) b. The S-Criterion is still met by the serving cell, according to the most recent measurements on the serving cell. c. If those conditions are met, the UE is allowed to use the last value for the serving cell measurements for cell reselection purpose until t-service is reached. Proposal 7: When a UE starts inter-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), and the inter-frequency neighbor cells associated to the same satellite are also configured with t-Service, then the UE is not required to measure these cells. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### RRM core requirements

### Sub-Topic 1-1: Time-based measurement initiation in IDLE mode

#### Issue 1-1-1: Skipping serving cell measurement before t-service

Proposals:

- Proposal 1 (Nokia): When a UE starts intra-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), the UE skips the serving cell measurements when the following side conditions are met:

a.	When skipping serving cell measurements reduces the value of Ksatellite (i.e. no neighbor cell is configured for measurements in the same satellite as the serving cell)

b.	The S-Criterion is still met by the serving cell, according to the most recent measurements on the serving cell.

c.	If those conditions are met, the UE is allowed to use the last value for the serving cell measurements for cell reselection purpose until t-service is reached.

Recommended WF: Discuss Proposal.

#### Issue 1-1-2: Measurement on inter-frequency neighbor cells associated to the same satellite

Proposals:

- Proposal 1 (Nokia):  When a UE starts inter-frequency neighbor cell measurements based on time-based measurement initiation (e.g. Ttrigger before t-service), and the inter-frequency neighbor cells associated to the same satellite are also configured with t-Service, then the UE is not required to measure these cells.

Recommended WF: Discuss Proposal.

### Sub-Topic 1-2: CONN mode neighbour cell measurements

#### Issue 1-2-1: For NB-IoT, frequencies to be measured

Proposals:

- Proposal 1 (Nokia): Capture in specification that for NB-IoT it is up for UE implementation which frequencies to be measured/prioritized in RRC_CONNECTED.

Recommended WF:

- Discuss proposal 1.

#### Issue 1-2-2: For NB-IoT, time-based neighbor cell measurements

Proposals:

- Proposal 1 (Nokia): NB-IoT UE shall start time-based neighbor cell measurements:

  - If no t-serviceStartNeigh is provided, [Y] DRX Cycles before t-service

  - If t-serviceStartNeigh is provided for the neighbor cells, measurements shall start at tinitiate_inter = min( [Y] DRX Cycles, tService-tServiceStartNeigh) before t-service

Recommended WF:

- Discuss proposal 1.

### Sub-Topic 1-3: GNSS re-acquisition gap in connected mode

#### Issue 1-3-1: GNSS-MG with (e)DRX

Proposals:

- Proposal 1 (Nokia): Refine the specification text such that, for the cases where the GNSS-MG is smaller than the eDRX cycle, the RLM requirements are still applicable.

- Proposal 2 (Nokia): When the GNSS-MG is shorter than the (e)DRX cycle and it collides with the on Duration part of one (e)DRX cycle, the time to evaluate requirements might be extended.

- Proposal 3 (Nokia): Proposal 3: When the UE is configured with eDRX cycle, and the GNSS-MG is larger than the eDRX cycle, the requirements applicable right after the GNSS-MG shall be corresponding to a DRX cycle of [1.28] s.

Recommended WF:

- Discuss proposals.

### RRM core part draft CRs

| T-doc number | Title | Source |
| --- | --- | --- |
| R4-2404993 | DraftCR on maintenance for R18 NB-IoT NTN enh | Huawei, HiSilicon |
| R4-2405603 | draftCR on measurement requirements for eMTC over NTN | Huawei, HiSilicon |

# Topic #2: RRM performance requirements (AI 7.3.4)

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2404689 | CMCC | Proposal 1: For NB-IoT/eMTC, introduce time-based and location-based measurement triggering test cases for RRC Idle and RRC connected modes in Earth Fixed cell scenarios. Observation 1: By means of constant ephemeris information, the scenario of earth-moving cell can not be emulated, the UE behavior of deriving the practical trajectory of serving cell with rough accuracy can not be tested. Proposal 2: Further study the feasibility of emulating the test scenario of earth-moving cell. Proposal 3: Introduce such test case to verify the GNSS measurement and UE behavior when GNSS gap and measurement gap overlapping, as long as RAN4 confirm that GNSS measurement can be supported by TE. |
| R4-2404994 | Huawei, HiSilicon | Observation 1: The exact time to start measurement before Ttrigger for NB-IoT is up to UE implementation. Proposal 1: Not to define TC for time-triggered measurement in connected mode for NB-IoT, and define TC for location-triggered measurement for intra-frequency case, as follows: Category; Titlle; Tag; Volunteer; Draft target date. Measurement Procedure in CONN (new); HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage; NB-CONN-1; Huawei; Phase 2 (2024.05). HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage; NB-CONN-2. Measurement Procedure in CONN with time/location triggering (new); HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering; NB-CONN-1T. HD-FDD InterIntra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering; NB-CONN-2D. Observation 2: For earth moving cell, it is not clear whether and how UE can estimate the reference location between two epoch time with one constant ephemeris. Proposal 2: Further discuss whether to define test case for location-based measurement in earth moving cell considering one constant ephemeris is used. Proposal 3: For GNSS re-acquisition, if the performance of measurement after early termination is to be verified, following test steps can be considered: - NW trigger GNSS measurement gap from T1. The configured GNSS gap length is longer than UE reported GNSS-PositionFixDuration. - T2 starts when UE has performed RACH procedure after GNSS fix. The target neighbour cell is turned on at T2. - UE shall be able to report measurement report with delay less than Tidentify from T2. |
| R4-2405434 | MediaTek inc. | Observation 1: For NB-IoT RRC Re-establishment over NTN, both the normal coverage and enhanced coverage have been tested in intra-frequency tests. Proposal 1: Regarding inter-frequency test cases for NB-IoT RRC Re-establishment over NTN, introduce only one test for enhanced coverage. I.e. drop the test NB-3-1. |
| R4-2405755 | Nokia | Proposal 1: For eMTC introduce a intra-frequency measurement test case where t-serviceStart is configured and 2 time instants are defined: a. T1: Initial test conditions, UE receives the RRC configuration for measuring the neighbor cell configured with t-serviceStart. T1 ends at t-serviceStart, and the UE shall be reachable by the eNB during the measurement gap opportunities in T1. b. T2: Initiated at t-serviceStart, when Cell 2 becomes available. UE shall start measurements on the neighbor cell and send the measurement report. Proposal 2: Do not introduce the test case for NB-IoT where t-serviceStart is configured. Proposal 3: Do not test time-based measurement triggering in connected mode for NB-IoT. Proposal 4: Do not test distance-based measurement triggering in connected mode for NB-IoT. Proposal 5: Do not introduce GNSS-MG test cases unless needed to verify a measurable UE behaviour. Proposal 6: For Idle and Connected mode mobility test cases, select half of the test cases to be tested with the neighbor cell associated to a different satellite. |

## Open issues summary

Before f2f meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-Topic 2-1: Test case list

#### Issue 2-1-1: For NB-IoT, inter-frequency test cases for RRC Re-establishment

Proposals:

- Proposal 1 (MTK): Regarding inter-frequency test cases for NB-IoT RRC Re-establishment over NTN, introduce only one test for enhanced coverage. I.e. drop the test NB-3-1.

| Category | Titlle | Tag |
| --- | --- | --- |
| RRC Re-establishment (A.13.3.1) | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage | NB-3-1 |
|  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage | NB-3-2 |

Recommended WF: Discuss the proposal.

#### Issue 2-1-2a: For NB-IoT, test case for time-based measurement triggering in connected mode

Proposals:

- Proposal 1 (Huawei, Nokia): Not to define TC for time-triggered measurement in connected mode for NB-IoT

| Category | Titlle | Tag | Volunteer | Draft target date |
| --- | --- | --- | --- | --- |
| Measurement Procedure in CONN (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-1 | Huawei | Phase 2 (2024.05) |
|  | HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage | NB-CONN-2 |  |  |
| Measurement Procedure in CONN with time/location triggering (new) | HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, time-based triggering | NB-CONN-1T |  |  |
|  | HD-FDD InterIntra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage, location-based triggering | NB-CONN-2D |  |  |

Recommended WF: Do not test for time-based measurement triggering in connected mode for NB-IoT.

#### Issue 2-1-2b: For NB-IoT, test case for location-based measurement triggering in connected mode

Proposals:

- Proposal 1 (Huawei): define TC for location-triggered measurement for intra-frequency case, as follows.

- Proposal 2 (Nokia): Proposal 4: Do not test distance-based measurement triggering in connected mode for NB-IoT.

Recommended WF: Discuss the proposal.

#### Issue 2-1-3: Test case upon t-ServiceStart

Proposals:

- Proposal 1 (Nokia): For eMTC introduce a intra-frequency measurement test case where t-serviceStart is configured and 2 time instants are defined:

  - T1: Initial test conditions, UE receives the RRC configuration for measuring the neighbor cell configured with t-serviceStart. T1 ends at t-serviceStart, and the UE shall be reachable by the eNB during the measurement gap opportunities in T1.

  - T2: Initiated at t-serviceStart, when Cell 2 becomes available. UE shall start measurements on the neighbor cell and send the measurement report.

- Proposal 2 (Nokia): For NB-IoT, do not introduce the test case where t-serviceStart is configured.

Recommended WF: Discuss the proposal.

#### Issue 2-1-4: For eMTC, test case with GNSS gap

Background: In WF in RAN4#110, R4-2403296

- FFS RAN4 to introduce tests as follows for verify the GNSS measurement when the GNSS gaps overlaps with the mobility measurement gaps for eMTC, considering selecting a short GNSS gap length to make test practical:

  - Subtest 1: gaps are colliding and GNSS measurements are performed and neighbour cell measurements are not performed.

  - Subtest 2: GNSS measurement is completed and UE performs neighbour cell measurements.

Proposals:

- Proposal 1 (Nokia): Do not introduce GNSS-MG test cases unless needed to verify a measurable UE behaviour.

- Proposal 2 (CMCC): Introduce such test case to verify the GNSS measurement and UE behavior when GNSS gap and measurement gap overlapping, as long as RAN4 confirm that GNSS measurement can be supported by TE.

- Proposal 3 (Huawei): For GNSS re-acquisition, if the performance of measurement after early termination is to be verified, following test steps can be considered:

  - NW trigger GNSS measurement gap from T1. The configured GNSS gap length is longer than UE reported GNSS-PositionFixDuration.

  - T2 starts when UE has performed RACH procedure after GNSS fix. The target neighbour cell is turned on at T2.

  - UE shall be able to report measurement report with delay less than Tidentify from T2.

Recommended WF: Discuss whether to introduce GNSS-MG test cases.

### Sub-Topic 2-2: Test set-up

#### Issue 2-2-1 Test configuration on Quasi-Earth-Fixed cell or Earth-Moving cell

Proposals:

- Proposal 1 (CMCC): For NB-IoT/eMTC, introduce time-based and location-based measurement triggering test cases for RRC Idle and RRC connected modes in Earth Fixed cell scenarios.

- Proposal 2a (CMCC): Further study the feasibility of emulating the test scenario of earth-moving cell.

- Proposal 2b (Huawei): Further discuss whether to define test case for location-based measurement in earth moving cell considering one constant ephemeris is used.

Recommended WF:

- Define test case with Quasi-Earth-Fixed cell as the baseline

- Further study the feasibility and whether to define test with earth-moving cell, considering one constant ephemeris is used.

#### Issue 2-2-2 Test configuration with neighbor satellite

Proposals:

- Proposal 1 (Nokia): For Idle and Connected mode mobility test cases, select half of the test cases to be tested with the neighbor cell associated to a different satellite.

Recommended WF: Discuss proposals.

### RRM perf part draft CRs

| T-doc number | Title | Source |
| --- | --- | --- |
| R4-2405432 | Draft CR on Test cases of cell re-selection and RRC reestablishment for NB IoT NTN enhancements | MediaTek inc. |
| R4-2404995 | Draft CR on TC for NB-IoT NTN CONN | Huawei, HiSilicon |
| R4-2405758 | DraftCR to 36.133 on Configuration of Cells in Neighbor Satellite | Nokia |
| R4-2404690 | (IoT_NTN_enh-Perf) draftCR to TS 36.133 Introduce of cell re-selection test cases for Cat-M1 UE | CMCC |
| R4-2405070 | Draft CR for 36.133: E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | Ericsson |
| R4-2405756 | DraftCR to 36.133 on Inter-Frequency HO for Cat-M1 UEs in CEModeA in NTN | Nokia |
| R4-2405757 | DraftCR to 36.133 on Intra-Frequency HO for Cat-M1 UEs in CEModeA in NTN | Nokia |
| R4-2405759 | DraftCR to 36.133 on Inter-Frequency Conditional HO for Cat-M1 UEs in NTN | Nokia |
| R4-2405433 | Draft CR on Test cases of RSRP for Cat M1 NTN enhancements. | MediaTek inc. |