# 3GPP TSG-RAN WG4 Meeting #110bis                                 R4-2404854

# Changsha, China, 15th – 19th April, 2024

Agenda item:	6.24.3.1

Source:	China Telecom

Title:	Discussion on RRM performance requirements for NES

Document for:	Discussion

- Introduction

In RAN4 #110 meeting, RRM performance requirements for NES was discussed and the WF [1] was approved. In this paper, we present our views about RRM performance requirements for NR network energy saving.

- Discussion

Status in the WF [1]:

Issue 3-1-2: Detailed configurations for FR1 inter-band SSB-less activation/deactivation - Proposals Non-DRX mode: - Option 1: Test cases for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined. (CTC, Ericsson) RTD - Option 1: 4µs for SCell 15kHz SCS and 2µs for SCell 30kHz SCS. (CMCC) - Option 2: RAN4 to define the following test cases with RTD<=CP corresponding to the SCS of SSB-less Cell. (Ericsson, Huawei) Power difference - Option 1: Set EPRE difference between PCell and SSB-less SCell as 9dB. (CMCC) - Option 1a: The EPRE difference between the reference Cell and SSB-less Cell is configured as 9 dB + ΔPL in the test cases, where ΔPL is the pathloss difference caused by frequency difference between two Cells. (Huawei) - Option 1b: Configure the target inter-band SSB-less SCell to have a power level that is [< 9 dB] higher than that of the PCell and use an AWGN channel for the setup. (MTK) Test configuration - Option 1: For TC of inter-band SSB-less, PCell/PSCell is considered as reference Cell, and the configurations for the SSB-less Cell and reference Cell shall be defined independently in the test cases, in terms as SCS/BW/duplex mode. (Huawei) - Option 1a: The legacy R15 test scenario configuration can be reused. For PCell and SCell, the combinations of any two configurations among 15kHz/≥10MHz FDD, 15kHz/≥10MHz TDD and 30kHz/≥40MHz TDD are supported. (CMCC) TRS/A-TRS - Option 1: Reuse existing TRS/A-TRS configuration. (CMCC, QC) - Option 1a: Reuse the TRS and A-TRS configuration in legacy Fast SCell activation test case for SSB-less SCell. (CMCC) - Option 1b: (QC) - Proposal: Test setup for A-TRS based and P-TRS based SSBless SCell activation is same except A-TRS configuration. Existing P-TRS configuration can be reused (e.g TRS.1.1 TDD/FDD for 15KHz, TRS.1.2 TDD/FDD for 30KHz) - Proposal: R17 fast SCell activation with two A-TRS burst “A.4.5.3.7 Fast SCell Activation of known SCell in FR1 for 640 ms SCell measurement cycle” can be reused for A-TRS configuration with two bursts. Others: - Option 1: Don’t configure the parameter SCell measurement cycle (measCycleSCell). (CMCC) - Option 2: Design two time periods (T1 and T2) to verify the SCell activation procedure for inter-band CA SSB-less SCell. 7s for T1 and 1s for T2 can be the starting point. (CMCC) - Option 3: The TCI.State.2 could be used for the RS(s) of the SSB-less SCell. (CMCC) - Option 4: The TCI.State.0 could be used for the TRS of the SSB-less SCell with an additional note that the reference signal is SSB0 from PCell. (CMCC) - Option 5: Don’t configure the parameter SSB configuration and SMTC configuration for SSB-less SCell. (CMCC) - Option 6: For configuration of the TC for SSB-less SCell operation, it is suggested to test the scenario where both timing reference cell indication and QCL-C configuration are available for the UE. (Vivo) Agreement: - Test cases for FR1 inter-band SSB-less activation based on TRS/A-TRS in non-DRX need to be defined - The legacy R15 test scenario configuration can be reused. For PCell and SCell, the combinations of any two configurations among 15kHz/≥10MHz FDD, 15kHz/≥10MHz TDD and 30kHz/≥40MHz TDD are supported - Reuse the TRS and A-TRS configuration in legacy Fast SCell activation test case for SSB-less SCell

For detailed configurations for FR1 inter-band SSB-less activation/deactivation, the side conditions are defined in the core part that the RTD between the target SCell and the collocated reference serving cell is within CP where CP is corresponding to the SCS of SSB-less SCell, and the [EPRE] difference at the UE is smaller than or equal to [9] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC], therefore, the RTD for the test cases can be defined with RTD<=CP corresponding to the SCS of SSB-less SCell, and the power difference for the test cases can be defined as smaller than or equal to [9] dB between SSB-less SCell and reference serving cell.

Proposal 1: RTD for the test cases can be defined with RTD<=CP corresponding to the SCS of SSB-less SCell.

Proposal 2: Power difference for the test cases can be defined as smaller than or equal to [9] dB between SSB-less SCell and reference serving cell.

- Conclusion

The contribution presents our views on RRM performance requirements for NES, with the following proposals:

Proposal 1: RTD for the test cases can be defined with RTD<=CP corresponding to the SCS of SSB-less SCell.

Proposal 2: Power difference for the test cases can be defined as smaller than or equal to [9] dB between SSB-less SCell and reference serving cell.

- References

- R4-2403526, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #110, February-March 2024.