# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2404915

Changsha, CN, April 15 – April 19, 2024

Title: 	Discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode

Source: 	OPPO

Agenda item:	9.6.2

Document for:	Discussion

Introduction

In RAN#103 meeting, a new WID on RRM enhancement Phase 5 was approved [1].

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast SCell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

It is observed that FR2-1 SSB based RRM requirements allow excessively long delay for UE in L3 measurement, according to the large Rx beam sweeping factor and/or CSSF factor as specified in corresponding requirement tables of TS38.13. In this contribution, we will provide initial discussion on FR2-1 SSB based L3 measurement delay reduction for connected mode.

Discussion

Rx beam sweeping factor

In R18, for both FR2 SCell activation delay reduction (in eFeRRM WI) and FR2 L1 measurement in Multi-Rx operation (in multi-Rx WI), the work for FR2 delay reduction has already started and faster beam sweeping are considered. It is straightforward to leverage the method of fast beam sweeping (e.g., for FR2 multi-Rx reception) for UE supporting multiple-Rx simultaneous reception on single carrier.

| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer-1 measurement | Supports beam sweeping factor reduction for SSB-based layer-1 measurement. | Per band | FR2-1 only | Candidate values for Component 2: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |

Due to possible enhanced UE capabilities and improved procedures, FR2 L3 measurement delay reduction is feasible under some new assumption, e.g., utilizing multi-Rx chain/multi-panel to perform fast beam sweeping.

Taking FR2 intra-frequency L3 measurements as example, the legacy M considers the beam sweep factor as 8 for different UE power classes, including Mpss/sss_sync_w/o_gaps, and Mmeas_period_w/o_gaps. Due to the signaling deriveSSB-IndexFromCell is always enabled for FR2-1, MSSB_index_intra is not considered.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

Proposal 1: RAN4 to define the requirements of TPSS/SSS_sync_intra and TSSB_measurement_period_intra and corresponding conditions for faster beam sweeping to reduce FR2-1 L3 measurement delay for UE supporting multi-Rx simultaneous reception.

For applicability rule, we support to consider power class 3 as first priority, and the requirements should be defined for UE under multi-Rx operation.

Proposal 2: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

RAN4 can first agree on reduction of L3 measurements delay on neighbour cells. For deactivated SCell and PSCell in FR2-1, the enhancement can also apply.

Proposal 3: For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply.

CSSFoutside_gap

For UE not in multiple-Rx simultaneous reception mode, RAN4 should discuss how to reduce FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC scenarios firstly. As baseline assumption on number of searchers is 2.

Proposal 4: RAN4 to consider EN-DC, NE-DC, SA and NR-DC for optimizing CSSFoutside_gap,i scaling factor.

CSSFintra is a carrier specific scaling factor and is determined according to CSSFoutside_gap,i in clause 9.1.5.1 for measurement conducted outside measurement gaps, i.e.

- Case 1: when intra-frequency SMTC is fully non-overlapping or partially overlapping with GAP, or

- Case 2: when intra-frequency SMTC is fully non-overlapping with GAP for UE indicating [no gap with interruption], or

- Case 3: when intra-frequency SMTC is fully non overlapping or partially overlapping with GAP for UE indicating [no gap without interruption].

Take CSSFoutside_gap,i for SA mode as example, the CSSFs are defined for FR2 carriers including:

- CSSFoutside_gap,i for FR2 PCC

- CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required

- CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required

- CSSFoutside_gap,i for inter-frequency MO with no measurement gap

- CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap

For the case of FR2 only intra-CA or FR2 only inter-band CA, FR2 PCell occupies one searcher and all neighbor FR2 Scell share another searcher. CSSFoutside_gap,i for FR2 SCC equals to NSCC_SSB +Y+Z+2* NSCC_CSIRS regardless neighbor cell measurement is required or not, where

NSCC_SSB equals to the number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG.

Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0.

Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0.

NSCC_CSIRS equals to the number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured

The candidate optimization could be done by reduce the components of CSSFoutside_gap,i. RAN4 needs to discuss the conditions of counting the number of configured SCell(s) or MOs without MG, because there may exist overlapping or close frequency layers of different Scell(s) or MO(s). By the way, inter-frequency MOs and E-UTRA inter-RAT MOs may share the same frequency layer and can be measured simultaneously.

Proposal 5: RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements.

In addition, it is also important to avoid new impact to UE performance of measurements in R19. RAN4 can consider to reuse existing accuracy requirements as baseline.

Proposal 6: RAN4 to consider existing accuracy requirements of FR2-1 SSB based measurement as baseline.

Conclusions

In this paper we provided our initial views on FR2-1 SSB based L3 measurement delay reduction for connected mode. The followings are provided.

Proposal 1: RAN4 to define the requirements of TPSS/SSS_sync_intra and TSSB_measurement_period_intra and corresponding conditions for faster beam sweeping to reduce FR2-1 L3 measurement delay for UE supporting multi-Rx simultaneous reception.

Proposal 2: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Proposal 3: For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply.

Proposal 4: RAN4 to consider EN-DC, NE-DC, SA and NR-DC for optimizing CSSFoutside_gap,i scaling factor.

Proposal 5: RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements.

Proposal 6: RAN4 to consider existing accuracy requirements of FR2-1 SSB based measurement as baseline.

Reference

- RP-240830, New WID: WI resulting from discussion on RRM enhancements