3GPP TSG-RAN WG4 Meeting #110bis	R4-2404929

Changsha, China, April 15th – 19th, 2024

Agenda item:	6.17.1.2

Source:	Intel Corporation

Title:	Power boosting features applicability to UL CA scenarios

Document for:	Discussion

# Introduction

The updated UE feature list for Rel-18 NR coverage enhancements work item was agreed in RAN4 #110 [1]. The majority of open issues were resolved, however, there are still open issues on applicable scenarios for power boosting (PAR/MPR enhancements) features and, especially, with respect to CA scenarios applicability [2]:

WF - RAN4 is still discussing the applicable scenarios. - The common understanding in RAN4 is that power boosting is only supported for single CC UL at least in Rel-18. - UL CA is not supported for this feature in Rel-18.

This contribution provides views on the applicable scenarios for Rel-18 power boosting features powerBoosting-pi2BPSK-QPSK-r18 and powerBoosting-pi2BPSK-QPSK-r18 and specifically on UL CA applicability.

# Discussion

During the WI stage the focus of technical analysis was primarily on enhancing power for UL single carrier scenarios. However, the comprehensive discussion on CA aspects was somewhat lacking. We believe it is crucial to address this gap, particularly in clarifying and finalizing UE behavior concerning UL CA support/configuration.

It is essential to recognize that the continuation of work on power enhancements in future releases appears improbable (e.g. no relevant objectives in Rel-19 work package). Therefore, it is helpful to seize the opportunity to define proper UE behavior concerning UL CA support at this point to maximize opportunities for the commercial feature adoption rather than leaving this undefined.

The key motivation behind supporting power boosting feature is to improve coverage performance, particularly for UEs operating at cell edges and utilizing lower-order modulations such as pi/2 BPSK and QPSK. However, it is crucial to note that being configured with coverage enhancements features not necessarily means that UE operates in single-carrier mode. In dynamic propagation conditions, networks may need to configure UEs to operate in CA mode consistently, while dynamically scheduling single-carrier or multi-carrier transmissions based on instantaneous propagation and traffic conditions. Limiting power boosting exclusively to single carrier case might impede network flexibility, potentially resulting in frequent UE CA (re)configurations or not enabling the feature in practical deployments.

Observation #1:	Limiting power boosting exclusively to single carrier case might impede network flexibility, potentially resulting in frequent UE CA (re)configurations or not enabling the feature in practical deployments.

Therefore, we see a value in enabling the power boosting feature in UL CA scenarios as long as it does not require substantial extra RAN4 work on the associated requirements definition.

Proposal #1:	Support power boosting enhancements/features for UL CA scenarios

Conditions to apply power boosting in UL CA scenarios

Acknowledging the complexities related to the support of simultaneous UL transmissions with enabled power boosting on multiple carriers (N > 1), it is evident that further extensive studies on associated RF requirements may be required to enable such scenario.

One pragmatic approach to address this issue to limit UE power boosting for CA scenarios to instances where UL CA is configured, but certain scheduling restrictions are applied in terms of simultaneous UL transmissions on different carriers. For instance, scenarios where only single-carrier UL transmissions are scheduled within a slot (e.g., PUSCH/PUCCH is scheduled on one carrier, while other carriers do not have active UL transmission). This approach ensures that network can seamlessly transition UE operation between single-carrier and multi-carrier transmissions as dictated by dynamic propagation conditions, without the need for CA reconfigurations, while the implementation complexity of power boosting remains approximately same as for the single carrier case. A simplified example of such approach is provided in Figure 1, when UL CA has 2 component carriers and the power boosting is applied depending on whether UL transmissions are scheduled in one or both carriers:

Figure 1. Power boosting for UL CA scenarios.

Another potential approach is to allow use of power boosting features for the case of UL CA under assumption that UE has a single active UL carrier per band (i.e. in case UL SCells were de-activated). The respective approach is less flexible comparing to the aforementioned and is less desirable.

Proposal #2:	For UL CA scenarios the existing power boosting requirements apply under assumption that there are no simultaneous UL transmissions on other UL carriers scheduled within the slot for the same UL band combination or for the band in the UL band combination.

RF requirements

Under assumptions of using UL scheduling restrictions to apply power boosting under UL CA scenarios no impact on the RF requirements is expected and single carrier requirements can be reused.

UE capabilities signalling

Based on current assumptions UE reports the power boosting capabilities with “per FS” (per band per band combination) granularity, which already provides sufficient flexibility for UE to report the respective capabilities and no changes are needed to support power boosting for UL CA. The proposed UE feature list are provided in [2].

# Conclusion

In this contribution we provided views on provides views on the applicability of Rel-18 power boosting features powerBoostRel18 and powerBoostTSRel18 and to UL CA scenarios and in summary make the following proposals:

Proposal #1:	Support power boosting enhancements/features for UL CA scenarios

Proposal #2:	For UL CA scenarios the existing power boosting requirements apply under assumption that there are no simultaneous UL transmissions on other UL carriers scheduled within the slot for the same UL band combination or for the band in the UL band combination.

# References

[1] R4-2403885, “WF on coverage enhancement for part 2”, Nokia, RAN4 #110, February 2024

[2] R4-2404928, “Views on RAN4 Rel-18 UE feature list”, Intel Corporation, RAN4 #110bis, April 2024