3GPP TSG-RAN4 Meeting #110bis	R4-2404942

# Changsha, China,  April 15th - April 19th 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5 on top of the running CR R4-2401116: Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-04-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This draft CR updates the endorsed running CR R4-2401116 for Rx requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Simplification and rewording of REFSENS requirement Addition of ACS, merging fixed and mobile VSAT as they should have sane ACS limit. Addition of Rx antenna off-axis performance which was missing, but specified in all ETSI ENs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NTN ka-bands won’t be correctly supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

# 10	Radiated receiver characteristics

## 10.1	General

Unless otherwise stated, the receiver characteristics are specified over the air (OTA) at the RIB for Ka bands fixed and mobile VSAT. The reference effective isotropic sensitivity (EIS), wanted signals and interference is defined assuming a 0 dBi reference antenna located at the center of the quiet zone.

## 10.2	Polarization characteristics

The minimum requirements on the receiver characteristics apply under one polarization.

## 10.3	OTA reference sensitivity level

### 10.3.1	General

The OTA REFSENS requirement is a directional requirement and is intended to ensure the minimum OTA reference sensitivity level at the centre of the quiet zone in the RX beam peak direction. The OTA reference sensitivity power level EISREFSENS is the minimum mean power received over the air at the RIB, at which the throughput shall meet or exceed the requirements for a specified reference measurement channel.

### 10.3.2	Minimum requirement

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as [specified in Annexes A.2.3.2 and A.3.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.2.1) with peak reference sensitivity specified in Table 10.3.2-1 and Table 10.3.2-2. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link Angle).

Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT

| NTN VSAT channel bandwidth (MHz) | UL/DL RB allocation | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- |
| 50, 100, 200, 400 | Full RB allocation NRB as specified in clause 5.3.2 | EISREFSENS_50MHz + 10log10(NRB x SCS x 12 / factor) (NOTE 1) |
| NOTE 1: The “factor” represents the normalized factor to scale EIS for different (Channel bandwidth, SCS) configurations. The value of factor is 66 RBs x 60 kHz SCS x 12, i.e. 47520 kHz. |  |  |

Table 10.3.2-2: EISREFSENS_50MHz  value per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | -126.8 |
|  |  | 3 | -115.6 |
| n512, n511, n510 | Mobile VSAT | 4, 5 | -126.8 |

<End of the change>

<Start of the change>

## 10.5	Adjacent channel selectivity

### 10.5.1	General

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both from the direction where peak gain is achieved.

The wanted and interfering signals apply to all supported polarizations, under the assumption of polarization match.

### 10.5.1	Minimum requirement

The NTN VSAT shall fulfil the minimum requirement specified in Table 10.5.1-1 for all values of an adjacent channel interferer up to [–25] dBm. However, it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 10.5.1-2 and Table 10.5.1-3 where the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.x.x.x. and A.x.x.x, with  one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.x.x.x. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 10.5.1-1: Adjacent channel selectivity

| Operating band | Units | Adjacent channel selectivity / Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| n512, n511, n510 | dB | TBD | TBD | TBD | TBD |

Table 10.5.1-2: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + TBD dB |  |  |  |
| PInterferer for band n512, n511, n510 | dBm | REFSENS + TBD dB | REFSENS + TBD dB | REFSENS + TBD dB | REFSENS + TBD dB |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 3 | 100 / -100 NOTE 3 | 200 / -200 NOTE 3 | 400 / -400 NOTE 3 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex x with one sided dynamic OCNG Pattern as described in Annex x and set-up according to Annex x. NOTE 2: The REFSENS power level is specified in Clause 10.3.2.s NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. |  |  |  |  |  |

Table 7.5-3: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration for band n512, n511, n510 | dBm | TBD | TBD | TBD | TBD |
| PInterferer | dBm | TBD |  |  |  |
| BWInterferer | MHz | 50 | 100 | 200 | 400 |
| FInterferer (offset) | MHz | 50 / -50 NOTE 2 | 100 / -100 NOTE 2 | 200 / -200 NOTE 2 | 400 / -400 NOTE 2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex x.x with one sided dynamic OCNG Pattern TDD as described in Annex x and set-up according to Annex x. NOTE 2: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 3: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in clause 9.2.3, with uplink configuration specified in Table 10.3.2-1. |  |  |  |  |  |

<End of the change>

<Start of the change>

## 10.8	Receiver antenna off-axis performance

The following requirements are applicable to NTN VSAT operating in band n512.

The receiver antenna off-axis gain of each co-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-1.

Table 10.8-1: Off-axis Co-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φmin ≤ φ ≤ 48° | 32 – 25logφ |
| 48° ≤ φ ≤ 85° | -10 |
| 85° ≤ φ ≤ 180° | 0 |
| Note: φmin = 1o or 100λ/D (degrees) whichever is the greater, for D/λ ≥ 50. φmin = 2o or 114(D/ λ)-1.09 (degrees) whichever is the greater, for D/λ < 50. where D is the nominal diameter of the antenna |  |

The receiver antenna off-axis gain of each cross-polarized components in any direction φ degrees from the antenna main beam shall not exceed the levels specified in Table 10.8-2.

Table 10.8-2: Off-axis Cross-polarized gain limit

| φ value (degree) | gain (dBi) |
| --- | --- |
| φr ≤ φ ≤ 7° | 23 – 20logφ |
| Note: φr = 1o or 100λ/D (degrees) whichever is the greater where D is the nominal diameter of the antenna |  |

<End of the change>