3GPP TSG-RAN WG4 Meeting #110bis	R4-2405071

Changsha, China, 15 – 19 April, 2024

Agenda Item:	4.2.2.3

Source:	Ericsson

Title:	(LTE_NBIOT_eMTC_NTN_req-Core) Discussions on open issues of IoT NTN performance requirements

Document for:	Discussion

# 1	Introduction

The IoT NTN WI was completed but there are still some unresolved issues in the specification that need to be resolved. In this paper we discuss those open issues of core and performance part and provide our view.

# 2	Discussion on performance part

PHR reporting

With regard to PHR reporting requirements, following agreements were reached [1]:

#### Issue 1: PHR reporting for NB-IoT in GEO - Agreements - Introduce a new PHR reporting table for NB-IoT in GEO and use legacy values in [] - Note: whether any revision of values is needed can be discussed in the maintenance stage

Based on the above agreement, the specification was updated to introduce separate PHR reporting requirements for GSO and NGSO respectively for NB-IoT as shown below [2]. RAN4 shall maintain this agreement and only update the values in the porting tables.

#### 9.1.23A.3 Report Mapping for UE Category NB1 for Satellite Access For UE category NB1 in GSO when the enhanced coverage level 0 is selected during the random access procedure [17], the report mapping is defined in Table 9.1.23A.3-1 for UEs not supporting enhanced PHR. Table 9.1.23A.3-1: Power headroom report mapping for UE category NB1 UEs in GSO not supporting enhanced PHR [31] when the enhanced coverage level 0 is selected during random access procedure [17] Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH [5]. POWER_HEADROOM_1; [5] PH [8]. POWER_HEADROOM_2; [8] PH [11]. POWER_HEADROOM_3; PH ≥ [11]. For UE category NB1 in GSO when enhanced coverage level other than 0 is selected during the random access procedure [17], the report mapping is defined in Table 9.1.23A.3-2 for UEs not supporting enhanced PHR. Table 9.1.23A.3-2: Power headroom report mapping for UE category NB1 UEs in GSO not supporting enhanced PHR [31] when the enhanced coverage level other than 0 is selected during random access procedure [17] Reported value; Measured quantity value (dB). POWER_HEADROOM_0; -54 PH [-10]. POWER_HEADROOM_1; [-10] PH [-2]. POWER_HEADROOM_2; [-2] PH [6]. POWER_HEADROOM_3; PH ≥ [6]. For UE category NB1 in GSO, the the report mapping defined in clause 9.1.23.3 applies for UEs supporting enhanced PHR. For UE category NB1 in NGSO, the report mapping defined in clause 9.1.23.3 applies.

- Observation 1: RAN4 has agreed to define separate PHR reporting tables for GSO and NGSO.

The values are in square brackets and need to be agreed. One important difference between GEO and LEO is in the altitude they are operating at. The geostationary satellites operate around 36 000 km above the earth while the LEO satellites operate at altitude around 500 km – 1200 km above the earth. This difference in altitude will be reflected on the power headroom reporting because of the differences in pathloss which were observed in the technical report in TR 38.821. Table 1 below shows the path loss for GEO and LEO satellites in S-band [3].

Table 1: Link budgets results

| Case | Transmission mode | Frequency [GHz] | TX: EIRP [dBm] | RX: G/T [dB/T] | Bandwidth [MHz] | Free space path loss [dB] | Atmospheric loss [dB] | Shadow fading margin [dB] | Scintillation Loss [dB] | Polarization loss [dB] | Additional losses [dB] | CNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SC4 | DL | 2.0 | 103.8 | -31.6 | 30.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC5 | DL | 2.0 | 99.0 | -31.6 | 10.0 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | 0.0 |
|  | UL | 2.0 | 23.0 | 19.0 | 0.4 | 190.6 | 0.2 | 3.0 | 2.2 | 0.0 | 0.0 | -10.9 |
| SC9 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC10 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 6.6 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 2.8 |
| SC14 | DL | 2.0 | 84.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC15 | DL | 2.0 | 80.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 7.2 |
|  | UL | 2.0 | 23.0 | 1.1 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -2.6 |
| SC19 | DL | 2.0 | 98.3 | -31.6 | 30.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC20 | DL | 2.0 | 93.5 | -31.6 | 10.0 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -5.2 |
|  | UL | 2.0 | 23.0 | 14.0 | 0.4 | 190.4 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -15.7 |
| SC24 | DL | 2.0 | 72.8 | -31.6 | 30.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC25 | DL | 2.0 | 68.0 | -31.6 | 10.0 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 0.6 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 159.1 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -3.2 |
| SC29 | DL | 2.0 | 78.8 | -31.6 | 30.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| SC30 | DL | 2.0 | 74.0 | -31.6 | 10.0 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | 1.2 |
|  | UL | 2.0 | 23.0 | -4.9 | 0.4 | 164.5 | 0.1 | 3.0 | 2.2 | 0.0 | 0.0 | -8.6 |
| NOTE: The link budget calculations including CIR and CINR results contributed by the companies are available in [24]. |  |  |  |  |  |  |  |  |  |  |  |  |

The table 1 shows the free space pathloss for GEO and LEO satellites for the scenarios defined in Table 6.1.1.1-9 in [3]. The scenarios are also included in the Appendix of this contribution for convenient reading. SC1-SC5, SC 16- shows the results GEO with different assumptions. SC6-SC15 and SDC21-SC30 show the results for LEO with different assumptions. Higher pathloss for GEO compared to LEO can be observed from these results. The UE may therefore use more transmit power for reaching a satellite at 36 00 km above the earth compared to reaching a satellite above 16 000 km above the earth. This means, for a UE with only 4 reportable values, it is therefore reasonable to assume more negative values for GEO compared to LEO since they can provide more accurate information about the actual power usage to the network. The network can use this information to perform link adaptation, e.g. to select more suitable transmission parameters.

We have made our proposals in Table 2 for normal coverage and Table 3 for enhanced coverage. For normal coverage, we have kept the lower range (-54) from legacy and have included 3 more negative values compared to the legacy table which is used for LEO. Similarly, for enhanced coverage, we have proposed to reuse the lower range from legacy table but to include more negative values to have better resolution in the lower range.

- Observation 2: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 500 - 1 200 km above the earth.

- For NB-IoT PC3 and PCE5 UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 2.

Table 2: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For NB-IoT PC3 and PC5 UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3.

Table 3 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

In addition, the PHR reporting tables for UE power class 6 contain square brackets. Following the logic from legacy, the PC3 and PC5 PHR reporting tables for normal coverage can be reused for PC6. The values of enhanced coverage table for PC6 were modified in legacy to include more negative values because of the lower transmit power. Because of same reason, values from the normal coverage are proposed to be reused for lower range while the maximum range is kept same as in legacy, i.e. 0.

- For NB-IoT PC6 UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values from normal coverage for PC3 and PC5 are reused.

- For NB-IoT PC6 UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 2.

Table 4: Power headroom report mapping for UE category NB1 in GSO not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 0 |
| POWER_HEADROOM_3 | PH ≥ 0 |

# Conclusion

In this contribution we have discussed and provided our view on the remaining RRM issues related to the core and performance part based on the way forward document agreed at last meeting. Based on the discussions, we have made following proposals:

- Observation 1: RAN4 has agreed to define separate PHR reporting tables for GSO and NGSO.

- Observation 2: GEO satellites operate around 36 000 km above the earth while LEO satellites operates around 500 - 1 200 km above the earth.

- For NB-IoT PC3 and PCE5 UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 2.

Table 2: Power headroom report mapping for UE category NB1 UEs for a cell (e.g. cell1) served by GEO satellite during random access procedure

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -10 |
| POWER_HEADROOM_1 | -10 PH -2 |
| POWER_HEADROOM_2 | -2 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For NB-IoT PC3 and PC5 UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 3.

Table 3 Power headroom report mapping for UE category NB1 UEs not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure and UE is served by GEO satellite

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 6 |
| POWER_HEADROOM_3 | PH ≥ 6 |

- For NB-IoT PC6 UE in normal coverage managed by GEO with only 4 reportable values, the PHR reporting values from normal coverage for PC3 and PC5 are reused.

- For NB-IoT PC6 UE in enhanced coverage managed by GEO with only 4 reportable values, the PHR reporting values are defined as shown in Table 2.

Table 4: Power headroom report mapping for UE category NB1 in GSO not supporting enhanced PHR when the enhanced coverage level other than 0 is selected during random access procedure [17] for UE PC6

| Reported value | Measured quantity value (dB) |
| --- | --- |
| POWER_HEADROOM_0 | -54 PH -40 |
| POWER_HEADROOM_1 | -40 PH -30 |
| POWER_HEADROOM_2 | -30 PH 0 |
| POWER_HEADROOM_3 | PH ≥ 0 |

# References

R4-2310098, " WF on RRM requirements for NTN NB-IoT/eMTC ", Moderator (MediaTek inc.)

TS 36.133 V18.3.0