3GPP TSG-RAN WG4 Meeting #110bis	R4-2405095

Changsha, China, 15 April –19 April 2024

Agenda Item:	6.8.7.1

Source:	Ericsson

Title:	UE demodulation requirements for NR less than 5MHz

Document for:	Discussion

# 1	Introduction

RAN4#110 agreed with the WF on the UE demodulation requirements for NR less than 5MHz [1], and RAN4 agreed to define PDCCH and PBCH demodulation requirements.

We continue to discuss the UE demodulation requirements for UE supporting NR less than 5MHz.

# 2	Discussion

## 2.1	PDCCH demodulation performance

### 2.1.1	Punctured PDCCH

#### 2.1.1.1	Simulation results

Table 1 shows the candidate simulation assumptions for PDCCH demodulation requirements [1]. Table 2 shows the detailed CORESET configurations. It is also important to specify where PDCCH with DCI 1_0 is allocated to avoid misalignment of the simulation results.

Table 1	Simulation assumptions for PDCCH with puncturing.

| Number of Tx | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x2 Low |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x2 Low |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x4 Low |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x4 Low |

Table 2	CORESET configuration for PDCCH decoding requirements.

| Parameters | Values | Notes |
| --- | --- | --- |
| CORESET0 configuration index (TS38.213 Table 13-0) | 8 | Non-interleaved, PR offset: 0, 3 symbols |
| REG bundle size (L) | 6 |  |
| Shift index (nshift) | NIDcell (see TS 38.211 7.3.2.2) |  |
| PDCCH CCEs allocation | 4, 5, 6, 7 |  |
| DCI payload size | 35 bits | DCI 1_0, 15RB |

Figure 1 shows our initial simulation results where we compare the cases with and without puncturing, that is, the non-puncturing PDCCH case corresponds to the CCE index set {0, 1, 2, 3} and the puncturing PDCCH case corresponds to the CCE index set {4, 5, 6, 7}, respectively. Table 3 summarizes the SNR (dB) to achieve 1% of BLER (or probability of missed downlink scheduling grant, Pm-dsg).

Figure 1	Punctured PDCCH simulation results.

Table 3	Summary of PDCCH simulation results

|  | 1Tx - 2Rx | 1Tx - 4Rx | 2Tx - 2Rx | 2Tx - 4Rx |
| --- | --- | --- | --- | --- |
| Punctured PDCCH | 3.8 | -2.6 | 1.1 | -3.8 |
| Non-punctured PDCCH (For information) | 3.5 | -3.7 | 0.0 | -5.1 |

#### 2.1.1.2	Discussion

Way forward: - Further discuss whether to define requirements with 1TX and/or 2TX.

One of the remaining open issues on the puncturing PDCCH demodulation requirements is the configuration of the number of Tx antennas: 1Tx and/or 2Tx. Firstly, we think it is sufficient to define one set of requirements since the UE demodulation algorithm is independent of the number of Tx antennas. We slight prefer to select 2Tx with the reason that 1) Tx diversity is used in commercial and 2) 2Tx case configures higher Doppler spread such as 100Hz.

Proposal 1: Configure 2Tx for the punctured PDCCH demodulation requirements.

### 2.1.2	High-speed train condition

Another open issue is whether to define PDCCH demodulation requirements with HST condition such as HST-417.

Issue 1-3-3: PDCCH requirements in HST conditions Way forward: Further discussion is needed: - Option 1: Introduce PDCCH requirements at 3MHz CBW in HST conditions. - Option 2: Not to introduce HST scenario for PDCCH requirements.

As we discussed in the past meetings, RAN1 ensures PDCCH-DMRS on every OFDM symbol, which does not need to perform the time-domain channel estimation interpolation, unlike PDSCH. This is also the reason why RAN4 has not introduced PDCCH demodulation requirements in HST condition in Rel-16/17/18 HST-related work items.

Figure 2	PDCCH demodulation performance comparison between HST-417 and AWGN.

Table 4	Simulation parameters for PDCCH with HST-417/AWGN.

| Number of Tx | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | DCI size | Propagation Condition | Antenna configuration and correlation Matrix |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 15 | 3 | 4 | 35 bits | HST-417 AWGN | 1x2 Low |

Figure 2 shows our PDCCH simulation results to evaluate the impact with HST channel compared with AWGN, according to the simulation parameters in Table 4, since HST-417 is based on AWGN channel model. As shown in the figure the impact with HST channel model is very small (< 0.2dB) at the SNR test point to achieve 1% BLER, which means PDCCH is robust to the Doppler shift caused by HST model.

Observation 1: PDCCH is robust to the Doppler shift caused by HST model.

With the observation, we don’t think RAN4 need to define new PDCCH demodulation requirements with HST channel model.

Proposal 2: Not to introduce HST scenario for PDCCH demodulation requirements.

## 2.2	PBCH demodulation performance

RAN4 has already agreed to define the punctured PBCH demodulation requirements with unknown SSB index for 2Rx/4Rx. Table 3 summarize our alignment simulation results and impairment simulation results to define PBCH demodulation requirements.

Table 5	Simulation results for PBCH demodulation with unknown SSB index.

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Alignment results | Impairment results |
| --- | --- | --- | --- | --- | --- | --- |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low | -5.8 | -3.8 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 4 Low | -8.5 | -6.5 |

# 3	Summary

Proposal 1: Configure 2Tx for the punctured PDCCH demodulation requirements.

Observation 1: PDCCH is robust to the Doppler shift caused by HST model.

Proposal 2: Not to introduce HST scenario for PDCCH demodulation requirements.

# References

- R4-2402863, “WF on [110][318] NR_FR1_lessthan_5MHz_BW_demod”, Nokia, Nokia Shanghai Bell.