# 3GPP TSG-RAN WG4 Meeting #110bis                             R4-2405119

# Changsha, China, April 15 – 19, 2024

# Source:        ZTE Corporation

# Title:           Discussion on UE RF requirements for LP-WUS/WUR

# Agenda Item:   9.14.2

# Document for:  Discussion

- Introduction

In RAN#103 meeting, the WID on LP-WUS/WUR was agreed and RAN4 scope is as follows[1].

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this contribution, we provided some preliminary simulation results on guard RBs for ASCS.

- Discussion

In SI phase, RAN4 agreed to adopt two approached for guard RB evaluation, i.e., filter suppression level analysis and link level simulation, and some related simulation assumptions has been listed in Table 1[2].

Table 1  LP-WUS evaluation parameters for WUS at centre

| NR RF channel BW | 20MHz/100MHz for 5MHz WUS |
| --- | --- |
| Guardband of NR channel | Unchanged, defined in Clause 5.3.3 in TS 38.101-1 |
| WUS BW within NR channel | Max 5.04 MHz |
| WUS RB allocation (Note 1) | 24 RB (15kHz SCS) or 12 RB (30kHz SCS) in 5.04 MHz; |
| WUS placement within NR channel | case 1: Center; |
| Guard RB size of LP-WUS | 0 RB, 0.5RB,1RB, 2RB, 3RB at each side Symmetric guard RB |
| ASCS interferer | 5MHz |
| Filter characteristic | 2nd to 5th order Butterworth lowpass@ baseband bandpass@100MHz bandpass@2.6GHz |
| Filter passband BW | Same as WUS bandwidth (number of RBs) |
| Target ACS | TBD |
| Target ASCS | TBD |
| Target WUS SNR | TBD |
| NR power level | Same PSD with WUS signal |
| RF impairment | Max. Center frequency offset = 5, 10, 50, 200 ppm. Initial search |
| Note 1: For easy evaluation, the number of allocated WUS RBs is fixed. The number of Guard RBs is changed. |  |

We have achieved an agreement that for ASCS guard RB, the required guard RB(s) should be blanked RB(s), which belong to WUS carrier/BW (i.e., WUS signal plus ASCS guard RB(s))[2]. The purpose of guard RBs for ASCS is to protect the WUS RB from the adjacent sub-carrier NR signal. For ASCS evaluation, it is feasible to consider the worst case that WUS is at the center of NR channel for ASCS evaluation as shown in Figure 1, where 5MHz BW for interference is configured.

Figure 1  LP-WUS at the centre of NR channel, only ASCS needs to be considered.

Here we provide some preliminary simulation results based on the assumption in Table 1. In Figure 2, we show the effect of 2nd to 5th Butterworth filters on LP-WUS and Interference with same PSD, where CBW=20MHz, SCS=15kHz. ASCS is defined as the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent sub-carrier(s) [2]. In Table 2, we show the ASCS results under different guard RBs, frequency offset and filter order.

Figure 2  The effect of 2nd to 5th Butterworth filters on LP-WUS and Interference with same PSD

Table 2  The ASCS results under different guard RBs, frequency offset and filter order

| Filter order |  |  | ASCS, BWinterference = 5MHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Guard RB CBW | Frequency Offset | 0RB | 0.5RB | 1RB | 2RB | 3RB |
| 5th | 20MHz, SCS = 15kHz | 0 ppm | -17.64 | -19.30 | -20.63 | -23.29 | -25.90 |
|  |  | 5 ppm | -17.67 | -19.08 | -20.45 | -23.13 | -25.72 |
|  |  | 10 ppm | -17.55 | -18.92 | -20.26 | -22.93 | -25.55 |
|  |  | 50 ppm | -16.19 | -17.46 | -18.81 | -21.52 | -24.15 |
|  |  | 200 ppm | -14.04 | -15.21 | -16.44 | -18.95 | -21.49 |
|  | 100MHz, SCS = 30kHz | 0 ppm | -19.24 | -23.20 | -25.58 | -29.97 | -33.93 |
|  |  | 5 ppm | -19.53 | -23.05 | -25.45 | -29.84 | -33.79 |
|  |  | 10 ppm | -19.81 | -22.89 | -25.31 | -29.70 | -33.68 |
|  |  | 50 ppm | -19.09 | -21.58 | -24.00 | -28.51 | -32.61 |
|  |  | 200 ppm | -14.03 | -16.34 | -18.78 | -23.59 | -28.11 |
| 4th | 20MHz, SCS = 15kHz | 0 ppm | -15.75 | -16.95 | -17.97 | -20.02 | -22.01 |
|  |  | 5 ppm | -15.74 | -16.80 | -17.85 | -19.87 | -21.86 |
|  |  | 10 ppm | -15.64 | -16.65 | -17.71 | -19.74 | -21.73 |
|  |  | 50 ppm | -14.59 | -15.58 | -16.57 | -18.63 | -20.66 |
|  |  | 200 ppm | -12.88 | -13.77 | -14.68 | -16.59 | -18.50 |
|  | 100MHz, SCS = 30kHz | 0 ppm | -17.03 | -19.75 | -21.56 | -24.90 | -27.96 |
|  |  | 5 ppm | -17.18 | -19.64 | -21.43 | -24.79 | -27.87 |
|  |  | 10 ppm | -17.28 | -19.48 | -21.31 | -24.68 | -27.77 |
|  |  | 50 ppm | -16.64 | -18.48 | -20.30 | -23.77 | -26.93 |
|  |  | 200 ppm | -12.74 | -14.52 | -16.30 | -19.93 | -23.39 |
| 3rd | 20MHz, SCS = 15kHz | 0 ppm | -13.44 | -14.25 | -14.98 | -16.39 | -17.80 |
|  |  | 5 ppm | -13.42 | -14.17 | -14.87 | -16.29 | -17.70 |
|  |  | 10 ppm | -13.35 | -14.06 | -14.79 | -16.20 | -17.61 |
|  |  | 50 ppm | -12.62 | -13.28 | -13.98 | -15.41 | -16.82 |
|  |  | 200 ppm | -11.32 | -11.96 | -12.59 | -13.86 | -15.18 |
|  | 100MHz, SCS = 30kHz | 0 ppm | -14.21 | -15.92 | -17.15 | -19.52 | -21.67 |
|  |  | 5 ppm | -14.26 | -15.83 | -17.08 | -19.44 | -21.61 |
|  |  | 10 ppm | -14.29 | -15.74 | -16.99 | -19.35 | -21.54 |
|  |  | 50 ppm | -13.76 | -15.03 | -16.29 | -18.72 | -20.94 |
|  |  | 200 ppm | -11.09 | -12.28 | -13.50 | -16.00 | -18.41 |
| 2nd | 20MHz, SCS = 15kHz | 0 ppm | -10.46 | -10.92 | -11.35 | -12.18 | -13.02 |
|  |  | 5 ppm | -10.42 | -10.87 | -11.30 | -12.13 | -12.97 |
|  |  | 10 ppm | -10.38 | -10.81 | -11.23 | -12.07 | -12.91 |
|  |  | 50 ppm | -9.95 | -10.34 | -10.76 | -11.60 | -12.45 |
|  |  | 200 ppm | -9.09 | -9.47 | -9.81 | -10.57 | -11.32 |
|  | 100MHz, SCS = 30kHz | 0 ppm | -10.61 | -11.53 | -12.26 | -13.66 | -14.95 |
|  |  | 5 ppm | -10.63 | -11.47 | -12.20 | -13.61 | -14.92 |
|  |  | 10 ppm | -10.61 | -11.42 | -12.16 | -13.55 | -14.87 |
|  |  | 50 ppm | -10.28 | -10.99 | -11.74 | -13.15 | -14.49 |
|  |  | 200 ppm | -8.76 | -9.42 | -10.10 | -11.54 | -12.95 |

It can be found that as the guard RB number increases, ASCS performance becomes better because of the attenuation function of the filter. In SI phase, we agreed a range of guard RBs for ASCS and in WI phase, we’d better give an accurate requirement on guard RBs.

However, before determining the guard RBs, it is needed to achieve an agreement on performance requirement for both filter suppression level analysis and link level simulation first. For example, the requirement of ASCS for filter suppression level analysis, or BLER requirement for link level simulation.

Proposal 1: It is needed to achieve an agreement on performance requirements for both filter suppression level analysis and link level simulation before determining guard RBs for ASCS.

- Conclusion

Propose 1: It is needed to achieve an agreement on performance requirements for both filter suppression level analysis and link level simulation before determining guard RBs.

- Reference

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony

- TR38.869(v18.0.0), Study on low-power wake-up signal and receiver for NR.