# 3GPP TSG-RAN WG4 Meeting # 110bis                                 R4-2405206

# Changsha, China, 15 April – 19 April, 2024

# Title: 	Simulation results for NES demodulation requirements

# Source: 	ZTE Corporation

# Agenda item:	6.24.4

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the NES demodulation requirements[1]. The results are provided to aid in determining where new requirements can/should be introduced.

# 2 Discussion

## 2.1 PDSCH requirements

According to the previous meeting[1] conclusions, we summarized the following Table 2-1 for PDSCH simulation assumptions.

- Table 2-1. PDSCH SSB-less SCell FR1 inter-band CA test setup

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| Channel model |  |  | {TDLA30-10} |
| Antenna configuration |  |  | 2x2, ULA Low |
| Fraction of maximum throughput (%) |  |  | 70 (Only SCell measured) |
| SCS/CBW |  |  | 30kHz/40MHz (106 PRB) |
| MCS |  |  | {16QAM, 0.48 (MCS13)} |
| Channel estimation |  |  | DM-RS based: non-ideal/practical |
| TO Estimation & Compensation |  |  | As per implementation |
| TO |  |  | {-0.9, -0.4, 0, 0.4, 0.9} CP |
| FFT window shift target |  |  | {As per implementation} |
| SSB/TRS |  |  | PCell: SSB and TRS (optional) SCell: TRS only |
| Precoder |  |  | Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Fill remaining DL symbols in the slot. 12 in normal DL slot 4 in special slot |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  | Antenna ports |  | {1000} |
| Number of HARQ Processes |  |  | 8 |
| Maximum number of HARQ transmissions |  |  | 4 |
| TDD UL-DL pattern |  |  | [30kHz SCS: FR1.30-1] |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS |  | k0=0 for CSI-RS resource 1,2,3,4 |
|  | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  | 'No CDM’ for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity | Slots | 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset | Slots | 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |

- Further, the PDSCH simulation results are shown in following Table 2-2 to Table 2-4 and simulation curves are shown in following Figure 2-1 to Figure2-3, which have different FFT window shift implementations .

Table 2-2  Simulation results for different timing offset when FFT window shift = CP

| TO as %CP | CP(us) | TO(us) | 70% TP(SNR dB) | Degradation(dB) |
| --- | --- | --- | --- | --- |
| 0 | 2.34 | 0 | 5.4 | NA |
| -0.4 | 2.34 | 1 | 7.3 | 1.9 |
| 0.4 | 2.34 | 1 | 6.0 | 0.6 |
| -0.9 | 2.34 | 2.2 | 12.1 | 6.5 |
| 0.9 | 2.34 | 2.2 | 6.5 | 1.1 |

Figure 2-1. Simulation curves for FFW window shift = CP

Table 2-3  Simulation results for different timing offset when FFT window shift = 0.5CP

| TO as %CP | CP(us) | TO(us) | 70% TP(SNR dB) | Degradation(dB) |
| --- | --- | --- | --- | --- |
| 0 | 2.34 | 0 | 5.5 | NA |
| -0.4 | 2.34 | 1 | 5.5 | 0 |
| 0.4 | 2.34 | 1 | 6.4 | 0.9 |
| -0.9 | 2.34 | 2.2 | 7.3 | 1.8 |
| 0.9 | 2.34 | 2.2 | 13.0 | 7.5 |

Figure 2-2. Simulation curves for FFW window shift = 0.5CP

Table 2-4  Simulation results for different timing offset when FFT window shift = 0*CP

| TO as %CP | CP(us) | TO(us) | 70% TP(SNR dB) | Degradation(dB) |
| --- | --- | --- | --- | --- |
| 0 | 2.34 | 0 | 5.4 | NA |
| -0.4 | 2.34 | 1 | 5.5 | 0.1 |
| 0.4 | 2.34 | 1 | 13.0 | 7.6 |
| -0.9 | 2.34 | 2.1 | 5.3 | -0.1 |
| 0.9 | 2.34 | 2.1 | NA | NA |

Figure 2-3. Simulation curves for FFW window shift = 0*CP

Observation 1. From our simulation results, we can find that different FFT window implementations have different performance degradation.

# 3 Conclusion

In this contribution, we give some simulation results for NR NES demodulation requirements.

Observation 1. From our simulation results, we can find that different FFT window implementations have different performance degradation.

# 4 References

- R4-2402872, WF on [109][330] Netw_Energy_NR_demod, Huawei, HiSilicon.