# 3GPP TSG-RAN WG4 Meeting # 110bis                                 R4-2405208

# Changsha, China, 15 April – 19 April, 2024

# Title: 	Simulation results for UE demodulation performance and CSI requirements for less than 5MHz

# Source: 	ZTE Corporation

# Agenda item:	6.8.7.1

# Document for:	Discussion

# 1 Introduction

This contribution contains initial simulation results for the <5MHz WID [1]. The results are provided to aid in determining where new requirements can/should be introduced[2].

# 2 Discussion

## 2.1 PBCH requirements

The simulation results for punctured PBCH were provided in following Table 2-1, in accordance with the conclusions of the previous meeting. Also simulation curves is shown in the following Figure 2-1.

Table 2-1 Minimum performance for punctured PBCH

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low | 1 | -6.1 |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 4 Low | 1 | -9.2 |

Figure 2-1. Simulation curves for punctured PDCCH CORESET#0

## 2.1 PDCCH requirements

The simulation results for punctured PDCCH were provided in following Table 2-2, in accordance with the conclusions of the last meeting[2]. Also simulation curves is shown in the following Figure 2-2.

Table 2-2 Minimum performance for punctured PDCCH

| Number of Tx | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x2 Low | 1 | 3.5 |
| 2 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x2 Low | 1 | -0.1 |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLA30-10 | 1x4 Low | 1 | -3.1 |
| 1 | 3 | 15 | 3 | 4 | [R.PDCCH. 1-2.x FDD] | TDLC300-100 | 2x4 Low | 1 | -4.9 |

Figure 2-2. Simulation curves for punctured PDCCH CORESET#0

# 3 Conclusion

In this contribution, we give some simulation results for less than 5MHz UE demodulation requirements.

# 4 References

- RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e.

- R4-2402863, WF on [110][318] NR_FR1_lessthan_5MHz_BW_demod. Nokia, Nokia Shanghai Bell.