3GPP TSG-RAN WG4 Meeting #110bis										            R4-2405289

Changsha, China, 15th – 19th April, 2024

Agenda item:			9.13.3

Source:	Moderator (CMCC)

Title:	Topic summary for [110bis][136] FS_Ambient_IoT_solutions

Document for:	Information

Introduction

This summary focuses on the R19 ambient IOT study item.  Companies contributions are listed as below.

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2405304 | A-IoT workplan | CMCC, Huawei, T-mobile |
| R4-2404867 | A-IoT general overview | Ericsson |
| R4-2405379 | General consideration for A-IOT | OPPO |
| R4-2405298 | A-IoT TR skeleton for RF part | Huawei, HiSilicon |
| R4-2404459 | Discussion on the general issues for AIoT | CATT |
| R4-2405891 | UE implementation aspects impacting work planning and study areas in RAN4 | Qualcomm Incorporated |
| R4-2404985 | Views on coexistence of Ambient IoT and NRLTE | Samsung |
| R4-2404549 | Discussion on the coexistence study of Ambient IoT and NR/LTE | Xiaomi |
| R4-2405305 | Discussion on A-IoT co-existence evaluation | CMCC |
| R4-2404868 | Coexisting study simulation assumptions for A-IoT | Ericsson |
| R4-2405299 | General discussion on A-IoT coexistence scenarios | Huawei, HiSilicon |
| R4-2404586 | Discussion on co-existence evaluation for A-IoT and NR/LTE | Spreadtrum Communications |
| R4-2405376 | Consideration on ambient IoT coexistence with NR/LTE | China Telecom |
| R4-2404671 | Discussion on the co-existence of the AIoT | vivo |
| R4-2404438 | Discussion on co-existence simulation methodology and scenarios for ambient IoT | CATT |
| R4-2404251 | Preliminary considerations on the ambient IoT device implementation and the co-existence analysis | Sony |
| R4-2405620 | Discussion on co-existence study for ambient IoT and NR/LTE | ZTE Corporation |
| R4-2405392 | Discussion on the Ambient IoT coexistence for NR | Qualcomm Incorporated |
| R4-2404355 | On coexistence between ambient IoT and NR/LTE | Apple |

Workplan and TR skeleton

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2405304 | A-IoT workplan | CMCC, Huawei, T-mobile |
| R4-2405298 | A-IoT TR skeleton for RF part | Huawei, HiSilicon |

Issue 1-1: Workplan

Recommended WF: approve the workplan in R4-2405304

Issue 1-2: TR skeleton

Recommended WF: approve the TR skeleton in R4-2405298

Co-existence evaluation

- Topic 2-1: Deployment scenario

According to the SID, following deployment scenarios are considered in SI:

Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor

Issue 2-1-1: deployment scenarios for D1T1

Moderator note: only deployment scenario is discussed under this issue, spectrum usage for each link will be discussed separately

Option 1-1 (CMCC, Samsung, Spreadtrum, Huawei): existing NR/LTE gNB are outdoor macro gNB while reader/CW/devices are all indoors. existing NR UE is only allowed outdoors

Option 1-2 (QC, Samsung): existing NR/LTE gNB are outdoor macro gNB while reader/CW/devices are all indoors. existing NR UE is indoor accessing to outdoor marco gNB

Option 2-1 (CMCC, Samsung): existing NR/LTE gNB are co-located with gNB like reader and CW. All are indoors. Reader /CW and existing gNB share same hardware

Option 2-2 (CMCC): existing NR/LTE gNB are co-located with gNB like reader and CW. All are indoors. Reader /CW and existing gNB do not share same hardware. (less limitation on the power boosting)

Recommended WF:

There are different considerations on the deployment scenario for D1T1 co-existence.

For option 1-1 and 1-2, one operator proposes to prioritize this deployment scenario considering that existing below 1GHz FDD bands are all deployed as outdoor macro BSs.

For option 2-1 and 2-2, there are some concerns on whether AIOT DL signal can be successfully detected, meanwhile there are some initial simulation results from companies to show the feasibility. It seems more study is required.

Recommend RAN4 to first evaluate co-existence for deployment scenario of option 1-1 and 1-2, and further study option 2-1 and 2-2.

Issue 2-1-2: deployment scenarios for D2T2

Moderator note: only deployment scenario is discussed under this issue, spectrum usage for each link will be discussed separately

Option 1-1: existing NR/LTE gNB are outdoor macro gNB, intermediate UE/CW/devices are all indoors. existing NR UE is only allowed outdoor.

Option 1-2: existing NR/LTE gNB are outdoor macro gNB, intermediate UE/CW/devices are all indoors. existing NR UE is indoor.

Recommended WF:

For D2T2 co-existence evaluation, existing NR/LTE gNB are outdoor macro gNB, intermediate UE/CW/devices are all indoors. FFS on whether existing NR UE is allowed for indoor or not.

- Topic 2-2: Spectrum usage

Moderator note: There are some companies prefer to wait for RAN1 further input on the spectrum and cases for evaluation, while some other companies provide analysis from RAN4 co-existence perspective. From moderator’s perspective, RAN1 physical layer design is irrespective of spectrum usage and it is not sure whether RAN1 will further down-select and provide input to RAN4. It is RAN4’s responsibility to evaluation the spectrum usage from co-existence perspective.  Hence, it is recommended to discuss these aspects in RAN4 and collect companies’ views.

Issue 2-2-1: Spectrum usage for R2D in D1T1

Option 1: FDD DL spectrum for R2D

Option 2: FDD UL spectrum for R2D

Recommended WF:

There are different preferences on the spectrum for R2D in D1T1. Discuss whether to prioritize FDD DL spectrum for R2D for D1T1.

Issue 2-2-2: Spectrum usage for CW transmission in D1T1 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Agreement (RAN1#116)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied.

· Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

· Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

· Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal of priority on the above cases:

- Option 1 (ZTE, Spreadtrum): case 1-2 and 1-4

- Option 2 (Qualcomm): case 1-1

- Option 3 (CMCC, vivo): case 1-1 and case 1-2

- Option 4 (Huawei): case 1-1 and case 1-4

Recommended WF:

Discuss whether to evaluate CW in both DL spectrum and UL spectrum.

Discuss whether to evaluate both inside and outside topology from co-existence perspective.

Issue 2-2-4: Spectrum usage for R2D in D2T2

Option 1: FDD UL spectrum for R2D

Recommended WF:

No company propose DL for R2D in D2T2. Use FDD UL spectrum for R2D in D2T2.

Issue 2-2-5: Spectrum usage for CW transmission in D2T2 for the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering

Agreement (RAN1#116)

For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied.

· Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

· Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

· Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal of priority on the above cases:

- Option 1 (ZTE, Huawei, Qualcomm): case 2-2 and 2-4

- Option 2 (Speadtrum): case 2-3

- Option 3 (vivo): case 2-2 and case 2-3

Recommended WF:

Discuss whether to prioritize UL spectrum.

Discuss whether to evaluate both inside and outside topology from co-existence perspective.

- Topic 2-3: Spectrum deployment mode

Issue 2-3-1: Spectrum deployment mode

Proposal (CMCC):

According to the SID, in-band/guard-band/standalone mode are classified as spectrum deployment mode as below:

“D. Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s).”

So in-band/guard-band/standalone mode are only categorized in terms of spectrum deployment mode which should be differentiated from operation mode concept, e.g. the operation mode definition for NB-IoT.

in-band/guard-band/standalone modes are categorized in terms of spectrum deployment mode and is the separate concept from the operation mode

•	in-band: A-IoT is operating in-band when it is located within a NR transmission bandwidth configuration but not within the NR minimum guard band.

•	guard-band: A-IoT is operating in guard band when it is located within a NR BS channel bandwidth but is not NB-IoT operation in NR in-band.

•	standalone: A-IoT utilize its own carrier. A-IoT and NR using different carriers.

For operation mode, i.e. whether reader of topology 1 and CW inside topology can share same hardware with existing gNB or not. This issue will impact final power boosting RF requirement definition.

Recommended WF:

Discuss above proposal

Issue 2-3-2: Priorities of spectrum deployment mode for co-existence evaluation

Option 1 (CMCC, vivo, Samsung, Spreadtrum): in-band

Option 2 (ZTE, CTC): in-band and guard-band

Option 3 (HW): guard-band

Recommended WF:

Slightly more companies support to prioritize in-band spectrum deployment mode considering that in-band is the worst case from co-existence evaluation perspective.

Discuss whether in-band spectrum deployment mode can be prioritized for co-existence evaluation.

- Topic 2-4: Evaluation methodology

Issue 2-4-1: Evaluation methodology

Option 1 (Spreadtrum): System level simulation

Section 5.3 in TR 38.803 is proposed as a starting point.

Adopt following simulation steps.

1.	Aggressor and victim network are generated.

-	UEs are distributed randomly across the network.

2.	UE associations: UEs are associated to base station based on coupling loss.

-	Associations are made assuming a single element at both UE and BS.

3.	Once association is done, round robin scheduling is used. BF weights are adjusted to point to the LOS direction between BS-UE. This is done for both victim and aggressor networks.

4.	Throughput is computed in the victim systems without considering ACI as below:

-	ThputNO ACIbpshz=fSINRICI=fSN+IICI, where IICI is the inter-cell interference.

5.	Throughput is computed considering ACI as below:

-	ThputACIbpshz=fSINRICI+ACI=fSN+IICI+IACI, where IACI is the adjacent channel interference.

6.	RF parameters are determined based on the degradation cause by ACI as below:

-	LossACI=1-ThputACIThputSINGLE.

In addition,

Option 2 (vivo): between NR and CW

- Discuss the distribution of BS/UE for D1T1/D2T2 and the power of CW.

- Run system simulation, get the CDF of CW power at receiver side.

- Check the performance degradation of receiver

Option 3 (CATT):

- For CW transmitting in DL spectrum, consider to use link level simulation for the DL in-band, guard band co-channel co-existence for Device 1 and Device 2a for the scenario of AIoT is victim. Further discuss the metric for the simulation.

- For CW transmitting in DL spectrum, system simulation is needed for UL in-band, guard band co-existence simulation for Device 1 and Device 2a. If throughput loss metric can be reused needs further discussion.

- For CW transmitting in UL spectrum, system simulation is needed for both UL and DL in-band, guard band co-existence simulation for Device 1 and Device 2a.

Option 4 (CMCC): If we assume all existing UE are outdoors, simulation is not needed. Instead, the calculation for the worst interference link is enough.

Option 5 (Samsung): Study the feasibility and performance to support coexistence of legacy NR/LTE DL signals and A-IoT signals on the same channel and on adjacent channel, including whether or not and how many Guard RB is needed between NR/LTE signal and A-IoT signal. Coexistence at Tag side and BS/intermediate side shall be discussed separately due to the different receiver capability and waveform design.

- The following definition of Guard RB for ACS and ASCS for Rel-18 LP-WUS study can be leveraged as starting point.

Recommended WF:

Use the Monte-Carlo method as baseline for co-existence evaluation, i.e. Section 5.3 in TR38.803

Depending on the discussion on deployment scenarios, for some cases, calculation for the worst interference link may be enough.

Discuss whether RAN4 needs to perform link level simulation for:

- CW transmitting in DL spectrum, consider to use link level simulation for the DL in-band, guard band co-channel co-existence for Device 1 and Device 2a for the scenario of AIoT is victim.

- Required guard RBs

Issue 2-4-2: Performance metric for AIOT

Option 1 (CMCC): it’s suggested to use 1% BLER as performance metric for all kinds of devices, reader of topology 1 and 2. Relationship between SINR and BLER are based on companies’ input. RAN4 need to discuss the outage probability if 1% BLER is used as performance metric. 5% worst victim is suggested as the candidate value.

Option 2 (Spreadtrum):

For NR/LTE UE and intermediate UE, BLER should be less than 1%.

For A-IoT UE, BLER is 1% is proposed as a starting point.

Option 3 (ZTE, vivo): SINR degradation

Option 4 (xiaomi): RAN4 can first research the SINR degradation with target ACS, ACLR and ACIR for Ambient IoT interfered in by Ambient IoT/ NR/LTE systems before deciding the mapping between SINR and BLER.

Recommended WF:

For NR/LTE system, use 5% throughput loss as performance metric as legacy.

For AIOT system, including reader, device, intermediate UE, further discuss the performance metric:

- Option 1: [1%] BLER, [5%] worst victim as outage probability.

- Option 2: SINR degradation

- Topic 2-5: Evaluation cases

Issue 2-5-1: device type

Option 1 (CMCC): prioritize device 1 and 2a for coexistence evaluation.

Option 2 (HW): Take Device 2a without a frequency shifter as the baseline for Device 2a coexistence analysis, because Device 2a without a frequency shifter brings more severe carrier-wave interference to the uplink receiver compared to the one with a frequency shifter.

Option 3 (CATT): RAN4 further discuss Device 2b RF architecture especially the assumption of RF envelope detector, AD/DA sampling rate, digital filter to decide the co-existence simulation methodology for Device 2b.

Recommended WF:

Discuss whether Device 2a without a frequency shifter is baseline for device 2a coexistence analysis.

Discuss whether device 1 and 2a can be prioritized for coexistence evaluation.

Issue 2-5-2: Evaluation cases for D1T1 for device 1 and 2a between NR and AIOT

Recommended WF:

Corresponding evaluation cases are listed for further discussion. Note that some duplicated cases are omitted in the table.

It seems that not necessary to consider interference between LTE and AIOT

In addition to the table below, spectrum operation mode (inband, guard band, standalone) and deployment scenario (outdoor NR + indoor A-IOT or indoor NR + indoor A-IOT) also need to be discussed.

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
|  | R2D: UL CW2D and D2R: UL | Reader | NR UL |
|  |  | NR UL | reader |
| Self interference cancelation is needed for reader · Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum · Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: DL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |
|  | R2D: UL CW2D and D2R: UL | reader | NR UL |
|  |  | NR UL | reader |
| Self interference cancellation is needed for reader · Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum | R2D: DL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | device and/or reader |
|  |  | reader | NR DL |
|  |  | NR DL | device |
|  | R2D: UL CW2D and D2R: UL | reader | NR UL |
|  |  | NR UL | reader |

Issue 2-5-3: Evaluation cases for D2T2 for device 1 and 2a between NR and AIOT

Recommended WF:

Corresponding evaluation cases are listed for further discussion. Note that some duplicated cases are omitted in the table.

It seems that not necessary to consider interference between LTE and AIOT

In addition to the table below, spectrum operation mode (inband, guard band, standalone) need to be discussed.

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| \ Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum Self interference cancelation is needed for reader | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
| Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum Self interference cancelation is needed for reader | R2D: UL CW2D and D2R: UL | CW and/or device | NR UL |
|  |  | NR UL | Device and/or reader |
|  |  | reader | NR UL |
|  |  | NR UL | device |
|  | R2D: UL CW2D and D2R: DL | CW and/or device | NR DL |
|  |  | NR DL | device and/or reader |

Issue 2-5-4: Evaluation cases for device 2b between AIOT and NR

There is proposal from HW to discuss co-existence evaluation cases for device 2b separately

| Deployment & Topology | Device Type | Description and constraints of the scenario |
| --- | --- | --- |
| D1T1-C | Device 2b | indoor BS + indoor device with active UL transmission R2D in DL spectrum D2R in UL spectrum (refer to R1-2401735) |
| D2T2-C | Device 2b | outdoor BS + Indoor Intermediate UE + Indoor device with active UL transmission R2D in UL spectrum D2R in UL spectrum (refer to R1-2401735) |

Recommended WF:

Discuss above proposal.

Corresponding evaluation cases are listed for further discussion.

| Deployment scenario and topology | spectrum | aggressor | victim |
| --- | --- | --- | --- |
| D1T1-C | R2D: DL D2R: UL | Reader | NR DL |
|  |  | NR DL | reader |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |
| D2T2-C | R2D: UL D2R: UL | Reader | NR UL |
|  |  | NR UL | Device |
|  |  | Device | NR UL |
|  |  | NR UL | Reader |

Issue 2-5-5: Interference between AIOT systems

Option 1 (Xiaomi)

| Cases | Operation mode | Aggressor | Victim | Direction |
| --- | --- | --- | --- | --- |
| 1 | Stand-alone | A-IoT- Device | A-IoT- BS | Uplink |
| 2 | Stand-alone | A-IoT- node | A-IoT-BS | Uplink |
| 3 | Stand-alone | A-IoT- Device | A-IoT-node | Uplink |

Option 2 (Qualcomm)

| No. | Category | Aggressor | Victim | Freq option | Impacted Requirements 1), 2), 3) |  |
| --- | --- | --- | --- | --- | --- | --- |
| 11 | Coex between A-IoT and A-IoT | CW2D | D2R | a), b) | CW ACLR | Reader ACS |
| 12 |  | CW2D | CW2D | a), b) | CW ACLR | CW ACS |
| 13 |  | R2D | R2D | a), b) | Reader ACLR | Device ACS |
| 14 |  | D2R | D2R | a), b) | Device ACLR | Reader ACS |

| Freq Option | R2D | CW2D | D2R | Scenario |
| --- | --- | --- | --- | --- |
| a) | DL | DL | DL | D1T1-A1, D1T1-A2, D1T1-B |
| b) | DL | UL | UL | D1T1-A1, D1T1-A2 |

Recommended WF:

For AIOT FDM operation, it seems more input from RAN1 is needed.

Recommend RAN4 to first evaluate interference between AIOT and NR.

Issue 2-5-6: Multi-operator scenario

Proposal (Spreadtrum): Multiple A-IoT operators co-existence scenario should be investigated in Rel-19 A-IoT.

Figure 2.5-1: one possible FDM candidate for multiple A-IoT operators scenario

Recommended WF:

More clarification is needed on the multiple A-IOT operator scenario.

- Topic 2-6: Evaluation parameters

Issue 2-6-1: General parameters

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| General Parameter | D1T1&D2T2 |
| --- | --- |
| Carrier frequency | 900 MHz |
| Channel BW for NR | 10MHz or 20MHz with 15KHz SCS |
| Channel BW for AIOT | DL: 180kHz with 15KHz SCS UL: 15KHz or 180KHz |
| Waveform | DL: OFDM based waveform UL: single carrier |
| A-IoT DL power control | No |
| A-IoT UL power control | No |
| Traffic model | Full buffer |
| Frequency reuse | 1 |

Issue 2-6-2: Layout for D1T1

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| Parameter | Assumptions for D1T1 |  |
| --- | --- | --- |
| Scenario | InF-DH | InF-SH |
| Channel model | Issue 2-6-9 |  |
| Hall size | 120x60 m | 300x150 m |
| Room height | 10 m | 10 m |
| AIOT reader deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. for the small hall (L=120m x W=60m): D=20m BS-height = 8 m | 18 BSs on a square lattice with spacing D, located D/2 from the walls. for the big hall (L=300m x W=150m): D=50m BS-height = 8 m |
| AIOT CW deployment | FFS: Model the CW node layer with a grid shift to the network layer |  |
| NR BS deployment | When NR BS are outdoors: Hexagonal grid, 19 macro sites, 3 sectors per site with wrap around, 1 AIOT indoor scenario per sector When NR BS are indoors: reuse AIOT reader deployment |  |
| NR BS Inter-site distance | 200 or 300 or 500 |  |
| Minimum NR BS – NR UE distance (2D) | 35 m |  |
| NR UE (D1T1) dropping | FFS |  |
| Device distribution | Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) |  |

Issue 2-6-3: Layout for D2T2

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| Parameter | Assumptions for D2T2 |  |
| --- | --- | --- |
| Scenario | InH-office | InF-DL |
| Channel model | TR 38.901 InH-office | TR 38.901 InF-DL |
| Hall size | 120 x50 m | 300x150 m |
| Room height | 3m | 10 m |
| Intermediate UE(D2T2) dropping | 12 UEs are distributed uniformly - L=120m x W=50m; D=20m - UE height = 1.5m | 18 UEs are distributed uniformly - L=300m x W=150m; D=50m - UE height = 1.5 m |
| NR BS deployment | hexagonal grid, 7 macro sites, 3 sectors per site with wrap around, 1 AIOT indoor scenario per sector |  |
| AIOT CW deployment | FFS |  |
| Intermediate UE height | 1.5m |  |
| NR UE dropping | FFS | FFS |
| Device distribution | Device Height= 1m or 1.5m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) | Device Height= 1.5m AIoT devices drop uniformly distributed over the horizontal area Number of A-IoTs = Total area × activated density (1.5 A-IOT devices/m²) |

Issue 2-6-4: AIOT micro-BS parameters for D1T1

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| A-IoT micro BS parameters | Recommended value | Note |
| --- | --- | --- |
| A-IoT micro-BS total Tx power | 33dBm baseline 38 dBm (optional) |  |
| A-IoT micro-BS receiver Noise Figure（dB） | 5 |  |
| A-IoT micro-BS antenna gain including feeder loss (dBi) | 5 or 8 |  |
| A-IoT micro-BS ACLR (dB) | ACLR1:40dB，ACLR2:50dB | Reference to TS 36.104 NB-IoT standalone |
| Antenna configuration | 2 or 4 antenna elements, with (M,N,P,Mg,Ng) = (1,1,2,1,1) or (M,N,P,Mg,Ng) = (2,1,2,1,1) |  |

Issue 2-6-4: Intermediate UE parameters for D2T2

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| intermediate UE parameters | Recommended value |
| --- | --- |
| intermediate UE total Tx power（dBm） | 23dBm baseline 26dBm |
| gain of antenna intermediate UE including feeder loss (dBi) | 0 |
| intermediate UE receiver Noise Figure（dB） | 7 |
| Antenna configuration | Omni direction antenna |

Issue 2-6-4: CW parameters

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| intermediate UE parameters | D1T1 | D2T2 |
| --- | --- | --- |
| Tx power（dBm） | If UL spectrum is used, UE Tx power is assumed, i.e. 23dBm/26dBm If DL spectrum is used, AIOT micro-BS Tx power is assumed. | Inter-mediate UE Tx power is assumed. |
| Other parameters | Same as AIOT micro-BS？ | Same as inter-mediate UE |

Issue 2-6-5: AIOT device parameters

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| A-IoT device parameters | Device 1 | Device 2a | Device 2b |
| --- | --- | --- | --- |
| A-IoT device Tx power (dBm) | <-10 | <-10 | [-10/-20] |
| A-IoT device effective antenna gain per Tx or Rx branch (dBi) | -3 or 0 | -3 or 0 | -3 or 0 |
| A-IoT device reflection （backscatter）loss (dB) Note: due to, e.g., impedance mismatch | -6 dB and 0 dB for OOK / FSK and BPSK | N/A | N/A |
| A-IoT device power gain of reflection amplifier (dB) | N/A | [10 / 20] | N/A |
| A-IoT Device receiver sensitivity (dBm) | -36 | -46 | [FFS] |
| BB filter type | 3/5 Butterworth filter | 3/5 Butterworth filter | 3/5 Butterworth filter |
| Cutoff frequency | FFS e.g. 1PRB or related to DL data rate | FFS e.g. 1PRB or related to DL data rate | FFS e.g. 1PRB or related to DL data rate |
| Guard band | 1PRB, 2PRB, etc increasing in steps of 1dB | 1PRB, 2PRB, etc increasing in steps of 1dB | 1PRB, 2PRB, etc increasing in steps of 1dB |

Issue 2-6-6: NR macro BS parameters

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| NR macro-BS Parameter | Recommended value |
| --- | --- |
| Macro-BS Tx power (dBm) | 46 |
| Height of macro NR BS (m) | 25 |
| NR Macro-BS Noise Figure(dB) | 5 |
| Network location | outdoor |

Issue 2-6-7: NR UE parameters

Recommended WF:

Discuss following parameters and strive to agree on some candidate values for evaluation in this meeting.

| NR UE Parameter | Recommended value | note |
| --- | --- | --- |
| UE TX power in dBm | -40 to 23 |  |
| NR UE Antenna gain (dBi) | 0 |  |
| NR UE noise floor (dBm) | -112.4 @ 180kHz | noise floor=-174+10*log (180*1000)+9 |
| NR UE ACLR（dB） | 30 | For power class 3 NR UE |
| NR UE Maximum input level（dBm） | -25 | TS38101-1 7.4 Maximum input level |
| NR UE Noise Figure（dB） | 9 |  |
| Antenna configuration | Omni direction antenna |  |

Issue 2-6-8: Adjacent RB/carrier Tx leakage and Rx suppression characteristics

Recommended WF:

There are proposals to discuss Adjacent RB/carrier Tx leakage and Rx suppression characteristics.

Further discuss the following table.

|  | In-band |  | Guard-band |  | standalone |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Tx | Rx | Tx | Rx | Tx | Rx |
| Intermediate UE/CW of topology 2 | Legacy UE IBE requirement | ICS=ACS value According to SBFD analysis, ICS could be equals to ACS | Legacy UE ACLR requirement? | Legacy UE ACS requirement? | Legacy UE ACLR requirement | Legacy UE ACS requirement |
| Existing gNB/reader of topology 1/CW of topology 1 | Based on companies’ input since there is no such requirements in 38.104 | Based on companies’ input since there is no such requirements in 38.104 | Legacy gNB ACLR requirement? | Legacy gNB ACS requirement? | Legacy gNB ACLR requirement | Legacy gNB ACS requirement |
| devices | Based on BB LPF performance |  |  |  |  |  |

Issue 2-6-9: Pathloss model for D1T1

Proposal:

- when the Tx node is high while Rx node is low, inF-DH is suggested.

- When the Tx node and Rx node both are high, inF-HH is suggested.

- When the Tx node and Rx node both are low, inF-DL is suggested.

Note: O-I and outdoor pathloss model needs to be considered additionally when we have agreements on the deployment scenarios

|  | D1T1-A1 | D1T1-A2 | D1T1-B | D1T1-C |
| --- | --- | --- | --- | --- |
| CW2D | inF-DH(reusing RAN1) | inF-DH(reusing RAN1) | inF-DL | - |
| R2D and D2R | inF-DH(reusing RAN1) | inF-DH(reusing RAN1) | inF-DH(reusing RAN1) | inF-DH |
| reader to UE | inF-DH | inF-DH | inF-DH | inF-DH |
| CW to UE | inF-DH | inF-DH | inF-DL |  |
| gNB to device | inF-DH | inF-DH | inF-DH | inF-DH |
| CW to gNB | inF-HH | inF-HH | inF-DH |  |
| reader to gNB | inF-HH | inF-HH | inF-HH | inF-HH |
| UE-to-device | inF-DL | inF-DL | inF-DL | inF-DL |

Recommended WF:

Further discuss the above proposal.

Issue 3-2-3: CW-interference cancellation capability when CW inside and outside topology

Proposal: following CW interference cancellation evaluation methodology is suggested. Besides, RAN4 needs to further discuss whether it is still necessary to evaluate -A2 evaluation case if CW interference cancellation capability is already much high, i.e. remaining CW interference is much lower than noise floor.

- Methodology: residual self-interference= Tx power - self-interference cancellation capability

- self-interference cancellation capability = spatial isolation + RF cancellation + digital cancellation if applicable

- spatial isolation is based on isolation distance assumption

Recommended WF:

- Further discuss the above proposal.

RF requirements impact

Moderator note: There are proposals to use RAN1 terminology of device types, align with RAN1 on the RF architecture and so on. From moderator perspective, this should be the common understanding for RAN4 study. Hence, the proposals are not explicitly listed for discussion in the summary.

- Topic 3-1: General

Issue 3-1-1: Scope of RF requirements discussion

Option 1 (QC): RF energy sources or energy harvesting of the tag are not in scope of the study

Option 2 (Ericsson): EH sensitivity can be studied in work item phase.

Option 3 (Sony): RAN4 shall study if there is any impact due to the energy harvesting/storage on co-existence, RF performance and testability of AIoT devices.

Recommended WF:

According to the following RAN plenary agreement in RAN#103 below, the EH is out of SI scope

- Confirm that study of design of energy harvesting signal/waveform is out of SI scope in Rel-19

It can be agreed that RF energy sources or energy harvesting of the tag are not in scope of the study.

- Topic 3-2: RF considerations

Issue 3-2-1: A-IOT devices

Option 1 (Ericsson):

- As the RF ED architecture does not provide inband selectivity, only out of band blocking requirement could be discussed for both device type 1 and type 2a.

- The system parameter of channel bandwidth needs to be discussed.

- The UL backscatter signal power level in relation to the received power specific to the A-IoT

Option 2 (CATT, CMCC):

- The assumptions for the RF BPF including: Operating frequency, cutoff frequency, performance assumption, etc.

- The feasibility and the performance for the RF BPF filter before the RF envelope detector should be studied in RAN4.

Option 3 (QC):

- Frequency confinement of a backscatter device and its definitions should be studied in RAN4

- RAN4 to study how to test a backscattering device, considering OTA methods and feasibility for conducted testing

Option 4 (Sony): The power consumption limit needs to be considered when RAN4 discusses the RF architecture and performance of the AIoT devices.

Option 5 (ZTE):

- specify different sets of RF requirement for Ambient IoT Device 1, Device 2a and Device 2b

- treat the reception of CW signals as part of Rx requirements for Ambient IoT Device 1, Device 2a.

Option 6 (OPPO): The regulation from FCC and EU can be a starting point. e.g. EN 302 208 RF requirements

| Device | Requirement |
| --- | --- |
| tag transmitter conformance requirements. | Radiated power (e.r.p.) |
|  | Unwanted emissions |

Recommended WF:

More discussion is needed.

Issue 3-2-2: AIOT reader (BS)

Option 1 (SS):

- Strive to minimize the impact for gNB hardware change.

- At BS/ intermediate node side, in terms of NB-IoT and NR co-existence with mixed numerology, FFS whether to define Guard RB or to leave it to implementation (e.g., separate digital filtering, internal gap), to mitigate the in-channel and adjacent-channel interference.

Option 2 (ZTE):

- specify the RF requirement (e.g. Tx and Rx RF requirements) for standalone A-IoT BS and follow the legacy BS RF requirement framework;

- discuss the applicability of the existing RF requirement for in-band /guard band A-IoT BS and discuss potential new RF requirements for A-IoT BS e.g. power boosting, EVM requirements for OOK signal etc.

- specify RF requirement for A-IoT BS transmitting CW signals within topology and specify RF requirement for CW source Node transmitting CW signals outside of topology.

Option 3 (OPPO): The regulation from FCC and EU can be a starting point. e.g. EN 302 208 RF requirements

| Device | Requirement | Device | Requirement |
| --- | --- | --- | --- |
| interrogator transmitter conformance requirement | Frequency error | interrogator receiver conformance requirement | Adjacent channel selectivity |
|  | Frequency stability under low voltage conditions |  | Blocking or desensitization |
|  | Effective radiated power |  | Receiver spurious response rejection |
|  | Transmitter antenna beam-width |  | Receiver sensitivity |
|  | Transmitter spectrum masks |  | Receiver radio-frequency intermodulation |
|  | Transmitter spurious emissions |  |  |
|  | Transmission times |  |  |

Recommended WF:

More discussion is needed.

Issue 4-2-3: AIOT reader (intermediate UE)

Option 1 (SS):

- At BS/ intermediate node side, in terms of NB-IoT and NR co-existence with mixed numerology, FFS whether to define Guard RB or to leave it to implementation (e.g., separate digital filtering, internal gap), to mitigate the in-channel and adjacent-channel interference.

Option 2 (ZTE):

- specify the Tx and Rx requirement for intermediate UE in addition to RF requirements for the legacy Uu link.

- specify RF requirement for A-IoT intermediate UE transmitting CW signals within topology and specify RF requirement for CW source Node transmitting CW signals outside of topology.

Option 3 (OPPO): The regulation from FCC and EU can be a starting point. e.g. EN 302 208 RF requirements

| Device | Requirement | Device | Requirement |
| --- | --- | --- | --- |
| interrogator transmitter conformance requirement | Frequency error | interrogator receiver conformance requirement | Adjacent channel selectivity |
|  | Frequency stability under low voltage conditions |  | Blocking or desensitization |
|  | Effective radiated power |  | Receiver spurious response rejection |
|  | Transmitter antenna beam-width |  | Receiver sensitivity |
|  | Transmitter spectrum masks |  | Receiver radio-frequency intermodulation |
|  | Transmitter spurious emissions |  |  |
|  | Transmission times |  |  |

Recommended WF:

More discussion is needed.