3GPP TSG-RAN WG4 Meeting #110bis	R4-2405328

Changsha, China, April 15 – April 19, 2024

Source:	Huawei, HiSilicon, ZTE

Title:	TP for TR 38.718-02-01 to introduce CA_n3A-n79C with BCS4&5

Agenda item:	5.10.2

Document for:	Approval

1 Background

This contribution provides the technical text proposal on the NR CA band combination CA_n3A-n79C with BCS4&5, which has been captured in the latest basket WI [1].

2 Text Proposal

---Start of changes TR 38.718-02-01 ---

5.x	CA_n3-n79

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1:  CA band combination CA_n3-n79

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n3 | 1710 | – | 1775 | 1815 | – | 1880 | FDD |
| n79 | 4400 | – | 5000 | 4400 | – | 5000 | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination CA_n3-n79

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n79C | CA_n79C CA_n3A-n79A CA_n3A-n79C | n3 | n3 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n79 | CA_n79C_BCS4 and 5 |  |

5.x.1.3	UE co-existence studies

The harmonic / harmonic mixing issue have been analysed in previous release.

IMD order for contiguous UL CA_n79C is calculated as below.

WGap = lowest band edge of n79 - highest DL band edge of n3 = 4400 – 1880 = 2520 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(200, 200) = 200 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 27.

There is no need to consider IMD27.

5.x.1.4	∆TIB,c and ∆RIB,c values

The TIB,c and RIB have been specified in current spec.

5.x.1.5	REFSENS requirements

For UL CA_n79C configuration, there is no additional REFSENS exception.

5.x.1.6	OOB blocking exception requirements

It has been specified in current spec.

5.x.2	Specific for 2 bands UL CA

5.x.2.1	Maximum output power for inter-band CA

Table 5.x.2.1-1: UE Power Class for uplink inter-band CA

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| CA_n3A-n79C | 23 | +2/-3 |

5.x.2.2	UE co-existence studies

The UE co-existence for 2UL CA n3A-n79A have already been analysed.

For UL CA_n3A-n79C, the triple beats are analysed below.

Table 5.x.2.2-1: Triple beat analysis for UL CA_n3A-n79C

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 4400 | 5000 | 1710 | 1775 |  |  |
| UE UL carriers | fx1_low | fx2_low | fx1_high | fx2_high | fy_low | fy_high |
| UL frequency (MHz) | 4400 | 4500 | 4900 | 5000 | 1710 | 1775 |
| TB1 | ¦fx1_low-fx2_low+fy_low ¦ | ¦fx1_low-fx2_low+fy_high ¦ | ¦fx1_high-fx2_high+fy_low ¦ | ¦fx1_high-fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 1610 | 1675 | 1610 | 1675 | 1610 | 1675 |
| TB2 | ¦-fx1_low+fx2_low+fy_low ¦ | ¦-fx1_low+fx2_low+fy_high ¦ | ¦-fx1_high+fx2_high+fy_low ¦ | ¦-fx1_high+fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 1810 | 1875 | 1810 | 1875 | 1810 | 1875 |
| TB3 | ¦-fx1_low-fx2_low+fy_low ¦ | ¦-fx1_low-fx2_low+fy_high ¦ | ¦-fx1_high-fx2_high+fy_low ¦ | ¦-fx1_high-fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 7190 | 7125 | 8190 | 8125 | 7125 | 8190 |
| TB4 | ¦fx1_low+fx2_low+fy_low ¦ | ¦fx1_low+fx2_low+fy_high ¦ | ¦fx1_high+fx2_high+fy_low ¦ | ¦fx1_high+fx2_high+fy_high ¦ | minFrequency limits | maxFrequency limits |
| TB IMD frequency limits (MHz) | 10610 | 10675 | 11610 | 11675 | 10610 | 11675 |

The 1st order triple beats interference may fall into the Rx band n3.

But based on the table 1 from WF R4-2220556, as band n3 belongs to FR1-2 (MB) and band n79 belongs to FR1-4 (VHB), this case is not eligible for the triple beat case (The two UL bands are not part of the same or adjacent band group).

Table 5.x.2.2-2 : Band group definition for adjacent band-group criterion

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 (LB) | FR1-2 (MB) | FR1-3 (HB) | FR1-4 (VHB) | FR1-5 (UHB) |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5250-7125 |
| Duplex mode | Mostly FDD | Mostly FDD | FDD and TDD | TDD only | TDD only |

5.x.2.3	REFSENS requirements

For UL CA_n3A-n79C configuration, there is no additional REFSENS exception.

---End of changes---

Reference

[1]	RP-240761, “Revised WID Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation