# 3GPP TSG-RAN WG4 Meeting #110bis	R4-2405386

Changsha, China, 15th – 19th April, 2024

Agenda Item:	9.9.3

Source: 	OPPO

Title: 	Initial discussion on Release-19 Sidelink intra-band contiguous CA

Document for:	Approval

- Introduction

During the RAN#103 meeting, the Rel0-19 Sidelink WID [1] has been agreed. The PC2 intra-band contiguous CA work is added as one of the objectives and hence we would like to have some initial discussion on this topic.

- Discussion

The objectives of Rel-19 sidelink WID are listed here for information.

The core part of the work item includes:

- Specify the RF requirements for NR sidelink CA in n47:

  - Intra-band non-contiguous CA with power class 2 and power class 3, with Component Carrier (CC) combinations 10MHz + 10MHz and 10MHz + 20MHz

  - Intra-band contiguous CA with power class 2

- Introduce necessary changes for release independence specification, if identified

Note: Work for power class 3 starts first in RAN4. Whether there is any RRM core requirement to be specified will be confirmed in Q3 2024.

For the intra-band contiguous CA work, it has been finished in Rel-18 sidelink enhancement WID as corresponding MPR/A-MPR and the other UE RF requirements have been defined for power class 3. To improve the uplink coverage, PC2 is quite important. Also for single carrier in ITS band n47, power class 2 has already been supported in Rel-17. In such case, to consider and work on the power class 2 for intra-band contiguous CA is reasonable and straightforward.

For the intra-band contiguous CA with PC2, firstly we would like to consider the co-existence scenario and corresponding work as this is the introduction of higher power class. As stated in previous paragraph, for single carrier PC2 work has been done in Rel-17. The co-existence simulation and conclusion has been captured in TR 38.785 sub-clause 5.1.1.

Observation 1: For sidelink on ITS band n47, single carrier PC2 work has been done in Rel-17 and captured in TR 38.785 sub-clause 5.1.1.

From the co-existence simulation perspective, the sidelink contiguous CA with PC2 can rely on the simulation results of Rel-17 work and hence it is propose that there is no additional co-existence simulation needed.

Proposal 1: No more co-existence simulation is needed for PC2 CA work.

Secondly, for the general consideration of PC2 sidelink CA, as the PC3 sidelink contiguous CA work has been finished hence the corresponding system parameter including the band combination and the channel bandwidth can be re-used. The only change of UE RF requirements for PC2 compared to PC3 for sidelink intra-band contiguous CA should be MPR and A-MPR requirements.

Observation 2: The only change of UE RF requirements for PC2 compared to PC3 for sidelink intra-band contiguous CA would be MPR and A-MPR requirements.

To this extend, we need to further discuss the simulation assumption for PC2 intra-band contiguous CA work. In Rel-17 NR SL PC2, as captured in current specification TS 38.101-1, the MPR and A-MPR assumption are captured in TS 38.786. Also compared to the intra-band contiguous CA simulation assumption as captured in TR 38.786, it is propose to reuse these assumptions for PC2 intra-band contiguous CA simulation.

For basic parameters, reuse the simulation assumptions in TR38.785 (Rel-17 enhanced NR sidelink). Other constraints for PSCCH/PSSCH/PSFCH/S-SSB can be assumed based on current RAN1’s agreement.

Table 6.3.2 Simulation assumption for NR SL CA operation

| Center frequency | 5.9GHz |
| --- | --- |
| Bandwidth | per CC: 10/20/30/40MHz Aggregated CBW: Table 5.2.3-1 (up to 70MHz CBW) |
| Maximum output power for aggregated CBW | 26dBm |
| Numerology | 15 kHz/30kHz/60kHz |
| Modulation per CC | QPSK/16QAM/64QAM/256QAM |
| Waveform | CP-OFDM |
| Carrier leakage | 34dBc |
| IQ image | 25dBc |
| CIM3 | 60dBc |
| PA calibration | PA calibrated to deliver 30dBc ACLR for a fully allocated RBs in 20MHz QPSK DFT- S-OFDM waveform at 1 dB MPR. This is based to share PA between LTE V2X and NR V2X at 5.9GHz as worst case. |

Proposal 2: Reuse the PC3 intra-band contiguous CA simulation assumption with the change of maximum output power change to 26dBm.

3	Conclusions

Observation 1: For sidelink on ITS band n47, single carrier PC2 work has been done in Rel-17 and captured in TR 38.785 sub-clause 5.1.1.

Observation 2: The only change of UE RF requirements for PC2 compared to PC3 for sidelink intra-band contiguous CA would be MPR and A-MPR requirements.

Proposal 1: No more co-existence simulation is needed for PC2 CA work.

Proposal 2: Reuse the PC3 intra-band contiguous CA simulation assumption with the change of maximum output power change to 26dBm.

4 References

[1] RP-240840 	New WID on NR Sidelink: Intra-band Carrier Aggregation in ITS band