3GPP TSG-RAN WG4 Meeting #110bis	R4-2405466 
Changsha, China, 15th – 19th April, 2024

Agenda item:	6.10.2

Source:	CAICT

Title:	Summary of 3GPP Rel-18 FR1 MIMO OTA lab alignment results

Document for:	Discussion

1	Introduction

The FR1 MIMO OTA lab alignment activity has made good progress [1-2]. At the last RAN4 meeting and this meeting, all 6 volunteer labs have submitted their measurement results of the performance alignment devices (PADs) [3]-[8].

At the last meeting, a CR to the test plan TS 38.151 is agreed in [8], which removed the statement that power validation reuslts can be considered a systematic offset to correct the final UE sensitivity vlaues. Accordingly, it was agreed in the WF [2] that the lab alignment outcome should be based on the measurement results without power validation systematic offsets.

Besides, it was agreed at the last meeting [9] that the pass/fail limit for Rel-18 lab alignment is updated based on the latest MU agreed in RAN5 (specified in Table B.1-1 of TS 38.551). For bands≤3GHz, the pass/fail limit is updated to 0.75*2.61=1.96 dB.

This contribution presents a summary of the Rel-18 FR1 MIMO OTA lab alignment results. The reference value of each PAD is derived by the linear average (in dB) of the measurement results submitted by all 6 labs. The pass/fail limit is specified as +/- 0.75* MU (+/- 1.96 dB). The detailed analysis data is attached in the Excel worksheet of this contribution.

2	Discussion

The FR1 MIMO OTA lab alignment results submitted by the 6 labs are summarized and analysed as shown in Table 1.

Table 1. Summary of the FR1 MIMO OTA lab alignment results (without systematic offsets)

| Device | Band | TRMS measurement result [dBm/15kHz] |  |  |  |  |  | Average approach | Average value | Max-Min deviation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | -89.34 | -90.12 | -90.43 | -88.26 | -89.22 | -91.75 | Linear average | -89.85 | 3.49 |
| PAD_2 | n28 | -86.46 | -87.93 | -87.91 | -85.50 | -85.86 | -87.74 |  | -86.90 | 2.43 |
| PAD_3 | n28 | -86.26 | -89.34 | -89.23 | -86.19 | -86.49 | -88.36 |  | -87.65 | 3.15 |
| Device | Band | TRMS offset [dBm/15kHz] |  |  |  |  |  | Pass/fail limit |  |  |
|  |  | Lab 1 | Lab 2 | Lab 3 | Lab 4 | Lab 5 | Lab 6 |  |  |  |
| PAD_1 | n28 | 0.52 | -0.27 | -0.58 | 1.59 | 0.63 | -1.90 | +/- 0.75 MU, i.e., +/- 1.96 dB |  |  |
| PAD_2 | n28 | 0.44 | -1.03 | -1.01 | 1.40 | 1.04 | -0.84 |  |  |  |
| PAD_3 | n28 | 1.38 | -1.69 | -1.59 | 1.46 | 1.15 | -0.71 |  |  |  |
| Lab alignment conclusion |  | Pass | Pass | Pass | Pass | Pass | Pass |  |  |  |

Figure 2 presents the TRMS measurement results (without systematic offsets) from all labs, and Figure 3 visualizes the deviations between TRMS measurement results from each lab and reference values.

Figure 2. TRMS measurement results from 6 labs (without systematic offsets)

Figure 3. Deviations between valid TRMS measurement results from each lab and reference values (without systematic offsets)

Observation 1: All 6 volunteer labs have submitted measurement results for the Rel-18 FR1 MIMO OTA lab alignment activity, no outlier value is identified.

Observation 2: As agreed, the reference value is derived by the linear average (in dB) of the measurement results submitted by all labs. The maximum deviation between measurement results and reference values is -1.9 dB.

Observation 3: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*MU (1.96 dB). All 6 labs with the valid measurement results are well aligned at band n28.

Observation 4: 3GPP Rel-18 FR1 MIMO OTA lab alignment activity can be successfully concluded with all 6 volunteer labs (Apple, CAICT, CMCC&BUPT, MediaTek, Xiaomi, OPPO) well aligned at band n28.

3	Conclusion

This contribution presents a summary of the Rel-18 FR1 MIMO OTA lab alignment results.

Observation 1: All 6 volunteer labs have submitted measurement results for the Rel-18 FR1 MIMO OTA lab alignment activity, no outlier value is identified.

Observation 2: As agreed, the reference value is derived by the linear average (in dB) of the measurement results submitted by all labs. The maximum deviation between measurement results and reference values is -1.9 dB.

Observation 3: The maximum deviation between measurement results and reference values is within the agreed pass/fail limit, +/- 0.75*MU (1.96 dB). All 6 labs with the valid measurement results are well aligned at band n28.

Observation 4: 3GPP Rel-18 FR1 MIMO OTA lab alignment activity can be successfully concluded with all 6 volunteer labs (Apple, CAICT, CMCC&BUPT, MediaTek, Xiaomi, OPPO) well aligned at band n28.

References

- R4-2321137, Updated Framework and time plan for FR1 MIMO OTA performance requirements development (Nov 2023), CAICT, 3GPP RAN4#109, Nov. 2023.

- R4-2402876, Way forward on [110][332] NR_MIMO_OTA_enh, CAICT, 3GPP RAN4#110, Feb 2024.

- R4-2320067, CAICT Rel-18 FR1 MIMO OTA Lab Alignment PAD Results, CAICT, 3GPP RAN4#109, Nov. 2023.

- R4-2318927, PADs measurement results for n28, CMCC, BUPT, 3GPP RAN4#109, Nov. 2023.

- R4-2318896, Measure results for 3GPP Rel-18 FR1 MIMO OTA Lab Alignment, Xiaomi, 3GPP RAN4#109, Nov. 2023.

- R4-2319919, 3GPP Rel-18 FR1 MIMO OTA PAD measurement, OPPO, 3GPP RAN4#109, Nov. 2023.

- R4-2403017, On FR1 MIMO OTA Lab Alignment PADs measurement results, Apple, 3GPP RAN4#110, Feb. 2024.

- R4-2404218, Re-submission of measurement results for low band alignment activity, MediaTek, 3GPP RAN4#110 bis, April 2024.

- R4-2400534, Update to FR1 Channel model validation, MVG, Spirent, Apple, ETS-Lindgren, Telecom Italia, Sony, 3GPP RAN4#110, Feb. 2024.