3GPP TSG-RAN WG4 Meeting #110bis	R4-2405538

Changsha, China, April 15-19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Less than 5Mhz test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases for Less than 5MHz are missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Radio Link Monitoring (SSB-based, FR1): - Non-DRX - Out-of-sync 15 PRBs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RLM feature is not tested with RLM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.X-1 A.6.5.1.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

#### A.3.X	Test parameters for FR1 for a UE operating on a cell with less than 5MHz BW

Table A.3.X-1: General test parameters for FR1 for a UE operating on a cell with less than 5MHz BW

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| Active PCell |  |  | Cell 1 |
| RF Channel Number |  |  | 1 |
| Duplex mode |  |  | FDD |
| BWchannel | Config 1 | MHz | 3: NRB,c = 12 |
|  | Config 2 |  | 3: NRB,c = 15 |
|  | Config 3 |  | 5: NRB,c = 20 |
| DL initial BWP configuration |  |  | DLBWP.0.1 |
| DL dedicated BWP configuration |  |  | DLBWP.1.1 |
| UL initial BWP configuration |  |  | ULBWP.0.1 |
| UL dedicated BWP configuration |  |  | ULBWP.1.1 |
| RMSI CORESET Reference Channel |  |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  |  | CCR.1.3 FDD |
| SSB Configuration |  |  | SSB.x FR1 |
| SMTC Configuration |  |  | SMTC.1 |
|  |  |  | SMTC.1 |
| PDSCH/PDCCH subcarrier spacing |  |  | 15 kHz |
| PRACH Configuration |  |  | Table A.3.8.2.1-1 |
|  |  |  | Table A.3.8.2.1-1 |
| SSB index assigned as RLM RS |  |  | 0 |
| OCNG parameters |  |  | OP.1 |
| CP length |  |  | Normal |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |
| DRX |  |  | OFF |
| Gap pattern ID |  |  | gp0 |
| Layer 3 filtering |  |  | Enabled |
| T310 timer |  | ms | 0 |
| T311 timer |  | ms | 1000 |
| N310 |  |  | 1 |
| N311 |  |  | 1 |
| T1 |  | s | [0.2] |
| T2 |  | s | [0.48] |
| T3 |  | s | [0.48] |
| D1 |  | s | [0.44] |
| Note 1: All configurations are assigned to the UE prior to the start of time period T1. Note 2: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

<End of change 1>

<Start of change 2>

#### A.6.5.1.x	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode for UE operating on a cell with less than 5MHz BW

##### A.6.5.1.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE operating on a cell with less than 5MHz BW properly detects the out of sync and in sync for the purpose of monitoring downlink radio link quality of the PCell. This test will partly verify the FR1 radio link monitoring requirements in clause 8.1.

The test parameters are the same as those specified in clause A.6.5.1.1 except those described in the following clause. Supported configurations are described in the Table A.3.X-1 and the test parameters are described in the following tables:

- Table A.3.X-1: General test parameters for FR1 PCell configured with channel BW of less than 5 MHz is used without changes.

- Table A.6.5.1.1.1-1: General test parameters for FR1 out-of-sync testing in non-DRX mode is used as a baseline. Table A.6.5.1.x.1-1 and Table A.3.X-1 describes parameters replacing the parameters in the baseline table.

- Table A.6.5.1.1.1-3: Cell specific test parameters for FR1 (Cell 1) for out-of-sync radio link monitoring tests in non-DRX mode is used without changes.

- Table A.6.5.1.1.1-4: Measurement gap configuration for out-of-sync tests in non-DRX mode is used without changes.

In the test, UE operating on a cell with less than 5MHz BW is configured to perform RLM on SSB, with detectionResource included in RadioLinkMonitoringRS set to SSB#0, and purpose set to ‘rlf’. There is one cell (Cell 1), which is the active NR cell operating on a cell with less than 5MHz BW, in the test. The test consists of three successive time periods, with time duration of T1, T2 and T3 respectively. Figure A.6.5.1.1.x-1 shows the variation of the downlink SNR in the active cell to emulate out-of-sync and in-sync states. Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. The UE is configured to perform inter-frequency measurements using Gap Pattern ID #0 (40ms) in test 1.

Table A.6.5.1.1.x-1: General test parameters for FR1 out-of-sync testing in non-DRX mode for a UE operating on a cell with less than 5MHz BW

| Attribute | Value for BLER Configuration #0 |  |  |
| --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |
| DCI format | 1-0 |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |
| Aggregation level (CCE) | [4] | [8] | [8] |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |
| DMRS precoder granularity | REG bundle size |  |  |
| REG bundle size | 6 |  |  |
| CP length | Normal |  |  |
| Mapping from REG to CCE | Distributed | [Non-Distributed] | [Distributed] |

Figure A.6.5.1.1.1-1: SNR variation for out-of-sync testing

##### A.6.5.1.x.2	Test Requirements

The UE behaviour in each test during time durations T1, T2 and T3 shall be as follows:

During the period from time point A to time point B the UE shall transmit uplink signal at least in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting.

The UE shall stop transmitting uplink signal no later than time point C (D1 second after the start of the time duration T3).

The rate of correct events observed during repeated tests shall be at least 90%.

<End of change 2>