3GPP TSG-RAN4 Meeting # 110-bis	R4-2405557

# Changsha, China,  15 – 19 April 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.181 | CR | xxxx | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for TS 38.181: FR2 introduction in Clause 9.4 OTA output power dynamics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of FR2-NTN and SAN Type 2-O in TS 38.108 implies update of TS 38.181 with similar features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of FR2-NTN support, also aligned with 38.141-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 support not existing in clause 9.4 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## -------- start of change ---------

## 9.4	OTA output power dynamics

### 9.4.1	General

The requirements in clause 9.4 apply during the transmitter ON period. Transmit signal quality (as specified in clause 9.6) shall be maintained for the output power dynamics requirements.

The OTA output power requirements are single direction requirements and apply to the beam peak directions over the OTA peak directions set.

### 9.4.2	OTA RE power control dynamic range

#### 9.4.2.1	Definition and applicability

The OTA RE power control dynamic range is the difference between the power of an RE and the average RE power for a SAN at maximum output power (Pmax,c,EIRP) for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

#### 9.4.2.2	Minimum requirement

The minimum requirement for SAN type 1-O is in TS 38.108 [2], clause 9.4.2.2.

#### 9.4.2.3	Method of test

No specific test or test requirements are defined for RE power control dynamic range. The Error Vector Magnitude test, as described in clause 9.6 provides sufficient test coverage for this requirement.

### 9.4.3	OTA total power dynamic range

#### 9.4.3.1	Definition and applicability

The OTA total power dynamic range is the difference between the maximum and the minimum transmit power of an OFDM symbol for a specified reference condition.

This requirement shall apply at each RIB supporting transmission in the operating band.

NOTE:	The upper limit of the OTA total power dynamic range is the SAN maximum carrier EIRP (Pmax,c,EIRP) when transmitting on all RBs. The lower limit of the OTA total power dynamic range is the average EIRP for single RB transmission in the same direction using the same beam. The OFDM symbols shall carry PDSCH and not contain PDCCH, RS or SSB.

#### 9.4.3.2	Minimum requirement

The minimum requirement for SAN type 1-O is in TS 38.108 [2], clause 9.4.3.2.

The minimum requirement for SAN type 2-O is in TS 38.108 [2], clause 9.4.3.3.

#### 9.4.3.3	Test purpose

The test purpose is to verify that the total power dynamic range is within the limits specified by the minimum requirement.

#### 9.4.3.4	Method of test

##### 9.4.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

Beams to be tested:	Declared beam with the highest intended EIRP for the narrowest intended beam corresponding to the smallest BeWθ, or for the narrowest intended beam corresponding to the smallest BeWϕ (D.3, D.11).

Directions to be tested: The OTA peak directions set reference beam direction pair (D.8).

##### 9.4.3.4.2	Procedure

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the SAN with the test system.

3)	Orient the positioner (and SAN) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the SAN according to the declared beam direction pair.

5)	For SAN type 1-O, set the SAN to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-SAN-FR1-TM3.1 if 64QAM is supported by SAN without power back off, or

-	NR-SAN-FR1-TM3.1 with all 64QAM PDSCH PRBs replaced by 16QAM PDSCH PRBs if 64QAM is supported by SAN with power back off, or

-	NR-SAN-FR1-TM3.1 with all 64QAM PDSCH PRBs replaced by 16QAM PDSCH PRBs if 64QAM is  not supported by SAN but 16QAM is supported by SAN, or

-	NR-SAN-FR1-TM3.1 with all 64QAM PDSCH PRBs replaced by QPSK PDSCH PRBs if 16QAM is not supported by SAN.

For SAN type 2-O, set the SAN to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-SAN-FR2-TM3.1 with 64QAM signals if 64QAM is supported by SAN without power back off;

-	NR-SAN-FR2-TM3.1 with highest modulation order supported without power back off if 64QAM is not supported by SAN, or;

-	NR-SAN-FR2-TM3.1with highest modulation order supported without power back off if 64QAM is supported by SAN with power back off;

6)	Measure the OFDM symbol TX power as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

7)	For SAN type 1-O, set the SAN to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-SAN-FR1-TM2 if 64QAM is supported by SAN, or

-	NR-SAN-FR1-TM2 with all 64QAM PDSCH PRBs replaced by 16QAM PDSCH PRBs if 64QAM is not supported by SAN and 16QAM is supported;

-	NR-SAN-FR1-TM2 with all 64QAM PDSCH PRBs replaced by QPSK PDSCH PRBs if 64QAM and 16QAM are both not supported;

For SAN type 2-O, set the SAN to transmit a signal according to the applicable test configuration in clause 4.8 using the corresponding test models:

-	NR-SAN-FR2-TM2 if 64QAM is supported by SAN, or;

-	NR-SAN-FR2-TM2 with highest modulation order supported if 64QAM is not supported by SAN;

8)	Measure the OFDM symbol TX power (OSTP) as defined in annex L by measuring the EIRP for any two orthogonal polarizations (denoted p1 and p2) and calculate total radiated transmit power for particular beam direction pair as EIRP = EIRPp1 + EIRPp2.

The measured OFDM symbols shall not contain RS or SSB.

#### 9.4.3.5	Test requirement

##### 9.4.3.5.1	SAN type 1-O

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.3.5.1-1.

Table 9.4.3.5.1-1: Total power dynamic range

| SAN channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |

NOTE:	Additional test requirements for the Error Vector Magnitude (EVM) at the lower limit of the dynamic range are defined in clause 6.6.

##### 9.4.3.5.2	SAN type 2-O

OTA total power dynamic range minimum requirement for SAN type 2-O is specified such as for each NR carrier it shall be larger than or equal to the levels specified in table 9.4.3.5.2-1.

Table 9.4.3.5.2-1: Minimum requirement for SAN type 2-O total power dynamic range

| SCS | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| (kHz) | OTA total power dynamic range (dB) |  |  |  |
| 60 | 17.7 | 20.8 | 23.8 | N/A |
| 120 | 14.6 | 17.7 | 20.8 | 23.8 |

NOTE:	Additional test requirements for the EVM at the lower limit of the dynamic range are defined in clause 6.6.

## 9.5	OTA transmit ON/OFF power

## -------- end of change ---------