3GPP TSG-RAN WG4 Meeting #110-bis	R4-2405589

Changsha, China, 15 – 19 April, 2024

Title: 	Additional simulation results for CPP

Source: 	Huawei, HiSilicon

Agenda item:	6.12.3.5

Document for:	Discussion

Introduction

Simulation assumption for evaluating the link level performance of CPP is updated in RAN4#109 [1]. In this paper we will provide our additional simulation results for CPP. Our initial simulation results are in [2].

Discussion

RSCPD

RSTD accuracy are defined under following side conditions

- 4-sample, Es/Iot of (-6,-13)dB, AWGN and TDL channel

- 1-sample, Es/Iot of (-3,-6)dB, AWGN channel

It is reasonable to define accuracy for RSCPD under same conditions, with the exception that TDL channel is changed to 2-tap channel, and timing is always based on 1-sample measurement. The latter is because when RSTD is measured with 4-sample combined, UE would report 4 RSCPD measurements, and each of the phase measurement should be based on single sample timing measurement instead of combined timing measurement.

The simulation results are shown in Table 1-6.

Table 1: RSCPD error for 1-sample, Es/Iot of (-6,-13)dB, AWGN, FR1

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 | 27.6 |
|  |  | 52 | 1 | 19.0 |
|  |  | 104 | 1 | 13.5 |
|  | 30 | 24 | 4 | 27.7 |
|  |  | 48 | 1 | 20.2 |
|  |  | 132 | 1 | 14.0 |
|  | 60 | 24 | 4 | 30.4 |
|  |  | 64 | 1 | 20.8 |
|  |  | 132 | 1 | 15.9 |

Table 2: RSCPD error for 1-sample, Es/Iot of (-6,-13)dB, 2-tap channel, FR1

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 | 48.3 |
|  |  | 52 | 1 | 36 |
|  |  | 104 | 1 | 32.4 |
|  | 30 | 24 | 4 | 56.1 |
|  |  | 48 | 1 | 44.3 |
|  |  | 132 | 1 | 28.1 |
|  | 60 | 24 | 4 | 47.0 |
|  |  | 64 | 1 | 26.1 |
|  |  | 132 | 1 | 22.4 |

Table 3: RSCPD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN channel, FR1

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 52 | 1 | 11.0 |
|  |  | 104 | 1 | 8.7 |
|  | 30 | 48 | 1 | 11.8 |
|  |  | 132 | 1 | 10.8 |
|  | 60 | 64 | 1 | 10.5 |
|  |  | 132 | 1 | 7.6 |

Table 4: RSCPD error for 1-sample, Es/Iot of (-6,-13)dB, AWGN, FR2

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 | 29.9 |
|  |  | 64 | 1 | 18.7 |
|  |  | 132 | 1 | 15.7 |
|  | 120 | 32 | 4 | 27.3 |
|  |  | 64 | 1 | 23.7 |
|  |  | 128 | 1 | 18.5 |

Table 5: RSCPD error for 1-sample, Es/Iot of (-6,-13)dB, 2-tap channel, FR2

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 | 47.4 |
|  |  | 64 | 1 | 26.4 |
|  |  | 132 | 1 | 22.1 |
|  | 120 | 32 | 4 | 38.1 |
|  |  | 64 | 1 | 34.9 |
|  |  | 128 | 1 | 28.2 |

Table 6: RSCPD error for 1-sample, Es/Iot of (-3,-6)dB, AWGN channel, FR2

| Parameter | Value |  |  | RSCPD error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 64 | 1 | 10.5 |
|  |  | 132 | 1 | 7.6 |
|  | 120 | 64 | 1 | 12.3 |
|  |  | 128 | 1 | 8.6 |

RSCP

Rx-Tx accuracy are defined under following side conditions

- 4-sample, Es/Iot of -3dB and -13dB, AWGN and TDL channel

- 1-sample, Es/Iot of 0dB and -6dB, AWGN channel

To define accuracy for relative RSCP, one way is to use Es/Iot combination same as RSCPD. In this case, the simulation results and requirements for RSCPD can be directly re-used. In last meeting, some companies proposed to align the Es/Iot to Rx-Tx. For this setup we conducted additional simulation for Es/Iot combination of (-3, -13) and (-13,-13) for AWGN and 2-tap channel, as well as (-6, -6) and (0,-6) for AWGN. Same as RSCPD, the timing is always based on 1-sample measurement.

The simulation results are shown in Table 7-12.

Table 7a: Relative RSCP error for 1-sample, Es/Iot of (-3,-13)dB, AWGN, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 24 | 4 |  |
|  |  | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 7b: Relative RSCP error for 1-sample, Es/Iot of (-13,-13)dB, AWGN, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 24 | 4 |  |
|  |  | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 8a: Relative RSCP error for 1-sample, Es/Iot of (-3,-13)dB, 2-tap channel, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 24 | 4 |  |
|  |  | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 8b: Relative RSCP error for 1-sample, Es/Iot of (-13,-13)dB, 2-tap channel, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 24 | 4 |  |
|  |  | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 24 | 4 |  |
|  |  | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 9a: Relative RSCP error for 1-sample, Es/Iot of (0,-6)dB, AWGN channel, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 9b: Relative RSCP error for 1-sample, Es/Iot of (-6,-6)dB, AWGN channel, FR1

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 15 | 52 | 1 |  |
|  |  | 104 | 1 |  |
|  | 30 | 48 | 1 |  |
|  |  | 132 | 1 |  |
|  | 60 | 64 | 1 |  |
|  |  | 132 | 1 |  |

Table 10a: Relative RSCP error for 1-sample, Es/Iot of (-3,-13)dB, AWGN, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 32 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 128 | 1 |  |

Table 10b: Relative RSCP error for 1-sample, Es/Iot of (-13,-13)dB, AWGN, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 32 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 128 | 1 |  |

Table 11a: Relative RSCP error for 1-sample, Es/Iot of (-3,-13)dB, 2-tap channel, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 32 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 128 | 1 |  |

Table 11b: Relative RSCP error for 1-sample, Es/Iot of (-13,-13)dB, 2-tap channel, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 24 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 32 | 4 |  |
|  |  | 64 | 1 |  |
|  |  | 128 | 1 |  |

Table 12a: Relative RSCP error for 1-sample, Es/Iot of (0,-6)dB, AWGN channel, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 64 | 1 |  |
|  |  | 128 | 1 |  |

Table 12b: Relative RSCP error for 1-sample, Es/Iot of (-6,-6)dB, AWGN channel, FR2

| Parameter | Value |  |  | Relative RSCP error |
| --- | --- | --- | --- | --- |
| SCS, RB num, Repetition | SCS (kHz) | RB num | Repetition (Note) | 90% |
|  | 60 | 64 | 1 |  |
|  |  | 132 | 1 |  |
|  | 120 | 64 | 1 |  |
|  |  | 128 | 1 |  |

Conclusions

In this paper we provided our additional simulation results for CPP. We suggest to take these results into account when defining the accuracy requirements for CPP.

Reference

- R4-2321459, Updated simulation assumptions for CPP measurement, CATT

- R4-2312843, Simulation results for CPP measurement, Huawei, HiSilicon