# 3GPP TSG- RAN WG4 Meeting # 110bis	R4-2405635

# Changsha, China, 15th–19th Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.115-1 Clause Annex |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To introduce the NCR into TS 38.115-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To add the conformance testing requirement for NCR in the Annex |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec cannot support the NCR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

Annex A (normative):
Repeater stimulus signals

A.1	Repeater stimulus signal 1

This repeater stimulus signal shall be used for the following tests:

-	Uplink maximum output power

-	Uplink operating band unwanted emissions

-	Uplink spurious emissions

Two uplink fixed reference channels for performance requirements (16QAM ¾) for FDD according to the TS38.141-1 [7], [A.4 table A.4-1, channel reference AX-X of 5] MHz bandwidth generated on separate centre frequencies with equal power and combined with a time difference of 266,7 us (4 OFDM symbols)

The PUSCH data payload shall contain only zeroes (0000 0000).

Each reference channel shall be subjected to time windowing and filtering so that it fulfils the spectral purity requirements defined in A.3.

A.2	Repeater stimulus signal 2

This repeater stimulus signal shall be used for the following tests:

-	Downlink operating band unwanted emissions

-	Downlink spurious emissions

Two NR-FR1-TM1.1 channels according to the TS38.141-1 [7] of 5 MHz bandwidth generated on separate centre frequencies with equal power and combined with a time difference of [1400 us (21 OFDM symbols)].

Each NR-FR1-TM1.1 channel shall be subjected to time windowing and filtering so that it fulfils the spectral purity requirements defined in A.3.

A.3	Repeater stimulus signal spectral purity requirements

The reference channels or test models constituting the repeater stimulus signal shall fulfil the spectral purity requirements defined in table A.3-1 and A.3-2, where:

-	the minimum spectral density suppression is related to the reference spectral density.

Table A.3-1: Repeater stimulus signal spectral purity requirements for nominal channel bandwidth of passband <=20MHz

| Center frequency of measurement filter | Minimum requirements | Measurement filter type and filter bandwidth | Integration bandwidth |
| --- | --- | --- | --- |
| 50.0 kHz outside from channel edge | 46.8 dBc (Note 1) | Square (100 kHz) | 100 kHz |
| Center of adjacent channel frequency with same nominal channel BW | 46.8 dBc (Note 2) | Square (nominal channel BW) | Nominal channel BW |
| NOTE 1: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of 100 kHz. NOTE 2: the reference spectral densigy shall be taken at the carrier center frequency with an integration bandwidth of nominal carrier BW |  |  |  |

Table A.3-2: Repeater stimulus signal spectral purity requirements for nominal channel bandwidth of passband >20MHz

| Center frequency of measurement filter | Minimum requirements | Measurement filter type and filter bandwidth | Integration bandwidth |
| --- | --- | --- | --- |
| 50.0 kHz outside from channel edge | 47.2 dBc (Note 1) | Square (100 kHz) | 100 kHz |
| Center of adjacent channel frequency with same nominal channel BW | 47.2 dBc (Note 2) | Square (nominal channel BW) | Nominal channel BW |
| NOTE 1: the reference spectral density shall be taken at the carrier center frequency with an integration bandwidth of 100 kHz. NOTE 2: the reference spectral densigy shall be taken at the carrier center frequency with an integration bandwidth of nominal carrier BW |  |  |  |

Annex B (normative):
Environmental requirements for the repeater

B.1	General

For each test in the present document, the environmental conditions under which the repeater is to be tested are defined.

B.2	Normal test environment

When a normal test environment is specified for a test, the test should be performed within the minimum and maximum limits of the conditions stated in table B.1.

Table B.1: Limits of conditions for normal test environment

| Condition | Minimum | Maximum |
| --- | --- | --- |
| Barometric pressure | 86 kPa | 106 kPa |
| Temperature | 15 C | 30 C |
| Relative humidity | 20 % | 85 % |
| Power supply | Nominal, as declared by the manufacturer |  |
| Vibration | Negligible |  |

The ranges of barometric pressure, temperature and humidity represent the maximum variation expected in the uncontrolled environment of a test laboratory. If it is not possible to maintain these parameters within the specified limits, the actual values shall be recorded in the test report.

NOTE:	This may, for instance, be the case for measurements of radiated emissions performed on an open field test site.

B.3	Extreme test environment

The manufacturer shall declare one of the following:

1)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-3 [16];

2)	The equipment class for the equipment under test, as defined in the IEC 60 721-3-4 [17];

3)	The equipment that does not comply with the mentioned classes, the relevant classes from IEC 60 721 [18] documentation for temperature, humidity and vibration shall be declared.

NOTE:	Reduced functionality for conditions that fall outside of the standard operational conditions is not tested in the present document. These may be stated and tested separately.

B.3.1	Extreme temperature

When an extreme temperature test environment is specified for a test, the test shall be performed at the standard minimum and maximum operating temperatures defined by the manufacturer's declaration for the equipment under test.

Minimum temperature:

The test shall be performed with the environment test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-1 [19].

Maximum temperature:

The test shall be performed with the environmental test equipment and methods including the required environmental phenomena into the equipment, conforming to the test procedure of IEC 60 068-2-2 [20].

NOTE:	It is recommended that the equipment is made fully operational prior to the equipment being taken to its lower operating temperature.

B.4	Vibration

When vibration conditions are specified for a test, the test shall be performed while the equipment is subjected to a vibration sequence as defined by the manufacturer's declaration for the equipment under test. This shall use the environmental test equipment and methods of inducing the required environmental phenomena in to the equipment, conforming to the test procedure of IEC 60 068-2-6 [21]. Other environmental conditions shall be within the ranges specified in annex B.2.

NOTE:	The higher levels of vibration may induce undue physical stress in to equipment after a prolonged series of tests. The testing body should only vibrate the equipment during the RF measurement process.

B.5	Power supply

When extreme power supply conditions are specified for a test, the test shall be performed at the standard upper and lower limits of operating voltage defined by manufacturer's declaration for the equipment under test.

Upper voltage limit:

The equipment shall be supplied with a voltage equal to the upper limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [19] Test Ab/Ad and IEC 60 068-2-2 [20] Test Bb/Bd: Dry heat.

Lower voltage limit:

The equipment shall be supplied with a voltage equal to the lower limit declared by the manufacturer (as measured at the input terminals to the equipment). The tests shall be carried out at the steady state minimum and maximum temperature limits declared by the manufacturer for the equipment, to the methods described in IEC 60 068-2-1 [19] Test Ab/Ad and IEC 60 068-2-2 [20] Test Bb/Bd: Dry heat.

B.6	Measurement of test environments

The measurement accuracy of the repeater test environments defined in annex B shall be:

Pressure:	5 kPa

Temperature:	2 degrees

Relative humidity:	5 %

DC voltage:	1.0 %

AC voltage:	1.5 %

Vibration:	10 %

Vibration frequency:	0.1 Hz

The above values shall apply unless the test environment is otherwise controlled and the specification for the control of the test environment specifies the uncertainty for the parameter.

Annex C (informative):
Test tolerances and derivation of test requirements

C.1	Measurement of conducted characteristics

Table C.1-1: Derivation of test requirements (conducted tests)

| Test | Minimum requirement in TS 38.106 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Repeater output power | See TS 38.106 [2], clause 6.2 | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 7.125GHz | Formula: Upper limit + TT, Lower limit – TT |
| 6.3 Frequency stability | See TS 38.106 [2], clause 6.3 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.4 Out of band gain | See TS 38.106 [2], clause 6.4 | 0.5 dB, f ≤ 3.0 GHz 0.8 dB, 3.0 GHz < f ≤ 7.125GHz | Formula: Minimum Requirement + TT |
| 6.5.2 ACLR/ CACLR | See TS 38.106 [2], clause 6.5.2 | Relative ACLR/CACLR: BW ≤ 20MHz: 0.8 dB BW > 20MHz: 1.2 dB Absolute ACLR/CACLR: 0 dB | Formula: ACLR Minimum Requirement - TT Absolute limit +TT |
| 6.5.3 Operating band unwanted emission | See TS 38.106 [2], clause 6.5.3 | Offsets < 10MHz: 1.5 dB, f ≤ 3.0GHz 1.8 dB, 3.0GHz < f ≤ 7.125 GHz Offsets ≥ 10MHz: 0 dB | Formula: Minimum Requirement + TT |
| 6.5.4 Spurious emissions | See TS 38.106 [2], clause 6.5.4 | 0 dB | Formula: Minimum Requirement + TT |
| 6.6 EVM | See TS 38.106 [2], clause 6.6 | 1.25% | Formula: EVM limit + TT |
| 6.7 Input intermodulation | See TS 38.106 [2], clause 6.7 | 0 dB | Formula: Minimum Requirement + TT |
| 6.8 Output intermodulation | See TS 38.106 [2], clause 6.8 | 0 dB | Formula: Minimum Requirement + TT |
| 6.9 ACRR | See TS 38.106 [2], clause 6.9 | 0.7 dB | Formula: Minimum Requirement – TT |
| 6.10.1 Transmit ON/OFF power | See TS 38.106 [2], clause 6.10.1 | 2.0 dB, f ≤ 3.0 GHz 2.5 dB, 3.0 GHz < f ≤ 7.125 GHz | Formula: Minimum Requirement + TT |
| 6.10.2 Transmitter transient period | See TS 38.106 [2], clause 6.10.2 | N/A |  |
| 6.11 Output power dynamics | See TS 38.106 [2], clause 6.11.2, 6..11.3 and 6.11.4 |  |  |
| 6.12 Transmit signal quality | See TS 38.106 [2], clause 6.12.2, 6..12.3 |  |  |
| 6.13 Transmit intermodulation | See TS 38.106 [2], clause 6.13.2 |  |  |
| 6.14 Conducted reference sensitivity | See TS 38.106 [2], clause 6.16.1.2 |  |  |
| 6.15 Conducted maximum input level | See TS 38.106 [2], clause 6.17 |  |  |
| 6.16 Conducted adjacent channel | See TS 38.106 [2], clause 6.18.1.2 and 6.18.1.3 |  |  |
| 6.17 Conducted blocking characteristics | See TS 38.106 [2], clause 6.19.2 and 6.19.3 |  |  |
| 6.18 Conducted spurious response | See TS 38.106 [2], clause 6.20.2 |  |  |
| 6.19 Conducted intermodulation characteristics | See TS 38.106 [2], clause 6.21.2 and See TS 38.106 [2], clause 6.21.3 |  |  |
| 6.20Conducted spurious emissions | See TS 38.106 [2], clause 6.21.2 and See TS 38.106 [2], clause 6.22.2 |  |  |
| NOTE: TT values are applicable for normal condition unless otherwise stated. |  |  |  |

Annex D (informative):
Measurement system set-up

Example of measurement system set-ups are attached below as an informative annex.

The following measurement system set up is applicable for both repeater type 1-C and NCR type 1-C.

For the measurement setup for NCR-MT type 1-H, the existing measurement setup in Annex D.1 and D.2 of TS 38.176-1 is applicable.

For the measurement setup for NCR-MT type 1-C, the existing measurement setup in Annex D.1 and D.2 of TS 38.176-1 is applicable with the TAB connectors replaced with antenna connectors.

D.1	Repeater output power and transmit ON/OFF power

Figure D.1-1: Measuring system set-up for repeater output power and tansmit ON/OFF power

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

Note 3:     for NCR-MT output power and transmit ON/OFF power measurement, the external signal generator is not needed and

D.2	Out of band gain

Figure D.2-1: Measuring system set-up for out of band gain

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:      This requirement is not applicable for NCR-MT.

D.3	Unwanted emission: Operating band unwanted emission, transmitter spurious emission, and ACLR

Figure D.3-1: Measuring system set-up for unwanted emission: Operating band unwanted emission, transmitter spurious emission, and ACLR

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

Note 3:     for NCR-MT output power and transmit ON/OFF power measurement, the external signal generator is not needed and

D.4	Modulation Accuracy: Repeater Error Vector Magnitude and Frequency Stability

Figure D.4-1: Measuring system set-up for repeater error vector magnitude and frequency stability

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

Note 3:      This requirement is not applicable for NCR-MT.

D.5	Input intermodulation

Figure D.5-1: Measuring system set-up for input intermodulation.

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:      This requirement is not applicable for NCR-MT.

D.6	Output Intermodulation

Figure D.6-1: Measuring system set-up for output intermodulation.

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

D.7	Adjacent Channel Rejection Ratio

Figure D.7-1: Measuring system set-up for Adjacent Channel Rejection Ratio

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

Note 3:     This requirement is not applicable for NCR-MT.

D.8	Rx spurious emission requirement

Figure D.8-1: Measuring system set-up for receiver spurious emission

Note 1:	The repeater is a bi-directional device. The signal generator may need protection.

Note 2:	UL/DL timing can be provided to the repeater.

Note 3:     for NCR-MT output power and transmit ON/OFF power measurement, the external signal generator is not needed and

<End of the change>