# 3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405710

# Changsha, China, April 15 – 19, 2024

Agenda item:		12.2.1

Source: 		Qualcomm Inc.

Title: 		Powerclasses and maximum output power

Document for:		Approval

## 1	Introduction

High power UE power classes and how the related RRC parameters apply have been discussed in RAN4 for multiple meetings. In this contribution we propose a compromise solution to conclude the topic, focusing on inter-band CA.

## 2	Discussion

The signaling parameters related to power class are summarized in Table 1. It is important to note that only ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 is a mandatory parameter. Therefore, specification should work when only this parameter is present. However, it cannot be expected that power class information is ideal and optimized in case UE does not take advantage of the other parameters, which allow providing information with higher granularity.

Table 1: Power Class related RRC-parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 For FR1, if the UE supports the different UE power class than the default UE power class as defined in clause 6.2 of TS 38.101-1 [2], the UE shall report the supported UE power class in this field. For FR2, UE shall report the supported UE power class as defined in clause 6 and 7 of TS 38.101-2 [3] in this field. UE indicating support for pc6 supports the enhanced intra-NR RRM and demodulation processing requirements for FR2 to support high speed up to 350 km/h as specified in TS 38.133 [5]. This capability is not applicable to IAB-MT. The power class pc7 is only applicable for RedCap UEs operation in FR2. | Band | Yes | N/A | N/A |
| powerClass, powerClass-v1610 Indicates power class the UE supports when operating according to this band combination. If the field is absent, the UE supports the default power class. If this power class is higher than the power class that the UE supports on the individual bands of this band combination (ue-PowerClass in BandNR), the latter determines maximum TX power available in each band. The UE sets the power class parameter only in band combinations that are applicable as specified in TS 38.101-1 [2] and TS 38.101-3 [4]. This capability is not applicable to IAB-MT. | BC | No | N/A | FR1 only |
| ue-PowerClassPerBandPerBC-r17 Indicates the UE power class per band per band combination. NOTE: It is not applicable to the case when UL-MIMO and intra-band UL CA are in operation at the same time. | FS | No | N/A | FR1 only |

It can be observed that powerClass, powerClass-v1610 provides the power class for band combination and according to the description in a case where powerClass is higher than ue-PowerClass in a given band, ue-PowerClass will take priority in defining the per band output power. The specifications on the other hand are not fully clear on what the approach should be when ue-PowerClass is higher than PowerClass.

A lot of the confusion in the power class discussion is originating the wording choices in 38.101-1. Some examples are given below:

#### 6.2A.1.1	UE maximum output power for Intra-band contiguous CA

For uplink intra-band contiguous carrier aggregation, the maximum output power is specified in Table 6.2A.1.1-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the NR band, the maximum output power is specified in Table 6.2.1-1 for power class 3 and other power classes if indicated in clause 5.5A.1

…

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in Table 6.2.1-1 apply for power class 3 and other power classes if indicated in clause 5.5A.3.

For inter-band carrier aggregation with two uplink contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.1 apply.

For inter-band carrier aggregation with two uplink non-contiguous carrier assigned to one NR band, the transmitter power requirements specified in subclause 6.2A.1.2 apply. For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is defined as the sum of maximum output power from each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

It can be seen that for contiguous CA carrier configuration is discussed while for interband CA “carrier assigned to” is used. Assigned can be interpreted as configured, activated or scheduled.

In RAN4#110 a WF on NR UE power classes was agreed [1]. For DL CA with single carrier UL following agreement was reached

For any DL CA with single-carrier UL, shall the UE mandatorily support the power class indicated in ue-PowerClass for the UL band if it’s applicable in the spec for the CA configuration?

down-select to the following two options

- Option 2: No. It is optional, subject to the power class capability reported by the UE.

- Option 3: Yes, except for Power Class 1.5 for which the UE shall at least meet the minimum requirements for Power Class 2.

There is no option left where UE output power would be limited to PowerClass, but instead the the remaining open question is whether it is optional or mandatory to meet ue-powerClass.

Given that the issue is existing since rel-16, the best approach would be to give an allowance for UE to exceed PowerClass. When this is formulated as an allowance, it enables UE implementations to apply either minimum between PowerClass and ue-PowerClass, but also enables better system performance and better cell coverage via higher power single-UL transmissions.

Similar to DL CA, the same approach can be used for UL CA to remove to ambiguity on what “carrier assigned to” means. Giving UE an allowance to transmit higher power for single carrier transmissions is best for system performance and as it is an allowance, implementations supported all the interpretations can be supported.

This allowance would mean, that in case UE only indicates the ue-PowerClass and PowerClass, there is ambiguity on what is the maximum output power when only a single carrier is scheduled. This ambiguity can be resolved with ue-PowerClassPerBandPerBC-r17, as it can be used to indicate the maximum single band output power capability when part of a band combination. This would mean that even if PowerClass is PC3, it would be possible to indicate that single UL transmission can reach higher output power. This means that network will have the accurate information of UE maximum power capabilities both for CA and non-CA transmissions. This applies for both DL CA and UL CA configurations.

Proposal 1: Explicitly allow UE to indicate higher power class than powerClass in ue-PowerClassPerBandPerBC-r17 when single band requirements are specified for said higher power class. UE shall meet ue-PowerClassPerBandPerBC-r17 for a band for non-CA transmissions when ue-PowerClassPerBandPerBC-r17 indicates higher power class than powerClass. If ue-PowerClassPerBandPerBC-r17 is not indicated to be higher than powerClass, UE output power in a band cannot exceed powerClass.

One possible concern is the relationship of this allowance and MSD specifications. It is good to note that MSD requirements are just a reference point for the minimum UE RF performance. The MSD is set for specific channel bandwidth, center frequency and RB allocation, while in the field operation different parameters can be used. Therefore, it would be sufficient to maintain the existing MSD requirements as a reference point, and specify that in case HPUE requirements are not yet specified for the higher power classes, MSD shall be verified at default power class power level.

Proposal 2: A UE indicating ue-PowerClassPerBandPerBC-r17 higher than powerClass for a band shall meet CA refsens and MSD requirements with the uplink power limited to the nominal maximum output power for the default power class, unless higher than default power class requirements are explicitly specified for the supported power class.

Specification changes for Tx requirements can be quite limited. Adding a general clause to describe the UE allowance is needed, as well as a change on configured maximum output power.

Proposal 3: Adopt the new general clause to TS 38.101-1 as shown below

6.2A.0	General

Non-CA UL transmissions are not bounded by the power class for the band combination indicated by powerClass and powerClass may be ignored in setting the configured maximum output power for CA with single CC UL transmission when ue-PowerClassPerBandPerBC-r17 indicates higher power class than PowerClass. In such scenario UE shall meet power class for a band indicated in ue-PowerClassPerBandPerBC-r17. If ue-PowerClassPerBandPerBC-r17 is not indicated to be higher than powerClass, UE output power in a band cannot exceed powerClass.

The non-CA and CA configurations for which minimum requirements have been evaluated for Power Class 2 or Power Class 1.5 operation are indicated in clause 5.5A.

Proposal 4: For higher limit of configured maximum output power PCMAX_H,f,c, UE shall ignore parameters PPowerClass,CA and ΔPPowerClass,CA for non-CA transmissions in case ue-PowerClassPerBandPerBC-r17 indicates a higher power class than powerClass

This provides a good compromise on enabling different UE implementations, as has been argued for multiple meetings and seen in proposals to limit transmission power to min(powerClass, ue-PowerClass). At the same time, this gives the UE an allowance to realize the full power of ue-PowerClass and therefore opportunity to improve system performance.

One consequence of this approach is that powerclass on which Pcmax,f,c is based on changes based on scheduling and even if output power of a carrier stays constant, power headroom will be different depending on transmissions on the other band. One possibility to resolve this is to separate Pcmax from power headroom reporting, and have the upper limit PCMAX_H,f,c to be always based on powerclass of the band. This means that the upper limit stays constant independent of transmission on the other band.

Proposal 5: If necessary, for power headroom reporting purposes PCMAX_H,f,c can be always based on powerclass of the band.

The required specification updates are provided in companion CR in [2]

## 3	Conclusion

In this contribution power class indication and RRC parameters were discussed and following proposals were made for interband CA.

Proposal 1: Explicitly allow UE to indicate higher power class than powerClass in ue-PowerClassPerBandPerBC-r17 when single band requirements are specified for said higher power class. UE shall meet ue-PowerClassPerBandPerBC-r17 for a band for non-CA transmissions when ue-PowerClassPerBandPerBC-r17 indicates higher power class than powerClass. If ue-PowerClassPerBandPerBC-r17 is not indicated to be higher than powerClass, UE output power in a band cannot exceed powerClass.

Proposal 2: A UE indicating ue-PowerClassPerBandPerBC-r17 higher than powerClass for a band shall meet CA refsens and MSD requirements with the uplink power limited to the nominal maximum output power for the default power class, unless higher than default power class requirements are explicitly specified for the supported power class

Proposal 3: Adopt the new general clause to TS 38.101-1 as shown below

6.2A.0	General

Non-CA UL transmissions are not bounded by the power class for the band combination indicated by powerClass and powerClass may be ignored in setting the configured maximum output power for CA with single CC UL transmission when ue-PowerClassPerBandPerBC-r17 indicates higher power class than PowerClass. In such scenario UE shall meet power class for a band indicated in ue-PowerClassPerBandPerBC-r17. If ue-PowerClassPerBandPerBC-r17 is not indicated to be higher than powerClass, UE output power in a band cannot exceed powerClass.

The non-CA and CA configurations for which minimum requirements have been evaluated for Power Class 2 or Power Class 1.5 operation are indicated in clause 5.5A.

Proposal 4: For higher limit of configured maximum output power PCMAX_H,f,c, UE shall ignore parameters PPowerClass,CA and ΔPPowerClass,CA for non-CA transmissions in case ue-PowerClassPerBandPerBC-r17 indicates a higher power class than powerClass

Proposal 5: If necessary, for power headroom reporting purposes PCMAX_H,f,c can be always based on powerclass of the band.

## 4	References

[1]  R4-2403891, WF on NR UE power classes, Huawei, HiSilicon

[2]	R4-2405711, [NR_PC2_CA_R17_2BDL_2BUL-Core] draft CR to TS 38.101-1: power class corrections, Qualcomm Inc.