| 3GPP TR 38.718-01-01 V0.9.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	TR maintenance	8

5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part	8

5.1	CA_2DL_n102B/C_2UL_n102B/C	8

5.1.1	Channel bandwidths per operating band for CA	8

5.1.2	UE maximum output power for Intra-band contiguous CA	8

5.1.3	UE additional maximum output power reduction for CA	8

5.1.4	Spurious emissions for UE co-existence for intra-band contiguous CA	9

5.1.5	Reference sensitivity power level for Intra-band contiguous CA	9

5.1.6	In-band blocking	9

5.1.7	Out-of-band blocking	9

5.1.8	Narrow band blocking	9

6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part	9

6.0	TEMPLATE - CA_xDL_a_yUL_b	9

6.0.1	Channel bandwidths per operating band for CA	9

6.0.2	Co-existence studies	9

6.0.2.1	Co-existence studies for FDD with 2 CC Uplink in Intra-Band CA	10

6.0.3	REFSENS	10

6.1	CA_3DL_n78(A-C)_1UL_n78A	11

6.1.1	Channel bandwidths per operating band for CA	11

6.1.2	Co-existence studies	12

6.1.3	REFSENS	12

6.2	CA_2DL_n26(2A)_1UL_n26A	12

6.2.1	Channel bandwidths per operating band for CA	12

6.2.2	Co-existence studies	13

6.2.3	REFSENS	13

7	Intra-Band Contiguous Carrier Aggregation FR2: Specific Band Combination Part	13

7.1	CA_xDL_a_yUL_b	13

7.1.1	Channel bandwidths per operating band for CA	13

7.1.2	UE co-existence studies	13

8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part	13

8.1	CA_xDL_a-a_yUL_b-b	13

8.1.1	Channel bandwidths per operating band for CA	13

8.1.2	UE co-existence studies	13

Annex A - Change history	14

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for NR Intra-band Carrier Aggregation Rel-18 for xDL/yUL including contiguous and non-contiguous spectrum under Rel-18 time frame. The purpose is to gather the relevant background information and studies to address NR Intra-band Carrier Aggregation requirements for the Rel-18 band combinations in Table 1-1, Table 1-2, Table 1-3 and Table 1-4.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-222105, “Revised WID: Rel-18 NR intra band Carrier Aggregation for xCC DL/yCC UL including contiguous and non-contiguous spectrum (x>=y)”, RAN#97-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for NR Intra-band Carrier Aggregation under Rel-18 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	Intra-Band Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 5.1	CA_2DL_n102B/C_2UL_n102B/C

### 5.1.1	Channel bandwidths per operating band for CA

Table 5.1.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n102 | n102 |

Table 5.1.1-2: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n102B | CA_n102B | 20, 40 | 20, 40, 60, 80 |  |  |  | 100 | 1 |
| CA_n102C | CA_n102C | 60, 80 | 40, 60, 80 |  |  |  | 160 | 1 |

### 5.1.2	UE maximum output power for Intra-band contiguous CA

Table 5.1.2.1-1: UE Power Class for intra-band contiguous CA

| NR CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n102B |  |  |  |  |  |  | 20 | +2/-3 |
| CA_n102C |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 5 is default power class unless otherwise stated. |  |  |  |  |  |  |  |  |

### 5.1.3	UE additional maximum output power reduction for CA

As commented at RAN#106 this type of combinations needs additional study if any of the defined MPR and/or A-MPR needs to be updated to take into account the additional channel bandwidth in UL. For this specific combination NS_58 needs to be checked.

Based on additional studies presented at RAN4#109 it is found that no additional A-MPR is needed for these configurations.

### 5.1.4	Spurious emissions for UE co-existence for intra-band contiguous CA

Spurious emissions requirements for UE coexistence are not applicable to bands restricted to stand-alone operation with shared spectrum channel access as identified in Table 5.2-1.

### 5.1.5	Reference sensitivity power level for Intra-band contiguous CA

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### 5.1.6	In-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### 5.1.7	Out-of-band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

### 5.1.8	Narrow band blocking

This update adds intra-band contiguous CA for the uplink, so no additional impact on receiver requirements.

# 6	Intra-Band Non-Contiguous Carrier Aggregation FR1: Specific Band Combination Part

## 6.0	TEMPLATE - CA_xDL_a_yUL_b

### 6.0.1	Channel bandwidths per operating band for CA

Table 6.0.1-1: Supported bandwidth combinations for CA_xDL_a_yUL_b

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| - | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.0.2	Co-existence studies

<Text will be added.>

#### 6.0.2.1	Co-existence studies for FDD with 2 CC Uplink in Intra-Band CA

Editor's note: This study is only needed for FDD operation with 2 CC uplink transmission in Intra-Band Carrier Aggregation.

Table 6.0.2.1-1: Co-existence studies for FDD with 2 CC Uplink in Intra-Band CA

| Configuration | Channel BW | Minimum channel separation | Maximum channel separation | Minimum frequency | Maximum frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| Data |  |  |  |  |  | - |
| CC location | fU1L | fU2L | fU3L | fU1H | fU2H | fU3H |
| Frequency |  |  |  |  |  |  |
| 2nd | I fU1L-fU2L I | I fU1L-fU3L I | fU1L + fU2L | fU1H+fU2H |  |  |
| Interference ranges |  |  |  |  |  |  |
| 3rd | 2*fU1L-fU3L | 2*fU1H-fU3H | 2*fU1L + fU2L | 2*fU1H + fU2H |  |  |
| Interference ranges |  |  |  |  |  |  |
| 4th | I 2*fU1L - 2*fU2L I | I 2*fU1H - 2*fU3H I | 3*fU1L - fU3L | 3*fU1H - fU3H | 3*fU1L + fU2L | 3*fU1H + fU2H |
| Interference ranges |  |  |  |  |  |  |
| 5th | I 3*fU1L-2*fU3L I | I 3*fU1H-2*fU3H I | 4*fU1L-fU3L | 4*fU1H-fU3H | 4*fU1L+fU2L | 4*fU1H+fU2H |
| Interference ranges |  |  |  |  |  |  |
| 6th | I 3*fU1L-3*fU2L I | I 3*fU1H-3*fU3H I | 4*fU1L-2*fU3L | 4*fU1H-2*fU3H | 5*fU1L-fU3L | 5*fUH1-fU3H |
| Interference ranges |  |  |  |  |  |  |
| 7th | I 4*fU1L-3*fU3L I | I 4*fU1H-3*fU3H I | 5*fU1L-2*fU3L | 5*fU1H-2*fU3H | 6*fU1L-fU3L | 6*fU1H-fU3H |
| Interference ranges |  |  |  |  |  |  |

Editor's note (This guidance shall be omitted when drafting TPs):

In the table the following abbreviations is used:

fU1L = minimum frequency of TX aggressor band of ULCC1 lower band range

fU2L = minimum frequency of TX aggressor band of ULCC2 lower band range

fU3L = maximum frequency of TX aggressor band of ULCC2 lower band range

fU1H = maximum frequency of TX aggressor band of ULCC1 higher band range

fU2H = minimum frequency of TX aggressor band of ULCC2 higher band range

fU3H = maximum frequency of TX aggressor band of ULCC2 higher band range

fD1L = minimum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD1H = maximum frequency of RX victim band of DLCC placed on the lower frequency side of the TX aggressor band

fD2L = minimum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

fD2H = maximum frequency of RX victim band of DLCC placed on the higher frequency side of the TX aggressor band

Channel BW = Channel bandwidth of the component carrier. Only equal channel BW is considered

Minimum channel separation = Minimum frequency separation between the two component carriers or the inter CC GB

Maximum channel separation = Maximum frequency separation between the two component carriers or aggregated uplink BW

### 6.0.3	REFSENS

<Text will be added.>

## 6.1	CA_3DL_n78(A-C)_1UL_n78A

### 6.1.1	Channel bandwidths per operating band for CA

Table 6.1.1-1: Supported bandwidth combinations for CA_3DL_n78(A-C)_1UL _n78A

| NR CA configuration | Uplink CA configuration or single uplink carrier4 | NR Band | Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| CA_n78(A-C) | - | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 0 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | n78 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | 1 |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.1.2	Co-existence studies

### There are no additional co-existence issues for this combination.

### 6.1.3	REFSENS

There are no REFSENS exceptions for this combination.

## 6.2	CA_2DL_n26(2A)_1UL_n26A

### 6.2.1	Channel bandwidths per operating band for CA

Table 6.2.1-1: Supported bandwidth combinations for CA_n26(2A)

| NR CA Configuration | Uplink CA Configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum Aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | - | 5, 10, 15 | 5, 10, 15 |  |  | 30 | 0 |

### 6.2.2	Co-existence studies

### There are no additional co-existence issues for this combination.

### 6.2.3	REFSENS

### The MSD value used is based on an average of the RAN4 #106 inputs from Ericsson, Murata and Skyworks.

Table 6.2.3-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation (LCRB) | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n26(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 74) | 25.2 | FDD |

# 7	Intra-Band Contiguous Carrier Aggregation FR2: Specific Band Combination Part

## 7.1	CA_xDL_a_yUL_b

### 7.1.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 7.1.2	UE co-existence studies

<Text will be added.>

# 8	Intra-Band Non-Contiguous Carrier Aggregation FR2: Specific Band Combination Part

## 8.1	CA_xDL_a-a_yUL_b-b

### 8.1.1	Channel bandwidths per operating band for CA

<Text will be added.>

### 8.1.2	UE co-existence studies

<Text will be added.>

# Annex A - Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-10 | 3GPP RAN4#104-bis-e | R4-2216085 |  |  |  | TR skeleton | 0.0.1 |
| 2022-11 | 3GPP RAN4#105 | R4-2218947 |  |  |  | Implemented TP’s from RAN4 #105: R4-2218520, TP for TR 38.718-01-01: CA_n78(A-C), SK Telecom | 0.1.0 |
| 2023-03 | 3GPP RAN4#106 | R4-2301059 |  |  |  | Implemented TP’s from RAN4 #106: R4-2303574, TP for 38.718-01-01 to include CA_n26(2A), Ericsson | 0.2.0 |
| 2023-04 | 3GPP RAN4#106bis-e | R4-2304857 |  |  |  | No TP’s approved RAN4#106bis-e | 0.3.0 |
| 2023-05 | 3GPP RAN4#107 | R4-2309385 |  |  |  | Implemented TP’s from RAN4 #107: R4-2310348, TP to TR 38.718.01-01 Addition of UE co-existence studies for FDD with 2 Uplink in Intra-Band CA, Nokia | 0.4.0 |
| 2023-08 | 3GPP RAN4#108 | R4-2311104 |  |  |  | No TP’s approved RAN4#108 | 0.5.0 |
| 2023-10 | 3GPP RAN4#108-bis | R4-2316413 |  |  |  | No TP’s approved RAN4#108-bis | 0.6.0 |
| 2023-11 | 3GPP RAN4#109 | R4-2320303 |  |  |  | No TP’s approved RAN4#109 | 0.7.0 |
| 2024-02 | 3GPP RAN4#110 | R4-2401469 |  |  |  | Implemented TP’s from RAN4 #110: R4-2402088, TP to TR 38.718.01-01 addition of CA_n102B and CA_n102C uplink | 0.8.0 |
| 2024-04 | 3GPP RAN4#110-bis | R4-2405762 |  |  |  | No TP’s approved RAN4#110-bis | 0.9.0 |