3GPP TSG-WG4 Meeting #110bisbis 	R4-2405799

# Changsha, China, April 15 - April 19

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR interruption requirements for measurements without gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DRX interruption behavior needs to be defined for UEs supporting Rel 18 need for interruption. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Definition of interruption requirements when DRX is configured |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Gapless measurements for UEs that need interruption cannot be use in combination with DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.19, 9.2.1, 9.2.5, 9.3.1, 9.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

##### 8.2.2.2.19	Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18]

When a UE supports [NeedForInterruptionInfoNR-R18] measurements and indicates [no-gap-with-interruption] on intra-frequency SSB-based or inter-frequency SSB-based measurements, the UE is allowed to cause interruptions while performing measurements on the frequency layers of the bands for which [no-gap-with-interruption] is indicated. Requirements in this section apply only when the UE is in SA operation mode.

The UE is allowed to cause interruption with interruption ratio no more than the requirements specified below upon UE measurements on a specific frequency layer that corresponds to the configured MO, where Tcycle,i is the interruption cycle on a certain frequency layer i, specified in Table 8.2.2.2.19-1, where CSSFoutside_gap,i is defined in clause 9.1.5.1 for measurement conducted outside measurement gaps.

Table 8.2.2.2.19-1: Tcycle,i length for inter/intra-frequency measurement target carrier i

| DRX cycle | TCycle,i |
| --- | --- |
| No DRX | max (80ms, SMTC period) x CSSFoutside_gap,i |
| DRX cycle ≤ 320ms | [max (80ms, DRX cycle, SMTC period) x CSSFoutside_gap,i] |
| DRX cycle>320ms | [DRX cycle x CSSFoutside_gap,] |

2LTcycle,i

When DRX is not configured,, the total allowed maximum interruption ratio (D) on each of the active serving cells due to UE measurements without gap applied in this sub-clause is specified as

D= i=1N2LTcycle,i

Where,

-	N is the total number of configured SSB based frequency layers to be measured outside gap including intra-frequency and inter-frequency target carriers where UE indicates that interruption is needed through [no-gap-with-interruption], and

-	L is the maximum interruption length for each interruption occasion specified in the Table 8.2.2.2.19-2 and 8.2.2.2.19-3.

The interruptions are allowed for all the active serving cells in the same FR as all NR MOs being measured with interruption if UE supports per-FR measurement gaps, and all the active serving cells if UE does not support per-FR measurement gaps.

When DRX is configured, the UE is allowed to cause an average of 2 interruptions of length L due to the measurement on target carrier i per TCycle,i, provided that the interruption follows the conditions:

- If the onDurationTimer is larger than [10] ms,

- Interruptions are allowed, except for the first and last slot containing PDCCH in the DRX ON duration

- Otherwise,

-if the minimum DRX cycle is smaller than [80] ms:

- Interruptions are not allowed in the DRX ON duration.

- Otherwise, if the minimum DRX cycle is larger than or equal to [80] ms:

- Interruptions are not allowed in the DRX ON duration.

where DRX cycle is the DRX short cycle, if configured, else the DRX long cycle.

Table 8.2.2.2.19-2: Interruption length L in FR1

|  | SCS (kHz) of victim cell | NR Slot length (ms) of victim cell | Number of interrupted slots in the victim cell (slots) | Interruption length L (ms) |
| --- | --- | --- | --- | --- |
| 0 | 15 | 1 | [1] | [1] |
| 1 | 30 | 0.5 | [2] | [1] |
| 2 | 60 | 0.25 | [4] | [1] |

Table 8.2.2.2.19-3: Interruption length L in FR2

|  | SCS (kHz) of victim cell | NR Slot length (ms) of victim cell | Number of interrupted slots in the victim cell (slots) | Interruption length L (ms) |
| --- | --- | --- | --- | --- |
| 2 | 60 | 0.25 | [3] | [0.75] |
| 3 | 120 | 0.125 | [6] | [0.75] |

## <End of Change #1>