3GPP TSG-RAN WG4 Meeting #110bis	R4-2405808

Changsha, China, April 15 – April 19, 2024

Source:	Nokia

Title:	TDCP RRM Requirements

Agenda item:	6.19.3.1

Document for:	Discussion

Introduction

From the previous RAN4# 110 meeting the following issues were raised regarding TDCP (Time Domain Channel Properties) RRM requirements and below is Nokia’s view.

TDCP RRM requirements discussion

In the last RAN4 meeting the following options we discussed regarding TDCP RRM requirements

Issue 3-1-3: Design for the test case of TDCP < Agreement>: - Test metric: - Step 1: Determine the configuration (e.g., SNR, doppler spread) for the test. - Step 2: for the configuration agreed in step 1, determine the TDCP accuracy range (i.e., TDCP accuracy value X1 to X2) from the simulation results from different companies. - Step 3: Define the test case for the configurations agreed in step 1 and the TDCP value UE should report to pass the test is determined from the range of values determined in step 2 (i.e., X1 to X2). [For the repeated tests UE should pass the test at least [Y2] % of the times. Y2 can be FFS.] - The text in [] is the baseline, and other proposal is not precluded. - Test Parameters: - Channel model: fading channel - As baseline, TDL-A30 - Pick one value from SNR = 10dB, 15dB, 20dB - The distance between two TRSs: 1 slot as baseline, further discuss on 10 slots

From our previous meeting results we saw that the case with 10 slots distance between two TRSs showed better differentiation in TDCP reports with low and high Doppler in comparison to the case of a lag of 1 slot. Therefore, we strongly recommend to include the test case with 10 slots distance between two TRSs.

10 slots lag between two TRSs showed better differentiation in TDCP reports with low and high Doppler in comparison to the case of a lag of 1 slot between two TRSs.

Include test case with 10 slots distance between two TRSs.

To harmonize and get fair values of X1 and X2, we can consider results of all companies and define these values as suggested in our proposals below. The X1 value can be defined as quantized minimum value among the 5th percentile among all companies -1 results. While, the X2 value is calculated as quantized maximum value among the 95th percentile among all companies +1 results. In table 1 below, we show the X1 and X2 values obtained using only Nokia’s results (attached in Appendix A) for the 1 slot, 1 sample case and the 10 slots, 4 samples case.

Table 1: X1 and X2 values using 5th and 95th percentiles.

| 1 slot, 1 sample, 5th and 95th percentile |  |  |  |  |  | 10 slot, 4 samples, 5th and 95th percentile |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Doppler | SNR | Quantized |  | Distance between X2 and X1 |  | Doppler | SNR | Quantized |  | Distance between X2 and X1 |
|  |  | X2 | X1 |  |  |  |  | X2 | X1 |  |
| 10 | 5 | 8 | 15 | 7 |  | 10 | 5 | 9 | 15 | 6 |
|  | 10 | 5 | 14 | 9 |  |  | 10 | 6 | 14 | 8 |
|  | 15 | 2 | 11 | 9 |  |  | 15 | 2 | 12 | 10 |
|  | 20 | 0 | 8 | 8 |  |  | 20 | 0 | 9 | 9 |
| 30 | 5 | 8 | 15 | 7 |  | 30 | 5 | 10 | 15 | 5 |
|  | 10 | 5 | 14 | 9 |  |  | 10 | 7 | 14 | 7 |
|  | 15 | 2 | 11 | 9 |  |  | 15 | 5 | 12 | 7 |
|  | 20 | 0 | 9 | 9 |  |  | 20 | 4 | 12 | 8 |
| 75 | 5 | 8 | 15 | 7 |  | 75 | 5 | 12 | 15 | 3 |
|  | 10 | 5 | 14 | 9 |  |  | 10 | 10 | 15 | 5 |
|  | 15 | 2 | 12 | 10 |  |  | 15 | 10 | 15 | 5 |
|  | 20 | 0 | 9 | 9 |  |  | 20 | 9 | 15 | 6 |
| 100 | 5 | 8 | 15 | 7 |  | 100 | 5 | 12 | 15 | 3 |
|  | 10 | 5 | 14 | 9 |  |  | 10 | 11 | 15 | 4 |
|  | 15 | 2 | 12 | 10 |  |  | 15 | 10 | 15 | 5 |
|  | 20 | 0 | 9 | 9 |  |  | 20 | 10 | 15 | 5 |
| 200 | 5 | 8 | 15 | 7 |  | 200 | 5 | 13 | 15 | 2 |
|  | 10 | 5 | 14 | 9 |  |  | 10 | 12 | 15 | 3 |
|  | 15 | 2 | 13 | 11 |  |  | 15 | 11 | 15 | 4 |
|  | 20 | 0 | 11 | 11 |  |  | 20 | 11 | 15 | 4 |
| 300 | 5 | 8 | 15 | 7 |  | 300 | 5 | 13 | 15 | 2 |
|  | 10 | 6 | 15 | 9 |  |  | 10 | 12 | 15 | 3 |
|  | 15 | 3 | 13 | 10 |  |  | 15 | 12 | 15 | 3 |
|  | 20 | 1 | 13 | 12 |  |  | 20 | 12 | 15 | 3 |

X1 is defined as quantized minimum value among the 5th percentile among all companies -1

X2 is calculated as quantized maximum value among the 95th percentile among all companies +1

Y2 is 10% of the samples.

A test is considered feasible, if X2-X1< 8 quantization levels

Issue 3-1-4: TDCP Measurement Report Mapping – amplitude < Agreement>: Define TDCP mapping table of amplitude in section 10 of RRM spec. - Option 1: ki; TDCP Range. 0; 0.9945< TDCP <=1. 1; 0.9922< TDCP <=0.9945. 2; 0.9890< TDCP <=0.9922. 3; 0.9844< TDCP <=0.9890. …; …. 12; 0.6464< TDCP <=0.75. 13; 0.5< TDCP <=0.6464. 14; 0.2929< TDCP <=0.5. 15; 0≤ TDCP <=0.2929. - Option 2: - use the RAN1 points as the middle (but not the centre due to non-uniform) of each range as Estimated TDCP value; Report index. 0.9953≤Estimated TDCP≤1; 0. 0.99335<Estimated TDCP<0.9953; 1. …; …. 0≤Estimated TDCP<0.39645; 15. - Other options are not precluded Issue 3-1-5: TDCP Measurement Report Mapping - phase < Agreement>: Define TDCP mapping table of phase in section 10 of RRM spec. - Option 1: - For mapping table of TDCP phase reporting, the 16 points divided [0 2pi] to 16 segments. the mapping table can use ej2πci16 as the boundary of each range. - Option 2: - the RAN1 points are in the middle of each range - Other options are not precluded

It was proposed in RAN4 to define measurement reporting table which uses specified by RAN1 normalized amplitude quantization levels as decision (quantization) boundaries. The shortcoming of this approach is that either highest or lowest level will be ignored as to distinguish 16 reporting levels, there should be 15 boundaries. Without the highest level 1 - 2-16 - 02≈0.996 being used in defining the quantization boundaries, the values that are arbitrarily close to level 1 - 2-16 - 12≈0.994  from right will be reported as the highest possible level.

As two alternatives we are proposing to define 15 decision (quantization) boundaries in between 16 quantization levels defined by quantization alphabet, which is a standard way of quantization.

a) Boundaries can be defined as average values between each neighboring pair of quantization alphabet values:

b1=1 - 2-16 - 02+1 - 2-16 - 122;

b2=1 - 2-16 - 12+1 - 2-16 - 222;

b3=1 - 2-16 - 22+1 - 2-16 - 322;

...

b14=1 - 2-16 - 132+1 - 2-16 - 1422;

b15=1 - 2-16 - 142+1 - 2-16 - 1522;

b) Boundaries can be defined as logarithmic averages between each neighboring pair of quantization alphabet values, utilizing the same principle that was used to create the quantization alphabet:

b(q) = 1-2-33-2*q4, where q=1...15;

The decision rules can be defined as follows:

q=0, if Al> b1

q=1, if b(2) < |A(l)| < b1

q=2, if b3 < |A(l)| < b2

...

q=13, if b(14) < |A(l)| < b(13)

q=14, if b(15) < |A(l)| < b(14)

q=15, if |A(l)| < b(15)

The normalized amplitude quantization levels as decision (quantization) boundaries approach has the shortcoming that either the highest or lowest level will be ignored since there should be 15 boundaries to distinguish 16 reporting levels.

Two alternatives we are proposing to define 15 decision (quantization) boundaries in between 16 quantization levels defined by quantization alphabet: (a) Boundaries can be defined as average values between each neighboring pair of quantization alphabet values. (b) Boundaries can be defined as logarithmic averages between each neighboring pair of quantization alphabet values

The quantization of TDCP magnitude can be done either as average values or logarithmic averages between neighboring quantization alphabet values.

In issue 3-1-4 agree on Option 2.

For phase average between neighboring levels is also the best option for TDCP phase boundaries:

Levels: ej2πq16, q=0,…,15  - blue lines

Boundaries: bc=ej2π(132+c16), c=0,…,15  - red dashed lines

The decision rules can be defined as follows:

q=0, if ∠b(15) <  ∠A(l) <  ∠b(0)

q=1, if ∠b(0) <  ∠A(l) <  ∠b(1)

q=2, if ∠b(2) <  ∠A(l) <  ∠b(2)

...

q=13, if ∠b(12) <  ∠A(l) <  ∠b(13)

q=14, if ∠b(13) <  ∠A(l) <  ∠b(14)

q=15, if ∠b(14) <  ∠A(l) <  ∠b(15)

Define TDCP phase boundaries by using phase average between neighboring levels.

In issue 3-1-5 agree on Option 2.

Conclusion

Hereafter is a summary of observations and proposals for TDCP RRM requirements discussion:

Observation 1: 10 slots lag between two TRSs showed better differentiation in TDCP reports with low and high Doppler in comparison to the case of a lag of 1 slot between two TRSs.

Proposal 1: Include test case with 10 slots distance between two TRSs.

Proposal 2: X1 is defined as quantized minimum value among the 5th percentile among all companies -1

Proposal 3: X2 is calculated as quantized maximum value among the 95th percentile among all companies +1

Proposal 4: Y2 is 10% of the samples.

Proposal 5: A test is considered feasible, if X2-X1< 8 quantization levels

Observation 2: The normalized amplitude quantization levels as decision (quantization) boundaries approach has the shortcoming that either the highest or lowest level will be ignored since there should be 15 boundaries to distinguish 16 reporting levels.

Observation 3: The quantization of TDCP magnitude can be done either as average values or logarithmic averages between neighboring quantization alphabet values.

Proposal 6: In issue 3-1-4 agree on Option 2.

Observation 4: Define TDCP phase boundaries by using phase average between neighboring levels.

Proposal 7: In issue 3-1-5 agree on Option 2.

- References

- R4-2402017, Discussion on Feasibility study of RRM TDCP accuracy requirements , Nokia, Nokia Shanghai Bell, RAN4#109 Chicago, USA, November 13 – November 17, 2023.

- R4-2402925, Discussion on Feasibility study of RRM TDCP accuracy requirements , Nokia, Nokia Shanghai Bell, RAN4#110 Athens, Greece, February 19 – March 01, 2024.

- Appendix A: Simulation results

Table : Simulation parameters

| Parameter | Value |
| --- | --- |
| Delay (between TRS symbols) | 1 slot |
| Channel model | TDL-A, delay spread=30ns |
| Doppler Spread | 10, 30, 75, 100, 200, 300 |
| SNR | 5:5:20 |
| Number of averaging samples: | one shot |
| Channel BW | 10MHz |
| SCS | 30KHz |
| Reference Channel estimation | LS base CE for TRS as baseline |
| Correlation matrix and antenna configuration | 1x2 Low |

Table 3: Simulation results for 1 slot delay

| Doppler | SNR | 5% |  |  | 10% |  |  | 90% |  |  |  | 95% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 slot |  |  | 1 slot |  |  | 1 slot |  |  |  | 1 slot |  |  |
|  |  | 1 sample | 2 samples | 4 samples | 1 sample | 2 samples | 4 samples | 1 sample | 2 samples | 4 samples | 1 sample |  | 2 samples | 4 samples |
| 10 | 5 | 0.3588 | 0.3594 | 0.3581 | 0.4371 | 0.4381 | 0.4318 | 0.8794 | 0.8743 | 0.8709 | 0.9035 |  | 0.9 | 0.896 |
|  | 10 | 0.6426 | 0.6429 | 0.6397 | 0.7112 | 0.712 | 0.7071 | 0.9578 | 0.9564 | 0.955 | 0.967 |  | 0.9659 | 0.9646 |
|  | 15 | 0.8513 | 0.8513 | 0.8502 | 0.886 | 0.8856 | 0.8855 | 0.9862 | 0.9857 | 0.9853 | 0.9893 |  | 0.989 | 0.9885 |
|  | 20 | 0.9477 | 0.9476 | 0.9473 | 0.9609 | 0.9607 | 0.9607 | 0.9956 | 0.9954 | 0.9953 | 0.9966 |  | 0.9965 | 0.9963 |
| 30 | 5 | 0.3583 | 0.3592 | 0.3624 | 0.4367 | 0.4356 | 0.4332 | 0.8793 | 0.874 | 0.8704 | 0.9029 |  | 0.8997 | 0.895 |
|  | 10 | 0.6424 | 0.6409 | 0.6453 | 0.7107 | 0.7088 | 0.7092 | 0.9578 | 0.9562 | 0.955 | 0.9668 |  | 0.9657 | 0.9642 |
|  | 15 | 0.8498 | 0.8493 | 0.8507 | 0.8854 | 0.8846 | 0.8846 | 0.9861 | 0.9856 | 0.9852 | 0.9892 |  | 0.9888 | 0.9884 |
|  | 20 | 0.9462 | 0.9459 | 0.9461 | 0.9598 | 0.9597 | 0.9595 | 0.9955 | 0.9954 | 0.9952 | 0.9965 |  | 0.9964 | 0.9963 |
| 75 | 5 | 0.355 | 0.3598 | 0.3774 | 0.4346 | 0.4353 | 0.4434 | 0.8788 | 0.8733 | 0.8668 | 0.9024 |  | 0.8981 | 0.8914 |
|  | 10 | 0.6348 | 0.6375 | 0.6496 | 0.7066 | 0.7058 | 0.711 | 0.9574 | 0.9554 | 0.9529 | 0.9665 |  | 0.9649 | 0.9626 |
|  | 15 | 0.8406 | 0.8423 | 0.8475 | 0.8793 | 0.879 | 0.8794 | 0.9859 | 0.9852 | 0.9844 | 0.989 |  | 0.9884 | 0.9876 |
|  | 20 | 0.9369 | 0.9372 | 0.9377 | 0.9535 | 0.953 | 0.9525 | 0.9952 | 0.995 | 0.9947 | 0.9963 |  | 0.9961 | 0.9958 |
| 100 | 5 | 0.3531 | 0.3603 | 0.3894 | 0.4333 | 0.4371 | 0.452 | 0.8783 | 0.8714 | 0.8626 | 0.9021 |  | 0.8969 | 0.8881 |
|  | 10 | 0.6291 | 0.6343 | 0.6558 | 0.7019 | 0.7022 | 0.7114 | 0.9569 | 0.9549 | 0.9511 | 0.9663 |  | 0.9642 | 0.961 |
|  | 15 | 0.8333 | 0.8352 | 0.8445 | 0.8732 | 0.8743 | 0.8756 | 0.9855 | 0.9848 | 0.9835 | 0.9888 |  | 0.988 | 0.987 |
|  | 20 | 0.9271 | 0.928 | 0.9303 | 0.9474 | 0.9469 | 0.9467 | 0.995 | 0.9947 | 0.9941 | 0.9961 |  | 0.9958 | 0.9954 |
| 200 | 5 | 0.3374 | 0.3629 | 0.4203 | 0.4157 | 0.4334 | 0.4768 | 0.8746 | 0.8641 | 0.8337 | 0.9006 |  | 0.8889 | 0.8644 |
|  | 10 | 0.5941 | 0.6145 | 0.6664 | 0.6742 | 0.6802 | 0.7125 | 0.9544 | 0.9499 | 0.9351 | 0.9648 |  | 0.9601 | 0.949 |
|  | 15 | 0.7805 | 0.7866 | 0.8263 | 0.8382 | 0.8365 | 0.8525 | 0.9835 | 0.9814 | 0.9745 | 0.9872 |  | 0.9855 | 0.9801 |
|  | 20 | 0.8659 | 0.8633 | 0.8941 | 0.9095 | 0.9018 | 0.9112 | 0.9933 | 0.9922 | 0.9882 | 0.9949 |  | 0.994 | 0.9911 |
| 300 | 5 | 0.305 | 0.3604 | 0.4292 | 0.3931 | 0.4271 | 0.4867 | 0.8702 | 0.8488 | 0.7939 | 0.8967 |  | 0.8779 | 0.8264 |
|  | 10 | 0.5386 | 0.5808 | 0.6552 | 0.6303 | 0.6471 | 0.6977 | 0.9507 | 0.9406 | 0.9081 | 0.9617 |  | 0.9533 | 0.925 |
|  | 15 | 0.6983 | 0.7238 | 0.7868 | 0.781 | 0.7826 | 0.8156 | 0.9803 | 0.9747 | 0.9556 | 0.9851 |  | 0.981 | 0.9645 |
|  | 20 | 0.7684 | 0.7898 | 0.8384 | 0.8461 | 0.8371 | 0.8653 | 0.9908 | 0.9871 | 0.9731 | 0.9931 |  | 0.9905 | 0.9795 |

Table 4: Simulation results for 10 slots delay

| Doppler | SNR | 5% |  |  | 10% |  |  | 90% |  |  | 95% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 slots |  |  | 10 slots |  |  | 10 slots |  |  | 10 slots |  |  |
|  |  | 1 sample | 2 samples | 4 samples | 1 sample | 2 samples | 4 samples | 1 sample | 2 samples | 4 samples | 1 sample | 2 samples | 4 samples |
| 10 | 5 | 0.3529 | 0.3608 | 0.3891 | 0.4326 | 0.4373 | 0.4518 | 0.8782 | 0.8716 | 0.8627 | 0.9021 | 0.897 | 0.8879 |
|  | 10 | 0.6296 | 0.6348 | 0.6558 | 0.702 | 0.7026 | 0.7115 | 0.957 | 0.9549 | 0.9512 | 0.9663 | 0.9643 | 0.961 |
|  | 15 | 0.8334 | 0.8355 | 0.8445 | 0.8731 | 0.8742 | 0.8757 | 0.9855 | 0.9848 | 0.9834 | 0.9888 | 0.988 | 0.987 |
|  | 20 | 0.9271 | 0.9279 | 0.9304 | 0.9473 | 0.9467 | 0.9465 | 0.9949 | 0.9947 | 0.9941 | 0.9961 | 0.9958 | 0.9954 |
| 30 | 5 | 0.3054 | 0.3606 | 0.4295 | 0.3919 | 0.4274 | 0.4869 | 0.8701 | 0.8489 | 0.7934 | 0.8967 | 0.8779 | 0.8264 |
|  | 10 | 0.5394 | 0.5811 | 0.6553 | 0.6294 | 0.647 | 0.6975 | 0.9508 | 0.9408 | 0.9079 | 0.9617 | 0.9533 | 0.925 |
|  | 15 | 0.6979 | 0.7239 | 0.7873 | 0.782 | 0.7824 | 0.8155 | 0.9804 | 0.9747 | 0.9557 | 0.9851 | 0.981 | 0.9644 |
|  | 20 | 0.7674 | 0.7902 | 0.8375 | 0.8461 | 0.8372 | 0.8647 | 0.9908 | 0.9871 | 0.9732 | 0.9931 | 0.9905 | 0.9795 |
| 75 | 5 | 0.1847 | 0.2281 | 0.2916 | 0.2645 | 0.2885 | 0.3375 | 0.827 | 0.7199 | 0.6322 | 0.8636 | 0.7646 | 0.67 |
|  | 10 | 0.2812 | 0.2967 | 0.3784 | 0.3947 | 0.3753 | 0.4324 | 0.9185 | 0.8381 | 0.7539 | 0.9384 | 0.8705 | 0.7872 |
|  | 15 | 0.3433 | 0.3233 | 0.4095 | 0.4725 | 0.412 | 0.4715 | 0.9549 | 0.8916 | 0.8137 | 0.9673 | 0.918 | 0.8437 |
|  | 20 | 0.3707 | 0.3309 | 0.4191 | 0.5042 | 0.432 | 0.4858 | 0.9686 | 0.9122 | 0.8379 | 0.9782 | 0.9357 | 0.8664 |
| 100 | 5 | 0.1647 | 0.1324 | 0.1333 | 0.233 | 0.1909 | 0.1853 | 0.7925 | 0.6773 | 0.5707 | 0.8361 | 0.73 | 0.6169 |
|  | 10 | 0.2367 | 0.1704 | 0.1652 | 0.3361 | 0.2349 | 0.2269 | 0.8917 | 0.7979 | 0.6867 | 0.9181 | 0.8395 | 0.7348 |
|  | 15 | 0.2863 | 0.1882 | 0.1789 | 0.3927 | 0.2588 | 0.2455 | 0.9332 | 0.8545 | 0.7448 | 0.9517 | 0.8891 | 0.7932 |
|  | 20 | 0.307 | 0.1945 | 0.1826 | 0.4185 | 0.2689 | 0.2541 | 0.9484 | 0.8766 | 0.7672 | 0.9646 | 0.9081 | 0.8151 |
| 200 | 5 | 0.16 | 0.1023 | 0.0802 | 0.2274 | 0.1441 | 0.1146 | 0.7695 | 0.6258 | 0.4958 | 0.8158 | 0.6862 | 0.5487 |
|  | 10 | 0.2285 | 0.1223 | 0.1006 | 0.3162 | 0.1748 | 0.1436 | 0.8726 | 0.7381 | 0.5903 | 0.9031 | 0.7957 | 0.6543 |
|  | 15 | 0.271 | 0.1295 | 0.1102 | 0.3729 | 0.1906 | 0.1571 | 0.917 | 0.7918 | 0.6383 | 0.9394 | 0.8455 | 0.7048 |
|  | 20 | 0.2868 | 0.1327 | 0.1139 | 0.396 | 0.1978 | 0.1623 | 0.9335 | 0.8119 | 0.656 | 0.9533 | 0.8641 | 0.7206 |
| 300 | 5 | 0.157 | 0.1028 | 0.0803 | 0.232 | 0.1449 | 0.1145 | 0.7686 | 0.6113 | 0.4702 | 0.8109 | 0.6688 | 0.5229 |
|  | 10 | 0.2288 | 0.1201 | 0.1008 | 0.3206 | 0.1757 | 0.1426 | 0.8699 | 0.7244 | 0.5666 | 0.8992 | 0.7803 | 0.6237 |
|  | 15 | 0.2698 | 0.1292 | 0.1116 | 0.3727 | 0.1874 | 0.1554 | 0.9152 | 0.7788 | 0.6146 | 0.9377 | 0.831 | 0.6757 |
|  | 20 | 0.2845 | 0.1326 | 0.1147 | 0.3939 | 0.1938 | 0.1596 | 0.9317 | 0.801 | 0.6325 | 0.9518 | 0.8502 | 0.6937 |