3GPP TSG-RAN WG4 Meeting #110bis             	R4-2405865

Changsha, China, 15th-19th April, 2024

Agenda item:	6.8.7.2

Source:	Samsung

Title:	Discussion and simulation results for BS demodulation requirements for less than 5MHz

Document for:     Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup for performance part of NR support for dedicated spectrum less than 5MHz for FR1 was under discussion. The related agreement was captured in the WF [1] as following

- General - Manufacturer declaration for 3MHz CBW - Reuse existing declarations (D.14 “NR supported channel bandwidths and SCS in TS 38.141-1” and D.7 “BS channel band width and SCS support” in TS 38.141-2 in Table 4.6-1) for less than 5 MHz CBW. - PUSCH requirement - A need for PUSCH requirement with less than 5MHz CBW - Do not define PUSCH requirements with 3MHz bandwidth (both in HST and non-HST condition) - A need for UL timing adjustment requirement with UE speed up to 500 km/h - Do not define new 3MHz requirements on UL TA in HST conditions - PUCCH requirement - Introduction of PUCCH requirements - Only introduce new demodulation requirement for PUCCH format 2 with 3MHz CBW - Applicability rule - An Applicability rule shall be introduced into TS 38.141-1/2 to enable a base station declaring to support less than 5MHz to conduct a new UCI BLER performance test with only format 2 for PUCCH, and skip the corresponding legacy 5MHz PUCCH format 2 tests - PRACH - Applicability rule or note for long RACH sequence - For BS supporting less than 5MHz carrier bandwidth, only test requirements relating to RACH preamble formats with 15KHz SCS with sequence length LRA =139, and PRACH formats with 1.25KHz SCS with sequence length LRA=839 shall apply

In this contribution, the ideal and impairment results for PUCCH format 2 are provided for requirement derivation.

- Discussion

PUCCH applicability rule

In the last meeting, an applicability rule to enable a base station declaring to support less than 5MHz to conduct a new UCI BLER performance test with only format 2 for PUCCH was introduced. For less than 5MHz, both 12 PRB and 15 PRB can be supported for different bands, and the related performance are introduced.

From baseband processing perspective, there is no different for 12 PRB or 15 PRB. Therefore, it may be not necessary to test PUCCH with both PRB. To reduce the test effort, only one test case with 3MHz should be enough. Similar as the applicability of requirements of different channel bandwidths, we propose that the test shall be done only with 15PRB if BS declare to support 3MHz channel bandwidth with both 12PRB and 15PRB maximum number of RB.

Proposal 1: Introduce the following applicability rule to test PUCCH requirement with 3MHz

- Unless otherwise stated, PUCCH requirements with 3MHz shall apply only for a BS declaring support of 3MHz (see D.XX) in table xxx. If both 12PRB and 15 PRB are declared to be supported for 3MHz, the test should be done 3MHz with 15 PRB.

- Simulation results

For this part, the ideal and impairment simulation results for PUCCH format 2 based on the agreed simulation assumption in the last meeting as following table

Table 1:  Simulation assumption for PUCCH format 2

| Parameters | Value |
| --- | --- |
| Modulation order | QPSK |
| First RB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs -1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| Frist symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Test metric | BLER |

Table 2: ideal and impairment simulation results for PUCCH format 2

| Test case | Format | SCS&BW | Maximum RB | Intra-slot Frequency hopping | UCI bits | Channel | Number of RB | Number of Tx and Rx | Number of symbols | BLER@1% (ideal) | BLER@1% (impairment) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Case 1 | Format 2 | 15KHz, 3MHz | 12 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | 0.23 | 2.23 |
| Case 2 | Format 2 | 15KHz 3MHz | 15 | Enabled | 22 | TDLC300-100 | 9 | 1T2R | 2 | -0.38 | 1.62 |

- Conclusion

In this paper, the view on remaining issue of PUCCH with 3MHz is provided, meanwhile, the ideal and impairment simulation results are provided for requirement derivation.

Proposal 1: Introduce the following applicability rule to test PUCCH requirement with 3MHz

- Unless otherwise stated, PUCCH requirements with 3MHz shall apply only for a BS declaring support of 3MHz (see D.XX) in table xxx. If both 12PRB and 15 PRB are declared to be supported for 3MHz, the test should be done 3MHz with 15 PRB.

# References

R4-2402863, WF on NR_FR1_lessthan_5MHz_BW_demod