3GPP TSG-RAN WG4 Meeting # 110bis	R4-2406341

China, Changsha, Apr. 15 – Apr. 19, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on MAC-CE based dual DL TCI state switching delay for sDCI in Multi-RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-4-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update and correct some of the TCI state switching requirements for multi-Rx. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following draftCRs related to 8.10E.1, 8.10E.2, 8.10E.3 are merged into this draftCR: - R4-2404582 - R4-2404786 Change 1: in 8.10E.1, Add PSCell and SCell options as per agremeent in the RAN4#110 meeting and correcting some of the terminology. Change 2: In 8.10E.2, Update dual TCI state definination in known condityion for Multi-RX. Change 3: In 8.10E.3.1, Modify dual DL TCI state switching delay for sDCI for Multi-RX. The delay requirement is defined for each MAC CE separately. Change 4: In 8.10E.3.2, Add reference to TS 38.321 as agreed in RAN4#110 meeting for MAC CE based TCI state switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current spec is not clear and correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10E.1,8.10E.2, 8.10E.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1>

8.10E	Active TCI state switching delay for UE operating in FR2-1 and configured with groupBasedBeamReporting-r17

8.10E.1	Introduction

The requirements in this clause apply for a UE configured with groupBasedBeamReporting-r17 and dual TCI state configurations on PCell, PSCell or SCell in standalone NR, when the UE is configured with a single serving cell (PCell, PSCell, or SCell) in FR2-1. The requirements apply when the dual target TCI states are known and the UE can receive the dual target TCI states simultaneously, where the known conditions are defined in 8.10E.2. UE shall complete the switch of active TCI states within the delay defined in this clause.

<End of Change #1>

<Start of Change #2>

8.10E.2	Known conditions for TCI state

Dual TCI states will be indicated in one or two TCI state switch commands. The dual target TCI states are known if the following conditions are met:

-	Dual target TCI states are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report

-	The dual target TCI states and all the RSs in the two QCL chains remain detectable during the TCI state switching period

-	SNR of the TCI state ≥ -3dB

-	RS resource group configured for dual target TCI states is reported in the last [1280] ms

Editor’s note: FFS whether additional conditions are needed for tests.

<End of Change #2>

<Start of Change #3>

8.10E.3	MAC-CE based dual DL TCI state switch delay

8.10E.3.1	MAC-CE based dual DL TCI state switching delay for sDCI

The requirement in this clause applies when UE is provided with twoQCLTypeDforPDCCHRepetition.

When a MAC CE command for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7]  indicating TCI state switch with CORESET index p are received in slot n, and if the TCI state is known, UE shall be able to receive on the target TCI state at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*( Tfirst-SSBp, + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI states until slot n+ THARQ + 3Nslotsubframe,µ ,

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tfirst-SSBp is time to first SSB transmission (associated with TCI state of CORESET p) after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

-	TSSB-proc = 2 ms.

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

When UE receive TCI state switch command for CORESET index p while UE is performing TCI state switch of CORESET index q, UE is expected to receive on new TCI states of CORESET index p and q after completing both the TCI state switch of CORESET p and q.

<End of Change #3>

<Start of Change #4>

8.10E.3.2	MAC-CE based dual DL TCI state switching delay for mDCI

The requirement in this clause applies when UE is capable of multiDCI-MultiTRP-r16 and configured with two CORESETPoolIndexes.

When a MAC CE command for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] indicating a TCI state switch for CORESETPoolIndex p is received at slot n, and if the TCI state is known, UE shall be able to receive on the target TCI state at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*( Tfirst-SSBp, + TSSB-proc) / NR slot length. The UE shall be able to receive PDCCH with the old TCI states until slot n+ THARQ + 3Nslotsubframe,µ.

Where:

-	THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

-	Tfirst-SSBp is time to first SSB transmission (i.e., SSB associated to CORESETPoolIndex p) after MAC CE command is decoded by the UE; The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state.

-	TSSB-proc = 2 ms.

-	TOk = 1 if target TCI state is not in the active TCI state list for PDSCH, 0 otherwise.

When UE receives TCI state switch command for CORESETPoolIndex p while UE is performing TCI state switch of CORESETPoolIndex q, UE is expected to receive on new TCI states of CORESETPoolIndex p and q after completing both the TCI state switch of CORESETPoolIndex p and q.

<End of Change #4>