3GPP TSG-RAN WG4 Meeting #110bis 					                                       R4-2406346

Changsha, China, April 15 – 19, 2024

Agenda item:			6.14.3

Source:	Apple

Title:	WF on R18 Further NR mobility enhancement – part 2

Document for:	Approval

# Core part maintenance

Issue 1-1: supported frequency range of solution based on existing measurement for validity check

- Agreement:

  - FG 39-x1 and 39-x2 apply for both FR1 and FR2.

Issue 1-2: whether to allow differentiation between FR1 and FR2 for the new UE capabilities 39-x1 and 39-x2.

- Agreement:

  - Allow differentiation between FR1 and FR2 for FG 39-x1 and 39-x2.

Issue 1-3: whether to remove inter-RAT measurement (idleInactiveEUTRA-MeasReport-r16) in solution based on existing measurement.

- Agreement:

  - No. Inter-RAT E-UTRAN measurement is supported in solution based on existing measurement for validity check.

Issue 1-4: applicability of validity check

- Agreement:

  - FFS:

    - Option 1: It is up to UE implementation whether to perform validity check for measurement which was performed during CONNECTED mode before UE enters IDLE/INACTIVE mode.

    - Option 2: UE is not allowed to perform validity check for measurement which was performed during CONNECTED mode before UE enters IDLE/INACTIVE mode.

  - It is RAN4 understanding that UE only needs to follow NW configuration for measurement and validity check.

    - FG 39-x2 does not apply to any measurements during IDLE/INACTIVE but no included in configuration of R18 CA/DC setup improvement.

      - UE is not required to report carriers which are not configured by NW.

Issue 1-5: X value and testing issue and impact to core requirements

- FFS:

  - Proposal 1: Extend X value with uncertainty in core requirement. (Nokia)

    - Msg1 transmission occasion is unknown to TE. RSRP variation from X sec before T1 based test design is not definable

# Topic #2: performance part

Issue 2-1: accuracy requirement of solution based on existing measurement for validity check

- Agreement:

  - Reported measurements shall meet the corresponding idle-mode (10.1.xB) accuracy requirements at the measurement instance.

Issue 2-2: test configuration/procedure for solution based on existing measurement for validity check

- Tentative Agreement:

  - Introduce test case to verify FG 39-x1 and 39-x2.

    - Candidate option: use different power levels before X window starts plus some time margin.

Issue 2-3: test scope of solution based on existing measurement for validity check

- FFS:

  - Proposal 1: Define test cases to support Rel-18 early measurement reporting of both EMR and cell-reselection measurements. (Nokia)

  - Proposal 2: Define also a test case where X is not configured to verify the accuracy of the reported measurements. (Nokia)

# UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 39. NR_Mob_enh2 | 39-x1 | Measurement validation based on EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup | idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 | Yes | N/A | UE does not support EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | Yes | N/A |  | Optional with capability signaling |
|  | 39-x2 | Measurement validation based on non-EMR measurement during connection setup/resume | Indicate UE supporting measurement validation based on non-EMR measurement during IDLE/INACTIVE state and reporting for fast CA/DC setup |  | Yes | N/A | UE does not support non-EMR measurement validation during connection setup/resume and reporting for fast CA/DC | Per-UE | No | Yes | N/A |  | Optional with capability signaling |

# Annex: test case list

Agreement:

| No. | Category | Configuration | Responsible company |
| --- | --- | --- | --- |
|  | NR-DC with selective activation of cell groups Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 1)+2) or 3)+4) | Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC | Apple |
|  |  | Inter-frequency CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC | ZTE |
|  |  | Intra-frequency CPC from FR1-FR2 NR-DC to FR1-FR2 NR-DC | CATT |
|  |  | Inter-frequency CPA from FR1-FR2 NR-DC to FR1-FR2 NR-DC | CMCC |
|  | FFS: Improvement on SCell/SCG setup delay | TBA | E/// |
|  | CHO including target MCG and target SCG in NR-DC (obj.3) Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 6) or 7) | FR1-FR1 NR-DC to FR1-FR1 NR-DC | Huawei |
|  |  | FR1-FR2 NR-DC to FR1-FR2 NR-DC | vivo |
|  | CHO including target MCG and candidate SCG in NR-DC (obj.4) Note: For UE capable of both FR1-FR1 DC and FR1-FR2 DC, UE only needs to pass either 8) or 9) | FR1-FR1 NR-DC to FR1-FR1 NR-DC | Nokia |
|  |  | FR1-FR2 NR-DC to FR1-FR2 NR-DC | OPPO |