3GPP TSG-RAN WG4 Meeting #110bis	R4-2406393

Changsha, China 15th ‒ 19th April, 2024

Title: 	WF on RRM requirements for NR_MIMO_evo_DL_UL

Agenda Item:	6.19.5

Source: 	Samsung

Document for:	Approval

<Topic 1 RRM core requirements maintenance >

Issue 1-1-2: Applicability of two TA timing requirements for initial access RACH and CBRA.

< Way forward >

- Option 1

  - The timing requirements with 2 TA for multi-DCI multi-TRP are not applicable to initial access RACH and contention based RACH.

Issue 1-2-2: For mDCI mTRP, OL definition?

< Agreement>

- if the first SSB which after decoding the MAC-CE overlaps or adjacent to the first SSB which after decoding another MAC-CE from other TRP in FR2 and SSB periodicity is equal to that of other TRP

- If the MAC CE arrived first, OL=0; Otherwise OL=1

- If the first SSB which after decoding the MAC-CE overlaps or adjacent to the first SSB which after decoding another MAC-CE from other TRP in FR2 and SSB periodicity is less than that of other TRP, OL=1

- Otherwise, OL=0

Issue 1-2-3: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs (RTD<CP and RTD>CP)?

< Way forward >

- Option 1

  - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

  - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

  - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

- Option 2

  - No additional DL RS tracking time for UL TCI state switching-

Issue 1-2-4-a: Scheduling restriction of L1-RSRP measurement when RTD>CP in FR1 for serving cell or cell with different PCI from serving cell:

< Agreement>

- Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1：no restriction

- Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing as PDSCH/PDCCH on FR1

  - Support simultaneousRxDataSSB-DiffNumerology: no restriction

  - not support simultaneousRxDataSSB-DiffNumerology

    - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on symbols which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement

Issue 1-2-4-b: Scheduling restriction of L1-RSRP measurement when RTD>CP in FR1 for cell with different PCI from serving cell:

< Agreement>

- The UE is not expected to transmit PUCCH/PUSCH/SRS on the symbols configured for L1-RSRP measurement which are overlapped or partially overlapped with SSB symbols configured for L1-RSRP measurement.

Issue 1-2-6: Whether to add scheduling restriction of DL and UL TCI state switch for mDCI?

< Way forward >

- Option1 (Xiaomi):

  - Define scheduling restriction for DL and UL TCI state switch, i.e. The UE is not expected to transmit or receive data on the SSB or CSI-RS symbols used for T/F measurement or pathloss measurement for FR1 with different SCS and FR2

<Topic 2 RRM performance requirement for TDCP >

Issue 2-1-1: Test metric of TDCP test cases:

< Agreement>	:

Agreements for the test case definition:

- Define the same test case for both 15kHz with FDD and and 30kHz with TDD in the same sub-clause.

- Channel models: TDL-A30

- Doppler:

  - Two TCs with low and high Doppler

    - TC1: low (10) for both 15kHz and 30kHz

    - TC2: high

      - Option 1: high (300)

      - Option 2: 100 for 15kHz SCS, and 200 for 30kHz SCS

  - BW:

    - 10MHz for FDD and TDD

  - SNR:

    - Option 1: 20dB for TC1

    - Option 2: 10dB or 20dB for TC2

  - The distance between two TRSs: 1

- Report index: Bring CR for both options, make decision in the next meting

  - Option 1:

    - lower Doppler: CDP at X1 is higher than Y1 = [90] %

    - high Doppler: CDP at X2 is lower than Y2= [10] %

    - X1, X2, Y1, and Y2 can be different for TDD and FDD

  - Option 2:

    - [X1, X2] for Y2= FFS

Issue 2-1-2: TDCP Measurement Report Mapping – amplitude

< Agreement>	:

    - Define the mapping table as: use RAN1 points for each boundary of the range.

| ki | TDCP Range |
| --- | --- |
| 0 | - 11282< TDCP <=1 |
| 1 | 1- 1/128< TDCP <=1- 11282 |
| 2 | 1-1642< TDCP <=1- 1/128 |
| … | … |
| 14 | 1- 12< TDCP <=1-1/2 |
| 15 | 0≤ TDCP <=1-12 |

Issue 2-1-3: TDCP Measurement Report Mapping – phase

- < Agreement>	:

    - Define the mapping table as: use RAN1 points for each boundary of the range

<Topic 3 other RRM performance requirement >

Issue 3-1-1: TCs design for two TAs

< Agreement>	:

- Only UE transmit timing from two TRPs

  - TC #1: FR1 UE transmit timing from two TRPs.

  - TC #2: FR2 UE transmit timing from two TRPs.

Issue 3-3-1: TCs design for s-DCI mTRP cases:

< Agreement>	:

| TC1 | separate DL TCI state switch |
| --- | --- |
| TC2 | separate UL TCI state switch |
| TC3 | Joint TCI state switching |

- Add the Note for the UE should pass which tests in the test purpose. Such as: UE can pass only one test or two tests depends on the UE capability.

Issue 3-3-2: Test configuration of for s-DCI mTRP cases:

- Proposals

  - Proposal 1: (Apple)

    - Dual TCI state switching

  - Proposal 2: (Ericsson, Nokia)

    - Single to dual TCI state switching

Issue 3-2-1: Whether to define TCs for m-DCI mTRP cases?

< Agreement>	:

For FR1 UE supports rxTimingDiff-r18 capability, define:

- Test case in Te

- Test case for “FR1 two TA, RTD>CP, joint TCI state switching TC”.

<Topic 4 CR split >

|  | Note | Source Company |
| --- | --- | --- |
| TDCP mapping tables | in Section 10 | Apple |
| TDCP tests | Subclauses in: A.4.x for EN-DC A.6.7.x for SA | Ericsson |
| TC: FR1 UE transmit timing from two TRPs. | A.4.x for EN-DC A.6.4.x for SA | Samsung |
| TC: FR2 UE transmit timing from two TRPs. | A.5.4.x for EN-DC A.7.4.x for SA | vivo |
| TC: sDCI mTRP, FR2 separate DL TCI state switching |  | Xiaomi |
| TC: sDCI mTRP, FR2 separate UL TCI state switching |  | Nokia |
| TC: sDCI mTRP, FR2 joint TCI state switching |  | MTK |
| TC: mDCI FR1 two TA, RTD>CP, joint TCI state switching |  | Huawei |

Note: please add new configurations in A.3 if necessary, such as new CSI-RS configuration etc.